3GPP TSG-CT WG4 Meeting #131	C4-254018

Sophia Antipolis, France, 13th – 17th October 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.002 | CR | 1276 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ASN.1 module version update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2023-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | ASN.1 modules should be uniquely identified by an Object Identifier. MAP Rel-19 modules (which are different from Rel-18) shall not use the same Object Identifier as Rel-18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Systematic replacement of "version 21 (21)" with "version22 (22)" for MAP modules |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ASN.1 modules are not uniquely identified. May lead to wrong import/export relationships and eventually to wrong protocol definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.3.3, 17.4, 17.5, 17.6.1, 17.6.2, 17.6.3, 17.6.4, 17.6.5, 17.6.6, 17.6.7, 17.6.8, 17.7.1, 17.7.2, 17.7.4, 17.7.5, 17.7.6, 17.7.7, 17.7.8, 17.7.9, 17.7.10, 17.7.11, 17.7.12, 17.7.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 17.3.3	ASN.1 Module for application-context-names

The following ASN.1 module summarises the application-context-name assigned to MAP application-contexts.

.$MAP-ApplicationContexts {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ApplicationContexts (2) version22 (22)}

DEFINITIONS

::=

BEGIN

-- EXPORTS everything

IMPORTS

gsm-NetworkId,

ac-Id

FROM MobileDomainDefinitions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

mobileDomainDefinitions (0) version1 (1)}

;

-- application-context-names

map-ac  OBJECT IDENTIFIER ::= {gsm-NetworkId ac-Id}

networkLocUpContext-v3  OBJECT IDENTIFIER ::=

{map-ac networkLocUp(1) version3(3)}

locationCancellationContext-v3  OBJECT IDENTIFIER ::=

{map-ac locationCancel(2) version3(3)}

roamingNumberEnquiryContext-v3  OBJECT IDENTIFIER ::=

{map-ac roamingNbEnquiry(3) version3(3)}

authenticationFailureReportContext-v3  OBJECT IDENTIFIER ::=

{map-ac authenticationFailureReport(39) version3(3)}

locationInfoRetrievalContext-v3  OBJECT IDENTIFIER ::=

{map-ac locInfoRetrieval(5) version3(3)}

resetContext-v3  OBJECT IDENTIFIER ::=

{map-ac reset(10) version3(3)}

handoverControlContext-v3  OBJECT IDENTIFIER ::=

{map-ac handoverControl(11) version3(3)}

equipmentMngtContext-v3  OBJECT IDENTIFIER ::=

{map-ac equipmentMngt(13) version3(3)}

infoRetrievalContext-v3  OBJECT IDENTIFIER ::=

{map-ac infoRetrieval(14) version3(3)}

interVlrInfoRetrievalContext-v3  OBJECT IDENTIFIER ::=

{map-ac interVlrInfoRetrieval(15) version3(3)}

subscriberDataMngtContext-v3  OBJECT IDENTIFIER ::=

{map-ac subscriberDataMngt(16) version3(3)}

tracingContext-v3  OBJECT IDENTIFIER ::=

{map-ac tracing(17) version3(3)}

networkFunctionalSsContext-v2  OBJECT IDENTIFIER ::=

{map-ac networkFunctionalSs(18) version2(2)}

networkUnstructuredSsContext-v2  OBJECT IDENTIFIER ::=

{map-ac networkUnstructuredSs(19) version2(2)}

shortMsgGatewayContext-v3  OBJECT IDENTIFIER ::=

{map-ac shortMsgGateway(20) version3(3)}

shortMsgMO-RelayContext-v3  OBJECT IDENTIFIER ::=

{map-ac shortMsgMO-Relay(21) version3(3)}

shortMsgAlertContext-v2  OBJECT IDENTIFIER ::=

{map-ac shortMsgAlert(23) version2(2)}

mwdMngtContext-v3  OBJECT IDENTIFIER ::=

{map-ac mwdMngt(24) version3(3)}

shortMsgMT-RelayContext-v3  OBJECT IDENTIFIER ::=

{map-ac shortMsgMT-Relay(25) version3(3)}

shortMsgMT-Relay-VGCS-Context-v3  OBJECT IDENTIFIER ::=

{map-ac shortMsgMT-Relay-VGCS(41) version3(3)}

imsiRetrievalContext-v2  OBJECT IDENTIFIER ::=

{map-ac imsiRetrieval(26) version2(2)}

msPurgingContext-v3  OBJECT IDENTIFIER ::=

{map-ac msPurging(27) version3(3)}

subscriberInfoEnquiryContext-v3  OBJECT IDENTIFIER ::=

{map-ac subscriberInfoEnquiry(28) version3(3)}

anyTimeInfoEnquiryContext-v3  OBJECT IDENTIFIER ::=

{map-ac anyTimeInfoEnquiry(29) version3(3)}

callControlTransferContext-v4  OBJECT IDENTIFIER ::=

{map-ac callControlTransfer(6) version4(4)}

ss-InvocationNotificationContext-v3  OBJECT IDENTIFIER ::=

{map-ac ss-InvocationNotification(36) version3(3)}

groupCallControlContext-v3  OBJECT IDENTIFIER ::=

{map-ac groupCallControl(31) version3(3)}

groupCallInfoRetrievalContext-v3  OBJECT IDENTIFIER ::=

{map-ac groupCallInfoRetrieval(45) version3(3)}

gprsLocationUpdateContext-v3  OBJECT IDENTIFIER ::=

{map-ac gprsLocationUpdate(32) version3(3)}

gprsLocationInfoRetrievalContext-v4  OBJECT IDENTIFIER ::=

{map-ac gprsLocationInfoRetrieval(33) version4(4)}

failureReportContext-v3  OBJECT IDENTIFIER ::=

{map-ac failureReport(34) version3(3)}

gprsNotifyContext-v3  OBJECT IDENTIFIER ::=

{map-ac gprsNotify(35) version3(3)}

reportingContext-v3  OBJECT IDENTIFIER ::=

{map-ac reporting(7) version3(3)}

callCompletionContext-v3  OBJECT IDENTIFIER ::=

{map-ac callCompletion(8) version3(3)}

istAlertingContext-v3  OBJECT IDENTIFIER ::=

{map-ac istAlerting(4) version3(3)}

serviceTerminationContext-v3  OBJECT IDENTIFIER ::=

{map-ac immediateTermination(9) version3(3)}

locationSvcGatewayContext-v3  OBJECT IDENTIFIER ::=

{map-ac locationSvcGateway(37) version3(3)}

locationSvcEnquiryContext-v3  OBJECT IDENTIFIER ::=

{map-ac locationSvcEnquiry(38) version3(3)}

# mm-EventReportingContext-v3  OBJECT IDENTIFIER ::=

{map-ac mm-EventReporting(42) version3(3)}

anyTimeInfoHandlingContext-v3  OBJECT IDENTIFIER ::=

{map-ac anyTimeInfoHandling(43) version3(3)}

subscriberDataModificationNotificationContext-v3  OBJECT IDENTIFIER ::=

{map-ac subscriberDataModificationNotification(22) version3(3)}

resourceManagementContext-v3  OBJECT IDENTIFIER ::=

{map-ac resourceManagement(44) version3(3)}

vcsgLocationUpdateContext-v3  OBJECT IDENTIFIER ::=

{map-ac vcsgLocationUpdate(46) version3(3)}

vcsgLocationCancellationContext-v3  OBJECT IDENTIFIER ::=

{map-ac vcsgLocationCancel(47) version3(3)}

-- The following Object Identifiers are reserved for application-contexts

--  existing in previous versions of the protocol

-- AC Name & Version	Object Identifier

--

-- networkLocUpContext-v1	map-ac networkLocUp (1)	version1 (1)

-- networkLocUpContext-v2	map-ac networkLocUp (1)	version2 (2)

-- locationCancellationContext-v1	map-ac locationCancellation (2)	version1 (1)

-- locationCancellationContext-v2	map-ac locationCancellation (2)	version2 (2)

-- roamingNumberEnquiryContext-v1	map-ac roamingNumberEnquiry (3)	version1 (1)

-- roamingNumberEnquiryContext-v2	map-ac roamingNumberEnquiry (3)	version2 (2)

-- locationInfoRetrievalContext-v1	map-ac locationInfoRetrieval (5)	version1 (1)

-- locationInfoRetrievalContext-v2	map-ac locationInfoRetrieval (5)	version2 (2)

-- resetContext-v1	map-ac reset (10)	version1 (1)

-- resetContext-v2	map-ac reset (10)	version2 (2)

-- handoverControlContext-v1	map-ac handoverControl (11)	version1 (1)

-- handoverControlContext-v2	map-ac handoverControl (11)	version2 (2)

-- sIWFSAllocationContext-v3	map-ac sIWFSAllocation (12)	version3 (3)

-- equipmentMngtContext-v1	map-ac equipmentMngt (13)	version1 (1)

-- equipmentMngtContext-v2	map-ac equipmentMngt (13)	version2 (2)

-- infoRetrievalContext-v1	map-ac infoRetrieval (14)	version1 (1)

-- infoRetrievalContext-v2	map-ac infoRetrieval (14)	version2 (2)

-- interVlrInfoRetrievalContext-v2	map-ac interVlrInfoRetrieval (15)	version2 (2)

-- subscriberDataMngtContext-v1	map-ac subscriberDataMngt (16)	version1 (1)

-- subscriberDataMngtContext-v2	map-ac subscriberDataMngt (16)	version2 (2)

-- tracingContext-v1	map-ac tracing (17)	version1 (1)

-- tracingContext-v2	map-ac tracing (17)	version2 (2)

-- networkFunctionalSsContext-v1	map-ac networkFunctionalSs (18)	version1 (1)

-- shortMsgGatewayContext-v1	map-ac shortMsgGateway (20)	version1 (1)

-- shortMsgGatewayContext-v2	map-ac shortMsgGateway (20)	version2 (2)

-- shortMsgRelayContext-v1	map-ac shortMsgRelay (21)	version1 (1)

-- shortMsgAlertContext-v1	map-ac shortMsgAlert (23)	version1 (1)

-- mwdMngtContext-v1	map-ac mwdMngt (24)	version1 (1)

-- mwdMngtContext-v2	map-ac mwdMngt (24)	version2 (2)

-- shortMsgMT-RelayContext-v2	map-ac shortMsgMT-Relay (25)	version2 (2)

-- msPurgingContext-v2	map-ac msPurging (27)	version2 (2)

-- callControlTransferContext-v3	map-ac callControlTransferContext (6)	version3 (3)

-- gprsLocationInfoRetrievalContext-v3	map-ac gprsLocationInfoRetrievalContext (33) version3 (3)

.#END

* * * Next Change * * * *

## 17.4	MAP Dialogue Information

.$MAP-DialogueInformation {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-DialogueInformation (3) version22 (22)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

map-DialogueAS,

MAP-DialoguePDU

;

IMPORTS

gsm-NetworkId,

as-Id

FROM MobileDomainDefinitions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

mobileDomainDefinitions (0) version1 (1)}

AddressString

FROM MAP-CommonDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network(1) modules (3) map-CommonDataTypes (18) version22 (22)}

ExtensionContainer

FROM MAP-ExtensionDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ExtensionDataTypes (21) version22 (22)}

;

-- abstract syntax name for MAP-DialoguePDU

map-DialogueAS  OBJECT IDENTIFIER ::=

{gsm-NetworkId as-Id map-DialoguePDU (1) version1 (1)}

MAP-DialoguePDU ::= CHOICE {

map-open	[0] MAP-OpenInfo,

map-accept	[1] MAP-AcceptInfo,

map-close	[2] MAP-CloseInfo,

map-refuse	[3] MAP-RefuseInfo,

map-userAbort	[4] MAP-UserAbortInfo,

map-providerAbort	[5] MAP-ProviderAbortInfo}

MAP-OpenInfo ::= SEQUENCE {

destinationReference	[0] AddressString	OPTIONAL,

originationReference	[1] AddressString	OPTIONAL,

...,

extensionContainer	ExtensionContainer	OPTIONAL

-- extensionContainer must not be used in version 2

}

MAP-AcceptInfo ::= SEQUENCE {

...,

extensionContainer	ExtensionContainer	OPTIONAL

-- extensionContainer must not be used in version 2

}

MAP-CloseInfo ::= SEQUENCE {

...,

extensionContainer	ExtensionContainer	OPTIONAL

-- extensionContainer must not be used in version 2

}

MAP-RefuseInfo ::= SEQUENCE {

reason	Reason,

...,

extensionContainer	ExtensionContainer	OPTIONAL,

-- extensionContainer must not be used in version 2

alternativeApplicationContext	OBJECT IDENTIFIER	OPTIONAL

-- alternativeApplicationContext must not be used in version 2

}

Reason ::= ENUMERATED {

noReasonGiven	(0),

invalidDestinationReference	(1),

invalidOriginatingReference	(2)}

MAP-UserAbortInfo ::= SEQUENCE {

map-UserAbortChoice	MAP-UserAbortChoice,

...,

extensionContainer	ExtensionContainer	OPTIONAL

-- extensionContainer must not be used in version 2

}

MAP-UserAbortChoice ::= CHOICE {

userSpecificReason	[0] NULL,

userResourceLimitation	[1] NULL,

resourceUnavailable	[2] ResourceUnavailableReason,

applicationProcedureCancellation	[3] ProcedureCancellationReason}

ResourceUnavailableReason ::= ENUMERATED {

shortTermResourceLimitation  (0),

longTermResourceLimitation  (1)}

ProcedureCancellationReason ::= ENUMERATED {

handoverCancellation  (0),

radioChannelRelease  (1),

networkPathRelease  (2),

callRelease  (3),

associatedProcedureFailure  (4),

tandemDialogueRelease  (5),

remoteOperationsFailure  (6)}

MAP-ProviderAbortInfo ::= SEQUENCE {

map-ProviderAbortReason	MAP-ProviderAbortReason,

...,

extensionContainer	ExtensionContainer	OPTIONAL

-- extensionContainer must not be used in version 2

}

MAP-ProviderAbortReason ::= ENUMERATED {

abnormalDialogue  (0),

invalidPDU  (1)}

.#END

* * * Next Change * * * *

## 17.5	MAP operation and error codes

.$MAP-Protocol {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Protocol (4) version22 (22)}

DEFINITIONS

::=

BEGIN

IMPORTS

OPERATION

FROM Remote-Operations-Information-Objects {

joint-iso-itu-t remote-operations(4) informationObjects(5) version1(0)}

updateLocation,

cancelLocation,

cancelVcsgLocation,

purgeMS,

sendIdentification,

updateGprsLocation,

updateVcsgLocation,

prepareHandover,

sendEndSignal,

processAccessSignalling,

forwardAccessSignalling,

prepareSubsequentHandover,

sendAuthenticationInfo,

authenticationFailureReport,

checkIMEI,

insertSubscriberData,

deleteSubscriberData,

reset,

forwardCheckSS-Indication,

restoreData,

provideSubscriberInfo,

anyTimeInterrogation,

# anyTimeSubscriptionInterrogation,

# anyTimeModification,

sendRoutingInfoForGprs,

failureReport,

noteMsPresentForGprs,

noteMM-Event,

noteSubscriberDataModified

FROM MAP-MobileServiceOperations {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MobileServiceOperations (5)

version22 (22)}

activateTraceMode,

deactivateTraceMode,

sendIMSI

FROM MAP-OperationAndMaintenanceOperations {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-OperationAndMaintenanceOperations (6)

version22 (22)}

sendRoutingInfo,

provideRoamingNumber,

resumeCallHandling,

setReportingState,

statusReport,

remoteUserFree,

ist-Alert,

ist-Command,

releaseResources

FROM MAP-CallHandlingOperations {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CallHandlingOperations (7)

version22 (22)}

registerSS,

eraseSS,

activateSS,

deactivateSS,

interrogateSS,

processUnstructuredSS-Request,

unstructuredSS-Request,

unstructuredSS-Notify,

registerPassword,

getPassword,

ss-InvocationNotification,

registerCC-Entry,

eraseCC-Entry

FROM MAP-SupplementaryServiceOperations {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SupplementaryServiceOperations (8)

version22 (22)}

sendRoutingInfoForSM,

mo-ForwardSM,

mt-ForwardSM,

reportSM-DeliveryStatus,

alertServiceCentre,

informServiceCentre,

readyForSM,

mt-ForwardSM-VGCS

FROM MAP-ShortMessageServiceOperations {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ShortMessageServiceOperations (9)

version22 (22)}

prepareGroupCall,

processGroupCallSignalling,

forwardGroupCallSignalling,

sendGroupCallEndSignal,

sendGroupCallInfo

FROM MAP-Group-Call-Operations {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Group-Call-Operations (22)

version22 (22)}

provideSubscriberLocation,

sendRoutingInfoForLCS,

subscriberLocationReport

FROM MAP-LocationServiceOperations {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-LocationServiceOperations (24)

version22 (22)}

;

Supported-MAP-Operations OPERATION ::= {updateLocation | cancelLocation | cancelVcsgLocation |

purgeMS |

sendIdentification | updateGprsLocation | updateVcsgLocation | prepareHandover | sendEndSignal |

processAccessSignalling | forwardAccessSignalling | prepareSubsequentHandover |

sendAuthenticationInfo | authenticationFailureReport | checkIMEI | insertSubscriberData |

deleteSubscriberData | reset | forwardCheckSS-Indication | restoreData | provideSubscriberInfo |

anyTimeInterrogation | anyTimeSubscriptionInterrogation | anyTimeModification |

sendRoutingInfoForGprs | failureReport |noteMsPresentForGprs | noteMM-Event |

noteSubscriberDataModified | activateTraceMode | deactivateTraceMode | sendIMSI |

sendRoutingInfo | provideRoamingNumber | resumeCallHandling | setReportingState | statusReport |

remoteUserFree | ist-Alert |

ist-Command | registerSS | eraseSS | activateSS | deactivateSS | interrogateSS |

processUnstructuredSS-Request | unstructuredSS-Request | unstructuredSS-Notify |

registerPassword | getPassword | ss-InvocationNotification | registerCC-Entry | eraseCC-Entry |

sendRoutingInfoForSM | mo-ForwardSM | mt-ForwardSM | reportSM-DeliveryStatus |

alertServiceCentre | informServiceCentre | readyForSM | prepareGroupCall |

processGroupCallSignalling | forwardGroupCallSignalling | sendGroupCallEndSignal |

provideSubscriberLocation | sendRoutingInfoForLCS | subscriberLocationReport |

releaseResources | mt-ForwardSM-VGCS | sendGroupCallInfo }

-- The following operation codes are reserved for operations

-- existing in previous versions of the protocol

-- Operation Name	AC used	Oper. Code

--

-- sendParameters	map-ac infoRetrieval (14) version1 (1)	local:9

-- processUnstructuredSS-Data	map-ac networkFunctionalSs (18) version1 (1)	local:19

-- performHandover	map-ac handoverControl (11) version1 (1)	local:28

-- performSubsequentHandover	map-ac handoverControl (11) version1 (1)	local:30

-- provideSIWFSNumber	map-ac sIWFSAllocation (12) version3 (3)	local:31

-- siwfs-SignallingModify	map-ac sIWFSAllocation (12) version3 (3)	local:32

-- noteInternalHandover	map-ac handoverControl (11) version1 (1)	local:35

-- noteSubscriberPresent	map-ac mwdMngt (24) version1 (1)	local:48

-- alertServiceCentreWithoutResult	map-ac shortMsgAlert (23) version1 (1)	local:49

-- traceSubscriberActivity	map-ac handoverControl (11) version1 (1)	local:52

-- beginSubscriberActivity	map-ac networkFunctionalSs (18) version1 (1)	local:54

-- The following error codes are reserved for errors

-- existing in previous versions of the protocol

-- Error Name	AC used	Error Code

--

-- unknownBaseStation	map-ac handoverControl (11) version1 (1)	local:2

-- invalidTargetBaseStation	map-ac handoverControl (11) version1 (1)	local:23

-- noRadioResourceAvailable	map-ac handoverControl (11) version1 (1)	local:24

.#END

* * * Next Change * * * *

### 17.6.1	Mobile Service Operations

.$MAP-MobileServiceOperations {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MobileServiceOperations (5)

version22 (22)}

DEFINITIONS

::=

BEGIN

EXPORTS

-- location registration operations

updateLocation,

cancelLocation,

purgeMS,

sendIdentification,

-- gprs location registration operations

updateGprsLocation,

-- vcsg location registration operations

updateVcsgLocation,

cancelVcsgLocation,

-- subscriber information enquiry operations

provideSubscriberInfo,

-- any time information enquiry operations

anyTimeInterrogation,

-- any time information handling operations

# anyTimeSubscriptionInterrogation,

# anyTimeModification,

-- subscriber data modification notification operations

noteSubscriberDataModified,

-- handover operations

prepareHandover,

sendEndSignal,

processAccessSignalling,

forwardAccessSignalling,

prepareSubsequentHandover,

-- authentication management operations

sendAuthenticationInfo,

authenticationFailureReport,

-- IMEI management operations

checkIMEI,

-- subscriber management operations

insertSubscriberData,

deleteSubscriberData,

-- fault recovery operations

reset,

forwardCheckSS-Indication,

restoreData,

-- gprs location information retrieval operations

sendRoutingInfoForGprs,

-- failure reporting operations

failureReport,

-- gprs notification operations

# noteMsPresentForGprs,

-- Mobility Management operations

noteMM-Event

;

IMPORTS

OPERATION

FROM Remote-Operations-Information-Objects {

joint-iso-itu-t remote-operations(4)

informationObjects(5) version1(0)}

systemFailure,

dataMissing,

unexpectedDataValue,

unknownSubscriber,

unknownMSC,

unidentifiedSubscriber,

unknownEquipment,

roamingNotAllowed,

ati-NotAllowed,

noHandoverNumberAvailable,

subsequentHandoverFailure,

# absentSubscriber,

# mm-EventNotSupported,

# atsi-NotAllowed,

# atm-NotAllowed,

bearerServiceNotProvisioned,

teleserviceNotProvisioned,

callBarred,

illegalSS-Operation,

ss-ErrorStatus,

ss-NotAvailable,

ss-Incompatibility,

ss-SubscriptionViolation,

informationNotAvailable,

targetCellOutsideGroupCallArea

FROM MAP-Errors {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Errors (10) version22 (22)}

UpdateLocationArg,

UpdateLocationRes,

CancelLocationArg,

CancelLocationRes,

PurgeMS-Arg,

PurgeMS-Res,

SendIdentificationArg,

SendIdentificationRes,

UpdateGprsLocationArg,

UpdateGprsLocationRes,

UpdateVcsgLocationArg,

UpdateVcsgLocationRes,

CancelVcsgLocationArg,

CancelVcsgLocationRes,

PrepareHO-Arg,

PrepareHO-Res,

ForwardAccessSignalling-Arg,

ProcessAccessSignalling-Arg,

SendEndSignal-Arg,

SendEndSignal-Res,

PrepareSubsequentHO-Res,

PrepareSubsequentHO-Arg,

SendAuthenticationInfoArg,

SendAuthenticationInfoRes,

AuthenticationFailureReportArg,

AuthenticationFailureReportRes,

CheckIMEI-Arg,

CheckIMEI-Res,

InsertSubscriberDataArg,

InsertSubscriberDataRes,

DeleteSubscriberDataArg,

DeleteSubscriberDataRes,

ResetArg,

RestoreDataArg,

RestoreDataRes,

ProvideSubscriberInfoArg,

ProvideSubscriberInfoRes,

# AnyTimeSubscriptionInterrogationArg,

# AnyTimeSubscriptionInterrogationRes,

# AnyTimeModificationArg,

# AnyTimeModificationRes,

NoteSubscriberDataModifiedArg,

NoteSubscriberDataModifiedRes,

AnyTimeInterrogationArg,

AnyTimeInterrogationRes,

SendRoutingInfoForGprsArg,

SendRoutingInfoForGprsRes,

FailureReportArg,

FailureReportRes,

NoteMsPresentForGprsArg,

# NoteMsPresentForGprsRes,

# NoteMM-EventArg,

# NoteMM-EventRes

FROM MAP-MS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version22 (22)}

;

-- location registration operations

updateLocation  OPERATION ::= {	--Timer m

ARGUMENT

UpdateLocationArg

RESULT

UpdateLocationRes

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

unknownSubscriber |

roamingNotAllowed}

CODE	local:2 }

cancelLocation  OPERATION ::= {	--Timer m

ARGUMENT

CancelLocationArg

RESULT

CancelLocationRes

-- optional

ERRORS {

dataMissing |

unexpectedDataValue}

CODE	local:3 }

purgeMS  OPERATION ::= {	--Timer m

ARGUMENT

PurgeMS-Arg

RESULT

PurgeMS-Res

-- optional

ERRORS{

dataMissing |

unexpectedDataValue|

unknownSubscriber}

CODE	local:67 }

sendIdentification  OPERATION ::= {	--Timer s

ARGUMENT

SendIdentificationArg

RESULT

SendIdentificationRes

ERRORS {

dataMissing |

unidentifiedSubscriber}

CODE	local:55 }

-- gprs location registration operations

updateGprsLocation  OPERATION ::= {	--Timer m

ARGUMENT

UpdateGprsLocationArg

RESULT

UpdateGprsLocationRes

ERRORS {

systemFailure |

unexpectedDataValue |

unknownSubscriber |

roamingNotAllowed}

CODE	local:23 }

-- subscriber information enquiry operations

provideSubscriberInfo  OPERATION ::= {	--Timer m

ARGUMENT

ProvideSubscriberInfoArg

RESULT

ProvideSubscriberInfoRes

ERRORS {

dataMissing |

unexpectedDataValue}

CODE	local:70 }

-- any time information enquiry operations

anyTimeInterrogation  OPERATION ::= {	--Timer m

ARGUMENT

AnyTimeInterrogationArg

RESULT

AnyTimeInterrogationRes

ERRORS {

systemFailure |

ati-NotAllowed |

dataMissing |

unexpectedDataValue |

unknownSubscriber}

CODE	local:71 }

-- any time information handling operations

anyTimeSubscriptionInterrogation  OPERATION ::= {	--Timer m

ARGUMENT

AnyTimeSubscriptionInterrogationArg

RESULT

AnyTimeSubscriptionInterrogationRes

ERRORS {

atsi-NotAllowed |

dataMissing |

unexpectedDataValue |

unknownSubscriber |

bearerServiceNotProvisioned |

teleserviceNotProvisioned |

callBarred |

illegalSS-Operation |

ss-NotAvailable |

informationNotAvailable}

CODE	local:62 }

anyTimeModification  OPERATION ::= {	--Timer m

# ARGUMENT

AnyTimeModificationArg

# RESULT

AnyTimeModificationRes

# ERRORS {

# atm-NotAllowed |

# dataMissing |

# unexpectedDataValue |

# unknownSubscriber |

# bearerServiceNotProvisioned |

# teleserviceNotProvisioned |

# callBarred |

# illegalSS-Operation |

# ss-SubscriptionViolation |

ss-ErrorStatus |

ss-Incompatibility |

# informationNotAvailable}

# CODE	local:65 }

-- subscriber data modification notification operations

noteSubscriberDataModified  OPERATION ::= {	--Timer m

ARGUMENT

NoteSubscriberDataModifiedArg

RESULT

NoteSubscriberDataModifiedRes

-- optional

ERRORS {

dataMissing |

unexpectedDataValue |

unknownSubscriber}

CODE	local:5 }

-- handover operations

prepareHandover  OPERATION ::= {	--Timer m

ARGUMENT

PrepareHO-Arg

RESULT

PrepareHO-Res

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

noHandoverNumberAvailable |

targetCellOutsideGroupCallArea }

CODE	local:68 }

sendEndSignal  OPERATION ::= {	--Timer l

ARGUMENT

SendEndSignal-Arg

RESULT

SendEndSignal-Res

CODE	local:29 }

processAccessSignalling  OPERATION ::= {	--Timer s

ARGUMENT

ProcessAccessSignalling-Arg

CODE	local:33 }

forwardAccessSignalling  OPERATION ::= {	--Timer s

ARGUMENT

ForwardAccessSignalling-Arg

CODE	local:34 }

prepareSubsequentHandover  OPERATION ::= {	--Timer m

ARGUMENT

PrepareSubsequentHO-Arg

RESULT

PrepareSubsequentHO-Res

ERRORS {

unexpectedDataValue |

dataMissing |

unknownMSC |

subsequentHandoverFailure}

CODE	local:69 }

-- authentication management operations

sendAuthenticationInfo  OPERATION ::= {	--Timer m

ARGUMENT

SendAuthenticationInfoArg

-- optional

-- within a dialogue sendAuthenticationInfoArg shall not be present in

-- subsequent invoke components. If received in a subsequent invoke component

-- it shall be discarded.

RESULT

SendAuthenticationInfoRes

-- optional

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

unknownSubscriber}

CODE	local:56 }

authenticationFailureReport  OPERATION ::= {	--Timer m

ARGUMENT

AuthenticationFailureReportArg

RESULT

AuthenticationFailureReportRes

-- optional

ERRORS {

systemFailure |

unexpectedDataValue |

unknownSubscriber}

CODE	local:15 }

-- IMEI management operations

checkIMEI  OPERATION ::= {	--Timer m

ARGUMENT

CheckIMEI-Arg

RESULT

CheckIMEI-Res

ERRORS {

systemFailure |

dataMissing |

unknownEquipment}

CODE	local:43 }

-- subscriber management operations

insertSubscriberData  OPERATION ::= {	--Timer m

ARGUMENT

InsertSubscriberDataArg

RESULT

InsertSubscriberDataRes

-- optional

ERRORS {

dataMissing |

unexpectedDataValue |

unidentifiedSubscriber}

CODE	local:7 }

deleteSubscriberData  OPERATION ::= {	--Timer m

ARGUMENT

DeleteSubscriberDataArg

RESULT

DeleteSubscriberDataRes

-- optional

ERRORS {

dataMissing |

unexpectedDataValue |

unidentifiedSubscriber}

CODE	local:8 }

-- fault recovery operations

reset  OPERATION ::= {	--Timer m

ARGUMENT

ResetArg

CODE	local:37 }

forwardCheckSS-Indication  OPERATION ::= {	--Timer s

CODE	local:38 }

restoreData  OPERATION ::= {	--Timer m

ARGUMENT

RestoreDataArg

RESULT

RestoreDataRes

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

unknownSubscriber}

CODE	local:57 }

-- gprs location information retrieval operations

sendRoutingInfoForGprs  OPERATION ::= {	--Timer m

ARGUMENT

SendRoutingInfoForGprsArg

RESULT

SendRoutingInfoForGprsRes

ERRORS {

absentSubscriber |

systemFailure |

dataMissing |

unexpectedDataValue |

unknownSubscriber |

callBarred }

CODE	local:24 }

-- failure reporting operations

failureReport  OPERATION ::= {	--Timer m

ARGUMENT

FailureReportArg

RESULT

FailureReportRes

-- optional

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

unknownSubscriber}

CODE	local:25 }

-- gprs notification operations

noteMsPresentForGprs  OPERATION ::= {	--Timer m

ARGUMENT

NoteMsPresentForGprsArg

RESULT

NoteMsPresentForGprsRes

-- optional

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

unknownSubscriber}

CODE	local:26 }

noteMM-Event  OPERATION ::= {	--Timer m

ARGUMENT

NoteMM-EventArg

RESULT

NoteMM-EventRes

# ERRORS {

# dataMissing |

# unexpectedDataValue |

# unknownSubscriber |

# mm-EventNotSupported}

# CODE	local:89 }

-- vcsg location registration operations

updateVcsgLocation  OPERATION ::= {	--Timer m

# ARGUMENT

# UpdateVcsgLocationArg

# RESULT

# UpdateVcsgLocationRes

# ERRORS {

systemFailure |

unexpectedDataValue |

unknownSubscriber}

CODE	local:53 }

cancelVcsgLocation  OPERATION ::= {	--Timer m

ARGUMENT

CancelVcsgLocationArg

RESULT

CancelVcsgLocationRes

-- optional

ERRORS {

dataMissing |

unexpectedDataValue}

CODE	local:36 }

.#END

* * * Next Change * * * *

### 17.6.2	Operation and Maintenance Operations

.$MAP-OperationAndMaintenanceOperations {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-OperationAndMaintenanceOperations (6)

version22 (22)}

DEFINITIONS

::=

BEGIN

EXPORTS

activateTraceMode,

deactivateTraceMode,

sendIMSI

;

IMPORTS

OPERATION

FROM Remote-Operations-Information-Objects {

joint-iso-itu-t remote-operations(4)

informationObjects(5) version1(0)}

systemFailure,

dataMissing,

unexpectedDataValue,

facilityNotSupported,

unknownSubscriber,

unidentifiedSubscriber,

tracingBufferFull

FROM MAP-Errors {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Errors (10) version22 (22)}

ActivateTraceModeArg,

ActivateTraceModeRes,

DeactivateTraceModeArg,

DeactivateTraceModeRes

FROM MAP-OM-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-OM-DataTypes (12) version22 (22)}

ISDN-AddressString,

IMSI

FROM MAP-CommonDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version22 (22)}

;

activateTraceMode  OPERATION ::= {	--Timer m

ARGUMENT

ActivateTraceModeArg

RESULT

ActivateTraceModeRes

-- optional

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

facilityNotSupported |

unidentifiedSubscriber |

tracingBufferFull}

CODE	local:50 }

deactivateTraceMode  OPERATION ::= {	--Timer m

ARGUMENT

DeactivateTraceModeArg

RESULT

DeactivateTraceModeRes

-- optional

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

facilityNotSupported |

unidentifiedSubscriber}

CODE	local:51 }

sendIMSI  OPERATION ::= {	--Timer m

ARGUMENT

ISDN-AddressString

RESULT

IMSI

ERRORS {

dataMissing |

unexpectedDataValue |

unknownSubscriber}

CODE	local:58 }

.#END

* * * Next Change * * * *

### 17.6.3	Call Handling Operations

.$MAP-CallHandlingOperations {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CallHandlingOperations (7)

version22 (22)}

DEFINITIONS

::=

BEGIN

EXPORTS

sendRoutingInfo,

provideRoamingNumber,

resumeCallHandling,

setReportingState,

statusReport,

remoteUserFree,

ist-Alert,

ist-Command,

releaseResources

;

IMPORTS

OPERATION

FROM Remote-Operations-Information-Objects {

joint-iso-itu-t remote-operations(4)

informationObjects(5) version1(0)}

systemFailure,

dataMissing,

unexpectedDataValue,

facilityNotSupported,

or-NotAllowed,

unknownSubscriber,

numberChanged,

bearerServiceNotProvisioned,

teleserviceNotProvisioned,

noRoamingNumberAvailable,

absentSubscriber,

busySubscriber,

noSubscriberReply,

callBarred,

forwardingViolation,

forwardingFailed,

cug-Reject,

resourceLimitation,

incompatibleTerminal,

unidentifiedSubscriber

FROM MAP-Errors {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Errors (10) version22 (22)}

SendRoutingInfoArg,

SendRoutingInfoRes,

ProvideRoamingNumberArg,

ProvideRoamingNumberRes,

ResumeCallHandlingArg,

ResumeCallHandlingRes,

SetReportingStateArg,

SetReportingStateRes,

StatusReportArg,

StatusReportRes,

RemoteUserFreeArg,

RemoteUserFreeRes,

IST-AlertArg,

IST-AlertRes,

IST-CommandArg,

IST-CommandRes,

ReleaseResourcesArg,

ReleaseResourcesRes

FROM MAP-CH-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CH-DataTypes (13) version22 (22)}

;

sendRoutingInfo  OPERATION ::= {	--Timer m

-- The timer is set to the upper limit of the range if the GMSC supports pre-paging.

ARGUMENT

SendRoutingInfoArg

RESULT

SendRoutingInfoRes

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

facilityNotSupported |

or-NotAllowed |

unknownSubscriber |

numberChanged |

bearerServiceNotProvisioned |

teleserviceNotProvisioned |

absentSubscriber |

busySubscriber |

noSubscriberReply |

callBarred |

cug-Reject |

forwardingViolation}

CODE	local:22 }

provideRoamingNumber  OPERATION ::= {	--Timer m

-- The timer is set to the upper limit of the range if the HLR supports pre-paging.

ARGUMENT

ProvideRoamingNumberArg

RESULT

ProvideRoamingNumberRes

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

facilityNotSupported |

or-NotAllowed |

absentSubscriber |

noRoamingNumberAvailable}

CODE	local:4 }

resumeCallHandling  OPERATION ::= {	--Timer m

ARGUMENT

ResumeCallHandlingArg

RESULT

ResumeCallHandlingRes

-- optional

ERRORS {

forwardingFailed |

or-NotAllowed |

# unexpectedDataValue |

dataMissing }

CODE	local:6 }

setReportingState  OPERATION ::= {	--Timer m

ARGUMENT

SetReportingStateArg

RESULT

SetReportingStateRes

-- optional

ERRORS {

systemFailure |

unidentifiedSubscriber |

unexpectedDataValue |

dataMissing |

resourceLimitation |

facilityNotSupported}

CODE	local:73 }

statusReport  OPERATION ::= {	--Timer m

ARGUMENT

StatusReportArg

RESULT

StatusReportRes

-- optional

ERRORS {

unknownSubscriber |

systemFailure |

unexpectedDataValue |

dataMissing}

CODE	local:74 }

remoteUserFree  OPERATION ::= {	--Timer ml

ARGUMENT

RemoteUserFreeArg

RESULT

RemoteUserFreeRes

ERRORS {

unexpectedDataValue |

dataMissing |

incompatibleTerminal |

absentSubscriber |

systemFailure |

busySubscriber}

CODE	local:75 }

ist-Alert  OPERATION ::= {	--Timer m

ARGUMENT

IST-AlertArg

RESULT

IST-AlertRes

-- optional

ERRORS {

unexpectedDataValue |

resourceLimitation |

unknownSubscriber |

systemFailure |

facilityNotSupported}

CODE	local:87 }

ist-Command  OPERATION::= {	--Timer m

ARGUMENT

IST-CommandArg

RESULT

IST-CommandRes

-- optional

ERRORS {

unexpectedDataValue |

resourceLimitation |

unknownSubscriber |

systemFailure |

facilityNotSupported}

CODE	local:88 }

releaseResources  OPERATION::= {	--Timer m

ARGUMENT

ReleaseResourcesArg

RESULT

ReleaseResourcesRes

-- optional

ERRORS {

unexpectedDataValue |

systemFailure }

CODE	local:20 }

.#END

* * * Next Change * * * *

### 17.6.4	Supplementary service operations

.$MAP-SupplementaryServiceOperations {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SupplementaryServiceOperations (8)

version22 (22)}

DEFINITIONS

::=

BEGIN

EXPORTS

registerSS,

eraseSS,

activateSS,

deactivateSS,

interrogateSS,

processUnstructuredSS-Request,

unstructuredSS-Request,

unstructuredSS-Notify,

registerPassword,

getPassword,

ss-InvocationNotification,

registerCC-Entry,

eraseCC-Entry

;

IMPORTS

OPERATION

FROM Remote-Operations-Information-Objects {

joint-iso-itu-t remote-operations(4)

informationObjects(5) version1(0)}

systemFailure,

dataMissing,

unexpectedDataValue,

unknownSubscriber,

bearerServiceNotProvisioned,

teleserviceNotProvisioned,

callBarred,

illegalSS-Operation,

ss-ErrorStatus,

ss-NotAvailable,

ss-SubscriptionViolation,

ss-Incompatibility,

pw-RegistrationFailure,

negativePW-Check,

numberOfPW-AttemptsViolation,

unknownAlphabet,

ussd-Busy,

absentSubscriber,

illegalSubscriber,

illegalEquipment,

shortTermDenial,

longTermDenial,

facilityNotSupported

FROM MAP-Errors {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Errors (10) version22 (22)}

RegisterSS-Arg,

SS-Info,

SS-ForBS-Code,

InterrogateSS-Res,

USSD-Arg,

USSD-Res,

Password,

GuidanceInfo,

SS-InvocationNotificationArg,

SS-InvocationNotificationRes,

RegisterCC-EntryArg,

RegisterCC-EntryRes,

EraseCC-EntryArg,

EraseCC-EntryRes

FROM MAP-SS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-DataTypes (14) version22 (22)}

SS-Code

FROM MAP-SS-Code {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-Code (15) version22 (22)}

;

-- supplementary service handling operations

registerSS  OPERATION ::= {	--Timer m

ARGUMENT

RegisterSS-Arg

RESULT

SS-Info

-- optional

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

bearerServiceNotProvisioned |

teleserviceNotProvisioned |

callBarred |

illegalSS-Operation |

ss-ErrorStatus |

ss-Incompatibility}

CODE	local:10 }

eraseSS  OPERATION ::= {	--Timer m

ARGUMENT

SS-ForBS-Code

RESULT

SS-Info

-- optional

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

bearerServiceNotProvisioned |

teleserviceNotProvisioned |

callBarred |

illegalSS-Operation |

ss-ErrorStatus

}

CODE	local:11 }

activateSS  OPERATION ::= {	--Timer m

ARGUMENT

SS-ForBS-Code

RESULT

SS-Info

-- optional

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

bearerServiceNotProvisioned |

teleserviceNotProvisioned |

callBarred |

illegalSS-Operation |

ss-ErrorStatus |

ss-SubscriptionViolation |

ss-Incompatibility |

negativePW-Check |

numberOfPW-AttemptsViolation}

CODE	local:12 }

deactivateSS  OPERATION ::= {	--Timer m

ARGUMENT

SS-ForBS-Code

RESULT

SS-Info

-- optional

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

bearerServiceNotProvisioned |

teleserviceNotProvisioned |

callBarred |

illegalSS-Operation |

ss-ErrorStatus |

ss-SubscriptionViolation |

negativePW-Check |

numberOfPW-AttemptsViolation}

CODE	local:13 }

interrogateSS  OPERATION ::= {	--Timer m

ARGUMENT

SS-ForBS-Code

RESULT

InterrogateSS-Res

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

bearerServiceNotProvisioned |

teleserviceNotProvisioned |

callBarred |

illegalSS-Operation |

ss-NotAvailable}

CODE	local:14 }

processUnstructuredSS-Request  OPERATION ::= {	--Timer 10 minutes

ARGUMENT

USSD-Arg

RESULT

USSD-Res

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

unknownAlphabet |

callBarred}

CODE	local:59 }

unstructuredSS-Request  OPERATION ::= {	--Timer ml

ARGUMENT

USSD-Arg

RESULT

USSD-Res

-- optional

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

absentSubscriber |

illegalSubscriber |

illegalEquipment |

unknownAlphabet |

ussd-Busy}

CODE	local:60 }

unstructuredSS-Notify  OPERATION ::= {	--Timer ml

ARGUMENT

USSD-Arg

RETURN RESULT TRUE

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

absentSubscriber |

illegalSubscriber |

illegalEquipment |

unknownAlphabet |

ussd-Busy}

CODE	local:61 }

registerPassword  OPERATION ::= {	--Timer ml

ARGUMENT

SS-Code

RESULT

Password

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

callBarred |

ss-SubscriptionViolation |

pw-RegistrationFailure |

negativePW-Check |

numberOfPW-AttemptsViolation}

--	LINKED {

--	getPassword}

CODE	local:17 }

getPassword  OPERATION ::= {	--Timer m

ARGUMENT

GuidanceInfo

RESULT

Password

CODE	local:18 }

ss-InvocationNotification  OPERATION ::= {	--Timer m

ARGUMENT

SS-InvocationNotificationArg

RESULT

SS-InvocationNotificationRes

-- optional

ERRORS {

dataMissing |

unexpectedDataValue |

unknownSubscriber}

CODE	local:72 }

registerCC-Entry  OPERATION ::= {	--Timer m

ARGUMENT

RegisterCC-EntryArg

RESULT

RegisterCC-EntryRes

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

callBarred |

illegalSS-Operation |

ss-ErrorStatus |

ss-Incompatibility |

shortTermDenial |

longTermDenial |

facilityNotSupported}

CODE	local:76 }

eraseCC-Entry  OPERATION ::= {	--Timer m

ARGUMENT

EraseCC-EntryArg

RESULT

EraseCC-EntryRes

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

callBarred |

illegalSS-Operation |

ss-ErrorStatus}

CODE	local:77 }

.#END

* * * Next Change * * * *

### 17.6.5	Short message service operations

.$MAP-ShortMessageServiceOperations {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ShortMessageServiceOperations (9)

version22 (22)}

DEFINITIONS

::=

BEGIN

EXPORTS

sendRoutingInfoForSM,

mo-ForwardSM,

mt-ForwardSM,

reportSM-DeliveryStatus,

alertServiceCentre,

informServiceCentre,

readyForSM,

mt-ForwardSM-VGCS

;

IMPORTS

OPERATION

FROM Remote-Operations-Information-Objects {

joint-iso-itu-t remote-operations(4)

informationObjects(5) version1(0)}

systemFailure,

dataMissing,

unexpectedDataValue,

facilityNotSupported,

unknownSubscriber,

unidentifiedSubscriber,

illegalSubscriber,

illegalEquipment,

teleserviceNotProvisioned,

callBarred,

subscriberBusyForMT-SMS,

sm-DeliveryFailure,

messageWaitingListFull,

absentSubscriberSM

FROM MAP-Errors {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Errors (10) version22 (22)}

RoutingInfoForSM-Arg,

RoutingInfoForSM-Res,

MO-ForwardSM-Arg,

MO-ForwardSM-Res,

MT-ForwardSM-Arg,

MT-ForwardSM-Res,

ReportSM-DeliveryStatusArg,

ReportSM-DeliveryStatusRes,

AlertServiceCentreArg,

InformServiceCentreArg,

ReadyForSM-Arg,

ReadyForSM-Res,

MT-ForwardSM-VGCS-Arg,

MT-ForwardSM-VGCS-Res

FROM MAP-SM-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SM-DataTypes (16) version22 (22)}

;

sendRoutingInfoForSM  OPERATION ::= {	--Timer m

ARGUMENT

RoutingInfoForSM-Arg

RESULT

RoutingInfoForSM-Res

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

facilityNotSupported |

unknownSubscriber |

teleserviceNotProvisioned |

callBarred |

absentSubscriberSM}

CODE	local:45 }

mo-ForwardSM  OPERATION ::= {	--Timer ml

ARGUMENT

MO-ForwardSM-Arg

RESULT

MO-ForwardSM-Res

-- optional

ERRORS {

systemFailure |

unexpectedDataValue |

facilityNotSupported |

sm-DeliveryFailure}

CODE	local:46 }

mt-ForwardSM  OPERATION ::= {	--Timer ml

-- the timer value may be subject to negotiation between GMSC and IP-SM-GW

ARGUMENT

MT-ForwardSM-Arg

RESULT

MT-ForwardSM-Res

-- optional

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

facilityNotSupported |

unidentifiedSubscriber |

illegalSubscriber |

illegalEquipment |

subscriberBusyForMT-SMS |

sm-DeliveryFailure |

absentSubscriberSM}

CODE	local:44 }

reportSM-DeliveryStatus  OPERATION ::= {	--Timer s

ARGUMENT

ReportSM-DeliveryStatusArg

RESULT

ReportSM-DeliveryStatusRes

-- optional

ERRORS {

dataMissing |

unexpectedDataValue |

unknownSubscriber |

messageWaitingListFull}

CODE	local:47 }

alertServiceCentre  OPERATION ::= {	--Timer s

ARGUMENT

AlertServiceCentreArg

RETURN RESULT TRUE

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue}

CODE	local:64 }

informServiceCentre  OPERATION ::= {	--Timer s

ARGUMENT

InformServiceCentreArg

CODE	local:63 }

readyForSM  OPERATION ::= {	--Timer m

ARGUMENT

ReadyForSM-Arg

RESULT

ReadyForSM-Res

-- optional

ERRORS {

dataMissing |

unexpectedDataValue |

facilityNotSupported |

unknownSubscriber}

CODE	local:66 }

mt-ForwardSM-VGCS  OPERATION ::= {	--Timer ml

ARGUMENT

MT-ForwardSM-VGCS-Arg

RESULT

MT-ForwardSM-VGCS-Res

-- optional

ERRORS {

systemFailure |

unexpectedDataValue }

CODE	local:21 }

.#END

* * * Next Change * * * *

### 17.6.6	Errors

.$MAP-Errors {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Errors (10) version22 (22)}

DEFINITIONS

::=

BEGIN

EXPORTS

-- generic errors

systemFailure,

dataMissing,

unexpectedDataValue,

facilityNotSupported,

incompatibleTerminal,

resourceLimitation,

-- identification and numbering errors

unknownSubscriber,

numberChanged,

unknownMSC,

unidentifiedSubscriber,

unknownEquipment,

-- subscription errors

roamingNotAllowed,

illegalSubscriber,

illegalEquipment,

bearerServiceNotProvisioned,

teleserviceNotProvisioned,

-- handover errors

noHandoverNumberAvailable,

subsequentHandoverFailure,

targetCellOutsideGroupCallArea,

-- operation and maintenance errors

tracingBufferFull,

-- call handling errors

or-NotAllowed,

noRoamingNumberAvailable,

busySubscriber,

noSubscriberReply,

absentSubscriber,

callBarred,

forwardingViolation,

forwardingFailed,

cug-Reject,

-- any time interrogation errors

ati-NotAllowed,

-- any time information handling errors

# atsi-NotAllowed,

# atm-NotAllowed,

informationNotAvailable,

-- supplementary service errors

illegalSS-Operation,

ss-ErrorStatus,

ss-NotAvailable,

ss-SubscriptionViolation,

ss-Incompatibility,

unknownAlphabet,

ussd-Busy,

pw-RegistrationFailure,

negativePW-Check,

numberOfPW-AttemptsViolation,

shortTermDenial,

longTermDenial,

-- short message service errors

subscriberBusyForMT-SMS,

sm-DeliveryFailure,

messageWaitingListFull,

absentSubscriberSM,

-- Group Call errors

noGroupCallNumberAvailable,

ongoingGroupCall,

-- location service errors

unauthorizedRequestingNetwork,

unauthorizedLCSClient,

positionMethodFailure,

# unknownOrUnreachableLCSClient,

-- Mobility Management errors

# mm-EventNotSupported

;

IMPORTS

ERROR

FROM Remote-Operations-Information-Objects {joint-iso-itu-t remote-operations(4)

informationObjects(5) version1(0) }

SS-Status

FROM MAP-SS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-DataTypes (14) version22 (22)}

SS-IncompatibilityCause,

PW-RegistrationFailureCause,

SM-DeliveryFailureCause,

SystemFailureParam,

DataMissingParam,

UnexpectedDataParam,

FacilityNotSupParam,

UnknownSubscriberParam,

NumberChangedParam,

UnidentifiedSubParam,

RoamingNotAllowedParam,

IllegalSubscriberParam,

IllegalEquipmentParam,

BearerServNotProvParam,

TeleservNotProvParam,

TracingBufferFullParam,

NoRoamingNbParam,

OR-NotAllowedParam,

AbsentSubscriberParam,

BusySubscriberParam,

NoSubscriberReplyParam,

CallBarredParam,

ForwardingViolationParam,

ForwardingFailedParam,

CUG-RejectParam,

ATI-NotAllowedParam,

SubBusyForMT-SMS-Param,

MessageWaitListFullParam,

AbsentSubscriberSM-Param,

ResourceLimitationParam,

NoGroupCallNbParam,

IncompatibleTerminalParam,

ShortTermDenialParam,

LongTermDenialParam,

UnauthorizedRequestingNetwork-Param,

UnauthorizedLCSClient-Param,

PositionMethodFailure-Param,

# UnknownOrUnreachableLCSClient-Param,

# MM-EventNotSupported-Param,

ATSI-NotAllowedParam,

ATM-NotAllowedParam,

IllegalSS-OperationParam,

SS-NotAvailableParam,

SS-SubscriptionViolationParam,

InformationNotAvailableParam,

TargetCellOutsideGCA-Param,

OngoingGroupCallParam

FROM MAP-ER-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ER-DataTypes (17) version22 (22)}

;

-- generic errors

systemFailure  ERROR ::= {

PARAMETER

SystemFailureParam

-- optional

CODE	local:34 }

dataMissing  ERROR ::= {

PARAMETER

DataMissingParam

-- optional

-- DataMissingParam must not be used in version <3

CODE	local:35 }

unexpectedDataValue  ERROR ::= {

PARAMETER

UnexpectedDataParam

-- optional

-- UnexpectedDataParam must not be used in version <3

CODE	local:36 }

facilityNotSupported  ERROR ::= {

PARAMETER

FacilityNotSupParam

-- optional

-- FacilityNotSupParam must not be used in version <3

CODE	local:21 }

incompatibleTerminal  ERROR ::= {

PARAMETER

IncompatibleTerminalParam

-- optional

CODE	local:28 }

resourceLimitation  ERROR ::= {

PARAMETER

ResourceLimitationParam

-- optional

CODE	local:51 }

-- identification and numbering errors

unknownSubscriber  ERROR ::= {

PARAMETER

UnknownSubscriberParam

-- optional

-- UnknownSubscriberParam must not be used in version <3

CODE	local:1 }

numberChanged  ERROR ::= {

PARAMETER

NumberChangedParam

-- optional

CODE	local:44 }

unknownMSC  ERROR ::= {

CODE	local:3 }

unidentifiedSubscriber  ERROR ::= {

PARAMETER

UnidentifiedSubParam

-- optional

-- UunidentifiedSubParam must not be used in version <3

CODE	local:5 }

unknownEquipment  ERROR ::= {

CODE	local:7 }

-- subscription errors

roamingNotAllowed  ERROR ::= {

PARAMETER

RoamingNotAllowedParam

CODE	local:8 }

illegalSubscriber  ERROR ::= {

PARAMETER

IllegalSubscriberParam

-- optional

-- IllegalSubscriberParam must not be used in version <3

CODE	local:9 }

illegalEquipment  ERROR ::= {

PARAMETER

IllegalEquipmentParam

-- optional

-- IllegalEquipmentParam must not be used in version <3

CODE	local:12 }

bearerServiceNotProvisioned  ERROR ::= {

PARAMETER

BearerServNotProvParam

-- optional

-- BearerServNotProvParam must not be used in version <3

CODE	local:10 }

teleserviceNotProvisioned  ERROR ::= {

PARAMETER

TeleservNotProvParam

-- optional

-- TeleservNotProvParam must not be used in version <3

CODE	local:11 }

-- handover errors

noHandoverNumberAvailable  ERROR ::= {

CODE	local:25 }

subsequentHandoverFailure  ERROR ::= {

CODE	local:26 }

targetCellOutsideGroupCallArea  ERROR ::= {

PARAMETER

TargetCellOutsideGCA-Param

-- optional

CODE	local:42 }

-- operation and maintenance errors

tracingBufferFull  ERROR ::= {

PARAMETER

TracingBufferFullParam

-- optional

CODE	local: 40 }

-- call handling errors

noRoamingNumberAvailable  ERROR ::= {

PARAMETER

NoRoamingNbParam

-- optional

CODE	local:39 }

absentSubscriber  ERROR ::= {

PARAMETER

AbsentSubscriberParam

-- optional

-- AbsentSubscriberParam must not be used in version <3

CODE	local:27 }

busySubscriber  ERROR ::= {

PARAMETER

BusySubscriberParam

-- optional

CODE	local:45 }

noSubscriberReply  ERROR ::= {

PARAMETER

NoSubscriberReplyParam

-- optional

CODE	local:46 }

callBarred  ERROR ::= {

PARAMETER

CallBarredParam

-- optional

CODE	local:13 }

forwardingViolation  ERROR ::= {

PARAMETER

ForwardingViolationParam

-- optional

CODE	local:14 }

forwardingFailed  ERROR ::= {

PARAMETER

ForwardingFailedParam

-- optional

CODE	local:47 }

cug-Reject  ERROR ::= {

PARAMETER

CUG-RejectParam

-- optional

CODE	local:15 }

or-NotAllowed  ERROR ::= {

PARAMETER

OR-NotAllowedParam

-- optional

CODE	local:48 }

-- any time interrogation errors

ati-NotAllowed  ERROR ::= {

PARAMETER

ATI-NotAllowedParam

-- optional

CODE	local:49 }

-- any time information handling errors

# atsi-NotAllowed  ERROR ::= {

# PARAMETER

ATSI-NotAllowedParam

-- optional

CODE	local:60 }

# atm-NotAllowed  ERROR ::= {

# PARAMETER

ATM-NotAllowedParam

-- optional

CODE	local:61 }

informationNotAvailable  ERROR ::= {

PARAMETER

InformationNotAvailableParam

-- optional

CODE	local:62 }

-- supplementary service errors

# illegalSS-Operation  ERROR ::= {

PARAMETER

IllegalSS-OperationParam

-- optional

-- IllegalSS-OperationParam must not be used in version <3

CODE	local:16 }

ss-ErrorStatus  ERROR ::= {

PARAMETER

SS-Status

-- optional

CODE	local:17 }

# ss-NotAvailable  ERROR ::= {

PARAMETER

SS-NotAvailableParam

-- optional

-- SS-NotAvailableParam must not be used in version <3

CODE	local:18 }

# ss-SubscriptionViolation  ERROR ::= {

PARAMETER

SS-SubscriptionViolationParam

-- optional

-- SS-SubscriptionViolationParam must not be used in version <3

CODE	local:19 }

ss-Incompatibility  ERROR ::= {

PARAMETER

SS-IncompatibilityCause

-- optional

CODE	local:20 }

unknownAlphabet  ERROR ::= {

CODE	local:71 }

ussd-Busy  ERROR ::= {

CODE	local:72 }

pw-RegistrationFailure  ERROR ::= {

PARAMETER

PW-RegistrationFailureCause

CODE	local:37 }

negativePW-Check  ERROR ::= {

CODE	local:38 }

numberOfPW-AttemptsViolation  ERROR ::= {

CODE	local:43 }

shortTermDenial  ERROR ::= {

PARAMETER

ShortTermDenialParam

-- optional

CODE	local:29 }

longTermDenial  ERROR ::= {

PARAMETER

LongTermDenialParam

-- optional

CODE	local:30 }

-- short message service errors

subscriberBusyForMT-SMS  ERROR ::= {

PARAMETER

SubBusyForMT-SMS-Param

-- optional

CODE	local:31 }

sm-DeliveryFailure  ERROR ::= {

PARAMETER

SM-DeliveryFailureCause

CODE	local:32 }

messageWaitingListFull  ERROR ::= {

PARAMETER

MessageWaitListFullParam

-- optional

CODE	local:33 }

absentSubscriberSM  ERROR ::= {

PARAMETER

AbsentSubscriberSM-Param

-- optional

CODE	local:6 }

-- Group Call errors

noGroupCallNumberAvailable  ERROR ::= {

PARAMETER

NoGroupCallNbParam

-- optional

CODE	local:50 }

ongoingGroupCall  ERROR ::= {

PARAMETER

OngoingGroupCallParam

-- optional

CODE	local:22 }

-- location service errors

unauthorizedRequestingNetwork  ERROR ::= {

PARAMETER

UnauthorizedRequestingNetwork-Param

-- optional

CODE	local:52 }

unauthorizedLCSClient  ERROR ::= {

PARAMETER

UnauthorizedLCSClient-Param

-- optional

CODE	local:53 }

positionMethodFailure  ERROR ::= {

PARAMETER

PositionMethodFailure-Param

-- optional

CODE	local:54 }

unknownOrUnreachableLCSClient  ERROR ::= {

PARAMETER

UnknownOrUnreachableLCSClient-Param

-- optional

CODE	local:58 }

# mm-EventNotSupported  ERROR ::= {

# PARAMETER

MM-EventNotSupported-Param

-- optional

CODE	local:59 }

.#END

* * * Next Change * * * *

### 17.6.7	Group Call operations

.$MAP-Group-Call-Operations {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Group-Call-Operations (22)

version22 (22)}

DEFINITIONS

::=

BEGIN

EXPORTS

prepareGroupCall,

sendGroupCallEndSignal,

forwardGroupCallSignalling,

processGroupCallSignalling,

sendGroupCallInfo

;

IMPORTS

OPERATION

FROM Remote-Operations-Information-Objects {

joint-iso-itu-t remote-operations(4)

informationObjects(5) version1(0)}

systemFailure,

unexpectedDataValue,

noGroupCallNumberAvailable,

ongoingGroupCall,

unknownSubscriber,

teleserviceNotProvisioned,

dataMissing

FROM MAP-Errors {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Errors (10) version22 (22)}

PrepareGroupCallArg,

PrepareGroupCallRes,

SendGroupCallEndSignalArg,

SendGroupCallEndSignalRes,

ForwardGroupCallSignallingArg,

ProcessGroupCallSignallingArg,

SendGroupCallInfoArg,

SendGroupCallInfoRes

FROM MAP-GR-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-GR-DataTypes (23) version22 (22)}

;

prepareGroupCall  OPERATION ::= {	--Timer m

ARGUMENT

PrepareGroupCallArg

RESULT

PrepareGroupCallRes

ERRORS {

systemFailure |

noGroupCallNumberAvailable |

unexpectedDataValue}

CODE	local:39 }

sendGroupCallEndSignal  OPERATION ::= {	--Timer l

ARGUMENT

SendGroupCallEndSignalArg

RESULT

SendGroupCallEndSignalRes

CODE	local:40 }

processGroupCallSignalling  OPERATION ::= {	--Timer s

ARGUMENT

ProcessGroupCallSignallingArg

CODE	local:41 }

forwardGroupCallSignalling  OPERATION ::= {	--Timer s

ARGUMENT

ForwardGroupCallSignallingArg

CODE	local:42 }

sendGroupCallInfo  OPERATION ::= {	--Timer m

ARGUMENT

SendGroupCallInfoArg

RESULT

SendGroupCallInfoRes

ERRORS {

systemFailure |

ongoingGroupCall |

unexpectedDataValue |

dataMissing |

teleserviceNotProvisioned |

unknownSubscriber}

CODE	local:84 }

.#END

* * * Next Change * * * *

### 17.6.8	Location service operations

.$MAP-LocationServiceOperations {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-LocationServiceOperations (24)

version22 (22)}

DEFINITIONS

::=

BEGIN

EXPORTS

provideSubscriberLocation,

sendRoutingInfoForLCS,

subscriberLocationReport

;

IMPORTS

OPERATION

FROM Remote-Operations-Information-Objects {

joint-iso-itu-t remote-operations(4)

informationObjects(5) version1(0)}

systemFailure,

dataMissing,

unexpectedDataValue,

facilityNotSupported,

unknownSubscriber,

absentSubscriber,

unauthorizedRequestingNetwork,

unauthorizedLCSClient,

positionMethodFailure,

resourceLimitation,

unknownOrUnreachableLCSClient,

unidentifiedSubscriber,

illegalEquipment,

illegalSubscriber

FROM MAP-Errors {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Errors (10) version22 (22)}

RoutingInfoForLCS-Arg,

RoutingInfoForLCS-Res,

ProvideSubscriberLocation-Arg,

ProvideSubscriberLocation-Res,

SubscriberLocationReport-Arg,

SubscriberLocationReport-Res

FROM MAP-LCS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-LCS-DataTypes (25) version22 (22)}

;

sendRoutingInfoForLCS  OPERATION ::= {	--Timer m

ARGUMENT

RoutingInfoForLCS-Arg

RESULT

RoutingInfoForLCS-Res

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

facilityNotSupported |

unknownSubscriber |

absentSubscriber |

unauthorizedRequestingNetwork }

CODE	local:85 }

provideSubscriberLocation  OPERATION ::= {	--Timer ml

ARGUMENT

ProvideSubscriberLocation-Arg

RESULT

ProvideSubscriberLocation-Res

ERRORS {

systemFailure |

dataMissing |

unexpectedDataValue |

facilityNotSupported |

unidentifiedSubscriber |

illegalSubscriber |

illegalEquipment |

absentSubscriber |

unauthorizedRequestingNetwork |

unauthorizedLCSClient |

positionMethodFailure }

CODE	local:83 }

subscriberLocationReport  OPERATION ::= {	--Timer m

ARGUMENT

SubscriberLocationReport-Arg

RESULT

SubscriberLocationReport-Res

ERRORS {

systemFailure |

dataMissing |

resourceLimitation |

unexpectedDataValue |

unknownSubscriber |

unauthorizedRequestingNetwork |

unknownOrUnreachableLCSClient}

CODE	local:86 }

.#END

* * * Next Change * * * *

### 17.7.1	Mobile Service data types

.$MAP-MS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version22 (22)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

-- location registration types

UpdateLocationArg,

UpdateLocationRes,

CancelLocationArg,

CancelLocationRes,

PurgeMS-Arg,

PurgeMS-Res,

SendIdentificationArg,

SendIdentificationRes,

UpdateGprsLocationArg,

UpdateGprsLocationRes,

IST-SupportIndicator,

SupportedLCS-CapabilitySets,

UpdateVcsgLocationArg,

UpdateVcsgLocationRes,

CancelVcsgLocationArg,

CancelVcsgLocationRes,

-- handover types

ForwardAccessSignalling-Arg,

PrepareHO-Arg,

PrepareHO-Res,

PrepareSubsequentHO-Arg,

PrepareSubsequentHO-Res,

ProcessAccessSignalling-Arg,

SendEndSignal-Arg,

SendEndSignal-Res,

-- authentication management types

SendAuthenticationInfoArg,

SendAuthenticationInfoRes,

AuthenticationFailureReportArg,

AuthenticationFailureReportRes,

-- security management types

Kc,

Cksn,

-- equipment management types

CheckIMEI-Arg,

CheckIMEI-Res,

-- subscriber management types

InsertSubscriberDataArg,

InsertSubscriberDataRes,

LSAIdentity,

DeleteSubscriberDataArg,

DeleteSubscriberDataRes,

Ext-QoS-Subscribed,

Ext2-QoS-Subscribed,

Ext3-QoS-Subscribed,

Ext4-QoS-Subscribed,

SubscriberData,

ODB-Data,

SubscriberStatus,

ZoneCodeList,

maxNumOfZoneCodes,

O-CSI,

D-CSI,

O-BcsmCamelTDPCriteriaList,

T-BCSM-CAMEL-TDP-CriteriaList,

SS-CSI,

ServiceKey,

DefaultCallHandling,

DefaultSMS-Handling,

DefaultGPRS-Handling,

CamelCapabilityHandling,

BasicServiceCriteria,

SupportedCamelPhases,

OfferedCamel4CSIs,

OfferedCamel4Functionalities,

maxNumOfCamelTDPData,

CUG-Index,

CUG-Info,

CUG-Interlock,

InterCUG-Restrictions,

IntraCUG-Options,

NotificationToMSUser,

QoS-Subscribed,

IST-AlertTimerValue,

T-CSI,

T-BcsmTriggerDetectionPoint,

APN,

AdditionalInfo,

-- fault recovery types

ResetArg,

RestoreDataArg,

RestoreDataRes,

-- provide subscriber info types

GeographicalInformation,

MS-Classmark2,

GPRSMSClass,

-- subscriber information enquiry types

ProvideSubscriberInfoArg,

ProvideSubscriberInfoRes,

SubscriberInfo,

LocationInformation,

LocationInformationGPRS,

SubscriberState,

GPRSChargingID,

MNPInfoRes,

RouteingNumber,

-- any time information enquiry types

AnyTimeInterrogationArg,

AnyTimeInterrogationRes,

-- any time information handling types

AnyTimeSubscriptionInterrogationArg,

AnyTimeSubscriptionInterrogationRes,

# AnyTimeModificationArg,

# AnyTimeModificationRes,

-- subscriber data modification notification types

NoteSubscriberDataModifiedArg,

NoteSubscriberDataModifiedRes,

-- gprs location information retrieval types

SendRoutingInfoForGprsArg,

SendRoutingInfoForGprsRes,

-- failure reporting types

FailureReportArg,

FailureReportRes,

-- gprs notification types

NoteMsPresentForGprsArg,

# NoteMsPresentForGprsRes,

-- Mobility Management types

NoteMM-EventArg,

NoteMM-EventRes,

NumberPortabilityStatus,

PagingArea,

-- VGCS / VBS types types

GroupId,

Long-GroupId,

AdditionalSubscriptions

;

IMPORTS

maxNumOfSS,

SS-SubscriptionOption,

SS-List,

SS-ForBS-Code,

Password,

OverrideCategory,

CliRestrictionOption

FROM MAP-SS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-DataTypes (14) version22 (22)}

SS-Code

FROM MAP-SS-Code {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-Code (15) version22 (22)}

Ext-BearerServiceCode

FROM MAP-BS-Code {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-BS-Code (20) version22 (22)}

Ext-TeleserviceCode

FROM MAP-TS-Code {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-TS-Code (19) version22 (22)}

AddressString,

ISDN-AddressString,

ISDN-SubaddressString,

FTN-AddressString,

AccessNetworkSignalInfo,

IMSI,

IMEI,

TMSI,

HLR-List,

LMSI,

Identity,

GlobalCellId,

CellGlobalIdOrServiceAreaIdOrLAI,

Ext-BasicServiceCode,

NAEA-PreferredCI,

EMLPP-Info,

MC-SS-Info,

SubscriberIdentity,

AgeOfLocationInformation,

LCSClientExternalID,

LCSClientInternalID,

Ext-SS-Status,

LCSServiceTypeID,

ASCI-CallReference,

TBCD-STRING,

LAIFixedLength,

PLMN-Id,

EMLPP-Priority,

GSN-Address,

DiameterIdentity,

Time,

E-UTRAN-CGI,

NR-CGI,

TA-Id,

NR-TA-Id,

RAIdentity,

NetworkNodeDiameterAddress

FROM MAP-CommonDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version22 (22)}

ExtensionContainer

FROM MAP-ExtensionDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ExtensionDataTypes (21) version22 (22)}

AbsentSubscriberDiagnosticSM

FROM MAP-ER-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ER-DataTypes (17) version22 (22)}

TracePropagationList

FROM MAP-OM-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-OM-DataTypes (12) version22 (22)}

;

-- location registration types

UpdateLocationArg ::= SEQUENCE {

imsi	IMSI,

msc-Number	[1] ISDN-AddressString,

vlr-Number	ISDN-AddressString,

lmsi	[10] LMSI	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

... ,

vlr-Capability	[6] VLR-Capability	OPTIONAL,

informPreviousNetworkEntity	[11]	NULL	OPTIONAL,

cs-LCS-NotSupportedByUE	[12]	NULL	OPTIONAL,

v-gmlc-Address	[2]	GSN-Address	OPTIONAL,

add-info	[13] ADD-Info	OPTIONAL,

pagingArea	[14] PagingArea	OPTIONAL,

skipSubscriberDataUpdate	[15] NULL	OPTIONAL,

-- The skipSubscriberDataUpdate parameter in the UpdateLocationArg and the ADD-Info

-- structures carry the same semantic.

restorationIndicator	[16]	NULL	OPTIONAL,

eplmn-List	[3] EPLMN-List	OPTIONAL,

mme-DiameterAddress	[4] NetworkNodeDiameterAddress	OPTIONAL

}

VLR-Capability ::= SEQUENCE{

supportedCamelPhases 	[0] SupportedCamelPhases	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

... ,

solsaSupportIndicator	[2] NULL	OPTIONAL,

istSupportIndicator	[1] IST-SupportIndicator	OPTIONAL,

superChargerSupportedInServingNetworkEntity	[3] SuperChargerInfo	OPTIONAL,

longFTN-Supported	[4]	NULL	OPTIONAL,

supportedLCS-CapabilitySets	[5]	SupportedLCS-CapabilitySets	OPTIONAL,

offeredCamel4CSIs	[6] OfferedCamel4CSIs	OPTIONAL,

supportedRAT-TypesIndicator	[7]	SupportedRAT-Types	OPTIONAL,

longGroupID-Supported	[8]	NULL	OPTIONAL,

mtRoamingForwardingSupported	[9]	NULL	OPTIONAL,

msisdn-lessOperation-Supported	[10]	NULL	OPTIONAL,

reset-ids-Supported	[11]	NULL	OPTIONAL }

SupportedRAT-Types::= BIT STRING {

utran  (0),

geran  (1),

gan    (2),

i-hspa-evolution (3),

e-utran	(4),

nb-iot	(5)} (SIZE (2..8))

-- exception handling: bits 6 to 7 shall be ignored if received and not understood

SuperChargerInfo ::= CHOICE {

sendSubscriberData	[0] NULL,

subscriberDataStored	[1] AgeIndicator }

AgeIndicator ::= OCTET STRING (SIZE (1..6))

-- The internal structure of this parameter is implementation specific.

IST-SupportIndicator ::=  ENUMERATED {

basicISTSupported	(0),

istCommandSupported	(1),

...}

-- exception handling:

-- reception of values > 1 shall be mapped to ' istCommandSupported '

SupportedLCS-CapabilitySets ::= BIT STRING {

lcsCapabilitySet1 (0),

lcsCapabilitySet2 (1),

lcsCapabilitySet3 (2),

lcsCapabilitySet4 (3) ,

lcsCapabilitySet5 (4) } (SIZE (2..16))

-- Core network signalling capability set1 indicates LCS Release98 or Release99 version.

-- Core network signalling capability set2 indicates LCS Release4.

-- Core network signalling capability set3 indicates LCS Release5.

-- Core network signalling capability set4 indicates LCS Release6.

-- Core network signalling capability set5 indicates LCS Release7 or later version.

-- A node shall mark in the BIT STRING all LCS capability sets it supports.

-- If no bit is set then the sending node does not support LCS.

-- If the parameter is not sent by an VLR then the VLR may support at most capability set1.

-- If the parameter is not sent by an SGSN then no support for LCS is assumed.

-- An SGSN is not allowed to indicate support of capability set1.

-- Other bits than listed above shall be discarded.

UpdateLocationRes ::= SEQUENCE {

hlr-Number	ISDN-AddressString,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

add-Capability	NULL	OPTIONAL,

pagingArea-Capability	[0]NULL	OPTIONAL }

ADD-Info ::= SEQUENCE {

imeisv	[0] IMEI,

skipSubscriberDataUpdate	[1] NULL	OPTIONAL,

-- The skipSubscriberDataUpdate parameter in the UpdateLocationArg and the ADD-Info

-- structures carry the same semantic.

...}

PagingArea ::= SEQUENCE SIZE (1..5) OF LocationArea

LocationArea ::= CHOICE {

laiFixedLength	[0] LAIFixedLength,

lac	[1] LAC}

LAC ::= OCTET STRING (SIZE (2))

-- Refers to Location Area Code of the Location Area Identification defined in

-- 3GPP TS 23.003 [17].

-- Location Area Code according to 3GPP TS 24.008 [35]

CancelLocationArg ::= [3] SEQUENCE {

identity	Identity,

cancellationType	CancellationType	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

typeOfUpdate	[0] TypeOfUpdate	OPTIONAL,

mtrf-SupportedAndAuthorized	[1] NULL	OPTIONAL,

mtrf-SupportedAndNotAuthorized	[2] NULL	OPTIONAL,

newMSC-Number	[3] ISDN-AddressString	OPTIONAL,

newVLR-Number	[4] ISDN-AddressString	OPTIONAL,

new-lmsi	[5] LMSI	OPTIONAL,

reattach-Required	[6] NULL	OPTIONAL

}

--mtrf-SupportedAndAuthorized and mtrf-SupportedAndNotAuthorized shall not

-- both be present

TypeOfUpdate ::= ENUMERATED {

sgsn-change (0),

mme-change (1),

...}

-- TypeOfUpdate shall be absent if CancellationType is different from updateProcedure

-- and initialAttachProcedure

CancellationType ::= ENUMERATED {

updateProcedure	(0),

subscriptionWithdraw	(1),

...,

initialAttachProcedure               (2)}

-- The HLR shall not send values other than listed above

CancelLocationRes ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

PurgeMS-Arg ::= [3] SEQUENCE {

imsi	IMSI,

vlr-Number	[0] ISDN-AddressString	OPTIONAL,

sgsn-Number	[1]	ISDN-AddressString	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

locationInformation	[2] LocationInformation	OPTIONAL,

locationInformationGPRS	[3] LocationInformationGPRS	OPTIONAL,

locationInformationEPS	[4] LocationInformationEPS	OPTIONAL }

PurgeMS-Res ::= SEQUENCE {

freezeTMSI	[0]	NULL	OPTIONAL,

freezeP-TMSI	[1]	NULL	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

freezeM-TMSI	[2]	NULL	OPTIONAL }

SendIdentificationArg ::= SEQUENCE {

tmsi	TMSI,

numberOfRequestedVectors	NumberOfRequestedVectors	OPTIONAL,

-- within a dialogue numberOfRequestedVectors shall be present in

-- the first service request and shall not be present in subsequent service requests.

-- If received in a subsequent service request it shall be discarded.

segmentationProhibited	NULL	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

msc-Number	ISDN-AddressString	OPTIONAL,

previous-LAI	[0] LAIFixedLength	OPTIONAL,

hopCounter	[1] HopCounter	OPTIONAL,

mtRoamingForwardingSupported	[2] NULL	OPTIONAL,

newVLR-Number	[3] ISDN-AddressString	OPTIONAL,

new-lmsi	[4] LMSI	OPTIONAL }

HopCounter ::= INTEGER (0..3)

SendIdentificationRes ::= [3] SEQUENCE {

imsi	IMSI	OPTIONAL,

-- IMSI shall be present in the first (or only) service response of a dialogue.

-- If multiple service requests are present in a dialogue then IMSI

-- shall not be present in any service response other than the first one.

authenticationSetList	AuthenticationSetList	OPTIONAL,

currentSecurityContext	[2]CurrentSecurityContext	OPTIONAL,

extensionContainer	[3] ExtensionContainer	OPTIONAL,

...,

lastUsedLtePLMN-Id	[4] PLMN-Id	OPTIONAL,

mtCallPendingFlag	[5]	NULL	OPTIONAL }

-- authentication management types

AuthenticationSetList ::= CHOICE {

tripletList	[0] TripletList,

quintupletList	[1] QuintupletList }

TripletList ::= SEQUENCE SIZE (1..5) OF

AuthenticationTriplet

QuintupletList ::= SEQUENCE SIZE (1..5) OF

AuthenticationQuintuplet

AuthenticationTriplet ::= SEQUENCE {

rand	RAND,

sres	SRES,

kc	Kc,

...}

AuthenticationQuintuplet ::= SEQUENCE {

rand	RAND,

xres	XRES,

ck	CK,

ik	IK,

autn	AUTN,

...}

CurrentSecurityContext ::= CHOICE {

gsm-SecurityContextData	[0] GSM-SecurityContextData,

umts-SecurityContextData	[1] UMTS-SecurityContextData }

GSM-SecurityContextData ::= SEQUENCE {

kc	Kc,

cksn	Cksn,

... }

UMTS-SecurityContextData ::= SEQUENCE {

ck	CK,

ik	IK,

ksi	KSI,

... }

RAND ::= OCTET STRING (SIZE (16))

SRES ::= OCTET STRING (SIZE (4))

Kc ::= OCTET STRING (SIZE (8))

XRES ::= OCTET STRING (SIZE (4..16))

CK ::= OCTET STRING (SIZE (16))

IK ::= OCTET STRING (SIZE (16))

AUTN ::= OCTET STRING (SIZE (16))

AUTS ::= OCTET STRING (SIZE (14))

Cksn ::= OCTET STRING (SIZE (1))

-- The internal structure is defined in 3GPP TS 24.008

KSI ::= OCTET STRING (SIZE (1))

-- The internal structure is defined in 3GPP TS 24.008

AuthenticationFailureReportArg ::= SEQUENCE {

imsi	IMSI,

failureCause	FailureCause,

extensionContainer	ExtensionContainer	OPTIONAL,

... ,

re-attempt	BOOLEAN	OPTIONAL,

accessType	AccessType	OPTIONAL,

rand	RAND	OPTIONAL,

vlr-Number	[0] ISDN-AddressString	OPTIONAL,

sgsn-Number	[1] ISDN-AddressString	OPTIONAL }

AccessType ::= ENUMERATED {

call (0),

emergencyCall (1),

locationUpdating (2),

supplementaryService (3),

shortMessage (4),

gprsAttach (5),

routingAreaUpdating (6),

serviceRequest (7),

pdpContextActivation (8),

pdpContextDeactivation (9),

...,

gprsDetach (10)}

-- exception handling:

-- received values greater than 10 shall be ignored.

AuthenticationFailureReportRes ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

FailureCause ::= ENUMERATED {

wrongUserResponse  (0),

wrongNetworkSignature  (1)}

-- gprs location registration types

UpdateGprsLocationArg ::= SEQUENCE {

imsi	IMSI,

sgsn-Number	ISDN-AddressString,

sgsn-Address	GSN-Address,

extensionContainer	ExtensionContainer	OPTIONAL,

... ,

sgsn-Capability	[0] SGSN-Capability	OPTIONAL,

informPreviousNetworkEntity	[1]	NULL	OPTIONAL,

ps-LCS-NotSupportedByUE	[2]	NULL	OPTIONAL,

v-gmlc-Address	[3]	GSN-Address	OPTIONAL,

add-info	[4]  ADD-Info	OPTIONAL,

eps-info	[5]	EPS-Info	OPTIONAL,

servingNodeTypeIndicator	[6]	NULL	OPTIONAL,

skipSubscriberDataUpdate	[7] NULL	OPTIONAL,

usedRAT-Type	[8] Used-RAT-Type	OPTIONAL,

gprsSubscriptionDataNotNeeded	[9] NULL	OPTIONAL,

nodeTypeIndicator	[10] NULL	OPTIONAL,

areaRestricted	[11] NULL	OPTIONAL,

ue-reachableIndicator	[12]	NULL	OPTIONAL,

epsSubscriptionDataNotNeeded	[13] NULL	OPTIONAL,

ue-srvcc-Capability	[14] UE-SRVCC-Capability	OPTIONAL,

eplmn-List	[15] EPLMN-List	OPTIONAL,

mmeNumberforMTSMS	[16] ISDN-AddressString	OPTIONAL,

smsRegisterRequest	[17] SMSRegisterRequest	OPTIONAL,

sms-Only	[18] NULL	OPTIONAL,

removalofMMERegistrationforSMS	[22] NULL	OPTIONAL,

sgsn-Name	[19] DiameterIdentity	OPTIONAL,

sgsn-Realm	[20]	DiameterIdentity	OPTIONAL,

lgd-supportIndicator	[21] NULL	OPTIONAL,

adjacentPLMN-List	[23] AdjacentPLMN-List	OPTIONAL }

SMSRegisterRequest::= ENUMERATED {

sms-registration-required  (0),

sms-registration-not-preferred  (1),

no-preference  (2),

...}

Used-RAT-Type::= ENUMERATED {

utran  (0),

geran  (1),

gan    (2),

i-hspa-evolution (3),

e-utran	(4),

...,

nb-iot	(5)}

-- The value e-utran indicates wide-band e-utran

EPS-Info ::= CHOICE{

pdn-gw-update	[0] PDN-GW-Update,

isr-Information	[1] ISR-Information }

PDN-GW-Update ::= SEQUENCE{

apn	[0] APN	OPTIONAL,

pdn-gw-Identity	[1] PDN-GW-Identity	OPTIONAL,

contextId	[2] ContextId                     OPTIONAL,

extensionContainer	[3] ExtensionContainer	OPTIONAL,

... }

--	The pdn-gw-update IE shall include the pdn-gw-Identity, and the apn or/and the contextID.

--	The HSS shall ignore the eps-info IE if it includes a pdn-gw-update IE which does not

--	include pdn-gw-Identity.

--	The pdn-gw-Identity is defined as OPTIONAL for backward compatility reason with

--	outdated earlier versions of this specification.

ISR-Information::= BIT STRING {

updateLocation  (0),

cancelSGSN  (1),

initialAttachIndicator  (2)} (SIZE (3..8))

-- exception handling: reception of unknown bit assignments in the

-- ISR-Information data type shall be discarded by the receiver

SGSN-Capability ::= SEQUENCE{

solsaSupportIndicator	NULL	OPTIONAL,

extensionContainer	[1] ExtensionContainer	OPTIONAL,

... ,

superChargerSupportedInServingNetworkEntity	[2] SuperChargerInfo	OPTIONAL ,

gprsEnhancementsSupportIndicator	[3] NULL	OPTIONAL,

supportedCamelPhases 	[4] SupportedCamelPhases	OPTIONAL,

supportedLCS-CapabilitySets	[5]  SupportedLCS-CapabilitySets	OPTIONAL,

offeredCamel4CSIs	[6] OfferedCamel4CSIs	OPTIONAL,

smsCallBarringSupportIndicator	[7]	NULL	OPTIONAL,	supportedRAT-TypesIndicator	[8]	SupportedRAT-Types	OPTIONAL,

supportedFeatures	[9] SupportedFeatures	OPTIONAL,

t-adsDataRetrieval	[10] NULL	OPTIONAL,

homogeneousSupportOfIMSVoiceOverPSSessions [11] BOOLEAN	OPTIONAL,

--	"true" indicates homogeneous support, "false" indicates homogeneous non-support

--	in the complete SGSN or MME area

cancellationTypeInitialAttach	[12]	NULL	OPTIONAL,

msisdn-lessOperation-Supported	[14]	NULL	OPTIONAL,

updateofHomogeneousSupportOfIMSVoiceOverPSSessions [15] NULL	OPTIONAL,

reset-ids-Supported	[16]	NULL	OPTIONAL,

ext-SupportedFeatures	[17]	Ext-SupportedFeatures	OPTIONAL

}

--	the supportedFeatures , t-adsDataRetrieval,

--	homogeneousSupportOfIMSVoiceOverPSSessions

--	/updateofHomogeneousSupportOfIMSVoiceOverPSSessions and

--	ext-SupportedFeatures are also applied to the MME/IWF

SupportedFeatures::= BIT STRING {

odb-all-apn (0),

odb-HPLMN-APN (1),

odb-VPLMN-APN (2),

odb-all-og (3),

odb-all-international-og (4),

odb-all-int-og-not-to-HPLMN-country (5),

odb-all-interzonal-og (6),

odb-all-interzonal-og-not-to-HPLMN-country (7),

odb-all-interzonal-og-and-internat-og-not-to-HPLMN-country (8),

regSub (9),

trace (10),

lcs-all-PrivExcep (11),

lcs-universal (12),

lcs-CallSessionRelated (13),

lcs-CallSessionUnrelated (14),

lcs-PLMN-operator (15),

lcs-ServiceType (16),

lcs-all-MOLR-SS (17),

lcs-basicSelfLocation (18),

lcs-autonomousSelfLocation (19),

lcs-transferToThirdParty (20),

sm-mo-pp (21),

barring-OutgoingCalls (22),

baoc (23),

boic (24),

boicExHC (25),

localTimeZoneRetrieval (26),

additionalMsisdn (27),

smsInMME (28),

smsInSGSN (29),

ue-Reachability-Notification (30),

state-Location-Information-Retrieval (31),

partialPurge (32),

gddInSGSN (33),

sgsnCAMELCapability (34),

pcscf-Restoration (35),

dedicatedCoreNetworks (36),

non-IP-PDN-Type-APNs (37),

non-IP-PDP-Type-APNs (38),

nrAsSecondaryRAT (39) } (SIZE (26..40))

--	for the definition and usage of the above features see the 3GPP TS 29.272 [144].

--	Additional supported features are encoded in Ext-SupportedFeatures bit string.

Ext-SupportedFeatures ::= BIT STRING {

unlicensedSpectrumAsSecondaryRAT (0) } (SIZE (1..40))

UE-SRVCC-Capability::= ENUMERATED {

ue-srvcc-not-supported  (0),

ue-srvcc-supported  (1),

...}

UpdateGprsLocationRes ::= SEQUENCE {

hlr-Number	ISDN-AddressString,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

add-Capability	NULL	OPTIONAL,

sgsn-mmeSeparationSupported	[0] NULL	OPTIONAL,

mmeRegisteredforSMS	[1] NULL	OPTIONAL }

EPLMN-List ::= SEQUENCE SIZE (1..50) OF

PLMN-Id

AdjacentPLMN-List ::= SEQUENCE SIZE (1..50) OF

PLMN-Id

-- handover types

ForwardAccessSignalling-Arg ::= [3] SEQUENCE {

an-APDU	AccessNetworkSignalInfo,

integrityProtectionInfo	[0] IntegrityProtectionInformation	OPTIONAL,

encryptionInfo	[1] EncryptionInformation	OPTIONAL,

keyStatus	[2]	KeyStatus	OPTIONAL,

allowedGSM-Algorithms	[4]	AllowedGSM-Algorithms	OPTIONAL,

allowedUMTS-Algorithms	[5]	AllowedUMTS-Algorithms	OPTIONAL,

radioResourceInformation	[6] RadioResourceInformation	OPTIONAL,

extensionContainer	[3]	ExtensionContainer	OPTIONAL,

...,

radioResourceList	[7]	RadioResourceList	OPTIONAL,

bssmap-ServiceHandover	[9]	BSSMAP-ServiceHandover	OPTIONAL,

ranap-ServiceHandover	[8]	RANAP-ServiceHandover	OPTIONAL,

bssmap-ServiceHandoverList	[10]	BSSMAP-ServiceHandoverList	OPTIONAL,

currentlyUsedCodec	[11] Codec	OPTIONAL,

iuSupportedCodecsList	[12] SupportedCodecsList	OPTIONAL,

rab-ConfigurationIndicator	[13] NULL	OPTIONAL,

iuSelectedCodec	[14]	Codec	OPTIONAL,

alternativeChannelType	[15]	RadioResourceInformation	OPTIONAL,

tracePropagationList	[17]	TracePropagationList	OPTIONAL,

aoipSupportedCodecsListAnchor	[18] AoIPCodecsList	OPTIONAL,

aoipSelectedCodecTarget	[19] AoIPCodec	OPTIONAL,

uesbi-Iu	[20]	UESBI-Iu	OPTIONAL,

imeisv	[21]	IMEI	OPTIONAL }

AllowedGSM-Algorithms ::= OCTET STRING (SIZE (1))

-- internal structure is coded as Algorithm identifier octet from

-- Permitted Algorithms defined in 3GPP TS 48.008

-- A node shall mark all GSM algorithms that are allowed in MSC-B

AllowedUMTS-Algorithms ::= SEQUENCE {

integrityProtectionAlgorithms	[0]	PermittedIntegrityProtectionAlgorithms	OPTIONAL,

encryptionAlgorithms	[1]	PermittedEncryptionAlgorithms	OPTIONAL,

extensionContainer	[2]	ExtensionContainer	OPTIONAL,

...}

PermittedIntegrityProtectionAlgorithms ::=

OCTET STRING (SIZE (1..maxPermittedIntegrityProtectionAlgorithmsLength))

-- Octets contain a complete PermittedIntegrityProtectionAlgorithms data type

-- as defined in 3GPP TS 25.413, encoded according to the encoding scheme

-- mandated by 3GPP TS 25.413.

-- Padding bits are included, if needed, in the least significant bits of the

-- last octet of the octet string.

maxPermittedIntegrityProtectionAlgorithmsLength INTEGER ::= 9

PermittedEncryptionAlgorithms ::=

OCTET STRING (SIZE (1..maxPermittedEncryptionAlgorithmsLength))

-- Octets contain a complete PermittedEncryptionAlgorithms data type

-- as defined in 3GPP TS 25.413, encoded according to the encoding scheme

-- mandated by 3GPP TS 25.413

-- Padding bits are included, if needed, in the least significant bits of the

-- last octet of the octet string.

maxPermittedEncryptionAlgorithmsLength INTEGER ::= 9

KeyStatus ::= ENUMERATED {

old  (0),

new  (1),

...}

-- exception handling:

-- received values in range 2-31 shall be treated as "old"

-- received values greater than 31 shall be treated as "new"

PrepareHO-Arg ::= [3] SEQUENCE {

targetCellId	[0] GlobalCellId	OPTIONAL,

ho-NumberNotRequired	NULL	OPTIONAL,

targetRNCId	[1] RNCId	OPTIONAL,

an-APDU	[2] AccessNetworkSignalInfo	OPTIONAL,

multipleBearerRequested	[3] NULL	OPTIONAL,

imsi	[4] IMSI	OPTIONAL,

integrityProtectionInfo	[5] IntegrityProtectionInformation	OPTIONAL,

encryptionInfo	[6] EncryptionInformation	OPTIONAL,

radioResourceInformation	[7] RadioResourceInformation	OPTIONAL,

allowedGSM-Algorithms	[9]	AllowedGSM-Algorithms	OPTIONAL,

allowedUMTS-Algorithms	[10]	AllowedUMTS-Algorithms	OPTIONAL,

radioResourceList	[11] RadioResourceList	OPTIONAL,

extensionContainer	[8] ExtensionContainer	OPTIONAL,

... ,

rab-Id	[12] RAB-Id	OPTIONAL,

bssmap-ServiceHandover	[13]	BSSMAP-ServiceHandover	OPTIONAL,

ranap-ServiceHandover	[14]	RANAP-ServiceHandover	OPTIONAL,

bssmap-ServiceHandoverList	[15]	BSSMAP-ServiceHandoverList	OPTIONAL,

asciCallReference	[20]	ASCI-CallReference	OPTIONAL,

geran-classmark	[16] GERAN-Classmark	OPTIONAL,

iuCurrentlyUsedCodec	[17] Codec	OPTIONAL,

iuSupportedCodecsList	[18] SupportedCodecsList	OPTIONAL,

rab-ConfigurationIndicator	[19] NULL	OPTIONAL,

uesbi-Iu	[21]	UESBI-Iu	OPTIONAL,

imeisv	[22]	IMEI	OPTIONAL,

alternativeChannelType	[23]	RadioResourceInformation	OPTIONAL,

tracePropagationList	[25]	TracePropagationList	OPTIONAL,

aoipSupportedCodecsListAnchor	[26] AoIPCodecsList	OPTIONAL,

regionalSubscriptionData	[27] ZoneCodeList	OPTIONAL,

lclsGlobalCallReference	[28]	LCLS-GlobalCallReference	OPTIONAL,

lcls-Negotiation	[29]	LCLS-Negotiation	OPTIONAL,

lcls-Configuration-Preference	[30]	LCLS-ConfigurationPreference	OPTIONAL,

csg-SubscriptionDataList	[31] CSG-SubscriptionDataList	OPTIONAL

}

LCLS-GlobalCallReference ::= OCTET STRING (SIZE (13..15))

-- Octets are coded as specified in 3GPP TS 29.205 [146]

LCLS-Negotiation::= BIT STRING {

permission-indicator-not-allowed-bit	(0),

permission-indicator-spare-bit	(1)} (SIZE (2..8))

--for definition and allowed combination of bits 0 and 1 see 3GPP TS 29.205

-- exception handling: bits 2 to 7 shall be ignored if received and not understood

LCLS-ConfigurationPreference::= BIT STRING {

forward-data-sending-indicator	(0),

backward-data-sending-indicator	(1),

forward-data-reception-indicator	(2),

backward-data-reception-indicator	(3)} (SIZE (4..8))

-- exception handling: bits 4 to 7 shall be ignored if received and not understood

BSSMAP-ServiceHandoverList ::= SEQUENCE SIZE (1.. maxNumOfServiceHandovers) OF

BSSMAP-ServiceHandoverInfo

BSSMAP-ServiceHandoverInfo ::= SEQUENCE {

bssmap-ServiceHandover	BSSMAP-ServiceHandover,

rab-Id	RAB-Id,

-- RAB Identity is needed to relate the service handovers with the radio access bearers.

...}

maxNumOfServiceHandovers  INTEGER ::= 7

BSSMAP-ServiceHandover ::= OCTET STRING (SIZE (1))

-- Octets are coded according the Service Handover information element in

-- 3GPP TS 48.008.

RANAP-ServiceHandover ::= OCTET STRING (SIZE (1))

-- Octet contains a complete Service-Handover data type

-- as defined in 3GPP TS 25.413, encoded according to the encoding scheme

-- mandated by 3GPP TS 25.413

-- Padding bits are included in the least significant bits.

RadioResourceList ::= SEQUENCE SIZE (1.. maxNumOfRadioResources) OF

RadioResource

RadioResource ::= SEQUENCE {

radioResourceInformation	RadioResourceInformation,

rab-Id	RAB-Id,

-- RAB Identity is needed to relate the radio resources with the radio access bearers.

...}

maxNumOfRadioResources  INTEGER ::= 7

PrepareHO-Res ::= [3] SEQUENCE {

handoverNumber	[0] ISDN-AddressString	OPTIONAL,

relocationNumberList	[1]	RelocationNumberList	OPTIONAL,

an-APDU	[2]	AccessNetworkSignalInfo	OPTIONAL,

multicallBearerInfo	[3]	MulticallBearerInfo	OPTIONAL,

multipleBearerNotSupported	NULL	OPTIONAL,

selectedUMTS-Algorithms	[5]	SelectedUMTS-Algorithms	OPTIONAL,

chosenRadioResourceInformation	[6] ChosenRadioResourceInformation	OPTIONAL,

extensionContainer	[4]	ExtensionContainer	OPTIONAL,

...,

iuSelectedCodec	[7] Codec	OPTIONAL,

iuAvailableCodecsList	[8] CodecList	OPTIONAL,

aoipSelectedCodecTarget	[9] AoIPCodec	OPTIONAL,

aoipAvailableCodecsListMap	[10] AoIPCodecsList	OPTIONAL }

SelectedUMTS-Algorithms ::= SEQUENCE {

integrityProtectionAlgorithm	[0]	ChosenIntegrityProtectionAlgorithm	OPTIONAL,

encryptionAlgorithm	[1]	ChosenEncryptionAlgorithm	OPTIONAL,

extensionContainer	[2]	ExtensionContainer	OPTIONAL,

...}

ChosenIntegrityProtectionAlgorithm ::= OCTET STRING (SIZE (1))

-- Octet contains a complete IntegrityProtectionAlgorithm data type

-- as defined in 3GPP TS 25.413, encoded according to the encoding scheme

-- mandated by 3GPP TS 25.413

-- Padding bits are included in the least significant bits.

ChosenEncryptionAlgorithm ::= OCTET STRING (SIZE (1))

-- Octet contains a complete EncryptionAlgorithm data type

-- as defined in 3GPP TS 25.413, encoded according to the encoding scheme

-- mandated by 3GPP TS 25.413

-- Padding bits are included in the least significant bits.

ChosenRadioResourceInformation ::= SEQUENCE {

chosenChannelInfo	[0] ChosenChannelInfo	OPTIONAL,

chosenSpeechVersion	[1] ChosenSpeechVersion	OPTIONAL,

...}

ChosenChannelInfo ::= OCTET STRING (SIZE (1))

-- Octets are coded according the Chosen Channel information element in 3GPP TS 48.008

ChosenSpeechVersion ::= OCTET STRING (SIZE (1))

-- Octets are coded according the Speech Version (chosen) information element in 3GPP TS

-- 48.008

PrepareSubsequentHO-Arg ::= [3] SEQUENCE {

targetCellId	[0] GlobalCellId	OPTIONAL,

targetMSC-Number	[1] ISDN-AddressString,

targetRNCId	[2] RNCId	OPTIONAL,

an-APDU	[3]	AccessNetworkSignalInfo	OPTIONAL,

selectedRab-Id	[4]	RAB-Id	OPTIONAL,

extensionContainer	[5]	ExtensionContainer	OPTIONAL,

...,

geran-classmark	[6] GERAN-Classmark	OPTIONAL,

rab-ConfigurationIndicator	[7] NULL	OPTIONAL }

PrepareSubsequentHO-Res ::= [3] SEQUENCE {

an-APDU	AccessNetworkSignalInfo,

extensionContainer	[0]	ExtensionContainer	OPTIONAL,

...}

ProcessAccessSignalling-Arg ::= [3] SEQUENCE {

an-APDU	AccessNetworkSignalInfo,

selectedUMTS-Algorithms	[1]	SelectedUMTS-Algorithms	OPTIONAL,

selectedGSM-Algorithm	[2]	SelectedGSM-Algorithm	OPTIONAL,

chosenRadioResourceInformation	[3] ChosenRadioResourceInformation OPTIONAL,

selectedRab-Id	[4] RAB-Id	OPTIONAL,

extensionContainer	[0]	ExtensionContainer	OPTIONAL,

...,

iUSelectedCodec	[5] Codec	OPTIONAL,

iuAvailableCodecsList	[6] CodecList	OPTIONAL,

aoipSelectedCodecTarget	[7] AoIPCodec	OPTIONAL,

aoipAvailableCodecsListMap	[8] AoIPCodecsList	OPTIONAL }

AoIPCodecsList ::= SEQUENCE {

codec1	[1] AoIPCodec,

codec2	[2] AoIPCodec	OPTIONAL,

codec3	[3] AoIPCodec	OPTIONAL,

codec4	[4] AoIPCodec	OPTIONAL,

codec5	[5] AoIPCodec	OPTIONAL,

codec6	[6] AoIPCodec	OPTIONAL,

codec7	[7] AoIPCodec	OPTIONAL,

codec8	[8] AoIPCodec	OPTIONAL,

extensionContainer	[9] ExtensionContainer	OPTIONAL,

...}

-- Codecs are sent in priority order where codec1 has highest priority

AoIPCodec ::= OCTET STRING (SIZE (1..3))

-- The internal structure is defined as follows:

-- octet 1	Coded as Speech Codec Elements in 3GPP TS 48.008

--	with the exception that FI, PI, PT and TF bits shall

--	be set to 0

-- octets 2,3	Optional; in case of AMR codec types it defines

--	the supported codec configurations as defined in

--	3GPP TS 48.008

SupportedCodecsList ::= SEQUENCE {

utranCodecList	[0] CodecList	OPTIONAL,

geranCodecList	[1] CodecList	OPTIONAL,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

...}

CodecList ::= SEQUENCE {

codec1	[1] Codec,

codec2	[2] Codec	OPTIONAL,

codec3	[3] Codec	OPTIONAL,

codec4	[4] Codec	OPTIONAL,

codec5	[5] Codec	OPTIONAL,

codec6	[6] Codec	OPTIONAL,

codec7	[7] Codec	OPTIONAL,

codec8	[8] Codec	OPTIONAL,

extensionContainer	[9] ExtensionContainer	OPTIONAL,

...}

-- Codecs are sent in priority order where codec1 has highest priority

Codec ::= OCTET STRING (SIZE (1..4))

-- The internal structure is defined as follows:

-- octet 1	Coded as Codec Identification code in 3GPP TS 26.103

-- octets 2,3,4	Parameters for the Codec as defined in 3GPP TS

--	26.103, if available, length depending on the codec

GERAN-Classmark ::= OCTET STRING (SIZE (2..87))

-- Octets are coded according the GERAN Classmark information element in 3GPP TS 48.008

SelectedGSM-Algorithm ::= OCTET STRING (SIZE (1))

-- internal structure is coded as Algorithm identifier octet from Chosen Encryption

-- Algorithm defined in 3GPP TS 48.008

-- A node shall mark only the selected GSM algorithm

SendEndSignal-Arg ::= [3] SEQUENCE {

an-APDU	AccessNetworkSignalInfo,

extensionContainer	[0]	ExtensionContainer	OPTIONAL,

...}

SendEndSignal-Res ::= SEQUENCE {

extensionContainer	[0]	ExtensionContainer	OPTIONAL,

...}

RNCId ::= OCTET STRING (SIZE (7))

-- The internal structure is defined as follows:

-- octet 1 bits 4321	Mobile Country Code 1st digit

--         bits 8765	Mobile Country Code 2nd digit

-- octet 2 bits 4321	Mobile Country Code 3rd digit

--         bits 8765	Mobile Network Code 3rd digit

--	or filler (1111) for 2 digit MNCs

-- octet 3 bits 4321	Mobile Network Code 1st digit

--         bits 8765	Mobile Network Code 2nd digit

-- octets 4 and 5	Location Area Code according to 3GPP TS 24.008

-- octets 6 and 7	RNC Id or Extended RNC Id value according to

--	3GPP TS 25.413

RelocationNumberList ::= SEQUENCE SIZE (1..maxNumOfRelocationNumber) OF

RelocationNumber

MulticallBearerInfo ::= INTEGER (1..maxNumOfRelocationNumber)

RelocationNumber ::= SEQUENCE {

handoverNumber	ISDN-AddressString,

rab-Id	RAB-Id,

-- RAB Identity is needed to relate the calls with the radio access bearers.

...}

RAB-Id ::= INTEGER (1..maxNrOfRABs)

maxNrOfRABs INTEGER ::= 255

maxNumOfRelocationNumber  INTEGER ::= 7

RadioResourceInformation ::= OCTET STRING (SIZE (3..13))

-- Octets are coded according the Channel Type information element in 3GPP TS 48.008

IntegrityProtectionInformation ::= OCTET STRING (SIZE (18..maxNumOfIntegrityInfo))

-- Octets contain a complete IntegrityProtectionInformation data type

-- as defined in 3GPP TS 25.413, encoded according to the encoding scheme

-- mandated by 3GPP TS 25.413

-- Padding bits are included, if needed, in the least significant bits of the

-- last octet of the octet string.

maxNumOfIntegrityInfo INTEGER ::= 100

EncryptionInformation ::= OCTET STRING (SIZE (18..maxNumOfEncryptionInfo))

-- Octets contain a complete EncryptionInformation data type

-- as defined in 3GPP TS 25.413, encoded according to the encoding scheme

-- mandated by 3GPP TS 25.413

-- Padding bits are included, if needed, in the least significant bits of the

-- last octet of the octet string.

maxNumOfEncryptionInfo INTEGER ::= 100

-- authentication management types

SendAuthenticationInfoArg ::= SEQUENCE {

imsi	[0] IMSI,

numberOfRequestedVectors	NumberOfRequestedVectors,

segmentationProhibited	NULL	OPTIONAL,

immediateResponsePreferred	[1] NULL	OPTIONAL,

re-synchronisationInfo	Re-synchronisationInfo	OPTIONAL,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

...,

requestingNodeType	[3] RequestingNodeType	OPTIONAL,

requestingPLMN-Id	[4] PLMN-Id	OPTIONAL,

numberOfRequestedAdditional-Vectors	[5] NumberOfRequestedVectors	OPTIONAL,

additionalVectorsAreForEPS	[6] NULL	OPTIONAL,

ueUsageTypeRequestIndication	[7] NULL	OPTIONAL }

NumberOfRequestedVectors ::= INTEGER (1..5)

Re-synchronisationInfo ::= SEQUENCE {

rand	RAND,

auts	AUTS,

...}

SendAuthenticationInfoRes ::= [3] SEQUENCE {

authenticationSetList	AuthenticationSetList	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

eps-AuthenticationSetList	[2] EPS-AuthenticationSetList	OPTIONAL,

ueUsageType	[3] UE-UsageType	OPTIONAL }

EPS-AuthenticationSetList ::= SEQUENCE SIZE (1..5) OF

EPC-AV

UE-UsageType::= OCTET STRING (SIZE (4))

-- octets are coded as defined in 3GPP TS 29.272 [144]

EPC-AV ::= SEQUENCE {

rand	RAND,

xres	XRES,

autn	AUTN,

kasme	KASME,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

KASME ::= OCTET STRING (SIZE (32))

RequestingNodeType ::= ENUMERATED {

vlr  (0),

sgsn  (1),

...,

s-cscf  (2),

bsf  (3),

gan-aaa-server  (4),

wlan-aaa-server  (5),

mme	(16),

mme-sgsn	(17)

}

-- the values 2, 3, 4 and 5 shall not be used on the MAP-D or Gr interfaces

-- exception handling:

-- received values in the range (6-15) shall be treated as "vlr"

-- received values greater than 17 shall be treated as "sgsn"

-- equipment management types

CheckIMEI-Arg ::= SEQUENCE {

imei	IMEI,

requestedEquipmentInfo	RequestedEquipmentInfo,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

CheckIMEI-Res ::= SEQUENCE {

equipmentStatus	EquipmentStatus	OPTIONAL,

bmuef	UESBI-Iu	OPTIONAL,

extensionContainer	[0] ExtensionContainer	OPTIONAL,

...}

RequestedEquipmentInfo::= BIT STRING {

equipmentStatus  (0),

bmuef  (1)} (SIZE (2..8))

-- exception handling: reception of unknown bit assignments in the

-- RequestedEquipmentInfo data type shall be discarded by the receiver

UESBI-Iu ::= SEQUENCE {

uesbi-IuA	[0] UESBI-IuA	OPTIONAL,

uesbi-IuB	[1] UESBI-IuB	OPTIONAL,

...}

UESBI-IuA	::= BIT STRING (SIZE(1..128))

-- See 3GPP TS 25.413

UESBI-IuB	::= BIT STRING (SIZE(1..128))

-- See 3GPP TS 25.413

EquipmentStatus ::= ENUMERATED {

permittedListed  (0),

prohibitedListed  (1),

trackingListed  (2)}

-- subscriber management types

InsertSubscriberDataArg ::= SEQUENCE {

imsi	[0] IMSI	OPTIONAL,

COMPONENTS OF	SubscriberData,

extensionContainer	[14] ExtensionContainer	OPTIONAL,

... ,

naea-PreferredCI	[15] NAEA-PreferredCI	OPTIONAL,

-- naea-PreferredCI is included at the discretion of the HLR operator.

gprsSubscriptionData	[16] GPRSSubscriptionData	OPTIONAL,

roamingRestrictedInSgsnDueToUnsupportedFeature [23]	NULL

OPTIONAL,

networkAccessMode	[24] NetworkAccessMode	OPTIONAL,

lsaInformation	[25] LSAInformation	OPTIONAL,

lmu-Indicator	[21]	NULL	OPTIONAL,

lcsInformation	[22]	LCSInformation	OPTIONAL,

istAlertTimer	[26] IST-AlertTimerValue	OPTIONAL,

superChargerSupportedInHLR	[27] AgeIndicator	OPTIONAL,

mc-SS-Info	[28] MC-SS-Info	OPTIONAL,

cs-AllocationRetentionPriority	[29] CS-AllocationRetentionPriority	OPTIONAL,

sgsn-CAMEL-SubscriptionInfo	[17] SGSN-CAMEL-SubscriptionInfo	OPTIONAL,

chargingCharacteristics	[18]	ChargingCharacteristics	OPTIONAL,

accessRestrictionData	[19] AccessRestrictionData	OPTIONAL,

ics-Indicator	[20]	BOOLEAN	OPTIONAL,

eps-SubscriptionData	[31]	EPS-SubscriptionData	OPTIONAL,

csg-SubscriptionDataList	[32] CSG-SubscriptionDataList	OPTIONAL,

ue-ReachabilityRequestIndicator	[33]	NULL	OPTIONAL,

sgsn-Number	[34]	ISDN-AddressString	OPTIONAL,

mme-Name	[35]	DiameterIdentity	OPTIONAL,

subscribedPeriodicRAUTAUtimer	[36]	SubscribedPeriodicRAUTAUtimer	OPTIONAL,

vplmnLIPAAllowed	[37]	NULL	OPTIONAL,

mdtUserConsent	[38]	BOOLEAN	OPTIONAL,

subscribedPeriodicLAUtimer	[39]	SubscribedPeriodicLAUtimer	OPTIONAL,

vplmn-Csg-SubscriptionDataList	[40]	VPLMN-CSG-SubscriptionDataList	OPTIONAL,

additionalMSISDN	[41]	ISDN-AddressString	OPTIONAL,

psAndSMS-OnlyServiceProvision	[42]	NULL	OPTIONAL,

smsInSGSNAllowed	[43]	NULL	OPTIONAL,

cs-to-ps-SRVCC-Allowed-Indicator	[44]	NULL	OPTIONAL,

pcscf-Restoration-Request	[45]	NULL	OPTIONAL,

adjacentAccessRestrictionDataList	[46] AdjacentAccessRestrictionDataList	OPTIONAL,

imsi-Group-Id-List	[47] IMSI-GroupIdList	OPTIONAL,

ueUsageType	[48] UE-UsageType	OPTIONAL,

userPlaneIntegrityProtectionIndicator	[49] NULL	OPTIONAL,

dl-Buffering-Suggested-Packet-Count	[50]	DL-Buffering-Suggested-Packet-Count	OPTIONAL,

reset-Id-List	[51]	Reset-Id-List	OPTIONAL,

eDRX-Cycle-Length-List	[52] EDRX-Cycle-Length-List	OPTIONAL,

ext-AccessRestrictionData	[53] Ext-AccessRestrictionData	OPTIONAL,

iab-Operation-Allowed-Indicator	[54]	NULL	OPTIONAL }

-- If the Network Access Mode parameter is sent, it shall be present only in

-- the first sequence if seqmentation is used

EDRX-Cycle-Length-List ::= SEQUENCE SIZE (1..8) OF

EDRX-Cycle-Length

EDRX-Cycle-Length ::= SEQUENCE {

rat-Type	[0]	Used-RAT-Type,

eDRX-Cycle-Length-Value	[1]	EDRX-Cycle-Length-Value,

...}

-- The eDRX-Cycle-Length contains the subscribed eDRX-Cycle-Length applicable to a

-- a specific RAT Type.

EDRX-Cycle-Length-Value ::= OCTET STRING (SIZE (1))

-- The EDRX-Cycle-Length-Value shall be encoded as specified in clause 7.3.216 of

-- 3GPP TS 29.272 [144].

Reset-Id-List ::= SEQUENCE SIZE (1..50) OF

Reset-Id

Reset-Id ::= OCTET STRING (SIZE (1..4))

-- Reset-Ids shall be unique within the HPLMN.

DL-Buffering-Suggested-Packet-Count ::= INTEGER (-1..2147483647)

-- values are defined in 3GPP TS 29.272 [144]

Group-Service-ID ::= INTEGER (0..4294967295)

-- values are defined in 3GPP TS 29.272 [144]

Local-GroupID ::= OCTET STRING (SIZE (1..10))

-- Refers to Local group ID defined by an operator identified by the PLMN-ID.

--	for details see 3GPP TS 29.272 [144]

IMSI-GroupIdList ::= SEQUENCE SIZE (1..50) OF

IMSI-GroupId

IMSI-GroupId ::= SEQUENCE {

group-Service-Id	[0]	Group-Service-ID,

plmnId	[1]	PLMN-Id,

local-Group-ID	[2]	Local-GroupID,

...}

SubscribedPeriodicRAUTAUtimer ::= INTEGER (0..4294967295)

-- This parameter carries the subscribed periodic TAU/RAU timer value in seconds as

-- specified in 3GPP TS 24.008 [35].

SubscribedPeriodicLAUtimer ::= INTEGER (0..4294967295)

-- This parameter carries the subscribed periodic LAU timer value in seconds as

-- specified in 3GPP TS 24.008 [35].

CSG-SubscriptionDataList ::= SEQUENCE SIZE (1..50) OF

CSG-SubscriptionData

CSG-SubscriptionData ::= SEQUENCE {

csg-Id	CSG-Id,

expirationDate	Time	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

lipa-AllowedAPNList	[0] LIPA-AllowedAPNList	OPTIONAL,

plmn-Id	[1] PLMN-Id	OPTIONAL

}

VPLMN-CSG-SubscriptionDataList ::= SEQUENCE SIZE (1..50) OF

CSG-SubscriptionData

CSG-Id ::= BIT STRING (SIZE (27))

-- coded according to 3GPP TS 23.003 [17].

LIPA-AllowedAPNList ::= SEQUENCE SIZE (1..maxNumOfLIPAAllowedAPN) OF

APN

maxNumOfLIPAAllowedAPN  INTEGER ::= 50

EPS-SubscriptionData ::= SEQUENCE {

apn-oi-Replacement	[0]	APN-OI-Replacement	OPTIONAL,

-- this apn-oi-Replacement refers to the UE level apn-oi-Replacement.

rfsp-id	[2]	RFSP-ID	OPTIONAL,

ambr	[3]	AMBR	OPTIONAL,

apn-ConfigurationProfile	[4]	APN-ConfigurationProfile	OPTIONAL,

stn-sr	[6]	ISDN-AddressString	OPTIONAL,

extensionContainer	[5]	ExtensionContainer	OPTIONAL,

...,

mps-CSPriority	[7]	NULL	OPTIONAL,

mps-EPSPriority	[8]	NULL	OPTIONAL,

subscribed-vsrvcc	[9]	NULL	OPTIONAL }

-- mps-CSPriority by its presence indicates that the UE is subscribed to the eMLPP in

-- the CS domain, referring to the 3GPP TS 29.272 [144] for details.

-- mps-EPSPriority by its presence indicates that the UE is subscribed to the MPS in

-- the EPS domain, referring to the 3GPP TS 29.272 [144] for details.

--

-- subscribed-vsrvcc by its presence indicates that the UE is subscribed to the vSRVCC in

-- the EPS domain, referring to the 3GPP TS 29.272 [144] for details.

APN-OI-Replacement ::=  OCTET STRING (SIZE (9..100))

-- Octets are coded as APN Operator Identifier according to TS 3GPP TS 23.003 [17]

RFSP-ID ::=  INTEGER (1..256)

APN-ConfigurationProfile ::= SEQUENCE {

defaultContext	ContextId,

completeDataListIncluded	NULL	OPTIONAL,

-- If segmentation is used, completeDataListIncluded may only be present in the

-- first segment of APN-ConfigurationProfile.

epsDataList	[1]	EPS-DataList,

extensionContainer	[2]	ExtensionContainer	OPTIONAL,

... ,

additionalDefaultContext	[3]	ContextId	OPTIONAL

--	for details see the 3GPP TS 29.272 [144].

}

EPS-DataList ::= SEQUENCE SIZE (1..maxNumOfAPN-Configurations) OF

APN-Configuration

maxNumOfAPN-Configurations  INTEGER ::= 50

APN-Configuration ::= SEQUENCE {

contextId	[0] ContextId,

pdn-Type	[1] PDN-Type,

servedPartyIP-IPv4-Address	[2] PDP-Address	OPTIONAL,

apn	[3] APN,

eps-qos-Subscribed	[4] EPS-QoS-Subscribed,

pdn-gw-Identity	[5] PDN-GW-Identity	OPTIONAL,

pdn-gw-AllocationType	[6] PDN-GW-AllocationType	OPTIONAL,

vplmnAddressAllowed	[7] NULL	OPTIONAL,

chargingCharacteristics	[8] ChargingCharacteristics	OPTIONAL,

ambr	[9] AMBR	OPTIONAL,

specificAPNInfoList	[10] SpecificAPNInfoList	OPTIONAL,	extensionContainer	[11] ExtensionContainer	OPTIONAL,

servedPartyIP-IPv6-Address	[12] PDP-Address	OPTIONAL,

...,

apn-oi-Replacement	[13] APN-OI-Replacement	OPTIONAL,

-- this apn-oi-Replacement refers to the APN level apn-oi-Replacement.

sipto-Permission	[14] SIPTO-Permission	OPTIONAL,

lipa-Permission	[15] LIPA-Permission	OPTIONAL,

restoration-Priority	[16] Restoration-Priority	OPTIONAL,

sipto-local-network-Permission	[17] SIPTO-Local-Network-Permission	OPTIONAL,

wlan-offloadability	[18] WLAN-Offloadability	OPTIONAL,

non-IP-PDN-Type-Indicator	[19]	NULL	OPTIONAL,

nIDD-Mechanism	[20]	NIDD-Mechanism	OPTIONAL,

sCEF-ID	[21]	FQDN	OPTIONAL,

pdn-ConnectionContinuity	[22]	PDN-ConnectionContinuity	OPTIONAL

-- absence of pdn-ConnectionContinuity indicates that the handling is left to

-- local VPLMN policy

}

PDN-ConnectionContinuity ::= ENUMERATED {

maintainPDN-Connection	(0),

disconnectPDN-ConnectionWithReactivationRequest	(1),

disconnectPDN-ConnectionWithoutReactivationRequest	(2)

}

NIDD-Mechanism ::= ENUMERATED {

sGi-based-data-delivery	(0),

sCEF-based-data-delivery	(1)

-- The default value, when this information element is not present, is

-- sGi-based-data-delivery (0)

}

PDN-Type ::= OCTET STRING (SIZE (1))

-- Octet is coded  as follows:

--	Bits

--	3 2 1

--	0 0 1 IPv4

--	0 1 0 IPv6

--	0 1 1 IPv4v6

--	1 0 0 IPv4_or_IPv6

--	Bits 8-4 shall be coded as zero.

--	for details see 3GPP TS 29.272 [144]

EPS-QoS-Subscribed ::= SEQUENCE {

qos-Class-Identifier	[0] QoS-Class-Identifier,

allocation-Retention-Priority	[1] Allocation-Retention-Priority,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

... }

AMBR ::= SEQUENCE {

max-RequestedBandwidth-UL	[0] Bandwidth,

max-RequestedBandwidth-DL	[1] Bandwidth,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

...,

extended-Max-RequestedBandwidth-UL	[3] BandwidthExt	OPTIONAL,

extended-Max-RequestedBandwidth-DL	[4] BandwidthExt	OPTIONAL

-- extended-Max-RequestedBandwidth-UL/DL shall be populated according to the

-- description of the corresponding parameters in 3GPP TS 29.272 [144]

}

SpecificAPNInfoList ::= SEQUENCE SIZE (1..maxNumOfSpecificAPNInfos) OF

SpecificAPNInfo

maxNumOfSpecificAPNInfos  INTEGER ::= 50

SpecificAPNInfo ::= SEQUENCE {

apn	[0] APN,

pdn-gw-Identity	[1] PDN-GW-Identity,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

... }

Bandwidth ::= INTEGER

-- bits per second

BandwidthExt ::= INTEGER

-- kilobits per second

QoS-Class-Identifier ::= INTEGER (1..9)

-- values are defined in  3GPP TS 29.212

Allocation-Retention-Priority ::= SEQUENCE {

priority-level	[0] INTEGER,

pre-emption-capability	[1] BOOLEAN	OPTIONAL,

pre-emption-vulnerability	[2] BOOLEAN	OPTIONAL,

extensionContainer	[3] ExtensionContainer	OPTIONAL,

... }

PDN-GW-Identity ::= SEQUENCE {

pdn-gw-ipv4-Address	[0] PDP-Address	OPTIONAL,

pdn-gw-ipv6-Address	[1] PDP-Address	OPTIONAL,

pdn-gw-name	[2] FQDN	OPTIONAL,

extensionContainer	[3] ExtensionContainer	OPTIONAL,

... }

FQDN ::=  OCTET STRING (SIZE (9..255))

PDN-GW-AllocationType ::= ENUMERATED {

static	(0),

dynamic	(1)}

WLAN-Offloadability ::= SEQUENCE {

wlan-offloadability-EUTRAN	[0] WLAN-Offloadability-Indication 	OPTIONAL,

wlan-offloadability-UTRAN	[1] WLAN-Offloadability-Indication	OPTIONAL,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

... }

WLAN-Offloadability-Indication ::= ENUMERATED {

notAllowed   (0),

allowed      (1)}

AccessRestrictionData ::= BIT STRING {

utranNotAllowed (0),

geranNotAllowed (1),

ganNotAllowed   (2),

i-hspa-evolutionNotAllowed (3),

wb-e-utranNotAllowed (4),

ho-toNon3GPP-AccessNotAllowed (5),

nb-iotNotAllowed (6),

enhancedCoverageNotAllowed (7) } (SIZE (2..8))

-- exception handling:

-- The VLR shall ignore the access restriction data related to an access type not

-- supported by the node.

-- The handling of the access restriction data by the SGSN is described in clause

-- 5.3.19 of TS 23.060, in clause 7.5.3 of TS 29.060 and clause 7.3.6 of TS 29.274.

-- Additional access restrictions are encoded in Ext-AccessRestrictionData bit string.

Ext-AccessRestrictionData ::= BIT STRING {

nrAsSecondaryRATNotAllowed (0),

unlicensedSpectrumAsSecondaryRATNotAllowed (1) } (SIZE (1..32))

AdjacentAccessRestrictionDataList ::= SEQUENCE SIZE (1..50) OF

AdjacentAccessRestrictionData

AdjacentAccessRestrictionData ::= SEQUENCE {

plmnId	[0]	PLMN-Id,

accessRestrictionData	[1]	AccessRestrictionData,

... ,

ext-AccessRestrictionData	[2]	Ext-AccessRestrictionData	OPTIONAL }

CS-AllocationRetentionPriority ::= OCTET STRING (SIZE (1))

-- This data type encodes each priority level defined in 3GPP TS 23.107 [154] as the

--binary value of the priority level.

IST-AlertTimerValue ::= INTEGER (15..255)

LCSInformation ::= SEQUENCE {

gmlc-List	[0]	GMLC-List	OPTIONAL,

lcs-PrivacyExceptionList	[1]	LCS-PrivacyExceptionList	OPTIONAL,

molr-List	[2]	MOLR-List	OPTIONAL,

...,

add-lcs-PrivacyExceptionList	[3]	LCS-PrivacyExceptionList	OPTIONAL }

-- add-lcs-PrivacyExceptionList may be sent only if lcs-PrivacyExceptionList is

-- present and contains four instances of LCS-PrivacyClass. If the mentioned condition

-- is not satisfied the receiving node shall discard add-lcs-PrivacyExceptionList.

-- If an LCS-PrivacyClass is received both in lcs-PrivacyExceptionList and in

-- add-lcs-PrivacyExceptionList with the same SS-Code, then the error unexpected

-- data value shall be returned.

GMLC-List ::= SEQUENCE SIZE (1..maxNumOfGMLC) OF

ISDN-AddressString

-- if segmentation is used, the complete GMLC-List shall be sent in one segment

maxNumOfGMLC  INTEGER ::= 5

NetworkAccessMode ::= ENUMERATED {

packetAndCircuit	(0),

onlyCircuit	(1),

onlyPacket	(2),

...}

-- if unknown values are received in NetworkAccessMode

-- they shall be discarded.

GPRSDataList ::= SEQUENCE SIZE (1..maxNumOfPDP-Contexts) OF

PDP-Context

maxNumOfPDP-Contexts  INTEGER ::= 50

PDP-Context ::= SEQUENCE {

pdp-ContextId	ContextId,

pdp-Type	[16] PDP-Type,

pdp-Address	[17] PDP-Address	OPTIONAL,

qos-Subscribed	[18] QoS-Subscribed,

vplmnAddressAllowed	[19] NULL	OPTIONAL,

apn	[20] APN,

extensionContainer	[21] ExtensionContainer	OPTIONAL,

... ,

ext-QoS-Subscribed	[0] Ext-QoS-Subscribed	OPTIONAL,

pdp-ChargingCharacteristics	[1] ChargingCharacteristics	OPTIONAL,

ext2-QoS-Subscribed	[2] Ext2-QoS-Subscribed	OPTIONAL,

-- ext2-QoS-Subscribed may be present only if ext-QoS-Subscribed is present.

ext3-QoS-Subscribed	[3] Ext3-QoS-Subscribed	OPTIONAL,

-- ext3-QoS-Subscribed may be present only if ext2-QoS-Subscribed is present.

ext4-QoS-Subscribed	[4] Ext4-QoS-Subscribed	OPTIONAL,

-- ext4-QoS-Subscribed may be present only if ext3-QoS-Subscribed is present.

apn-oi-Replacement	[5]	APN-OI-Replacement	OPTIONAL,

-- this apn-oi-Replacement refers to the APN level apn-oi-Replacement and has

-- higher priority than UE level apn-oi-Replacement.

ext-pdp-Type	[6] Ext-PDP-Type	OPTIONAL,

-- contains the value IPv4v6 defined in 3GPP TS 29.060 [105], if the PDP can be

-- accessed by dual-stack UEs

ext-pdp-Address	[7] PDP-Address	OPTIONAL,

-- contains an additional IP address in case of dual-stack static IP address assignment

-- for the UE.

-- it may contain an IPv4 or an IPv6 address/prefix, and it may be present

-- only if pdp-Address is present; if both are present, each parameter shall

-- contain a different type of address (IPv4 or IPv6).

ambr	[10] AMBR	OPTIONAL,

-- this ambr contains the AMBR associated to the APN included in the

-- PDP-Context (APN-AMBR).

sipto-Permission	[8] SIPTO-Permission	OPTIONAL,

lipa-Permission	[9] LIPA-Permission	OPTIONAL,

restoration-Priority	[11] Restoration-Priority	OPTIONAL,

sipto-local-network-Permission	[12] SIPTO-Local-Network-Permission	OPTIONAL,

nIDD-Mechanism	[13]	NIDD-Mechanism	OPTIONAL,

sCEF-ID	[14]	FQDN	OPTIONAL

}

Restoration-Priority ::= OCTET STRING (SIZE (1))

-- Octet 1:

--  Restoration Priority. This octet encodes the Restoration Priority,

--  as the binary value of the Restoration-Priority described in 3GPP TS 29.272 [144].

SIPTO-Permission ::= ENUMERATED {

siptoAboveRanAllowed  (0),

siptoAboveRanNotAllowed  (1)

}

SIPTO-Local-Network-Permission ::= ENUMERATED {

siptoAtLocalNetworkAllowed  (0),

siptoAtLocalNetworkNotAllowed  (1)

}

LIPA-Permission ::= ENUMERATED {

lipaProhibited  (0),

lipaOnly  (1),

lipaConditional  (2)

}

ContextId ::= INTEGER (1..maxNumOfPDP-Contexts)

GPRSSubscriptionData ::= SEQUENCE {

completeDataListIncluded	NULL	OPTIONAL,

-- If segmentation is used, completeDataListIncluded may only be present in the

-- first segment of GPRSSubscriptionData.

gprsDataList	[1]	GPRSDataList,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

...,

apn-oi-Replacement	[3]	APN-OI-Replacement	OPTIONAL

-- this apn-oi-Replacement refers to the UE level apn-oi-Replacement.

}

# SGSN-CAMEL-SubscriptionInfo ::= SEQUENCE {

gprs-CSI	[0]	GPRS-CSI	OPTIONAL,

mo-sms-CSI	[1]	SMS-CSI	OPTIONAL,

extensionContainer	[2]	ExtensionContainer	OPTIONAL,

...,

mt-sms-CSI	[3]	SMS-CSI	OPTIONAL,

mt-smsCAMELTDP-CriteriaList	[4]	MT-smsCAMELTDP-CriteriaList	OPTIONAL,

mg-csi	[5]	MG-CSI	OPTIONAL

}

# GPRS-CSI ::= SEQUENCE {

gprs-CamelTDPDataList	[0] GPRS-CamelTDPDataList	OPTIONAL,

camelCapabilityHandling	[1] CamelCapabilityHandling	OPTIONAL,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

notificationToCSE	[3]	NULL	OPTIONAL,

csi-Active	[4]	NULL	OPTIONAL,

...}

--	notificationToCSE and csi-Active shall not be present when GPRS-CSI is sent to SGSN.

--	They may only be included in ATSI/ATM ack/NSDC message.

--	GPRS-CamelTDPData and  camelCapabilityHandling shall be present in

--	the GPRS-CSI sequence.

--	If GPRS-CSI is segmented, gprs-CamelTDPDataList and camelCapabilityHandling shall be

--	present in the first segment

# GPRS-CamelTDPDataList ::= SEQUENCE SIZE (1..maxNumOfCamelTDPData) OF

# GPRS-CamelTDPData

--	GPRS-CamelTDPDataList shall not contain more than one instance of

--	GPRS-CamelTDPData containing the same value for gprs-TriggerDetectionPoint.

# GPRS-CamelTDPData ::= SEQUENCE {

gprs-TriggerDetectionPoint	[0] GPRS-TriggerDetectionPoint,

serviceKey	[1] ServiceKey,

gsmSCF-Address	[2] ISDN-AddressString,

defaultSessionHandling	[3] DefaultGPRS-Handling,

extensionContainer	[4] ExtensionContainer	OPTIONAL,

...

}

# DefaultGPRS-Handling ::= ENUMERATED {

continueTransaction (0) ,

releaseTransaction (1) ,

...}

-- exception handling:

-- reception of values in range 2-31 shall be treated as "continueTransaction"

-- reception of values greater than 31 shall be treated as "releaseTransaction"

# GPRS-TriggerDetectionPoint ::= ENUMERATED {

attach	(1),

attachChangeOfPosition	(2),

pdp-ContextEstablishment	(11),

pdp-ContextEstablishmentAcknowledgement	(12),

pdp-ContextChangeOfPosition	(14),

... }

-- exception handling:

-- For GPRS-CamelTDPData sequences containing this parameter with any

-- other value than the ones listed the receiver shall ignore the whole

-- GPRS-CamelTDPDatasequence.

APN ::=  OCTET STRING (SIZE (2..63))

-- Octets are coded according to TS 3GPP TS 23.003 [17]

PDP-Type ::= OCTET STRING (SIZE (2))

-- Octets are coded according to TS 3GPP TS 29.060 [105]

-- Only the values PPP, IPv4, IPv6 and Non-IP are allowed for this parameter.

Ext-PDP-Type ::= OCTET STRING (SIZE (2))

-- Octets are coded, similarly to PDP-Type, according to TS 3GPP TS 29.060 [105].

-- Only the value IPv4v6 is allowed for this parameter.

PDP-Address ::= OCTET STRING (SIZE (1..16))

-- Octets are coded according to TS 3GPP TS 29.060 [105]

-- The possible size values are:

-- 1-7 octets  X.25 address type

--  4  octets  IPv4 address type

-- 16  octets  Ipv6 address type

QoS-Subscribed ::= OCTET STRING (SIZE (3))

-- Octets are coded according to TS 3GPP TS 24.008 [35] Quality of Service Octets

-- 3-5.

Ext-QoS-Subscribed ::= OCTET STRING (SIZE (1..9))

-- OCTET 1:

--  Allocation/Retention Priority (This octet encodes each priority level defined in

--     3GPP TS 23.107 [154] as the binary value of the priority level, declaration in

--     3GPP TS 29.060 [105])

-- Octets 2-9 are coded according to 3GPP TS 24.008 [35] Quality of Service Octets

-- 6-13.

Ext2-QoS-Subscribed ::= OCTET STRING (SIZE (1..3))

-- Octets 1-3 are coded according to 3GPP TS 24.008 [35] Quality of Service Octets 14-16.

-- If Quality of Service information is structured with 14 octet length, then

-- Octet 1 is coded according to 3GPP TS 24.008 [35] Quality of Service Octet 14.

Ext3-QoS-Subscribed ::= OCTET STRING (SIZE (1..2))

-- Octets 1-2 are coded according to 3GPP TS 24.008 [35] Quality of Service Octets 17-18.

Ext4-QoS-Subscribed ::= OCTET STRING (SIZE (1))

-- Octet 1:

--  Evolved Allocation/Retention Priority. This octet encodes the Priority Level (PL),

--  the Preemption Capability (PCI) and Preemption Vulnerability  (PVI) values, as

--  described in 3GPP TS 29.060 [105].

ChargingCharacteristics ::= OCTET STRING (SIZE (2))

-- Octets are coded according to 3GPP TS 32.215.

LSAOnlyAccessIndicator ::= ENUMERATED {

accessOutsideLSAsAllowed  (0),

accessOutsideLSAsRestricted (1)}

LSADataList ::= SEQUENCE SIZE (1..maxNumOfLSAs) OF

LSAData

maxNumOfLSAs  INTEGER ::= 20

LSAData ::= SEQUENCE {

lsaIdentity	[0] LSAIdentity,

lsaAttributes	[1] LSAAttributes,

lsaActiveModeIndicator	[2] NULL	OPTIONAL,

extensionContainer	[3] ExtensionContainer	OPTIONAL,

...}

LSAInformation ::= SEQUENCE {

completeDataListIncluded	NULL	OPTIONAL,

-- If segmentation is used, completeDataListIncluded may only be present in the

-- first segment.

lsaOnlyAccessIndicator	[1]	LSAOnlyAccessIndicator	OPTIONAL,

lsaDataList	[2]	LSADataList	OPTIONAL,

extensionContainer	[3] ExtensionContainer	OPTIONAL,

...}

LSAIdentity ::= OCTET STRING (SIZE (3))

-- Octets are coded according to TS 3GPP TS 23.003 [17]

LSAAttributes ::= OCTET STRING (SIZE (1))

-- Octets are coded according to TS 3GPP TS 48.008 [49]

SubscriberData ::= SEQUENCE {

msisdn	[1] ISDN-AddressString	OPTIONAL,

category	[2] Category	OPTIONAL,

subscriberStatus	[3] SubscriberStatus	OPTIONAL,

bearerServiceList	[4] BearerServiceList	OPTIONAL,

-- The exception handling for reception of unsupported / not allocated

-- bearerServiceCodes is defined in clause 8.8.1

teleserviceList	[6] TeleserviceList	OPTIONAL,

-- The exception handling for reception of unsupported / not allocated

-- teleserviceCodes is defined in clause 8.8.1

provisionedSS	[7] Ext-SS-InfoList	OPTIONAL,

odb-Data	[8] ODB-Data	OPTIONAL,

roamingRestrictionDueToUnsupportedFeature  [9] NULL	OPTIONAL,

regionalSubscriptionData	[10] ZoneCodeList	OPTIONAL,

vbsSubscriptionData	[11] VBSDataList	OPTIONAL,

vgcsSubscriptionData	[12] VGCSDataList	OPTIONAL,

vlrCamelSubscriptionInfo	[13] VlrCamelSubscriptionInfo	OPTIONAL

}

Category ::= OCTET STRING (SIZE (1))

-- The internal structure is defined in ITU-T Rec Q.763.

SubscriberStatus ::= ENUMERATED {

serviceGranted  (0),

operatorDeterminedBarring  (1)}

BearerServiceList ::= SEQUENCE SIZE (1..maxNumOfBearerServices) OF

Ext-BearerServiceCode

maxNumOfBearerServices  INTEGER ::= 50

TeleserviceList ::= SEQUENCE SIZE (1..maxNumOfTeleservices) OF

Ext-TeleserviceCode

maxNumOfTeleservices  INTEGER ::= 20

ODB-Data ::= SEQUENCE {

odb-GeneralData	ODB-GeneralData,

odb-HPLMN-Data	ODB-HPLMN-Data	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

ODB-GeneralData ::= BIT STRING {

allOG-CallsBarred  (0),

internationalOGCallsBarred  (1),

internationalOGCallsNotToHPLMN-CountryBarred  (2),

interzonalOGCallsBarred (6),

interzonalOGCallsNotToHPLMN-CountryBarred (7),

interzonalOGCallsAndInternationalOGCallsNotToHPLMN-CountryBarred (8),

premiumRateInformationOGCallsBarred  (3),

premiumRateEntertainementOGCallsBarred  (4),

ss-AccessBarred  (5),

allECT-Barred (9),

chargeableECT-Barred (10),

internationalECT-Barred (11),

interzonalECT-Barred (12),

doublyChargeableECT-Barred (13),

multipleECT-Barred (14),

allPacketOrientedServicesBarred (15),

roamerAccessToHPLMN-AP-Barred  (16),

roamerAccessToVPLMN-AP-Barred  (17),

roamingOutsidePLMNOG-CallsBarred  (18),

allIC-CallsBarred  (19),

roamingOutsidePLMNIC-CallsBarred  (20),

roamingOutsidePLMNICountryIC-CallsBarred  (21),

roamingOutsidePLMN-Barred  (22),

roamingOutsidePLMN-CountryBarred  (23),

registrationAllCF-Barred  (24),

registrationCFNotToHPLMN-Barred  (25),

registrationInterzonalCF-Barred  (26),

registrationInterzonalCFNotToHPLMN-Barred  (27),

registrationInternationalCF-Barred  (28)} (SIZE (15..32))

-- exception handling: reception of unknown bit assignments in the

-- ODB-GeneralData type shall be treated like unsupported ODB-GeneralData

-- When the ODB-GeneralData type is removed from the HLR for a given subscriber,

-- in NoteSubscriberDataModified operation sent toward the gsmSCF

-- all bits shall be set to "O".

ODB-HPLMN-Data ::= BIT STRING {

plmn-SpecificBarringType1  (0),

plmn-SpecificBarringType2  (1),

plmn-SpecificBarringType3  (2),

plmn-SpecificBarringType4  (3)} (SIZE (4..32))

-- exception handling: reception of unknown bit assignments in the

-- ODB-HPLMN-Data type shall be treated like unsupported ODB-HPLMN-Data

-- When the ODB-HPLMN-Data type is removed from the HLR for a given subscriber,

-- in NoteSubscriberDataModified operation sent toward the gsmSCF

-- all bits shall be set to "O".

Ext-SS-InfoList ::= SEQUENCE SIZE (1..maxNumOfSS) OF

Ext-SS-Info

Ext-SS-Info ::= CHOICE {

forwardingInfo	[0] Ext-ForwInfo,

callBarringInfo	[1] Ext-CallBarInfo,

cug-Info	[2] CUG-Info,

ss-Data	[3] Ext-SS-Data,

emlpp-Info	[4] EMLPP-Info}

Ext-ForwInfo ::= SEQUENCE {

ss-Code	SS-Code,

forwardingFeatureList	Ext-ForwFeatureList,

extensionContainer	[0] ExtensionContainer	OPTIONAL,

...}

Ext-ForwFeatureList ::= SEQUENCE SIZE (1..maxNumOfExt-BasicServiceGroups) OF

Ext-ForwFeature

Ext-ForwFeature ::= SEQUENCE {

basicService	Ext-BasicServiceCode	OPTIONAL,

ss-Status	[4] Ext-SS-Status,

forwardedToNumber	[5] ISDN-AddressString	OPTIONAL,

-- When this data type is sent from an HLR which supports CAMEL Phase 2

-- to a VLR that supports CAMEL Phase 2 the VLR shall not check the

-- format of the number

forwardedToSubaddress	[8] ISDN-SubaddressString	OPTIONAL,

forwardingOptions	[6] Ext-ForwOptions	OPTIONAL,

noReplyConditionTime	[7] Ext-NoRepCondTime	OPTIONAL,

extensionContainer	[9] ExtensionContainer	OPTIONAL,

...,

longForwardedToNumber	[10] FTN-AddressString	OPTIONAL }

Ext-ForwOptions ::= OCTET STRING (SIZE (1..5))

-- OCTET 1:

--  bit 8: notification to forwarding party

--	0  no notification

--	1  notification

--  bit 7: redirecting presentation

--	0 no presentation

--	1  presentation

--  bit 6: notification to calling party

--	0  no notification

--	1  notification

--  bit 5: 0 (unused)

--  bits 43: forwarding reason

--	00  ms not reachable

--	01  ms busy

--	10  no reply

--	11  unconditional

-- bits 21: 00 (unused)

-- OCTETS 2-5: reserved for future use. They shall be discarded if

-- received and not understood.

Ext-NoRepCondTime ::= INTEGER (1..100)

-- Only values 5-30 are used.

-- Values in the ranges 1-4 and 31-100 are reserved for future use

-- If received:

--	values 1-4 shall be mapped on to value 5

--	values 31-100 shall be mapped on to value 30

Ext-CallBarInfo ::= SEQUENCE {

ss-Code	SS-Code,

callBarringFeatureList	Ext-CallBarFeatureList,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

Ext-CallBarFeatureList ::= SEQUENCE SIZE (1..maxNumOfExt-BasicServiceGroups) OF

Ext-CallBarringFeature

Ext-CallBarringFeature ::= SEQUENCE {

basicService	Ext-BasicServiceCode	OPTIONAL,

ss-Status	[4] Ext-SS-Status,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

CUG-Info ::= SEQUENCE {

cug-SubscriptionList	CUG-SubscriptionList,

cug-FeatureList	CUG-FeatureList	OPTIONAL,

extensionContainer	[0] ExtensionContainer	OPTIONAL,

...}

CUG-SubscriptionList ::= SEQUENCE SIZE (0..maxNumOfCUG) OF

CUG-Subscription

CUG-Subscription ::= SEQUENCE {

cug-Index	CUG-Index,

cug-Interlock	CUG-Interlock,

intraCUG-Options	IntraCUG-Options,

basicServiceGroupList	Ext-BasicServiceGroupList	OPTIONAL,

extensionContainer	[0] ExtensionContainer	OPTIONAL,

...}

CUG-Index ::= INTEGER (0..32767)

-- The internal structure is defined in ETS 300 138.

CUG-Interlock ::= OCTET STRING (SIZE (4))

IntraCUG-Options ::= ENUMERATED {

noCUG-Restrictions  (0),

cugIC-CallBarred  (1),

cugOG-CallBarred  (2)}

maxNumOfCUG  INTEGER ::= 10

CUG-FeatureList ::= SEQUENCE SIZE (1..maxNumOfExt-BasicServiceGroups) OF

CUG-Feature

Ext-BasicServiceGroupList ::= SEQUENCE SIZE (1..maxNumOfExt-BasicServiceGroups) OF

Ext-BasicServiceCode

maxNumOfExt-BasicServiceGroups  INTEGER ::= 32

CUG-Feature ::= SEQUENCE {

basicService	Ext-BasicServiceCode	OPTIONAL,

preferentialCUG-Indicator	CUG-Index	OPTIONAL,

interCUG-Restrictions	InterCUG-Restrictions,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

InterCUG-Restrictions ::= OCTET STRING (SIZE (1))

-- bits 876543: 000000 (unused)

-- Exception handling:

-- bits 876543 shall be ignored if received and not understood

-- bits 21

--	00  CUG only facilities

--	01  CUG with outgoing access

--	10  CUG with incoming access

--	11  CUG with both outgoing and incoming access

Ext-SS-Data ::= SEQUENCE {

ss-Code	SS-Code,

ss-Status	[4] Ext-SS-Status,

ss-SubscriptionOption	SS-SubscriptionOption	OPTIONAL,

basicServiceGroupList	Ext-BasicServiceGroupList	OPTIONAL,

extensionContainer	[5] ExtensionContainer	OPTIONAL,

...}

LCS-PrivacyExceptionList ::= SEQUENCE SIZE (1..maxNumOfPrivacyClass) OF

LCS-PrivacyClass

maxNumOfPrivacyClass  INTEGER ::= 4

LCS-PrivacyClass ::= SEQUENCE {

ss-Code	SS-Code,

ss-Status	Ext-SS-Status,

notificationToMSUser	[0] NotificationToMSUser	OPTIONAL,

-- notificationToMSUser may be sent only for SS-codes callSessionRelated

-- and callSessionUnrelated. If not received for SS-codes callSessionRelated

-- and callSessionUnrelated,

-- the default values according to 3GPP TS 23.271 shall be assumed.

externalClientList	[1] ExternalClientList	OPTIONAL,

-- externalClientList may be sent only for SS-code callSessionUnrelated to a

-- visited node that does not support LCS Release 4 or later versions.

-- externalClientList may be sent only for SS-codes callSessionUnrelated and

-- callSessionRelated to a visited node that supports LCS Release 4 or later versions.

plmnClientList	[2] PLMNClientList	OPTIONAL,

-- plmnClientList may be sent only for SS-code plmnoperator.

extensionContainer	[3] ExtensionContainer	OPTIONAL,

...,

ext-externalClientList	[4] Ext-ExternalClientList	OPTIONAL,

-- Ext-externalClientList may be sent only if the visited node supports LCS Release 4 or

-- later versions, the user did specify more than 5 clients, and White Book SCCP is used.

serviceTypeList	[5]	ServiceTypeList	OPTIONAL

-- serviceTypeList may be sent only for SS-code serviceType and if the visited node

-- supports LCS Release 5 or later versions.

--

-- if segmentation is used, the complete LCS-PrivacyClass shall be sent in one segment

}

ExternalClientList ::= SEQUENCE SIZE (0..maxNumOfExternalClient) OF

ExternalClient

maxNumOfExternalClient  INTEGER ::= 5

PLMNClientList ::= SEQUENCE SIZE (1..maxNumOfPLMNClient) OF

LCSClientInternalID

maxNumOfPLMNClient  INTEGER ::= 5

Ext-ExternalClientList ::= SEQUENCE SIZE (1..maxNumOfExt-ExternalClient) OF

ExternalClient

maxNumOfExt-ExternalClient  INTEGER ::= 35

ExternalClient ::= SEQUENCE {

clientIdentity	LCSClientExternalID,

gmlc-Restriction	[0] GMLC-Restriction	OPTIONAL,

notificationToMSUser	[1] NotificationToMSUser	OPTIONAL,

-- If notificationToMSUser is not received, the default value according to

-- 3GPP TS 23.271 shall be assumed.

extensionContainer	[2] ExtensionContainer	OPTIONAL,

... }

GMLC-Restriction ::= ENUMERATED {

gmlc-List	(0),

home-Country	(1) ,

... }

-- exception handling:

-- At reception of any other value than the ones listed the receiver shall ignore

-- GMLC-Restriction.

NotificationToMSUser ::= ENUMERATED {

notifyLocationAllowed	(0),

notifyAndVerify-LocationAllowedIfNoResponse	(1),

notifyAndVerify-LocationNotAllowedIfNoResponse	(2),

...,

locationNotAllowed (3) }

-- exception handling:

-- At reception of any other value than the ones listed the receiver shall ignore

-- NotificationToMSUser.

ServiceTypeList ::= SEQUENCE SIZE (1..maxNumOfServiceType) OF

ServiceType

maxNumOfServiceType  INTEGER ::= 32

ServiceType ::= SEQUENCE {

serviceTypeIdentity	LCSServiceTypeID,

gmlc-Restriction	[0] GMLC-Restriction	OPTIONAL,

notificationToMSUser	[1] NotificationToMSUser	OPTIONAL,

-- If notificationToMSUser is not received, the default value according to

-- 3GPP TS 23.271 shall be assumed.

extensionContainer	[2] ExtensionContainer	OPTIONAL,

... }

MOLR-List ::= SEQUENCE SIZE (1..maxNumOfMOLR-Class) OF

MOLR-Class

maxNumOfMOLR-Class  INTEGER ::= 3

MOLR-Class ::= SEQUENCE {

ss-Code	SS-Code,

ss-Status	Ext-SS-Status,

extensionContainer	[0] ExtensionContainer	OPTIONAL,

...}

ZoneCodeList ::= SEQUENCE SIZE (1..maxNumOfZoneCodes)

OF ZoneCode

ZoneCode ::= OCTET STRING (SIZE (2))

-- internal structure is defined in TS 3GPP TS 23.003 [17]

maxNumOfZoneCodes  INTEGER ::= 10

InsertSubscriberDataRes ::= SEQUENCE {

teleserviceList	[1] TeleserviceList	OPTIONAL,

bearerServiceList	[2] BearerServiceList	OPTIONAL,

ss-List	[3] SS-List	OPTIONAL,

odb-GeneralData	[4] ODB-GeneralData	OPTIONAL,

regionalSubscriptionResponse	[5] RegionalSubscriptionResponse	OPTIONAL,

supportedCamelPhases	[6] SupportedCamelPhases	OPTIONAL,

extensionContainer	[7] ExtensionContainer	OPTIONAL,

... ,

offeredCamel4CSIs	[8] OfferedCamel4CSIs	OPTIONAL,

supportedFeatures	[9] SupportedFeatures	OPTIONAL,

ext-SupportedFeatures	[10] Ext-SupportedFeatures	OPTIONAL }

RegionalSubscriptionResponse ::= ENUMERATED {

networkNode-AreaRestricted	(0),

tooManyZoneCodes	(1),

zoneCodesConflict	(2),

regionalSubscNotSupported	(3)}

DeleteSubscriberDataArg ::= SEQUENCE {

imsi	[0] IMSI,

basicServiceList	[1] BasicServiceList	OPTIONAL,

-- The exception handling for reception of unsupported/not allocated

-- basicServiceCodes is defined in clause 6.8.2

ss-List	[2] SS-List	OPTIONAL,

roamingRestrictionDueToUnsupportedFeature [4] NULL	OPTIONAL,

regionalSubscriptionIdentifier	[5] ZoneCode	OPTIONAL,

vbsGroupIndication	[7] NULL	OPTIONAL,

vgcsGroupIndication	[8] NULL	OPTIONAL,

camelSubscriptionInfoWithdraw	[9] NULL	OPTIONAL,

extensionContainer	[6] ExtensionContainer	OPTIONAL,

...,

gprsSubscriptionDataWithdraw	[10] GPRSSubscriptionDataWithdraw	OPTIONAL,

roamingRestrictedInSgsnDueToUnsuppportedFeature [11] NULL	OPTIONAL,

lsaInformationWithdraw	[12] LSAInformationWithdraw	OPTIONAL,

gmlc-ListWithdraw	[13]	NULL	OPTIONAL,

istInformationWithdraw	[14] NULL	OPTIONAL,

specificCSI-Withdraw	[15] SpecificCSI-Withdraw	OPTIONAL,

chargingCharacteristicsWithdraw	[16] NULL	OPTIONAL,

stn-srWithdraw	[17] NULL	OPTIONAL,

epsSubscriptionDataWithdraw	[18] EPS-SubscriptionDataWithdraw	OPTIONAL,

apn-oi-replacementWithdraw	[19] NULL	OPTIONAL,

csg-SubscriptionDeleted	[20]	NULL	OPTIONAL,

subscribedPeriodicTAU-RAU-TimerWithdraw	[22]	NULL	OPTIONAL,

subscribedPeriodicLAU-TimerWithdraw	[23]	NULL	OPTIONAL,

subscribed-vsrvccWithdraw	[21] NULL	OPTIONAL,

vplmn-Csg-SubscriptionDeleted	[24]	NULL	OPTIONAL,

additionalMSISDN-Withdraw	[25]	NULL	OPTIONAL,

cs-to-ps-SRVCC-Withdraw	[26]	NULL	OPTIONAL,

imsiGroupIdList-Withdraw	[27]	NULL	OPTIONAL,

userPlaneIntegrityProtectionWithdraw	[28] NULL	OPTIONAL,

dl-Buffering-Suggested-Packet-Count-Withdraw	[29] NULL	OPTIONAL,

ue-UsageTypeWithdraw	[30] NULL	OPTIONAL,

reset-idsWithdraw	[31]	NULL	OPTIONAL,

iab-OperationWithdraw	[32]	NULL	OPTIONAL }

SpecificCSI-Withdraw ::= BIT STRING {

o-csi (0),

ss-csi (1),

tif-csi (2),

d-csi (3),

vt-csi (4),

mo-sms-csi (5),

m-csi (6),

gprs-csi (7),

t-csi (8),

mt-sms-csi (9),

mg-csi (10),

o-IM-CSI (11),

d-IM-CSI (12),

vt-IM-CSI (13) } (SIZE(8..32))

-- exception handling:

-- bits 11 to 31 shall be ignored if received by a non-IP Multimedia Core Network entity.

-- bits 0-10 and 14-31 shall be ignored if received by an IP Multimedia Core Network entity.

-- bits 11-13 are only applicable in an IP Multimedia Core Network.

-- Bit 8 and bits 11-13 are only applicable for the NoteSubscriberDataModified operation.

GPRSSubscriptionDataWithdraw ::= CHOICE {

allGPRSData	NULL,

contextIdList	ContextIdList}

EPS-SubscriptionDataWithdraw ::= CHOICE {

allEPS-Data	NULL,

contextIdList	ContextIdList}

ContextIdList ::= SEQUENCE SIZE (1..maxNumOfPDP-Contexts) OF

ContextId

LSAInformationWithdraw ::= CHOICE {

allLSAData	NULL,

lsaIdentityList	LSAIdentityList }

LSAIdentityList ::= SEQUENCE SIZE (1..maxNumOfLSAs) OF

LSAIdentity

BasicServiceList ::= SEQUENCE SIZE (1..maxNumOfBasicServices) OF

Ext-BasicServiceCode

maxNumOfBasicServices  INTEGER ::= 70

DeleteSubscriberDataRes ::= SEQUENCE {

regionalSubscriptionResponse	[0] RegionalSubscriptionResponse	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

VlrCamelSubscriptionInfo ::= SEQUENCE {

o-CSI	[0] O-CSI	OPTIONAL,

extensionContainer	[1] ExtensionContainer	OPTIONAL,

...,

ss-CSI	[2] SS-CSI	OPTIONAL,

o-BcsmCamelTDP-CriteriaList	[4] O-BcsmCamelTDPCriteriaList	OPTIONAL,

tif-CSI	[3] NULL	OPTIONAL,

m-CSI	[5] M-CSI	OPTIONAL,

mo-sms-CSI	[6] SMS-CSI	OPTIONAL,

vt-CSI	[7] T-CSI	OPTIONAL,

t-BCSM-CAMEL-TDP-CriteriaList	[8] T-BCSM-CAMEL-TDP-CriteriaList	OPTIONAL,

d-CSI	[9] D-CSI	OPTIONAL,

mt-sms-CSI	[10] SMS-CSI	OPTIONAL,

mt-smsCAMELTDP-CriteriaList	[11]	MT-smsCAMELTDP-CriteriaList	OPTIONAL

}

MT-smsCAMELTDP-CriteriaList ::= SEQUENCE SIZE (1.. maxNumOfCamelTDPData) OF

MT-smsCAMELTDP-Criteria

MT-smsCAMELTDP-Criteria ::= SEQUENCE {

sms-TriggerDetectionPoint	SMS-TriggerDetectionPoint,

tpdu-TypeCriterion	[0]	TPDU-TypeCriterion	OPTIONAL,

... }

TPDU-TypeCriterion ::= SEQUENCE SIZE (1..maxNumOfTPDUTypes) OF

MT-SMS-TPDU-Type

maxNumOfTPDUTypes INTEGER ::= 5

MT-SMS-TPDU-Type ::= ENUMERATED {

sms-DELIVER	(0),

sms-SUBMIT-REPORT	(1),

sms-STATUS-REPORT	(2),

... }

--	exception handling:

--	For TPDU-TypeCriterion sequences containing this parameter with any

--	other value than the ones listed above the receiver shall ignore

--	the whole TPDU-TypeCriterion sequence.

--	In CAMEL phase 4, sms-SUBMIT-REPORT shall not be used and a received TPDU-TypeCriterion

--	sequence containing sms-SUBMIT-REPORT shall be wholly ignored.

D-CSI ::= SEQUENCE {

dp-AnalysedInfoCriteriaList	[0] DP-AnalysedInfoCriteriaList	OPTIONAL,

camelCapabilityHandling	[1] CamelCapabilityHandling	OPTIONAL,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

notificationToCSE	[3]	NULL	OPTIONAL,

csi-Active	[4]	NULL	OPTIONAL,

...}

--	notificationToCSE and csi-Active shall not be present when D-CSI is sent to VLR/GMSC.

--	They may only be included in ATSI/ATM ack/NSDC message.

--	DP-AnalysedInfoCriteria and  camelCapabilityHandling shall be present in

--	the D-CSI sequence.

--	If D-CSI is segmented, then the first segment shall contain dp-AnalysedInfoCriteriaList

--	and camelCapabilityHandling. Subsequent segments shall not contain

--	camelCapabilityHandling, but may contain dp-AnalysedInfoCriteriaList.

DP-AnalysedInfoCriteriaList  ::= SEQUENCE SIZE (1..maxNumOfDP-AnalysedInfoCriteria) OF

DP-AnalysedInfoCriterium

maxNumOfDP-AnalysedInfoCriteria INTEGER ::= 10

DP-AnalysedInfoCriterium ::= SEQUENCE {

dialledNumber	ISDN-AddressString,

serviceKey	ServiceKey,

gsmSCF-Address	ISDN-AddressString,

defaultCallHandling	DefaultCallHandling,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

SS-CSI ::= SEQUENCE {

ss-CamelData	SS-CamelData,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

notificationToCSE	[0]	NULL	OPTIONAL,

csi-Active	[1]	NULL	OPTIONAL

--	notificationToCSE and csi-Active shall not be present when SS-CSI is sent to VLR.

--	They may only be included in ATSI/ATM ack/NSDC message.

}

SS-CamelData  ::= SEQUENCE {

ss-EventList	SS-EventList,

gsmSCF-Address	ISDN-AddressString,

extensionContainer	[0] ExtensionContainer	OPTIONAL,

...}

SS-EventList  ::= SEQUENCE SIZE (1..maxNumOfCamelSSEvents) OF SS-Code

-- Actions for the following SS-Code values are defined in CAMEL Phase 3:

-- ect	SS-Code ::= '00110001'B

-- multiPTY	SS-Code ::= '01010001'B

-- cd	SS-Code ::= '00100100'B

-- ccbs	SS-Code ::= '01000100'B

-- all other SS codes shall be ignored

-- When SS-CSI is sent to the VLR, it shall not contain a marking for ccbs.

-- If the VLR receives SS-CSI containing a marking for ccbs, the VLR shall discard the

-- ccbs marking in SS-CSI.

maxNumOfCamelSSEvents INTEGER ::= 10

O-CSI ::= SEQUENCE {

o-BcsmCamelTDPDataList	O-BcsmCamelTDPDataList,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

camelCapabilityHandling	[0] CamelCapabilityHandling	OPTIONAL,

notificationToCSE	[1]	NULL	OPTIONAL,

csiActive	[2]	NULL	OPTIONAL}

--	notificationtoCSE and csiActive shall not be present when O-CSI is sent to VLR/GMSC.

--	They may only be included in ATSI/ATM ack/NSDC message.

--	O-CSI shall not be segmented.

O-BcsmCamelTDPDataList ::= SEQUENCE SIZE (1..maxNumOfCamelTDPData) OF

O-BcsmCamelTDPData

-- O-BcsmCamelTDPDataList shall not contain more than one instance of

-- O-BcsmCamelTDPData containing the same value for o-BcsmTriggerDetectionPoint.

-- For CAMEL Phase 2, this means that only one instance of O-BcsmCamelTDPData is allowed

-- with o-BcsmTriggerDetectionPoint being equal to DP2.

maxNumOfCamelTDPData  INTEGER ::= 10

O-BcsmCamelTDPData ::= SEQUENCE {

o-BcsmTriggerDetectionPoint	O-BcsmTriggerDetectionPoint,

serviceKey	ServiceKey,

gsmSCF-Address	[0] ISDN-AddressString,

defaultCallHandling	[1] DefaultCallHandling,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

...

}

ServiceKey ::= INTEGER (0..2147483647)

O-BcsmTriggerDetectionPoint ::= ENUMERATED {

collectedInfo (2),

...,

routeSelectFailure (4) }

-- exception handling:

-- For O-BcsmCamelTDPData sequences containing this parameter with any

-- other value than the ones listed the receiver shall ignore the whole

-- O-BcsmCamelTDPDatasequence.

-- For O-BcsmCamelTDP-Criteria sequences containing this parameter with any

-- other value than the ones listed the receiver shall ignore the whole

-- O-BcsmCamelTDP-Criteria sequence.

O-BcsmCamelTDPCriteriaList ::= SEQUENCE SIZE (1..maxNumOfCamelTDPData) OF

O-BcsmCamelTDP-Criteria

T-BCSM-CAMEL-TDP-CriteriaList ::= SEQUENCE SIZE (1..maxNumOfCamelTDPData) OF

T-BCSM-CAMEL-TDP-Criteria

O-BcsmCamelTDP-Criteria ::= SEQUENCE {

o-BcsmTriggerDetectionPoint	O-BcsmTriggerDetectionPoint,

destinationNumberCriteria	[0] DestinationNumberCriteria	OPTIONAL,

basicServiceCriteria	[1] BasicServiceCriteria	OPTIONAL,

callTypeCriteria	[2] CallTypeCriteria	OPTIONAL,

...,

o-CauseValueCriteria	[3] O-CauseValueCriteria	OPTIONAL,

extensionContainer	[4] ExtensionContainer	OPTIONAL }

T-BCSM-CAMEL-TDP-Criteria ::= SEQUENCE {

t-BCSM-TriggerDetectionPoint	T-BcsmTriggerDetectionPoint,

basicServiceCriteria	[0] BasicServiceCriteria	OPTIONAL,

t-CauseValueCriteria	[1] T-CauseValueCriteria	OPTIONAL,

... }

DestinationNumberCriteria  ::= SEQUENCE {

matchType	[0] MatchType,

destinationNumberList	[1] DestinationNumberList	OPTIONAL,

destinationNumberLengthList	[2] DestinationNumberLengthList	OPTIONAL,

-- one or both of destinationNumberList and destinationNumberLengthList

-- shall be present

...}

DestinationNumberList  ::= SEQUENCE SIZE	(1..maxNumOfCamelDestinationNumbers) OF

ISDN-AddressString

-- The receiving entity shall not check the format of a number in

-- the dialled number list

DestinationNumberLengthList  ::= SEQUENCE SIZE (1..maxNumOfCamelDestinationNumberLengths) OF

INTEGER(1..maxNumOfISDN-AddressDigits)

BasicServiceCriteria   ::= SEQUENCE SIZE(1..maxNumOfCamelBasicServiceCriteria) OF

Ext-BasicServiceCode

maxNumOfISDN-AddressDigits  INTEGER ::= 15

maxNumOfCamelDestinationNumbers  INTEGER ::= 10

maxNumOfCamelDestinationNumberLengths  INTEGER ::= 3

maxNumOfCamelBasicServiceCriteria  INTEGER ::= 5

CallTypeCriteria       ::= ENUMERATED {

forwarded	(0),

notForwarded	(1)}

MatchType       ::= ENUMERATED {

inhibiting	(0),

enabling	(1)}

O-CauseValueCriteria   ::= SEQUENCE SIZE(1..maxNumOfCAMEL-O-CauseValueCriteria) OF

CauseValue

T-CauseValueCriteria   ::= SEQUENCE SIZE(1..maxNumOfCAMEL-T-CauseValueCriteria) OF

CauseValue

maxNumOfCAMEL-O-CauseValueCriteria  INTEGER ::= 5

maxNumOfCAMEL-T-CauseValueCriteria  INTEGER ::= 5

CauseValue ::= OCTET STRING (SIZE(1))

-- Type extracted from Cause parameter in ITU-T Recommendation Q.763.

-- For the use of cause value refer to ITU-T Recommendation Q.850.

DefaultCallHandling ::= ENUMERATED {

continueCall (0) ,

releaseCall (1) ,

...}

-- exception handling:

-- reception of values in range 2-31 shall be treated as "continueCall"

-- reception of values greater than 31 shall be treated as "releaseCall"

CamelCapabilityHandling ::= INTEGER(1..16)

-- value 1 = CAMEL phase 1,

-- value 2 = CAMEL phase 2,

-- value 3 = CAMEL Phase 3,

-- value 4 = CAMEL phase 4:

-- reception of values greater than 4 shall be treated as CAMEL phase 4.

SupportedCamelPhases ::= BIT STRING {

phase1 (0),

phase2 (1),

phase3 (2),

phase4 (3)} (SIZE (1..16))

-- A node shall mark in the BIT STRING all CAMEL Phases it supports.

-- Other values than listed above shall be discarded.

OfferedCamel4CSIs ::= BIT STRING {

o-csi	(0),

d-csi	(1),

vt-csi	(2),

t-csi	(3),

mt-sms-csi	(4),

mg-csi	(5),

psi-enhancements	(6)

} (SIZE (7..16))

-- A node supporting Camel phase 4 shall mark in the BIT STRING all Camel4 CSIs

-- it offers.

-- Other values than listed above shall be discarded.

OfferedCamel4Functionalities ::= BIT STRING {

initiateCallAttempt	(0),

splitLeg	(1),

moveLeg	(2),

disconnectLeg	(3),

entityReleased	(4),

dfc-WithArgument	(5),

playTone	(6),

dtmf-MidCall	(7),

chargingIndicator	(8),

alertingDP	(9),

locationAtAlerting	(10),

changeOfPositionDP	(11),

or-Interactions	(12),

warningToneEnhancements	(13),

cf-Enhancements	(14),

subscribedEnhancedDialledServices	(15),

servingNetworkEnhancedDialledServices (16),

criteriaForChangeOfPositionDP	(17),

serviceChangeDP	(18),

collectInformation	(19)

} (SIZE (15..64))

-- A node supporting Camel phase 4 shall mark in the BIT STRING all CAMEL4

-- functionalities it offers.

-- Other values than listed above shall be discarded.

# SMS-CSI ::= SEQUENCE {

sms-CAMEL-TDP-DataList	[0] SMS-CAMEL-TDP-DataList	OPTIONAL,

camelCapabilityHandling	[1] CamelCapabilityHandling	OPTIONAL,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

notificationToCSE	[3] NULL	OPTIONAL,

csi-Active	[4] NULL	OPTIONAL,

...}

--	notificationToCSE and csi-Active shall not be present

--	when MO-SMS-CSI or MT-SMS-CSI is sent to VLR or SGSN.

--	They may only be included in ATSI/ATM ack/NSDC message.

--	SMS-CAMEL-TDP-Data and  camelCapabilityHandling shall be present in

--	the SMS-CSI sequence.

--	If SMS-CSI is segmented, sms-CAMEL-TDP-DataList and camelCapabilityHandling shall be

--	present in the first segment

# SMS-CAMEL-TDP-DataList ::= SEQUENCE SIZE (1..maxNumOfCamelTDPData) OF

# SMS-CAMEL-TDP-Data

--	SMS-CAMEL-TDP-DataList shall not contain more than one instance of

--	SMS-CAMEL-TDP-Data containing the same value for sms-TriggerDetectionPoint.

# SMS-CAMEL-TDP-Data ::= SEQUENCE {

sms-TriggerDetectionPoint	[0] SMS-TriggerDetectionPoint,

serviceKey	[1] ServiceKey,

gsmSCF-Address	[2] ISDN-AddressString,

defaultSMS-Handling	[3] DefaultSMS-Handling,

extensionContainer	[4] ExtensionContainer	OPTIONAL,

...

}

# SMS-TriggerDetectionPoint ::= ENUMERATED {

sms-CollectedInfo (1),

...,

sms-DeliveryRequest (2)

}

--	exception handling:

--	For SMS-CAMEL-TDP-Data and MT-smsCAMELTDP-Criteria sequences containing this

--	parameter with any other value than the ones listed the receiver shall ignore

--	the whole sequence.

--

--	If this parameter is received with any other value than sms-CollectedInfo

--	in an SMS-CAMEL-TDP-Data sequence contained in mo-sms-CSI, then the receiver shall

--	ignore the whole SMS-CAMEL-TDP-Data sequence.

--

--	If this parameter is received with any other value than sms-DeliveryRequest

--	in an SMS-CAMEL-TDP-Data sequence contained in mt-sms-CSI then the receiver shall

--	ignore the whole SMS-CAMEL-TDP-Data sequence.

--

--	If this parameter is received with any other value than sms-DeliveryRequest

--	in an MT-smsCAMELTDP-Criteria sequence then the receiver shall

--	ignore the whole MT-smsCAMELTDP-Criteria sequence.

# DefaultSMS-Handling ::= ENUMERATED {

continueTransaction (0) ,

releaseTransaction (1) ,

...}

--	exception handling:

--	reception of values in range 2-31 shall be treated as "continueTransaction"

--	reception of values greater than 31 shall be treated as "releaseTransaction"

# M-CSI ::= SEQUENCE {

mobilityTriggers	MobilityTriggers,

serviceKey	ServiceKey,

gsmSCF-Address	[0]	ISDN-AddressString,

extensionContainer	[1]	ExtensionContainer	OPTIONAL,

notificationToCSE	[2] NULL	OPTIONAL,

csi-Active	[3] NULL	OPTIONAL,

...}

--	notificationToCSE and csi-Active shall not be present when M-CSI is sent to VLR.

--	They may only be included in ATSI/ATM ack/NSDC message.

# MG-CSI ::= SEQUENCE {

mobilityTriggers	MobilityTriggers,

serviceKey	ServiceKey,

gsmSCF-Address	[0]	ISDN-AddressString,

extensionContainer	[1]	ExtensionContainer	OPTIONAL,

notificationToCSE	[2] NULL	OPTIONAL,

csi-Active	[3] NULL	OPTIONAL,

...}

--	notificationToCSE and csi-Active shall not be present when MG-CSI is sent to SGSN.

--	They may only be included in ATSI/ATM ack/NSDC message.

# MobilityTriggers  ::= SEQUENCE SIZE (1..maxNumOfMobilityTriggers) OF

# MM-Code

maxNumOfMobilityTriggers INTEGER ::= 10

# MM-Code ::= OCTET STRING (SIZE (1))

--	This type is used to indicate a Mobility Management event.

--	Actions for the following MM-Code values are defined in CAMEL Phase 4:

--

--	CS domain MM events:

--	Location-update-in-same-VLR	MM-Code ::= '00000000'B

--	Location-update-to-other-VLR	MM-Code ::= '00000001'B

--	IMSI-Attach	MM-Code ::= '00000010'B

--	MS-initiated-IMSI-Detach	MM-Code ::= '00000011'B

--	Network-initiated-IMSI-Detach	MM-Code ::= '00000100'B

--

--	PS domain MM events:

--	Routeing-Area-update-in-same-SGSN	MM-Code ::= '10000000'B

--	Routeing-Area-update-to-other-SGSN-update-from-new-SGSN

--	MM-Code ::= '10000001'B

--	Routeing-Area-update-to-other-SGSN-disconnect-by-detach

--	MM-Code ::= '10000010'B

--	GPRS-Attach	MM-Code ::= '10000011'B

--	MS-initiated-GPRS-Detach	MM-Code ::= '10000100'B

--	Network-initiated-GPRS-Detach	MM-Code ::= '10000101'B

--	Network-initiated-transfer-to-MS-not-reachable-for-paging

--	MM-Code ::= '10000110'B

--

--	If the MSC receives any other MM-code than the ones listed above for the

--	CS domain, then the MSC shall ignore that MM-code.

--	If the SGSN receives any other MM-code than the ones listed above for the

--	PS domain, then the SGSN shall ignore that MM-code.

T-CSI ::= SEQUENCE {

t-BcsmCamelTDPDataList	T-BcsmCamelTDPDataList,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

camelCapabilityHandling	[0] CamelCapabilityHandling	OPTIONAL,

notificationToCSE	[1] NULL	OPTIONAL,

csi-Active	[2] NULL	OPTIONAL}

--	notificationToCSE and csi-Active shall not be present when VT-CSI/T-CSI is sent

--	to VLR/GMSC.

--	They may only be included in ATSI/ATM ack/NSDC message.

--	T-CSI shall not be segmented.

T-BcsmCamelTDPDataList ::= SEQUENCE SIZE (1..maxNumOfCamelTDPData) OF

T-BcsmCamelTDPData

--- T-BcsmCamelTDPDataList shall not contain more than one instance of

--- T-BcsmCamelTDPData containing the same value for t-BcsmTriggerDetectionPoint.

--- For CAMEL Phase 2, this means that only one instance of T-BcsmCamelTDPData is allowed

--- with t-BcsmTriggerDetectionPoint being equal to DP12.

--- For CAMEL Phase 3, more TDP's are allowed.

T-BcsmCamelTDPData ::= SEQUENCE {

t-BcsmTriggerDetectionPoint	T-BcsmTriggerDetectionPoint,

serviceKey	ServiceKey,

gsmSCF-Address	[0] ISDN-AddressString,

defaultCallHandling	[1] DefaultCallHandling,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

...}

T-BcsmTriggerDetectionPoint ::= ENUMERATED {

termAttemptAuthorized (12),

... ,

tBusy (13),

tNoAnswer (14)}

-- exception handling:

-- For T-BcsmCamelTDPData sequences containing this parameter with any other

-- value than the ones listed above, the receiver shall ignore the whole

-- T-BcsmCamelTDPData sequence.

-- gprs location information retrieval types

SendRoutingInfoForGprsArg ::= SEQUENCE {

imsi	[0] IMSI,

ggsn-Address	[1] GSN-Address	OPTIONAL,

ggsn-Number	[2]	ISDN-AddressString,

extensionContainer	[3] ExtensionContainer	OPTIONAL,

...}

SendRoutingInfoForGprsRes ::= SEQUENCE {

sgsn-Address	[0] GSN-Address,

ggsn-Address	[1]	GSN-Address	OPTIONAL,

mobileNotReachableReason	[2]	AbsentSubscriberDiagnosticSM	OPTIONAL,

extensionContainer	[3] ExtensionContainer	OPTIONAL,

...}

-- failure report types

FailureReportArg ::= SEQUENCE {

imsi	[0] IMSI,

ggsn-Number	[1] ISDN-AddressString	,

ggsn-Address	[2] GSN-Address	OPTIONAL,

extensionContainer	[3] ExtensionContainer	OPTIONAL,

...}

FailureReportRes ::= SEQUENCE {

ggsn-Address	[0] GSN-Address	OPTIONAL,

extensionContainer	[1] ExtensionContainer	OPTIONAL,

...}

-- gprs notification types

NoteMsPresentForGprsArg ::= SEQUENCE {

imsi	[0] IMSI,

sgsn-Address	[1] GSN-Address,

ggsn-Address	[2] GSN-Address	OPTIONAL,

extensionContainer	[3] ExtensionContainer	OPTIONAL,

...}

NoteMsPresentForGprsRes ::= SEQUENCE {

extensionContainer	[0] ExtensionContainer	OPTIONAL,

...}

-- fault recovery types

ResetArg ::= SEQUENCE {

sendingNodenumber	SendingNode-Number,

hlr-List	HLR-List	OPTIONAL,

-- The hlr-List parameter shall only be applicable for a restart of the HSS/HLR.

extensionContainer	[0] ExtensionContainer	OPTIONAL,

...,

reset-Id-List	[1]	Reset-Id-List	OPTIONAL,

subscriptionData	[2]	InsertSubscriberDataArg	OPTIONAL,

subscriptionDataDeletion	[3]	DeleteSubscriberDataArg	OPTIONAL}

SendingNode-Number ::= CHOICE {

hlr-Number	ISDN-AddressString,

css-Number	[1] ISDN-AddressString}

RestoreDataArg ::= SEQUENCE {

imsi	IMSI,

lmsi	LMSI	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

... ,

vlr-Capability	[6] VLR-Capability	OPTIONAL,

restorationIndicator	[7]	NULL	OPTIONAL

}

RestoreDataRes ::= SEQUENCE {

hlr-Number	ISDN-AddressString,

msNotReachable	NULL	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

-- VBS/VGCS types

VBSDataList ::= SEQUENCE SIZE (1..maxNumOfVBSGroupIds) OF

VoiceBroadcastData

VGCSDataList ::= SEQUENCE SIZE (1..maxNumOfVGCSGroupIds) OF

VoiceGroupCallData

maxNumOfVBSGroupIds  INTEGER ::= 50

maxNumOfVGCSGroupIds  INTEGER ::= 50

VoiceGroupCallData  ::= SEQUENCE {

groupId	GroupId,

-- groupId shall be filled with six TBCD fillers (1111)if the longGroupId is present

extensionContainer	ExtensionContainer	OPTIONAL,

...,

additionalSubscriptions	AdditionalSubscriptions	OPTIONAL,

additionalInfo	[0] AdditionalInfo	OPTIONAL,

longGroupId	[1] Long-GroupId	OPTIONAL }

-- VoiceGroupCallData containing a longGroupId shall not be sent to VLRs that did not

-- indicate support of long Group IDs within the Update Location or Restore Data

-- request message

AdditionalInfo ::= BIT STRING (SIZE (1..136))

--	Refers to Additional Info as specified in 3GPP TS 43.068

AdditionalSubscriptions ::= BIT STRING {

privilegedUplinkRequest (0),

emergencyUplinkRequest (1),

emergencyReset (2)} (SIZE (3..8))

-- Other bits than listed above shall be discarded.

VoiceBroadcastData ::= SEQUENCE {

groupid	GroupId,

-- groupId shall be filled with six TBCD fillers (1111)if the longGroupId is present

broadcastInitEntitlement	NULL	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

longGroupId	[0] Long-GroupId	OPTIONAL }

-- VoiceBroadcastData containing a longGroupId shall not be sent to VLRs that did not

-- indicate support of long Group IDs within the Update Location or Restore Data

-- request message

GroupId  ::= TBCD-STRING (SIZE (3))

-- When Group-Id is less than six characters in length, the TBCD filler (1111)

-- is used to fill unused half octets.

-- Refers to the Group Identification as specified in 3GPP TS 23.003

-- and 3GPP TS 43.068/ 43.069

Long-GroupId  ::= TBCD-STRING (SIZE (4))

-- When Long-Group-Id is less than eight characters in length, the TBCD filler (1111)

-- is used to fill unused half octets.

-- Refers to the Group Identification as specified in 3GPP TS 23.003

-- and 3GPP TS 43.068/ 43.069

-- provide subscriber info types

ProvideSubscriberInfoArg ::= SEQUENCE {

imsi	[0] IMSI,

lmsi	[1] LMSI	OPTIONAL,

requestedInfo	[2] RequestedInfo,

extensionContainer	[3] ExtensionContainer	OPTIONAL,

...,

callPriority	[4]	EMLPP-Priority	OPTIONAL

}

ProvideSubscriberInfoRes ::= SEQUENCE {

subscriberInfo	SubscriberInfo,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

SubscriberInfo ::= SEQUENCE {

locationInformation	[0] LocationInformation	OPTIONAL,

subscriberState	[1] SubscriberState	OPTIONAL,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

... ,

locationInformationGPRS	[3] LocationInformationGPRS	OPTIONAL,

ps-SubscriberState	[4] PS-SubscriberState	OPTIONAL,

imei	[5] IMEI	OPTIONAL,

ms-Classmark2	[6] MS-Classmark2	OPTIONAL,

gprs-MS-Class	[7] GPRSMSClass	OPTIONAL,

mnpInfoRes	[8] MNPInfoRes	OPTIONAL,

imsVoiceOverPS-SessionsIndication	[9] IMS-VoiceOverPS-SessionsInd	OPTIONAL,

lastUE-ActivityTime	[10] Time	OPTIONAL,

lastRAT-Type	[11] Used-RAT-Type	OPTIONAL,

eps-SubscriberState	[12] PS-SubscriberState	OPTIONAL,

locationInformationEPS	[13] LocationInformationEPS	OPTIONAL,

timeZone	[14] TimeZone	OPTIONAL,

daylightSavingTime	[15] DaylightSavingTime	OPTIONAL,

locationInformation5GS	[16] LocationInformation5GS	OPTIONAL }

--	If the HLR receives locationInformation, subscriberState or ms-Classmark2 from an SGSN or

--	MME (via an IWF), it shall discard them.

--	If the HLR receives locationInformationGPRS, ps-SubscriberState, gprs-MS-Class or

--	locationInformationEPS (outside the locationInformation IE) from a VLR, it shall

--	discard them.

--	If the HLR receives parameters which it has not requested, it shall discard them.

--	The locationInformation5GS IE should be absent if UE did not access via 5GS and IM-SSF.

# IMS-VoiceOverPS-SessionsInd ::= ENUMERATED {

imsVoiceOverPS-SessionsNotSupported	(0),

imsVoiceOverPS-SessionsSupported	(1),

unknown	(2)

}

--	"unknown" shall not be used within ProvideSubscriberInfoRes

TimeZone ::= OCTET STRING (SIZE (2..3))

--	Refer to the 3GPP TS 29.272 [144] for details.

DaylightSavingTime ::= ENUMERATED {

noAdjustment	(0),

plusOneHourAdjustment	(1),

plusTwoHoursAdjustment	(2)

}

--	Refer to the 3GPP TS 29.272 [144] for details.

MNPInfoRes ::= SEQUENCE {

routeingNumber	[0] RouteingNumber	OPTIONAL,

imsi	[1] IMSI	OPTIONAL,

msisdn	[2] ISDN-AddressString	OPTIONAL,

numberPortabilityStatus	[3] NumberPortabilityStatus	OPTIONAL,

extensionContainer	[4] ExtensionContainer	OPTIONAL,

... }

--	The IMSI parameter contains a generic IMSI, i.e. it is not tied necessarily to the

--	Subscriber. MCC and MNC values in this IMSI shall point to the Subscription Network of

--	the Subscriber. See 3GPP TS 23.066 [108].

RouteingNumber ::= TBCD-STRING (SIZE (1..5))

# NumberPortabilityStatus ::= ENUMERATED {

notKnownToBePorted	(0),

ownNumberPortedOut	(1),

foreignNumberPortedToForeignNetwork	(2),

...,

ownNumberNotPortedOut	(4),

foreignNumberPortedIn	(5)

}

--	exception handling:

--  reception of other values than the ones listed the receiver shall ignore the

--  whole NumberPortabilityStatus;

--  ownNumberNotPortedOut or foreignNumberPortedIn may only be included in Any Time

--  Interrogation message.

MS-Classmark2 ::= OCTET STRING (SIZE (3))

-- This parameter carries the value part of the MS Classmark 2 IE defined in

-- 3GPP TS 24.008 [35].

GPRSMSClass ::= SEQUENCE {

mSNetworkCapability	[0] MSNetworkCapability,

mSRadioAccessCapability	[1] MSRadioAccessCapability	OPTIONAL

}

MSNetworkCapability ::= OCTET STRING (SIZE (1..8))

-- This parameter carries the value part of the MS Network Capability IE defined in

-- 3GPP TS 24.008 [35].

MSRadioAccessCapability ::= OCTET STRING (SIZE (1..50))

-- This parameter carries the value part of the MS Radio Access Capability IE defined in

-- 3GPP TS 24.008 [35].

RequestedInfo ::= SEQUENCE {

locationInformation	[0] NULL	OPTIONAL,

subscriberState	[1] NULL	OPTIONAL,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

...,

currentLocation	[3] NULL	OPTIONAL,

requestedDomain	[4] DomainType	OPTIONAL,

imei	[6] NULL	OPTIONAL,

ms-classmark	[5] NULL	OPTIONAL,

mnpRequestedInfo	[7] NULL	OPTIONAL,

locationInformationEPS-Supported	[11] NULL	OPTIONAL,

t-adsData	[8] NULL	OPTIONAL,

requestedNodes	[9] RequestedNodes	OPTIONAL,

servingNodeIndication	[10] NULL	OPTIONAL,

localTimeZoneRequest	[12] NULL	OPTIONAL

}

--	currentLocation and locationInformationEPS-Supported shall be absent if

--	locationInformation is absent

--	t-adsData shall be absent in messages sent to the VLR

--	requestedNodes shall be absent if requestedDomain is "cs-Domain"

--	servingNodeIndication shall be absent if locationInformation is absent;

--	servingNodeIndication shall be absent if current location is present;

--	servingNodeIndication indicates by its presence that only the serving node's

--	address (MME-Name or SGSN-Number or VLR-Number) is requested.

DomainType ::=  ENUMERATED {

cs-Domain	(0),

ps-Domain	(1),

...}

-- exception handling:

-- reception of values > 1 shall be mapped to 'cs-Domain'

RequestedNodes ::= BIT STRING {

mme	(0),

sgsn	(1)} (SIZE (1..8))

-- Other bits than listed above shall be discarded.

LocationInformation ::= SEQUENCE {

ageOfLocationInformation	AgeOfLocationInformation	OPTIONAL,

geographicalInformation	[0] GeographicalInformation	OPTIONAL,

vlr-number	[1] ISDN-AddressString	OPTIONAL,

locationNumber	[2] LocationNumber	OPTIONAL,

cellGlobalIdOrServiceAreaIdOrLAI	[3] CellGlobalIdOrServiceAreaIdOrLAI	OPTIONAL,

extensionContainer	[4] ExtensionContainer	OPTIONAL,

... ,

selectedLSA-Id	[5] LSAIdentity	OPTIONAL,

msc-Number	[6] ISDN-AddressString	OPTIONAL,

geodeticInformation	[7] GeodeticInformation	OPTIONAL,

currentLocationRetrieved	[8] NULL	OPTIONAL,

sai-Present	[9] NULL	OPTIONAL,

locationInformationEPS	[10] LocationInformationEPS	OPTIONAL,

userCSGInformation	[11] UserCSGInformation	OPTIONAL }

-- sai-Present indicates that the cellGlobalIdOrServiceAreaIdOrLAI parameter contains

-- a Service Area Identity.

-- currentLocationRetrieved shall be present

-- if the location information were retrieved after a successfull paging.

-- if the locationinformationEPS IE is present then the cellGlobalIdOrServiceAreaIdOrLAI IE,

-- the ageOfLocationInformation IE, the geographicalInformation IE, the geodeticInformation IE

-- and the currentLocationRetrieved IE (outside the locationInformationEPS IE) shall be

-- absent. As an exception, both the cellGlobalIdOrServiceAreaIdOrLAI IE including an LAI and

-- the locationinformationEPS IE may be present in a MAP-NOTE-MM-EVENT.

-- UserCSGInformation contains the CSG ID, Access mode, and the CSG Membership Indication in

-- the case the Access mode is Hybrid Mode.

-- The locationInformationEPS IE should be absent if locationInformationEPS-Supported was not

-- received in the RequestedInfo IE.

LocationInformationEPS ::= SEQUENCE {

e-utranCellGlobalIdentity	[0] E-UTRAN-CGI	OPTIONAL,

trackingAreaIdentity	[1] TA-Id	OPTIONAL,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

geographicalInformation	[3] GeographicalInformation	OPTIONAL,

geodeticInformation	[4] GeodeticInformation	OPTIONAL,

currentLocationRetrieved	[5] NULL	OPTIONAL,

ageOfLocationInformation	[6] AgeOfLocationInformation	OPTIONAL,

...,

mme-Name	[7] DiameterIdentity	OPTIONAL }

-- currentLocationRetrieved shall be present if the location information

-- was retrieved after successful paging.

LocationInformationGPRS ::= SEQUENCE {

cellGlobalIdOrServiceAreaIdOrLAI	[0] CellGlobalIdOrServiceAreaIdOrLAI OPTIONAL,

routeingAreaIdentity	[1] RAIdentity	OPTIONAL,

geographicalInformation	[2] GeographicalInformation	OPTIONAL,

sgsn-Number	[3] ISDN-AddressString	OPTIONAL,

selectedLSAIdentity	[4] LSAIdentity	OPTIONAL,

extensionContainer	[5] ExtensionContainer	OPTIONAL,

...,

sai-Present	[6] NULL	OPTIONAL,

geodeticInformation	[7] GeodeticInformation	OPTIONAL,

currentLocationRetrieved	[8] NULL	OPTIONAL,

ageOfLocationInformation	[9] AgeOfLocationInformation	OPTIONAL,

userCSGInformation	[10] UserCSGInformation	OPTIONAL }

-- sai-Present indicates that the cellGlobalIdOrServiceAreaIdOrLAI parameter contains

-- a Service Area Identity.

-- currentLocationRetrieved shall be present if the location information

-- was retrieved after successful paging.

-- UserCSGInformation contains the CSG ID, Access mode, and the CSG Membership Indication in

-- the case the Access mode is Hybrid Mode.

LocationInformation5GS ::= SEQUENCE {

nrCellGlobalIdentity	[0] NR-CGI	OPTIONAL,

e-utranCellGlobalIdentity	[1] E-UTRAN-CGI	OPTIONAL,

geographicalInformation	[2] GeographicalInformation	OPTIONAL,

geodeticInformation	[3] GeodeticInformation	OPTIONAL,

amf-address	[4] FQDN	OPTIONAL,

trackingAreaIdentity	[5] TA-Id	OPTIONAL,

currentLocationRetrieved	[6] NULL	OPTIONAL,

ageOfLocationInformation	[7] AgeOfLocationInformation	OPTIONAL,

vplmnId	[8] PLMN-Id	OPTIONAL,

localtimeZone	[9] TimeZone	OPTIONAL,

rat-Type	[10] Used-RAT-Type	OPTIONAL,

extensionContainer	[11] ExtensionContainer	OPTIONAL,

...,

nrTrackingAreaIdentity	[12] NR-TA-Id	OPTIONAL

}

-- currentLocationRetrieved shall be present if the location information

-- was retrieved after successful paging.

UserCSGInformation ::= SEQUENCE {

csg-Id	[0] CSG-Id,

extensionContainer	[1] ExtensionContainer	OPTIONAL,

...,

accessMode	[2] OCTET STRING (SIZE(1))	OPTIONAL,

cmi	[3] OCTET STRING (SIZE(1))	OPTIONAL }

-- The encoding of the accessMode and cmi parameters are as defined in 3GPP TS 29.060 [105].

GeographicalInformation ::= OCTET STRING (SIZE (8))

--	Refers to geographical Information defined in 3GPP TS 23.032.

--	Only the description of an ellipsoid point with uncertainty circle

--	as specified in 3GPP TS 23.032 is allowed to be used

--	The internal structure according to 3GPP TS 23.032 is as follows:

--	Type of shape (ellipsoid point with uncertainty circle)	1 octet

--	Degrees of Latitude	3 octets

--	Degrees of Longitude	3 octets

--	Uncertainty code	1 octet

GeodeticInformation ::= OCTET STRING (SIZE (10))

--	Refers to Calling Geodetic Location defined in Q.763 (1999).

--	Only the description of an ellipsoid point with uncertainty circle

--	as specified in Q.763 (1999) is allowed to be used

--	The internal structure according to Q.763 (1999) is as follows:

--	Screening and presentation indicators	1 octet

--	Type of shape (ellipsoid point with uncertainty circle)	1 octet

--	Degrees of Latitude	3 octets

--	Degrees of Longitude	3 octets

--	Uncertainty code	1 octet

--	Confidence	1 octet

LocationNumber ::= OCTET STRING (SIZE (2..10))

-- the internal structure is defined in ITU-T Rec Q.763

SubscriberState ::= CHOICE {

assumedIdle	[0] NULL,

camelBusy	[1] NULL,

netDetNotReachable	NotReachableReason,

notProvidedFromVLR	[2] NULL}

PS-SubscriberState ::= CHOICE {

notProvidedFromSGSNorMME	[0] NULL,

ps-Detached	[1] NULL,

ps-AttachedNotReachableForPaging	[2] NULL,

ps-AttachedReachableForPaging	[3] NULL,

ps-PDP-ActiveNotReachableForPaging	[4] PDP-ContextInfoList,

ps-PDP-ActiveReachableForPaging	[5] PDP-ContextInfoList,

netDetNotReachable	NotReachableReason }

PDP-ContextInfoList ::= SEQUENCE SIZE (1..maxNumOfPDP-Contexts) OF

PDP-ContextInfo

PDP-ContextInfo ::= SEQUENCE {

pdp-ContextIdentifier	[0] ContextId,

pdp-ContextActive	[1] NULL	OPTIONAL,

pdp-Type	[2] PDP-Type,

pdp-Address	[3] PDP-Address	OPTIONAL,

apn-Subscribed	[4] APN	OPTIONAL,

apn-InUse	[5] APN	OPTIONAL,

nsapi	[6] NSAPI	OPTIONAL,

transactionId	[7] TransactionId	OPTIONAL,

teid-ForGnAndGp	[8] TEID	OPTIONAL,

teid-ForIu	[9] TEID	OPTIONAL,

ggsn-Address	[10] GSN-Address	OPTIONAL,

qos-Subscribed	[11] Ext-QoS-Subscribed	OPTIONAL,

qos-Requested	[12] Ext-QoS-Subscribed	OPTIONAL,

qos-Negotiated	[13] Ext-QoS-Subscribed	OPTIONAL,

chargingId	[14] GPRSChargingID	OPTIONAL,

chargingCharacteristics	[15] ChargingCharacteristics	OPTIONAL,

rnc-Address	[16] GSN-Address	OPTIONAL,

extensionContainer	[17] ExtensionContainer	OPTIONAL,

...,

qos2-Subscribed	[18] Ext2-QoS-Subscribed	OPTIONAL,

-- qos2-Subscribed may be present only if qos-Subscribed is present.

qos2-Requested	[19] Ext2-QoS-Subscribed	OPTIONAL,

-- qos2-Requested may be present only if qos-Requested is present.

qos2-Negotiated	[20] Ext2-QoS-Subscribed	OPTIONAL,

-- qos2-Negotiated may be present only if qos-Negotiated is present.

qos3-Subscribed	[21] Ext3-QoS-Subscribed	OPTIONAL,

-- qos3-Subscribed may be present only if qos2-Subscribed is present.

qos3-Requested	[22] Ext3-QoS-Subscribed	OPTIONAL,

-- qos3-Requested may be present only if qos2-Requested is present.

qos3-Negotiated	[23] Ext3-QoS-Subscribed	OPTIONAL,

-- qos3-Negotiated may be present only if qos2-Negotiated is present.

qos4-Subscribed	[25] Ext4-QoS-Subscribed	OPTIONAL,

-- qos4-Subscribed may be present only if qos3-Subscribed is present.

qos4-Requested	[26] Ext4-QoS-Subscribed	OPTIONAL,

-- qos4-Requested may be present only if qos3-Requested is present.

qos4-Negotiated	[27] Ext4-QoS-Subscribed	OPTIONAL,

-- qos4-Negotiated may be present only if qos3-Negotiated is present.

ext-pdp-Type	[28] Ext-PDP-Type	OPTIONAL,

-- contains the value IPv4v6 defined in 3GPP TS 29.060 [105], if the PDP can be

-- accessed by dual-stack UEs.

ext-pdp-Address	[29] PDP-Address	OPTIONAL

-- contains an additional IP address in case of dual-stack static IP address assignment

-- for the UE.

-- it may contain an IPv4 or an IPv6 address/prefix, and it may be present

-- only if pdp-Address is present; if both are present, each parameter shall

-- contain a different type of address (IPv4 or IPv6).

}

NSAPI ::= INTEGER (0..15)

--	This type is used to indicate the Network layer Service Access Point

# TransactionId ::= OCTET STRING (SIZE (1..2))

--	This type carries the value part of the transaction identifier which is used in the

--	session management messages on the access interface. The encoding is defined in

--	3GPP TS 24.008

# TEID ::= OCTET STRING (SIZE (4))

--	This type carries the value part of the Tunnel Endpoint Identifier which is used to

--	distinguish between different tunnels between the same pair of entities which communicate

--	using the GPRS Tunnelling Protocol The encoding is defined in 3GPP TS 29.060.

# GPRSChargingID ::= OCTET STRING (SIZE (4))

--	The Charging ID is a unique four octet value generated by the GGSN when

--	a PDP Context is activated. A Charging ID is generated for each activated context.

--	The encoding is defined in 3GPP TS 29.060.

NotReachableReason ::= ENUMERATED {

msPurged (0),

imsiDetached (1),

restrictedArea (2),

notRegistered (3)}

-- any time interrogation info types

AnyTimeInterrogationArg ::= SEQUENCE {

subscriberIdentity	[0] SubscriberIdentity,

requestedInfo	[1] RequestedInfo,

gsmSCF-Address	[3] ISDN-AddressString,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

...}

AnyTimeInterrogationRes ::= SEQUENCE {

subscriberInfo	SubscriberInfo,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

-- any time information handling types

AnyTimeSubscriptionInterrogationArg ::= SEQUENCE {

subscriberIdentity	[0] SubscriberIdentity,

requestedSubscriptionInfo	[1] RequestedSubscriptionInfo,

gsmSCF-Address	[2] ISDN-AddressString,

extensionContainer	[3] ExtensionContainer	OPTIONAL,

longFTN-Supported	[4]	NULL	OPTIONAL,

...}

AnyTimeSubscriptionInterrogationRes ::= SEQUENCE {

callForwardingData	[1] CallForwardingData	OPTIONAL,

callBarringData	[2] CallBarringData	OPTIONAL,

odb-Info	[3] ODB-Info	OPTIONAL,

camel-SubscriptionInfo	[4] CAMEL-SubscriptionInfo	OPTIONAL,

supportedVLR-CAMEL-Phases	[5] SupportedCamelPhases	OPTIONAL,

supportedSGSN-CAMEL-Phases	[6] SupportedCamelPhases	OPTIONAL,

extensionContainer	[7] ExtensionContainer	OPTIONAL,

... ,

offeredCamel4CSIsInVLR	[8] OfferedCamel4CSIs	OPTIONAL,

offeredCamel4CSIsInSGSN	[9] OfferedCamel4CSIs	OPTIONAL,

msisdn-BS-List	[10] MSISDN-BS-List	OPTIONAL,

csg-SubscriptionDataList	[11] CSG-SubscriptionDataList	OPTIONAL,

cw-Data	[12]	CallWaitingData	OPTIONAL,

ch-Data	[13]	CallHoldData	OPTIONAL,

clip-Data	[14] ClipData	OPTIONAL,

clir-Data	[15]	ClirData	OPTIONAL,

ect-data	[16] EctData	OPTIONAL }

CallWaitingData ::= SEQUENCE {

cwFeatureList	[1] Ext-CwFeatureList,

notificationToCSE	[2] NULL	OPTIONAL,

... }

Ext-CwFeatureList ::= SEQUENCE SIZE (1..maxNumOfExt-BasicServiceGroups) OF

Ext-CwFeature

Ext-CwFeature ::= SEQUENCE {
	basicService	[1] Ext-BasicServiceCode,

ss-Status	[2] Ext-SS-Status,

... }

ClipData ::= SEQUENCE {

ss-Status	[1] Ext-SS-Status,

overrideCategory	[2] OverrideCategory,

notificationToCSE	[3] NULL	OPTIONAL,

... }

ClirData ::= SEQUENCE {

ss-Status	[1] Ext-SS-Status,

cliRestrictionOption	[2] CliRestrictionOption	OPTIONAL,

notificationToCSE	[3] NULL	OPTIONAL,

... }

CallHoldData ::= SEQUENCE {

ss-Status	[1] Ext-SS-Status,

notificationToCSE	[2] NULL	OPTIONAL,

... }

EctData ::= SEQUENCE {

ss-Status	[1] Ext-SS-Status,

notificationToCSE	[2] NULL	OPTIONAL,

... }

RequestedSubscriptionInfo ::= SEQUENCE {

requestedSS-Info	[1] SS-ForBS-Code	OPTIONAL,

odb	[2] NULL	OPTIONAL,

requestedCAMEL-SubscriptionInfo	[3] RequestedCAMEL-SubscriptionInfo	OPTIONAL,

supportedVLR-CAMEL-Phases	[4] NULL	OPTIONAL,

supportedSGSN-CAMEL-Phases	[5] NULL	OPTIONAL,

extensionContainer	[6] ExtensionContainer	OPTIONAL,

...,

additionalRequestedCAMEL-SubscriptionInfo

[7] AdditionalRequestedCAMEL-SubscriptionInfo

OPTIONAL,

msisdn-BS-List	[8] NULL	OPTIONAL,

csg-SubscriptionDataRequested	[9] NULL	OPTIONAL,

cw-Info	[10]	NULL	OPTIONAL,

clip-Info	[11] NULL	OPTIONAL,

clir-Info	[12] NULL	OPTIONAL,

hold-Info	[13] NULL	OPTIONAL,

ect-Info	[14] NULL	OPTIONAL }

MSISDN-BS-List ::= SEQUENCE SIZE (1..maxNumOfMSISDN) OF

MSISDN-BS

maxNumOfMSISDN  INTEGER ::= 50

MSISDN-BS ::= SEQUENCE {

msisdn	ISDN-AddressString,

basicServiceList	[0]	BasicServiceList	OPTIONAL,

extensionContainer	[1]	ExtensionContainer	OPTIONAL,

...}

RequestedCAMEL-SubscriptionInfo ::= ENUMERATED {

o-CSI	(0),

t-CSI	(1),

vt-CSI	(2),

tif-CSI	(3),

gprs-CSI	(4),

mo-sms-CSI	(5),

ss-CSI	(6),

m-CSI	(7),

d-csi	(8)}

AdditionalRequestedCAMEL-SubscriptionInfo ::= ENUMERATED {

mt-sms-CSI	(0),

mg-csi	(1),

o-IM-CSI	(2),

d-IM-CSI	(3),

vt-IM-CSI	(4),

...}

--	exception handling: unknown values shall be discarded by the receiver.

CallForwardingData ::= SEQUENCE {

forwardingFeatureList	Ext-ForwFeatureList,

notificationToCSE	NULL	OPTIONAL,

extensionContainer	[0] ExtensionContainer	OPTIONAL,

...}

CallBarringData ::= SEQUENCE {

callBarringFeatureList	Ext-CallBarFeatureList,

password	Password	OPTIONAL,

wrongPasswordAttemptsCounter	WrongPasswordAttemptsCounter	OPTIONAL,

notificationToCSE	NULL	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

WrongPasswordAttemptsCounter ::= INTEGER (0..4)

ODB-Info ::= SEQUENCE {

odb-Data	ODB-Data,

notificationToCSE	NULL	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

CAMEL-SubscriptionInfo ::= SEQUENCE {

o-CSI	[0]	O-CSI	OPTIONAL,

o-BcsmCamelTDP-CriteriaList	[1]	O-BcsmCamelTDPCriteriaList	OPTIONAL,

d-CSI	[2]	D-CSI	OPTIONAL,

t-CSI	[3]	T-CSI	OPTIONAL,

t-BCSM-CAMEL-TDP-CriteriaList	[4]	T-BCSM-CAMEL-TDP-CriteriaList	OPTIONAL,

vt-CSI	[5]	T-CSI	OPTIONAL,

vt-BCSM-CAMEL-TDP-CriteriaList	[6]	T-BCSM-CAMEL-TDP-CriteriaList	OPTIONAL,

tif-CSI	[7]	NULL	OPTIONAL,

tif-CSI-NotificationToCSE	[8]	NULL	OPTIONAL,

gprs-CSI	[9]	GPRS-CSI	OPTIONAL,

mo-sms-CSI	[10]	SMS-CSI	OPTIONAL,

ss-CSI	[11]	SS-CSI	OPTIONAL,

m-CSI	[12]	M-CSI	OPTIONAL,

extensionContainer	[13]	ExtensionContainer	OPTIONAL,

...,

specificCSIDeletedList	[14]	SpecificCSI-Withdraw	OPTIONAL,

mt-sms-CSI	[15]	SMS-CSI	OPTIONAL,

mt-smsCAMELTDP-CriteriaList	[16]	MT-smsCAMELTDP-CriteriaList	OPTIONAL,

mg-csi	[17]	MG-CSI	OPTIONAL,

o-IM-CSI	[18] O-CSI	OPTIONAL,

o-IM-BcsmCamelTDP-CriteriaList	[19] O-BcsmCamelTDPCriteriaList	OPTIONAL,

d-IM-CSI	[20] D-CSI	OPTIONAL,

vt-IM-CSI	[21] T-CSI	OPTIONAL,

vt-IM-BCSM-CAMEL-TDP-CriteriaList	[22]	T-BCSM-CAMEL-TDP-CriteriaList	OPTIONAL

}

# AnyTimeModificationArg ::= SEQUENCE {

subscriberIdentity	[0]	SubscriberIdentity,

gsmSCF-Address	[1]	ISDN-AddressString,

modificationRequestFor-CF-Info	[2]	ModificationRequestFor-CF-Info OPTIONAL,

modificationRequestFor-CB-Info	[3]	ModificationRequestFor-CB-Info OPTIONAL,

modificationRequestFor-CSI	[4]	ModificationRequestFor-CSI	OPTIONAL,

extensionContainer	[5]	ExtensionContainer	OPTIONAL,

longFTN-Supported	[6]	NULL	OPTIONAL,

...,

modificationRequestFor-ODB-data	[7]	ModificationRequestFor-ODB-data OPTIONAL,

modificationRequestFor-IP-SM-GW-Data	[8]	ModificationRequestFor-IP-SM-GW-Data OPTIONAL,

activationRequestForUE-reachability	[9]	RequestedServingNode	OPTIONAL,

modificationRequestFor-CSG	[10]	ModificationRequestFor-CSG	OPTIONAL,

modificationRequestFor-CW-Data	[11] ModificationRequestFor-CW-Info	OPTIONAL,

modificationRequestFor-CLIP-Data	[12] ModificationRequestFor-CLIP-Info	OPTIONAL,

modificationRequestFor-CLIR-Data	[13] ModificationRequestFor-CLIR-Info	OPTIONAL,

modificationRequestFor-HOLD-Data	[14] ModificationRequestFor-CH-Info	OPTIONAL,

modificationRequestFor-ECT-Data	[15] ModificationRequestFor-ECT-Info	OPTIONAL }

ModificationRequestFor-CW-Info ::= SEQUENCE {

basicService	[0]	Ext-BasicServiceCode	OPTIONAL,

ss-Status	[1]	Ext-SS-Status	OPTIONAL,

modifyNotificationToCSE	[2]	ModificationInstruction	OPTIONAL,

extensionContainer	[3]	ExtensionContainer	OPTIONAL,

...}

ModificationRequestFor-CH-Info ::= SEQUENCE {

ss-Status	[0]	Ext-SS-Status	OPTIONAL,

modifyNotificationToCSE	[1]	ModificationInstruction	OPTIONAL,

extensionContainer	[2]	ExtensionContainer	OPTIONAL,

...}

ModificationRequestFor-ECT-Info ::= SEQUENCE {

ss-Status	[0]	Ext-SS-Status	OPTIONAL,

modifyNotificationToCSE	[1]	ModificationInstruction	OPTIONAL,

extensionContainer	[2]	ExtensionContainer	OPTIONAL,

...}

ModificationRequestFor-CLIR-Info ::= SEQUENCE {

ss-Status	[0]	Ext-SS-Status	OPTIONAL,

cliRestrictionOption	[1]  CliRestrictionOption	OPTIONAL,

modifyNotificationToCSE	[2]	ModificationInstruction	OPTIONAL,

extensionContainer	[3]	ExtensionContainer	OPTIONAL,

...}

ModificationRequestFor-CLIP-Info ::= SEQUENCE {

ss-Status	[0]	Ext-SS-Status	OPTIONAL,

overrideCategory	[1]  OverrideCategory	OPTIONAL,

modifyNotificationToCSE	[2]	ModificationInstruction	OPTIONAL,

extensionContainer	[3]	ExtensionContainer	OPTIONAL,

...}

ModificationRequestFor-CSG ::= SEQUENCE {

modifyNotificationToCSE	[0]	ModificationInstruction	OPTIONAL,

extensionContainer	[1]	ExtensionContainer	OPTIONAL,

...}

RequestedServingNode ::= BIT STRING {

mmeAndSgsn  (0)} (SIZE (1..8))

ServingNode ::= BIT STRING {

mme (0),

sgsn (1)} (SIZE (2..8))

-- Other bits than listed above shall be discarded.

# AnyTimeModificationRes ::= SEQUENCE {

ss-InfoFor-CSE	[0]	Ext-SS-InfoFor-CSE	OPTIONAL,

camel-SubscriptionInfo	[1]	CAMEL-SubscriptionInfo	OPTIONAL,

extensionContainer	[2]	ExtensionContainer	OPTIONAL,

...,

odb-Info	[3]	ODB-Info	OPTIONAL,

cw-Data	[4]	CallWaitingData	OPTIONAL,

ch-Data	[5]	CallHoldData	OPTIONAL,

clip-Data	[6] ClipData	OPTIONAL,

clir-Data	[7]	ClirData	OPTIONAL,

ect-data	[8] EctData	OPTIONAL,

serviceCentreAddress	[9] AddressString	OPTIONAL

}

ModificationRequestFor-CF-Info ::= SEQUENCE {

ss-Code	[0]	SS-Code,

basicService	[1]	Ext-BasicServiceCode	OPTIONAL,

ss-Status	[2]	Ext-SS-Status	OPTIONAL,

forwardedToNumber	[3]	AddressString	OPTIONAL,

forwardedToSubaddress	[4]	ISDN-SubaddressString	OPTIONAL,

noReplyConditionTime	[5]	Ext-NoRepCondTime	OPTIONAL,

modifyNotificationToCSE	[6]	ModificationInstruction	OPTIONAL,

extensionContainer	[7]	ExtensionContainer	OPTIONAL,

...}

ModificationRequestFor-CB-Info ::= SEQUENCE {

ss-Code	[0]	SS-Code,

basicService	[1]	Ext-BasicServiceCode	OPTIONAL,

ss-Status	[2]	Ext-SS-Status	OPTIONAL,

password	[3]	Password	OPTIONAL,

wrongPasswordAttemptsCounter	[4]	WrongPasswordAttemptsCounter	OPTIONAL,

modifyNotificationToCSE	[5]	ModificationInstruction	OPTIONAL,

extensionContainer	[6]	ExtensionContainer	OPTIONAL,

...}

ModificationRequestFor-ODB-data ::= SEQUENCE {

odb-data	[0]	ODB-Data	OPTIONAL,

modifyNotificationToCSE	[1]	ModificationInstruction	OPTIONAL,

extensionContainer	[2]	ExtensionContainer	OPTIONAL,

...}

ModificationRequestFor-CSI ::= SEQUENCE {

requestedCamel-SubscriptionInfo	[0]	RequestedCAMEL-SubscriptionInfo,

modifyNotificationToCSE	[1]	ModificationInstruction	OPTIONAL,

modifyCSI-State	[2]	ModificationInstruction	OPTIONAL,

extensionContainer	[3]	ExtensionContainer	OPTIONAL,

...,

additionalRequestedCAMEL-SubscriptionInfo

[4] AdditionalRequestedCAMEL-SubscriptionInfo

OPTIONAL }

-- requestedCamel-SubscriptionInfo shall be discarded if

-- additionalRequestedCAMEL-SubscriptionInfo is received

ModificationRequestFor-IP-SM-GW-Data ::= SEQUENCE {

modifyRegistrationStatus	[0]	ModificationInstruction	OPTIONAL,

extensionContainer	[1]	ExtensionContainer	OPTIONAL,

...,

ip-sm-gw-DiameterAddress	[2]	NetworkNodeDiameterAddress	OPTIONAL

-- ip-sm-gw-DiameterAddress may be present when ModificationInstruction is "activate"

}

ModificationInstruction ::= ENUMERATED {

deactivate	(0),

activate	(1)}

-- subscriber data modification notification types

NoteSubscriberDataModifiedArg ::= SEQUENCE {

imsi	IMSI,

msisdn	ISDN-AddressString,

forwardingInfoFor-CSE	[0] Ext-ForwardingInfoFor-CSE	OPTIONAL,

callBarringInfoFor-CSE	[1] Ext-CallBarringInfoFor-CSE	OPTIONAL,

odb-Info	[2] ODB-Info	OPTIONAL,

camel-SubscriptionInfo	[3] CAMEL-SubscriptionInfo	OPTIONAL,

allInformationSent	[4] NULL	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

ue-reachable	[5] ServingNode	OPTIONAL,

csg-SubscriptionDataList	[6] CSG-SubscriptionDataList	OPTIONAL,

cw-Data	[7]	CallWaitingData	OPTIONAL,

ch-Data	[8]	CallHoldData	OPTIONAL,

clip-Data	[9] ClipData	OPTIONAL,

clir-Data	[10]	ClirData	OPTIONAL,

ect-data	[11] EctData	OPTIONAL }

NoteSubscriberDataModifiedRes ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

-- mobility management event notificatioon info types

# NoteMM-EventArg::= SEQUENCE {

serviceKey	ServiceKey,

eventMet	[0]	MM-Code,

imsi	[1]	IMSI,

msisdn	[2]	ISDN-AddressString,

locationInformation	[3]	LocationInformation	OPTIONAL,

supportedCAMELPhases	[5]	SupportedCamelPhases	OPTIONAL,

extensionContainer	[6]	ExtensionContainer	OPTIONAL,

...,

locationInformationGPRS	[7]	LocationInformationGPRS	OPTIONAL,

offeredCamel4Functionalities	[8] OfferedCamel4Functionalities	OPTIONAL

}

# NoteMM-EventRes ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

Ext-SS-InfoFor-CSE ::= CHOICE {

forwardingInfoFor-CSE	[0] Ext-ForwardingInfoFor-CSE,

callBarringInfoFor-CSE	[1] Ext-CallBarringInfoFor-CSE

}

Ext-ForwardingInfoFor-CSE ::= SEQUENCE {

ss-Code	[0]	SS-Code,

forwardingFeatureList	[1]	Ext-ForwFeatureList,

notificationToCSE	[2]	NULL	OPTIONAL,

extensionContainer	[3]	ExtensionContainer	OPTIONAL,

...}

Ext-CallBarringInfoFor-CSE ::= SEQUENCE {

ss-Code	[0]	SS-Code,

callBarringFeatureList	[1]	Ext-CallBarFeatureList,

password	[2]	Password	OPTIONAL,

wrongPasswordAttemptsCounter	[3]	WrongPasswordAttemptsCounter	OPTIONAL,

notificationToCSE	[4]	NULL	OPTIONAL,

extensionContainer	[5]	ExtensionContainer	OPTIONAL,

...}

-- vcsg location registration types

# UpdateVcsgLocationArg ::= SEQUENCE {

imsi	IMSI,

msisdn	[2] ISDN-AddressString	OPTIONAL,

vlr-Number	[0] ISDN-AddressString	OPTIONAL,

sgsn-Number	[1] ISDN-AddressString	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

... }

UpdateVcsgLocationRes ::= SEQUENCE {

temporaryEmptySubscriptiondataIndicator	NULL	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

... }

CancelVcsgLocationArg ::= SEQUENCE {

identity	Identity,

extensionContainer	ExtensionContainer	OPTIONAL,

...

}

CancelVcsgLocationRes ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

... }

.#END

* * * Next Change * * * *

### 17.7.2	Operation and maintenance data types

.$MAP-OM-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-OM-DataTypes (12) version22 (22)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

ActivateTraceModeArg,

ActivateTraceModeRes,

DeactivateTraceModeArg,

DeactivateTraceModeRes,

TracePropagationList

;

IMPORTS

AddressString,

IMSI,

GSN-Address,

GlobalCellId,

# E-UTRAN-CGI,

# TA-Id,

# RAIdentity,

LAIFixedLength,

PLMN-Id

FROM MAP-CommonDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version22 (22)}

ExtensionContainer

FROM MAP-ExtensionDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ExtensionDataTypes (21) version22 (22)}

;

ActivateTraceModeArg ::= SEQUENCE {

imsi	[0] IMSI	OPTIONAL,

traceReference	[1] TraceReference,

traceType	[2] TraceType,

omc-Id	[3] AddressString	OPTIONAL,

extensionContainer	[4] ExtensionContainer	OPTIONAL,

...,

traceReference2	[5] TraceReference2	OPTIONAL,

traceDepthList	[6] TraceDepthList	OPTIONAL,

traceNE-TypeList	[7] TraceNE-TypeList	OPTIONAL,

traceInterfaceList	[8] TraceInterfaceList	OPTIONAL,

traceEventList	[9] TraceEventList	OPTIONAL,

traceCollectionEntity	[10] GSN-Address	OPTIONAL,

mdt-Configuration	[11] MDT-Configuration	OPTIONAL

}

MDT-Configuration ::= SEQUENCE {

jobType	JobType,

areaScope	AreaScope	OPTIONAL,

listOfMeasurements	ListOfMeasurements	OPTIONAL,

reportingTrigger	[0] ReportingTrigger	OPTIONAL,

reportInterval	ReportInterval	OPTIONAL,

reportAmount	[1] ReportAmount	OPTIONAL,

eventThresholdRSRP	EventThresholdRSRP	OPTIONAL,

eventThresholdRSRQ	[2] EventThresholdRSRQ	OPTIONAL,

loggingInterval	[3] LoggingInterval	OPTIONAL,

loggingDuration	[4] LoggingDuration	OPTIONAL,

extensionContainer	[5] ExtensionContainer	OPTIONAL,

...,

measurementPeriodUMTS	[6] PeriodUMTS	OPTIONAL,

measurementPeriodLTE	[7] PeriodLTE	OPTIONAL,

collectionPeriodRRM-UMTS	[8] PeriodUMTS	OPTIONAL,

collectionPeriodRRM-LTE	[9] PeriodLTE	OPTIONAL,

positioningMethod	[10] PositioningMethod	OPTIONAL,

measurementQuantity	[11] MeasurementQuantity	OPTIONAL,

eventThreshold1F	[12] EventThreshold1F	OPTIONAL,

eventThreshold1I	[13] EventThreshold1I	OPTIONAL,

mdt-Allowed-PLMN-List	[14] MDT-Allowed-PLMNId-List	OPTIONAL }

MDT-Allowed-PLMNId-List ::= SEQUENCE SIZE (1..16) OF

PLMN-Id

PeriodUMTS ::= ENUMERATED {

d250ms (0),

d500ms (1),

d1000ms (2),

d2000ms (3),

d3000ms (4),

d4000ms (5),

d6000ms (6),

d8000ms (7),

d12000ms (8),

d16000ms (9),

d20000ms (10),

d24000ms (11),

d28000ms (12),

d32000ms (13),

d64000ms (14)}

PeriodLTE ::= ENUMERATED {

d1024ms (0),

d1280ms (1),

d2048ms (2),

d2560ms (3),

d5120ms (4),

d10240ms (5),

d1min (6)}

PositioningMethod ::= OCTET STRING (SIZE (1))

-- Octet is coded as described in 3GPP TS 32.422 [132].

MeasurementQuantity ::= OCTET STRING (SIZE (1))

-- Octet is coded as described in 3GPP TS 32.422 [132].

EventThreshold1F ::= INTEGER

(-120..165)

EventThreshold1I ::= INTEGER

(-120..-25)

JobType ::= ENUMERATED {

immediate-MDT-only (0),

logged-MDT-only (1),

trace-only (2),

immediate-MDT-and-trace (3)}

AreaScope ::= SEQUENCE {

cgi-List	[0] CGI-List	OPTIONAL,

e-utran-cgi-List	[1] E-UTRAN-CGI-List	OPTIONAL,

routingAreaId-List	[2] RoutingAreaId-List	OPTIONAL,

locationAreaId-List	[3] LocationAreaId-List	OPTIONAL,

trackingAreaId-List	[4] TrackingAreaId-List	OPTIONAL,

extensionContainer	[5] ExtensionContainer	OPTIONAL,

... }

CGI-List ::= SEQUENCE SIZE (1..32) OF

GlobalCellId

E-UTRAN-CGI-List ::= SEQUENCE SIZE (1..32) OF

E-UTRAN-CGI

RoutingAreaId-List ::= SEQUENCE SIZE (1..8) OF

RAIdentity

LocationAreaId-List ::= SEQUENCE SIZE (1..8) OF

LAIFixedLength

TrackingAreaId-List ::= SEQUENCE SIZE (1..8) OF

TA-Id

ListOfMeasurements ::= OCTET STRING (SIZE (4))

-- Octets are coded as described in 3GPP TS 32.422.

ReportingTrigger ::= OCTET STRING (SIZE (1))

-- Octet is coded as described in 3GPP TS 32.422.

ReportInterval ::= ENUMERATED {

umts250ms (0),

umts500ms (1),

umts1000ms (2),

umts2000ms (3),

umts3000ms (4),

umts4000ms (5),

umts6000ms (6),

umts8000ms (7),

umts12000ms (8),

umts16000ms (9),

umts20000ms (10),

umts24000ms (11),

umts28000ms (12),

umts32000ms (13),

umts64000ms (14),

lte120ms (15),

lte240ms (16),

lte480ms (17),

lte640ms (18),

lte1024ms (19),

lte2048ms (20),

lte5120ms (21),

lte10240ms (22),

lte1min (23),

lte6min (24),

lte12min (25),

lte30min (26),

lte60min (27)}

ReportAmount ::= ENUMERATED {

d1 (0),

d2 (1),

d4 (2),

d8 (3),

d16 (4),

d32 (5),

d64 (6),

infinity (7)}

EventThresholdRSRP ::= INTEGER

(0..97)

EventThresholdRSRQ ::= INTEGER

(0..34)

LoggingInterval ::= ENUMERATED {

d1dot28 (0),

d2dot56 (1),

d5dot12 (2),

d10dot24 (3),

d20dot48 (4),

d30dot72 (5),

d40dot96 (6),

d61dot44 (7)}

LoggingDuration ::= ENUMERATED {

d600sec (0),

d1200sec (1),

d2400sec (2),

d3600sec (3),

d5400sec (4),

d7200sec (5)}

TraceReference ::= OCTET STRING (SIZE (1..2))

TraceReference2 ::= OCTET STRING (SIZE (3))

TraceRecordingSessionReference ::= OCTET STRING (SIZE (2))

TraceType ::= INTEGER

(0..255)

-- Trace types are fully defined in  3GPP TS 52.008. [61]

TraceDepthList ::= SEQUENCE {

msc-s-TraceDepth	[0] TraceDepth	OPTIONAL,

mgw-TraceDepth	[1] TraceDepth	OPTIONAL,

sgsn-TraceDepth	[2] TraceDepth	OPTIONAL,

ggsn-TraceDepth	[3] TraceDepth	OPTIONAL,

rnc-TraceDepth	[4] TraceDepth	OPTIONAL,

bmsc-TraceDepth	[5] TraceDepth	OPTIONAL,

... ,

mme-TraceDepth	[6] TraceDepth	OPTIONAL,

sgw-TraceDepth	[7] TraceDepth	OPTIONAL,

pgw-TraceDepth	[8] TraceDepth	OPTIONAL,

eNB-TraceDepth	[9] TraceDepth	OPTIONAL,

msc-s-TraceDepthExtension	[10] TraceDepthExtension	OPTIONAL,

mgw-TraceDepthExtension	[11] TraceDepthExtension	OPTIONAL,

sgsn-TraceDepthExtension	[12] TraceDepthExtension	OPTIONAL,

ggsn-TraceDepthExtension	[13] TraceDepthExtension	OPTIONAL,

rnc-TraceDepthExtension	[14] TraceDepthExtension	OPTIONAL,

bmsc-TraceDepthExtension	[15] TraceDepthExtension	OPTIONAL,

mme-TraceDepthExtension	[16] TraceDepthExtension	OPTIONAL,

sgw-TraceDepthExtension	[17] TraceDepthExtension	OPTIONAL,

pgw-TraceDepthExtension	[18] TraceDepthExtension	OPTIONAL,

eNB-TraceDepthExtension	[19] TraceDepthExtension	OPTIONAL }

-- If one of the TraceDepthExtension types is sent, the corresponding TraceDepth type

-- shall also be sent with the same enumeration value to allow the receiver not supporting

-- the Extension to fall back to the non extended type.

-- If one of the TraceDepthExtension types is received and supported, the corresponding

-- TraceDepth type shall be ignored.

TraceDepth ::= ENUMERATED {

minimum (0),

medium (1),

maximum (2),

...}

-- The value medium is applicable only for RNC. For other network elements, if value medium

-- is received, value minimum shall be applied.

TraceDepthExtension ::= ENUMERATED {

minimumWithoutVendorSpecificExtension (0),

mediumWithoutVendorSpecificExtension (1),

maximumWithoutVendorSpecificExtension (2),

...}

-- The value mediumWithoutVendorSpecificExtension is applicable only for RNC. For other

-- network elements, if value mediumWithoutVendorSpecificExtension is received, value

-- minimumWithoutVendorSpecificExtension shall be applied.

TraceNE-TypeList ::= BIT STRING {

msc-s (0),

mgw (1),

sgsn (2),

ggsn (3),

rnc (4),

bm-sc (5) ,

mme (6),

sgw (7),

pgw (8),

eNB (9)} (SIZE (6..16))

-- Other bits than listed above shall be discarded.

TraceInterfaceList ::= SEQUENCE {

msc-s-List	[0] MSC-S-InterfaceList	OPTIONAL,

mgw-List	[1] MGW-InterfaceList	OPTIONAL,

sgsn-List	[2] SGSN-InterfaceList	OPTIONAL,

ggsn-List	[3] GGSN-InterfaceList	OPTIONAL,

rnc-List	[4] RNC-InterfaceList	OPTIONAL,

bmsc-List	[5] BMSC-InterfaceList	OPTIONAL,

...,

mme-List	[6] MME-InterfaceList	OPTIONAL,

sgw-List	[7] SGW-InterfaceList	OPTIONAL,

pgw-List	[8] PGW-InterfaceList	OPTIONAL,

eNB-List	[9] ENB-InterfaceList	OPTIONAL}

MSC-S-InterfaceList ::= BIT STRING {

a (0),

iu (1),

mc (2),

map-g (3),

map-b (4),

map-e (5),

map-f (6),

cap (7),

map-d (8),

map-c (9)} (SIZE (10..16))

-- Other bits than listed above shall be discarded.

MGW-InterfaceList ::= BIT STRING {

mc (0),

nb-up (1),

iu-up (2)} (SIZE (3..8))

-- Other bits than listed above shall be discarded.

SGSN-InterfaceList ::= BIT STRING {

gb (0),

iu (1),

gn (2),

map-gr (3),

map-gd (4),

map-gf (5),

gs (6),

ge (7),

s3 (8),

s4 (9),

s6d (10)} (SIZE (8..16))

-- Other bits than listed above shall be discarded.

GGSN-InterfaceList ::= BIT STRING {

gn (0),

gi (1),

gmb (2)} (SIZE (3..8))

-- Other bits than listed above shall be discarded.

RNC-InterfaceList ::= BIT STRING {

iu (0),

iur (1),

iub (2),

uu (3)} (SIZE (4..8))

-- Other bits than listed above shall be discarded.

BMSC-InterfaceList ::= BIT STRING {

gmb (0)} (SIZE (1..8))

-- Other bits than listed above shall be discarded.

MME-InterfaceList ::= BIT STRING {

s1-mme (0),

s3 (1),

s6a (2),

s10 (3),

s11 (4)} (SIZE (5..8))

-- Other bits than listed above shall be discarded.

SGW-InterfaceList ::= BIT STRING {

s4 (0),

s5 (1),

s8b (2),

s11 (3),

gxc (4)} (SIZE (5..8))

-- Other bits than listed above shall be discarded.

PGW-InterfaceList ::= BIT STRING {

s2a (0),

s2b (1),

s2c (2),

s5 (3),

s6b (4),

gx (5),

s8b (6),

sgi (7)} (SIZE (8..16))

-- Other bits than listed above shall be discarded.

ENB-InterfaceList ::= BIT STRING {

s1-mme (0),

x2 (1),

uu (2)} (SIZE (3..8))

-- Other bits than listed above shall be discarded.

TraceEventList ::= SEQUENCE {

msc-s-List	[0] MSC-S-EventList	OPTIONAL,

mgw-List	[1] MGW-EventList	OPTIONAL,

sgsn-List	[2] SGSN-EventList	OPTIONAL,

ggsn-List	[3] GGSN-EventList	OPTIONAL,

bmsc-List	[4] BMSC-EventList	OPTIONAL,

...,

mme-List	[5] MME-EventList	OPTIONAL,

sgw-List	[6] SGW-EventList	OPTIONAL,

pgw-List	[7] PGW-EventList	OPTIONAL}

MSC-S-EventList ::= BIT STRING {

mo-mtCall (0),

mo-mt-sms (1),

lu-imsiAttach-imsiDetach (2),

handovers (3),

ss (4)} (SIZE (5..16))

-- Other bits than listed above shall be discarded.

MGW-EventList ::= BIT STRING {

context (0)} (SIZE (1..8))

-- Other bits than listed above shall be discarded.

SGSN-EventList ::= BIT STRING {

pdpContext (0),

mo-mt-sms (1),

rau-gprsAttach-gprsDetach (2),

mbmsContext (3)} (SIZE (4..16))

-- Other bits than listed above shall be discarded.

GGSN-EventList ::= BIT STRING {

pdpContext (0),

mbmsContext (1)} (SIZE (2..8))

-- Other bits than listed above shall be discarded.

BMSC-EventList ::= BIT STRING {

mbmsMulticastServiceActivation (0)} (SIZE (1..8))

-- Other bits than listed above shall be discarded.

MME-EventList ::= BIT STRING {

ue-initiatedPDNconectivityRequest (0),

serviceRequestts (1),

initialAttachTrackingAreaUpdateDetach (2),

ue-initiatedPDNdisconnection (3),

bearerActivationModificationDeletion (4),

handover (5)} (SIZE (6..8))

-- Other bits than listed above shall be discarded.

SGW-EventList ::= BIT STRING {

pdn-connectionCreation (0),

pdn-connectionTermination (1),

bearerActivationModificationDeletion (2)} (SIZE (3..8))

-- Other bits than listed above shall be discarded.

PGW-EventList ::= BIT STRING {

pdn-connectionCreation (0),

pdn-connectionTermination (1),

bearerActivationModificationDeletion (2)} (SIZE (3..8))

-- Other bits than listed above shall be discarded.

TracePropagationList ::= SEQUENCE {

traceReference	[0] TraceReference	OPTIONAL,

traceType	[1] TraceType	OPTIONAL,

traceReference2	[2] TraceReference2	OPTIONAL,

traceRecordingSessionReference	[3] TraceRecordingSessionReference OPTIONAL,

rnc-TraceDepth	[4] TraceDepth	OPTIONAL,

rnc-InterfaceList	[5] RNC-InterfaceList	OPTIONAL,

msc-s-TraceDepth	[6] TraceDepth	OPTIONAL,

msc-s-InterfaceList	[7] MSC-S-InterfaceList	OPTIONAL,

msc-s-EventList	[8] MSC-S-EventList	OPTIONAL,

mgw-TraceDepth	[9] TraceDepth	OPTIONAL,

mgw-InterfaceList	[10] MGW-InterfaceList	OPTIONAL,

mgw-EventList	[11] MGW-EventList	OPTIONAL,

...,

rnc-TraceDepthExtension	[12] TraceDepthExtension	OPTIONAL,

msc-s-TraceDepthExtension	[13] TraceDepthExtension	OPTIONAL,

mgw-TraceDepthExtension	[14] TraceDepthExtension	OPTIONAL

}

-- If one of the TraceDepthExtension types is sent, the corresponding TraceDepth type

-- shall also be sent with the same enumeration value to allow the receiver not supporting

-- the Extension to fall back to the non extended type.

-- If one of the TraceDepthExtension types is received and supported, the corresponding

-- TraceDepth type shall be ignored.

ActivateTraceModeRes ::= SEQUENCE {

extensionContainer	[0] ExtensionContainer	OPTIONAL,

...,

traceSupportIndicator	[1]	NULL	OPTIONAL

}

DeactivateTraceModeArg ::= SEQUENCE {

imsi	[0] IMSI	OPTIONAL,

traceReference	[1] TraceReference,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

...,

traceReference2	[3] TraceReference2	OPTIONAL

}

DeactivateTraceModeRes ::= SEQUENCE {

extensionContainer	[0] ExtensionContainer	OPTIONAL,

...}

.#END

* * * Next Change * * * *

### 17.7.3	Call handling data types

.$MAP-CH-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CH-DataTypes (13) version22 (22)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

SendRoutingInfoArg,

SendRoutingInfoRes,

ProvideRoamingNumberArg,

ProvideRoamingNumberRes,

ResumeCallHandlingArg,

ResumeCallHandlingRes,

NumberOfForwarding,

SuppressionOfAnnouncement,

CallReferenceNumber,

SetReportingStateArg,

SetReportingStateRes,

StatusReportArg,

StatusReportRes,

RemoteUserFreeArg,

RemoteUserFreeRes,

IST-AlertArg,

IST-AlertRes,

IST-CommandArg,

IST-CommandRes,

UU-Data,

ReleaseResourcesArg,

ReleaseResourcesRes

;

IMPORTS

SubscriberInfo,

SupportedCamelPhases,

OfferedCamel4CSIs,

CUG-Interlock,

O-CSI,

D-CSI,

O-BcsmCamelTDPCriteriaList,

T-BCSM-CAMEL-TDP-CriteriaList,

IST-SupportIndicator,

IST-AlertTimerValue,

T-CSI,

NumberPortabilityStatus,

PagingArea

FROM MAP-MS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version22 (22)}

ForwardingOptions,

SS-List,

CCBS-Feature

FROM MAP-SS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-DataTypes (14) version22 (22)}

ISDN-AddressString,

ISDN-SubaddressString,

FTN-AddressString,

ExternalSignalInfo,

Ext-ExternalSignalInfo,

IMSI,

LMSI,

Ext-BasicServiceCode,

AlertingPattern,

NAEA-PreferredCI,

EMLPP-Priority,

PLMN-Id

FROM MAP-CommonDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version22 (22)}

ExtensionContainer

FROM MAP-ExtensionDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ExtensionDataTypes (21) version22 (22)}

;

CUG-CheckInfo ::= SEQUENCE {

cug-Interlock	CUG-Interlock,

cug-OutgoingAccess	NULL	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

NumberOfForwarding ::= INTEGER (1..5)

SendRoutingInfoArg ::= SEQUENCE {

msisdn	[0] ISDN-AddressString,

cug-CheckInfo	[1] CUG-CheckInfo	OPTIONAL,

numberOfForwarding	[2] NumberOfForwarding	OPTIONAL,

interrogationType	[3] InterrogationType,

or-Interrogation	[4] NULL	OPTIONAL,

or-Capability	[5] OR-Phase	OPTIONAL,

gmsc-OrGsmSCF-Address	[6] ISDN-AddressString,

callReferenceNumber	[7] CallReferenceNumber	OPTIONAL,

forwardingReason	[8] ForwardingReason	OPTIONAL,

basicServiceGroup	[9] Ext-BasicServiceCode	OPTIONAL,

networkSignalInfo	[10] ExternalSignalInfo	OPTIONAL,

camelInfo	[11] CamelInfo	OPTIONAL,

suppressionOfAnnouncement	[12] SuppressionOfAnnouncement	OPTIONAL,

extensionContainer	[13] ExtensionContainer	OPTIONAL,

...,

alertingPattern	[14] AlertingPattern	OPTIONAL,

ccbs-Call	[15] NULL	OPTIONAL,

supportedCCBS-Phase	[16]	SupportedCCBS-Phase	OPTIONAL,

additionalSignalInfo	[17] Ext-ExternalSignalInfo	OPTIONAL,

istSupportIndicator	[18] IST-SupportIndicator	OPTIONAL,

pre-pagingSupported	[19]	NULL	OPTIONAL,

callDiversionTreatmentIndicator	[20]	CallDiversionTreatmentIndicator	OPTIONAL,

longFTN-Supported	[21]	NULL	OPTIONAL,

suppress-VT-CSI	[22]	NULL	OPTIONAL,

suppressIncomingCallBarring	[23]	NULL	OPTIONAL,

gsmSCF-InitiatedCall	[24]	NULL	OPTIONAL,

basicServiceGroup2	[25] Ext-BasicServiceCode	OPTIONAL,

networkSignalInfo2	[26] ExternalSignalInfo	OPTIONAL,

suppressMTSS	[27] SuppressMTSS	OPTIONAL,

mtRoamingRetrySupported	[28] NULL	OPTIONAL,

callPriority	[29]	EMLPP-Priority	OPTIONAL

}

SuppressionOfAnnouncement ::= NULL

# SuppressMTSS ::= BIT STRING {

suppressCUG	(0),

suppressCCBS	(1) } (SIZE (2..16))

--	Other bits than listed above shall be discarded

InterrogationType ::= ENUMERATED {

basicCall  (0),

forwarding  (1)}

OR-Phase ::= INTEGER (1..127)

CallReferenceNumber ::= OCTET STRING (SIZE (1..8))

ForwardingReason ::= ENUMERATED {

notReachable  (0),

busy  (1),

noReply  (2)}

SupportedCCBS-Phase ::= INTEGER (1..127)

-- exception handling:

-- Only value 1 is used.

-- Values in the ranges 2-127 are reserved for future use.

-- If received values 2-127 shall be mapped on to value 1.

CallDiversionTreatmentIndicator ::= OCTET STRING (SIZE(1))

--	callDiversionAllowed (xxxx xx01)

--	callDiversionNotAllowed (xxxx xx10)

--	network default is call diversion allowed

SendRoutingInfoRes ::= [3] SEQUENCE {

imsi	[9] IMSI	OPTIONAL,

-- IMSI must be present if SendRoutingInfoRes is not segmented.

-- If the TC-Result-NL segmentation option is taken the IMSI must be

-- present in one segmented transmission of SendRoutingInfoRes.

extendedRoutingInfo	ExtendedRoutingInfo	OPTIONAL,

cug-CheckInfo	[3] CUG-CheckInfo	OPTIONAL,

cugSubscriptionFlag	[6] NULL	OPTIONAL,

subscriberInfo	[7] SubscriberInfo	OPTIONAL,

ss-List	[1] SS-List	OPTIONAL,

basicService	[5] Ext-BasicServiceCode	OPTIONAL,

forwardingInterrogationRequired	[4] NULL	OPTIONAL,

vmsc-Address	[2] ISDN-AddressString	OPTIONAL,

extensionContainer	[0] ExtensionContainer	OPTIONAL,

... ,

naea-PreferredCI	[10] NAEA-PreferredCI	OPTIONAL,

-- naea-PreferredCI is included at the discretion of the HLR operator.

ccbs-Indicators	[11] CCBS-Indicators	OPTIONAL,

msisdn	[12] ISDN-AddressString	OPTIONAL,

numberPortabilityStatus	[13] NumberPortabilityStatus	OPTIONAL,

istAlertTimer	[14] IST-AlertTimerValue	OPTIONAL,

supportedCamelPhasesInVMSC	[15]	SupportedCamelPhases	OPTIONAL,

offeredCamel4CSIsInVMSC	[16] OfferedCamel4CSIs	OPTIONAL,

routingInfo2	[17] RoutingInfo	OPTIONAL,

ss-List2	[18] SS-List	OPTIONAL,

basicService2	[19] Ext-BasicServiceCode	OPTIONAL,

allowedServices	[20] AllowedServices	OPTIONAL,

unavailabilityCause	[21] UnavailabilityCause	OPTIONAL,

releaseResourcesSupported	[22] NULL	OPTIONAL,

gsm-BearerCapability	[23] ExternalSignalInfo	OPTIONAL

}

# AllowedServices ::= BIT STRING {

firstServiceAllowed	(0),

secondServiceAllowed	(1) } (SIZE (2..8))

--	firstService is the service indicated in the networkSignalInfo

--	secondService is the service indicated in the networkSignalInfo2

--	Other bits than listed above shall be discarded

# UnavailabilityCause ::= ENUMERATED {

bearerServiceNotProvisioned	(1),

teleserviceNotProvisioned	(2),

absentSubscriber	(3),

busySubscriber	(4),

callBarred	(5),

cug-Reject	(6),

...}

--	exception handling:

--	Reception of other values than the ones listed shall result in the service

--	being unavailable for that call.

CCBS-Indicators ::= SEQUENCE {

ccbs-Possible	[0]	NULL	OPTIONAL,

keepCCBS-CallIndicator	[1]	NULL	OPTIONAL,

extensionContainer	[2]	ExtensionContainer	OPTIONAL,

...}

RoutingInfo ::= CHOICE {

roamingNumber	ISDN-AddressString,

forwardingData	ForwardingData}

ForwardingData ::= SEQUENCE {

forwardedToNumber	[5] ISDN-AddressString	OPTIONAL,

-- When this datatype is sent from an HLR which supports CAMEL Phase 2

-- to a GMSC which supports CAMEL Phase 2 the GMSC shall not check the

-- format of the number

forwardedToSubaddress	[4] ISDN-SubaddressString	OPTIONAL,

forwardingOptions	[6] ForwardingOptions	OPTIONAL,

extensionContainer	[7] ExtensionContainer	OPTIONAL,

...,

longForwardedToNumber	[8] FTN-AddressString	OPTIONAL}

ProvideRoamingNumberArg ::= SEQUENCE {

imsi	[0] IMSI,

msc-Number	[1] ISDN-AddressString,

msisdn	[2] ISDN-AddressString	OPTIONAL,

lmsi	[4] LMSI	OPTIONAL,

gsm-BearerCapability	[5] ExternalSignalInfo	OPTIONAL,

networkSignalInfo	[6] ExternalSignalInfo	OPTIONAL,

suppressionOfAnnouncement	[7] SuppressionOfAnnouncement	OPTIONAL,

gmsc-Address	[8] ISDN-AddressString	OPTIONAL,

callReferenceNumber	[9] CallReferenceNumber	OPTIONAL,

or-Interrogation	[10] NULL	OPTIONAL,

extensionContainer	[11] ExtensionContainer	OPTIONAL,

... ,

alertingPattern	[12] AlertingPattern	OPTIONAL,

ccbs-Call	[13] NULL	OPTIONAL,

supportedCamelPhasesInInterrogatingNode	[15] SupportedCamelPhases	OPTIONAL,

additionalSignalInfo	[14] Ext-ExternalSignalInfo	OPTIONAL,

orNotSupportedInGMSC	[16] NULL	OPTIONAL,

pre-pagingSupported	[17] NULL	OPTIONAL,

longFTN-Supported	[18]	NULL	OPTIONAL,

suppress-VT-CSI	[19]	NULL	OPTIONAL,

offeredCamel4CSIsInInterrogatingNode	[20] OfferedCamel4CSIs	OPTIONAL,

mtRoamingRetrySupported	[21] NULL	OPTIONAL,

pagingArea	[22] PagingArea	OPTIONAL,

callPriority	[23]	EMLPP-Priority	OPTIONAL,

mtrf-Indicator	[24] NULL	OPTIONAL,

oldMSC-Number	[25] ISDN-AddressString	OPTIONAL,

lastUsedLtePLMN-Id	[26] PLMN-Id	OPTIONAL

}

ProvideRoamingNumberRes ::= SEQUENCE {

roamingNumber	ISDN-AddressString,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

releaseResourcesSupported	NULL	OPTIONAL,

vmsc-Address	ISDN-AddressString	OPTIONAL }

ResumeCallHandlingArg ::= SEQUENCE {

callReferenceNumber	[0] CallReferenceNumber	OPTIONAL,

basicServiceGroup	[1] Ext-BasicServiceCode	OPTIONAL,

forwardingData	[2] ForwardingData	OPTIONAL,

imsi	[3] IMSI	OPTIONAL,

cug-CheckInfo	[4] CUG-CheckInfo	OPTIONAL,

o-CSI	[5] O-CSI	OPTIONAL,

extensionContainer	[7] ExtensionContainer	OPTIONAL,

ccbs-Possible	[8]	NULL	OPTIONAL,

msisdn	[9]	ISDN-AddressString	OPTIONAL,

uu-Data	[10] UU-Data	OPTIONAL,

allInformationSent	[11] NULL	OPTIONAL,

...,

d-csi	[12]	D-CSI	OPTIONAL,

o-BcsmCamelTDPCriteriaList	[13]	O-BcsmCamelTDPCriteriaList	OPTIONAL,

basicServiceGroup2	[14] Ext-BasicServiceCode	OPTIONAL,

mtRoamingRetry	[15] NULL	OPTIONAL

}

UU-Data ::= SEQUENCE {

uuIndicator	[0] UUIndicator	OPTIONAL,

uui	[1] UUI	OPTIONAL,

uusCFInteraction	[2] NULL	OPTIONAL,

extensionContainer	[3] ExtensionContainer	OPTIONAL,

...}

UUIndicator ::= OCTET STRING (SIZE (1))

-- Octets are coded according to ETS 300 356

UUI  ::= OCTET STRING (SIZE (1..131))

-- Octets are coded according to ETS 300 356

ResumeCallHandlingRes ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

CamelInfo ::= SEQUENCE {

supportedCamelPhases	SupportedCamelPhases,

suppress-T-CSI	NULL	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

... ,

offeredCamel4CSIs	[0] OfferedCamel4CSIs	OPTIONAL }

ExtendedRoutingInfo ::= CHOICE {

routingInfo	RoutingInfo,

camelRoutingInfo	[8] CamelRoutingInfo}

CamelRoutingInfo ::= SEQUENCE {

forwardingData	ForwardingData	OPTIONAL,

gmscCamelSubscriptionInfo	[0] GmscCamelSubscriptionInfo,

extensionContainer	[1] ExtensionContainer	OPTIONAL,

...}

GmscCamelSubscriptionInfo ::= SEQUENCE {

t-CSI	[0] T-CSI	OPTIONAL,

o-CSI	[1] O-CSI	OPTIONAL,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

...,

o-BcsmCamelTDP-CriteriaList	[3] O-BcsmCamelTDPCriteriaList	OPTIONAL,

t-BCSM-CAMEL-TDP-CriteriaList	[4]	T-BCSM-CAMEL-TDP-CriteriaList	OPTIONAL,

d-csi	[5]	D-CSI	OPTIONAL}

SetReportingStateArg ::= SEQUENCE {

imsi	[0]	IMSI	OPTIONAL,

lmsi	[1]	LMSI	OPTIONAL,

ccbs-Monitoring	[2]	ReportingState	OPTIONAL,

extensionContainer	[3]	ExtensionContainer	OPTIONAL,

...}

ReportingState ::= ENUMERATED {

stopMonitoring	(0),

startMonitoring	(1),

...}

-- exception handling:

-- reception of values 2-10 shall be mapped to 'stopMonitoring'

-- reception of values > 10 shall be mapped to 'startMonitoring'

SetReportingStateRes ::= SEQUENCE{

ccbs-SubscriberStatus	[0]	CCBS-SubscriberStatus	OPTIONAL,

extensionContainer	[1]	ExtensionContainer	OPTIONAL,

...}

CCBS-SubscriberStatus ::= ENUMERATED {

ccbsNotIdle	(0),

ccbsIdle	(1),

ccbsNotReachable	(2),

...}

--  exception handling:

--  reception of values 3-10 shall be mapped to 'ccbsNotIdle'

--  reception of values 11-20 shall be mapped to 'ccbsIdle'

--  reception of values > 20 shall be mapped to 'ccbsNotReachable'

StatusReportArg ::= SEQUENCE{

imsi	[0]	IMSI,

eventReportData	[1]	EventReportData	OPTIONAL,

callReportdata	[2]	CallReportData	OPTIONAL,

extensionContainer	[3]	ExtensionContainer	OPTIONAL,

...}

EventReportData ::= SEQUENCE{

ccbs-SubscriberStatus	[0]	CCBS-SubscriberStatus	OPTIONAL,

extensionContainer	[1]	ExtensionContainer	OPTIONAL,

...}

CallReportData ::= SEQUENCE{

monitoringMode	[0]	MonitoringMode	OPTIONAL,

callOutcome	[1]	CallOutcome	OPTIONAL,

extensionContainer	[2]	ExtensionContainer	OPTIONAL,

...}

MonitoringMode ::= ENUMERATED {

a-side	(0),

b-side	(1),

...}

--	exception handling:

--  reception of values 2-10 shall be mapped 'a-side'

--  reception of values > 10 shall be mapped to 'b-side'

CallOutcome ::= ENUMERATED {

success	(0),

failure	(1),

busy	(2),

...}

--	exception handling:

--  reception of values 3-10 shall be mapped to 'success'

--  reception of values 11-20 shall be mapped to 'failure'

--  reception of values > 20 shall be mapped to 'busy'

StatusReportRes ::= SEQUENCE {

extensionContainer	[0]	ExtensionContainer	OPTIONAL,

...}

RemoteUserFreeArg ::= SEQUENCE{

imsi	[0]	IMSI,

callInfo	[1]	ExternalSignalInfo,

ccbs-Feature	[2]	CCBS-Feature,

translatedB-Number	[3]	ISDN-AddressString,

replaceB-Number	[4]	NULL	OPTIONAL,

alertingPattern	[5]	AlertingPattern	OPTIONAL,

extensionContainer	[6]	ExtensionContainer	OPTIONAL,

...}

RemoteUserFreeRes ::= SEQUENCE{

ruf-Outcome	[0]	RUF-Outcome,

extensionContainer	[1]	ExtensionContainer	OPTIONAL,

...}

RUF-Outcome ::= ENUMERATED{

accepted (0),

rejected (1),

noResponseFromFreeMS (2), -- T4 Expiry

noResponseFromBusyMS (3), -- T10 Expiry

udubFromFreeMS (4),

udubFromBusyMS (5),

...}

-- exception handling:

-- reception of values 6-20 shall be mapped to 'accepted'

-- reception of values 21-30 shall be mapped to 'rejected'

-- reception of values 31-40 shall be mapped to 'noResponseFromFreeMS'

-- reception of values 41-50 shall be mapped to 'noResponseFromBusyMS'

-- reception of values 51-60 shall be mapped to 'udubFromFreeMS'

-- reception of values > 60 shall be mapped to 'udubFromBusyMS'

IST-AlertArg ::= SEQUENCE{

imsi	[0]	IMSI,

extensionContainer	[1]	ExtensionContainer	OPTIONAL,

...}

IST-AlertRes ::= SEQUENCE{

istAlertTimer	[0]	IST-AlertTimerValue	OPTIONAL,

istInformationWithdraw	[1]	NULL	OPTIONAL,

callTerminationIndicator	[2]	CallTerminationIndicator	OPTIONAL,

extensionContainer	[3]	ExtensionContainer	OPTIONAL,

...}

IST-CommandArg ::= SEQUENCE{

imsi	[0]	IMSI,

extensionContainer	[1]	ExtensionContainer	OPTIONAL,

...}

IST-CommandRes ::= SEQUENCE{

extensionContainer	ExtensionContainer	OPTIONAL,

...}

CallTerminationIndicator ::= ENUMERATED {

terminateCallActivityReferred	(0),

terminateAllCallActivities	(1),

...}

-- exception handling:

-- reception of values 2-10 shall be mapped to ' terminateCallActivityReferred '

-- reception of values > 10 shall be mapped to ' terminateAllCallActivities '

-- In MSCs not supporting linkage of all call activities, any value received shall

-- be interpreted as ' terminateCallActivityReferred '

ReleaseResourcesArg ::= SEQUENCE{

msrn	ISDN-AddressString,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

ReleaseResourcesRes ::= SEQUENCE{

extensionContainer	ExtensionContainer	OPTIONAL,

...}

.#END

* * * Next Change * * * *

### 17.7.4	Supplementary service data types

.$MAP-SS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-DataTypes (14) version22 (22)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

RegisterSS-Arg,

SS-Info,

SS-Status,

SS-SubscriptionOption,

SS-ForBS-Code,

InterrogateSS-Res,

USSD-Arg,

USSD-Res,

USSD-DataCodingScheme,

USSD-String,

Password,

GuidanceInfo,

SS-List,

SS-InfoList,

OverrideCategory,

CliRestrictionOption,

NoReplyConditionTime,

ForwardingOptions,

maxNumOfSS,

SS-Data,

SS-InvocationNotificationArg,

SS-InvocationNotificationRes,

CCBS-Feature,

RegisterCC-EntryArg,

RegisterCC-EntryRes,

EraseCC-EntryArg,

EraseCC-EntryRes

;

IMPORTS

AddressString,

ISDN-AddressString,

ISDN-SubaddressString,

FTN-AddressString,

IMSI,

BasicServiceCode,

AlertingPattern,

EMLPP-Priority,

MaxMC-Bearers,

MC-Bearers,

ExternalSignalInfo

FROM MAP-CommonDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version22 (22)}

ExtensionContainer

FROM MAP-ExtensionDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ExtensionDataTypes (21) version22 (22)}

SS-Code

FROM MAP-SS-Code {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-Code (15) version22 (22)}

;

RegisterSS-Arg ::= SEQUENCE {

ss-Code	SS-Code,

basicService	BasicServiceCode	OPTIONAL,

forwardedToNumber	[4] AddressString	OPTIONAL,

forwardedToSubaddress	[6] ISDN-SubaddressString	OPTIONAL,

noReplyConditionTime	[5] NoReplyConditionTime	OPTIONAL,

...,

defaultPriority	[7] EMLPP-Priority	OPTIONAL,

nbrUser	[8] MC-Bearers	OPTIONAL,

longFTN-Supported	[9]	NULL	OPTIONAL }

NoReplyConditionTime ::= INTEGER (5..30)

SS-Info ::= CHOICE {

forwardingInfo	[0] ForwardingInfo,

callBarringInfo	[1] CallBarringInfo,

ss-Data	[3] SS-Data}

ForwardingInfo ::= SEQUENCE {

ss-Code	SS-Code	OPTIONAL,

forwardingFeatureList	ForwardingFeatureList,

...}

ForwardingFeatureList ::=

SEQUENCE SIZE (1..maxNumOfBasicServiceGroups) OF

ForwardingFeature

ForwardingFeature ::= SEQUENCE {

basicService	BasicServiceCode	OPTIONAL,

ss-Status	[4] SS-Status	OPTIONAL,

forwardedToNumber	[5] ISDN-AddressString	OPTIONAL,

forwardedToSubaddress	[8] ISDN-SubaddressString	OPTIONAL,

forwardingOptions	[6] ForwardingOptions	OPTIONAL,

noReplyConditionTime	[7] NoReplyConditionTime	OPTIONAL,

...,

longForwardedToNumber	[9] FTN-AddressString	OPTIONAL }

SS-Status ::= OCTET STRING (SIZE (1))

-- bits 8765: 0000 (unused)

-- bits 4321: Used to convey the "P bit","R bit","A bit" and "Q bit",

--	 representing supplementary service state information

--	 as defined in TS 3GPP TS 23.011 [22]

-- bit 4: "Q bit"

-- bit 3: "P bit"

-- bit 2: "R bit"

-- bit 1: "A bit"

ForwardingOptions ::= OCTET STRING (SIZE (1))

-- bit 8: notification to forwarding party

--	0  no notification

--	1  notification

-- bit 7: redirecting presentation

--	0 no presentation

--	1  presentation

-- bit 6: notification to calling party

--	0  no notification

--	1  notification

-- bit 5: 0 (unused)

-- bits 43: forwarding reason

--	00  ms not reachable

--	01  ms busy

--	10  no reply

--	11  unconditional when used in a SRI Result,

--	 or call deflection when used in a RCH Argument

-- bits 21: 00 (unused)

CallBarringInfo ::= SEQUENCE {

ss-Code	SS-Code	OPTIONAL,

callBarringFeatureList	CallBarringFeatureList,

...}

CallBarringFeatureList ::= SEQUENCE SIZE (1..maxNumOfBasicServiceGroups) OF

CallBarringFeature

CallBarringFeature ::= SEQUENCE {

basicService	BasicServiceCode	OPTIONAL,

ss-Status	[4] SS-Status	OPTIONAL,

...}

SS-Data ::= SEQUENCE {

ss-Code	SS-Code	OPTIONAL,

ss-Status	[4] SS-Status	OPTIONAL,

ss-SubscriptionOption	SS-SubscriptionOption	OPTIONAL,

basicServiceGroupList	BasicServiceGroupList	OPTIONAL,

...,

defaultPriority	EMLPP-Priority	OPTIONAL,

nbrUser	[5] MC-Bearers	OPTIONAL

}

SS-SubscriptionOption ::= CHOICE {

cliRestrictionOption	[2] CliRestrictionOption,

overrideCategory	[1] OverrideCategory}

CliRestrictionOption ::= ENUMERATED {

permanent  (0),

temporaryDefaultRestricted  (1),

temporaryDefaultAllowed  (2)}

OverrideCategory ::= ENUMERATED {

overrideEnabled  (0),

overrideDisabled  (1)}

SS-ForBS-Code ::= SEQUENCE {

ss-Code	SS-Code,

basicService	BasicServiceCode	OPTIONAL,

...,

longFTN-Supported	[4]	NULL	OPTIONAL }

GenericServiceInfo ::= SEQUENCE {

ss-Status	SS-Status,

cliRestrictionOption	CliRestrictionOption	OPTIONAL,

...,

maximumEntitledPriority	[0] EMLPP-Priority	OPTIONAL,

defaultPriority	[1] EMLPP-Priority	OPTIONAL,

ccbs-FeatureList	[2] CCBS-FeatureList	OPTIONAL,

nbrSB	[3] MaxMC-Bearers	OPTIONAL,

nbrUser	[4] MC-Bearers	OPTIONAL,

nbrSN	[5] MC-Bearers	OPTIONAL }

CCBS-FeatureList ::= SEQUENCE SIZE (1..maxNumOfCCBS-Requests) OF

CCBS-Feature

maxNumOfCCBS-Requests  INTEGER ::= 5

CCBS-Feature ::= SEQUENCE {

ccbs-Index	[0] CCBS-Index	OPTIONAL,

b-subscriberNumber	[1] ISDN-AddressString	OPTIONAL,

b-subscriberSubaddress	[2] ISDN-SubaddressString	OPTIONAL,

basicServiceGroup	[3] BasicServiceCode	OPTIONAL,

...}

CCBS-Index  ::= INTEGER (1..maxNumOfCCBS-Requests)

InterrogateSS-Res ::= CHOICE {

ss-Status	[0] SS-Status,

basicServiceGroupList	[2] BasicServiceGroupList,

forwardingFeatureList	[3] ForwardingFeatureList,

genericServiceInfo	[4]	GenericServiceInfo }

USSD-Arg ::= SEQUENCE {

ussd-DataCodingScheme	USSD-DataCodingScheme,

ussd-String	USSD-String,

... ,

alertingPattern	AlertingPattern	OPTIONAL,

msisdn	[0] ISDN-AddressString	OPTIONAL }

USSD-Res ::= SEQUENCE {

ussd-DataCodingScheme	USSD-DataCodingScheme,

ussd-String	USSD-String,

...}

USSD-DataCodingScheme ::= OCTET STRING (SIZE (1))

-- The structure of the USSD-DataCodingScheme is defined by

-- the Cell Broadcast Data Coding Scheme as described in

-- TS 3GPP TS 23.038 [25]

USSD-String ::= OCTET STRING (SIZE (1..maxUSSD-StringLength))

-- The structure of the contents of the USSD-String is dependent

-- on the USSD-DataCodingScheme as described in TS 3GPP TS 23.038 [25].

maxUSSD-StringLength  INTEGER ::= 160

Password ::= NumericString

(FROM ("0"|"1"|"2"|"3"|"4"|"5"|"6"|"7"|"8"|"9"))

(SIZE (4))

GuidanceInfo ::= ENUMERATED {

enterPW  (0),

enterNewPW  (1),

enterNewPW-Again  (2)}

-- How this information is really delivered to the subscriber

-- (display, announcement, ...) is not part of this

-- specification.

SS-List ::= SEQUENCE SIZE (1..maxNumOfSS) OF

SS-Code

maxNumOfSS  INTEGER ::= 30

SS-InfoList ::= SEQUENCE SIZE (1..maxNumOfSS) OF

SS-Info

BasicServiceGroupList ::= SEQUENCE SIZE (1..maxNumOfBasicServiceGroups) OF

BasicServiceCode

maxNumOfBasicServiceGroups  INTEGER ::= 13

SS-InvocationNotificationArg ::= SEQUENCE {

imsi	[0] IMSI,

msisdn	[1] ISDN-AddressString,

ss-Event	[2] SS-Code,

-- The following SS-Code values are allowed :

-- ect	SS-Code ::= '00110001'B

-- multiPTY	SS-Code ::= '01010001'B

-- cd	SS-Code ::= '00100100'B

-- ccbs	SS-Code ::= '01000100'B

ss-EventSpecification	[3] SS-EventSpecification	OPTIONAL,

extensionContainer	[4] ExtensionContainer	OPTIONAL,

...,

b-subscriberNumber	[5]	ISDN-AddressString	OPTIONAL,

ccbs-RequestState	[6]	CCBS-RequestState	OPTIONAL

}

CCBS-RequestState ::= ENUMERATED {

request 	(0),

recall 	(1),

active 	(2),

completed	(3),

suspended	(4),

frozen	(5),

deleted	(6)

}

SS-InvocationNotificationRes ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...

}

SS-EventSpecification ::= SEQUENCE SIZE (1..maxEventSpecification) OF

AddressString

maxEventSpecification  INTEGER ::= 2

RegisterCC-EntryArg ::= SEQUENCE {

ss-Code	[0]	SS-Code,

ccbs-Data	[1]	CCBS-Data	OPTIONAL,

...}

CCBS-Data ::= SEQUENCE {

ccbs-Feature	[0]	CCBS-Feature,

translatedB-Number	[1]	ISDN-AddressString,

serviceIndicator	[2]	ServiceIndicator	OPTIONAL,

callInfo	[3]	ExternalSignalInfo,

networkSignalInfo	[4]	ExternalSignalInfo,

...}

ServiceIndicator ::= BIT STRING {

clir-invoked (0),

camel-invoked (1)} (SIZE(2..32))

-- exception handling:

-- bits 2 to 31 shall be ignored if received and not understood

RegisterCC-EntryRes ::= SEQUENCE {

ccbs-Feature	[0] CCBS-Feature	OPTIONAL,

...}

EraseCC-EntryArg ::= SEQUENCE {

ss-Code	[0]	SS-Code,

ccbs-Index	[1]	CCBS-Index	OPTIONAL,

...}

EraseCC-EntryRes ::= SEQUENCE {

ss-Code	[0]	SS-Code,

ss-Status	[1] SS-Status	OPTIONAL,

...}

.#END

* * * Next Change * * * *

### 17.7.5	Supplementary service codes

.$MAP-SS-Code {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-Code (15) version22 (22)}

DEFINITIONS

::=

BEGIN

SS-Code ::= OCTET STRING (SIZE (1))

-- This type is used to represent the code identifying a single

-- supplementary service, a group of supplementary services, or

-- all supplementary services. The services and abbreviations

-- used are defined in TS 3GPP TS 22.004 [5]. The internal structure is

-- defined as follows:

--

-- bits 87654321: group (bits 8765), and specific service

-- (bits 4321)

allSS	SS-Code ::= '00000000'B

-- reserved for possible future use

-- all SS

allLineIdentificationSS	SS-Code ::= '00010000'B

-- reserved for possible future use

-- all line identification SS

clip	SS-Code ::= '00010001'B

-- calling line identification presentation

clir	SS-Code ::= '00010010'B

-- calling line identification restriction

colp	SS-Code ::= '00010011'B

-- connected line identification presentation

colr	SS-Code ::= '00010100'B

-- connected line identification restriction

mci	SS-Code ::= '00010101'B

-- reserved for possible future use

-- malicious call identification

allNameIdentificationSS	SS-Code ::= '00011000'B

-- all name identification SS

cnap	SS-Code ::= '00011001'B

-- calling name presentation

-- SS-Codes '00011010'B to '00011111'B are reserved for future

-- NameIdentification Supplementary Service use.

allForwardingSS	SS-Code ::= '00100000'B

-- all forwarding SS

cfu	SS-Code ::= '00100001'B

-- call forwarding unconditional

allCondForwardingSS	SS-Code ::= '00101000'B

-- all conditional forwarding SS

cfb	SS-Code ::= '00101001'B

-- call forwarding on mobile subscriber busy

cfnry	SS-Code ::= '00101010'B

-- call forwarding on no reply

cfnrc	SS-Code ::= '00101011'B

-- call forwarding on mobile subscriber not reachable

cd	SS-Code ::= '00100100'B

-- call deflection

allCallOfferingSS	SS-Code ::= '00110000'B

-- reserved for possible future use

-- all call offering SS includes also all forwarding SS

ect	SS-Code ::= '00110001'B

-- explicit call transfer

mah	SS-Code ::= '00110010'B

-- reserved for possible future use

-- mobile access hunting

allCallCompletionSS	SS-Code ::= '01000000'B

-- reserved for possible future use

-- all Call completion SS

cw	SS-Code ::= '01000001'B

-- call waiting

hold	SS-Code ::= '01000010'B

-- call hold

ccbs-A	SS-Code ::= '01000011'B

-- completion of call to busy subscribers, originating side

-- this SS-Code is used only in InsertSubscriberData, DeleteSubscriberData

-- and InterrogateSS

ccbs-B	SS-Code ::= '01000100'B

-- completion of call to busy subscribers, destination side

-- this SS-Code is used only in InsertSubscriberData and DeleteSubscriberData

mc	SS-Code ::= '01000101'B

-- multicall

allMultiPartySS	SS-Code ::= '01010000'B

-- reserved for possible future use

-- all multiparty SS

multiPTY	SS-Code ::= '01010001'B

-- multiparty

allCommunityOfInterest-SS	SS-Code ::= '01100000'B

-- reserved for possible future use

-- all community of interest SS

cug	SS-Code ::= '01100001'B

-- closed user group

allChargingSS	SS-Code ::= '01110000'B

-- reserved for possible future use

-- all charging SS

aoci	SS-Code ::= '01110001'B

-- advice of charge information

aocc	SS-Code ::= '01110010'B

-- advice of charge charging

allAdditionalInfoTransferSS	SS-Code ::= '10000000'B

-- reserved for possible future use

-- all additional information transfer SS

uus1	SS-Code ::= '10000001'B

-- UUS1 user-to-user signalling

uus2	SS-Code ::= '10000010'B

-- UUS2 user-to-user signalling

uus3	SS-Code ::= '10000011'B

-- UUS3 user-to-user signalling

allBarringSS	SS-Code ::= '10010000'B

-- all barring SS

barringOfOutgoingCalls	SS-Code ::= '10010001'B

-- barring of outgoing calls

baoc	SS-Code ::= '10010010'B

-- barring of all outgoing calls

boic	SS-Code ::= '10010011'B

-- barring of outgoing international calls

boicExHC	SS-Code ::= '10010100'B

-- barring of outgoing international calls except those directed

-- to the home PLMN Country

barringOfIncomingCalls	SS-Code ::= '10011001'B

-- barring of incoming calls

baic	SS-Code ::= '10011010'B

-- barring of all incoming calls

bicRoam	SS-Code ::= '10011011'B

-- barring of incoming calls when roaming outside home PLMN

-- Country

allPLMN-specificSS	SS-Code ::= '11110000'B

plmn-specificSS-1	SS-Code ::= '11110001'B

plmn-specificSS-2	SS-Code ::= '11110010'B

plmn-specificSS-3	SS-Code ::= '11110011'B

plmn-specificSS-4	SS-Code ::= '11110100'B

plmn-specificSS-5	SS-Code ::= '11110101'B

plmn-specificSS-6	SS-Code ::= '11110110'B

plmn-specificSS-7	SS-Code ::= '11110111'B

plmn-specificSS-8	SS-Code ::= '11111000'B

plmn-specificSS-9	SS-Code ::= '11111001'B

plmn-specificSS-A	SS-Code ::= '11111010'B

plmn-specificSS-B	SS-Code ::= '11111011'B

plmn-specificSS-C	SS-Code ::= '11111100'B

plmn-specificSS-D	SS-Code ::= '11111101'B

plmn-specificSS-E	SS-Code ::= '11111110'B

plmn-specificSS-F	SS-Code ::= '11111111'B

allCallPrioritySS	SS-Code ::= '10100000'B

-- reserved for possible future use

-- all call priority SS

emlpp	SS-Code ::= '10100001'B

-- enhanced Multilevel Precedence Pre-emption (EMLPP) service

allLCSPrivacyException	SS-Code ::= '10110000'B

-- all LCS Privacy Exception Classes

universal	SS-Code ::= '10110001'B

-- allow location by any LCS client

callSessionRelated	SS-Code ::= '10110010'B

-- allow location by any value added LCS client to which a call/session

-- is established from the target MS

callSessionUnrelated	SS-Code ::= '10110011'B

-- allow location by designated external value added LCS clients

plmnoperator	SS-Code ::= '10110100'B

-- allow location by designated PLMN operator LCS clients

serviceType	SS-Code ::= '10110101'B

-- allow location by LCS clients of a designated LCS service type

allMOLR-SS	SS-Code ::= '11000000'B

-- all Mobile Originating Location Request Classes

basicSelfLocation	SS-Code ::= '11000001'B

-- allow an MS to request its own location

autonomousSelfLocation	SS-Code ::= '11000010'B

-- allow an MS to perform self location without interaction

-- with the PLMN for a predetermined period of time

transferToThirdParty	SS-Code ::= '11000011'B

-- allow an MS to request transfer of its location to another LCS client

.#END

* * * Next Change * * * *

### 17.7.6	Short message data types

.$MAP-SM-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SM-DataTypes (16) version22 (22)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

RoutingInfoForSM-Arg,

RoutingInfoForSM-Res,

MO-ForwardSM-Arg,

MO-ForwardSM-Res,

MT-ForwardSM-Arg,

MT-ForwardSM-Res,

ReportSM-DeliveryStatusArg,

ReportSM-DeliveryStatusRes,

AlertServiceCentreArg,

InformServiceCentreArg,

ReadyForSM-Arg,

ReadyForSM-Res,

SM-DeliveryOutcome,

AlertReason,

Additional-Number,

MT-ForwardSM-VGCS-Arg,

MT-ForwardSM-VGCS-Res

;

IMPORTS

AddressString,

ISDN-AddressString,

SignalInfo,

IMSI,

LMSI,

ASCI-CallReference,

Time,

NetworkNodeDiameterAddress,

HLR-Id

FROM MAP-CommonDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version22 (22)}

AbsentSubscriberDiagnosticSM

FROM MAP-ER-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ER-DataTypes (17) version22 (22)}

ExtensionContainer

FROM MAP-ExtensionDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ExtensionDataTypes (21) version22 (22)}

;

RoutingInfoForSM-Arg ::= SEQUENCE {

msisdn	[0] ISDN-AddressString,

sm-RP-PRI	[1] BOOLEAN,

serviceCentreAddress	[2] AddressString,

extensionContainer	[6] ExtensionContainer	OPTIONAL,

... ,

gprsSupportIndicator	[7]	NULL	OPTIONAL,

-- gprsSupportIndicator is set only if the SMS-GMSC supports

-- receiving of two numbers from the HLR

sm-RP-MTI	[8] SM-RP-MTI	OPTIONAL,

sm-RP-SMEA	[9] SM-RP-SMEA	OPTIONAL,

sm-deliveryNotIntended	[10] SM-DeliveryNotIntended	OPTIONAL,

ip-sm-gwGuidanceIndicator	[11] NULL	OPTIONAL,

imsi	[12] IMSI	OPTIONAL,

t4-Trigger-Indicator	[14] NULL	OPTIONAL,

singleAttemptDelivery	[13]	NULL	OPTIONAL,

correlationID	[15] CorrelationID	OPTIONAL,

smsf-supportIndicator	[16] NULL	OPTIONAL }

SM-DeliveryNotIntended ::= ENUMERATED {

onlyIMSI-requested  (0),

onlyMCC-MNC-requested  (1),

...}

SM-RP-MTI ::= INTEGER (0..10)

-- 0 SMS Deliver

-- 1 SMS Status Report

-- other values are reserved for future use and shall be discarded if

-- received

SM-RP-SMEA ::= OCTET STRING (SIZE (1..12))

-- this parameter contains an address field which is encoded

-- as defined in 3GPP TS 23.040. An address field contains 3 elements :

--	address-length

--	type-of-address

--	address-value

RoutingInfoForSM-Res ::= SEQUENCE {

imsi	IMSI,

locationInfoWithLMSI	[0] LocationInfoWithLMSI,

extensionContainer	[4] ExtensionContainer	OPTIONAL,

...,

ip-sm-gwGuidance	[5] IP-SM-GW-Guidance	OPTIONAL }

IP-SM-GW-Guidance ::= SEQUENCE {

minimumDeliveryTimeValue	SM-DeliveryTimerValue,

recommendedDeliveryTimeValue	SM-DeliveryTimerValue,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

LocationInfoWithLMSI ::= SEQUENCE {

networkNode-Number	[1] ISDN-AddressString,

lmsi	LMSI	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

gprsNodeIndicator	[5]	NULL	OPTIONAL,

-- gprsNodeIndicator is set only if the SGSN number is sent as the

-- Network Node Number

additional-Number	[6] Additional-Number	OPTIONAL,

networkNodeDiameterAddress	[7] NetworkNodeDiameterAddress	OPTIONAL,

additionalNetworkNodeDiameterAddress	[8] NetworkNodeDiameterAddress	OPTIONAL,

thirdNumber	[9] Additional-Number	OPTIONAL,

thirdNetworkNodeDiameterAddress	[10] NetworkNodeDiameterAddress	OPTIONAL,

imsNodeIndicator	[11] NULL	OPTIONAL,

-- gprsNodeIndicator and imsNodeIndicator shall not both be present.

-- additionalNumber and thirdNumber shall not both contain the same type of number.

smsf-3gpp-Number	[12]	ISDN-AddressString	OPTIONAL,

smsf-3gpp-DiameterAddress	[13]	NetworkNodeDiameterAddress	OPTIONAL,

smsf-non-3gpp-Number	[14]	ISDN-AddressString	OPTIONAL,

smsf-non-3gpp-DiameterAddress	[15]	NetworkNodeDiameterAddress	OPTIONAL,

smsf-3gpp-address-indicator	[16]	NULL	OPTIONAL,

smsf-non-3gpp-address-indicator	[17]	NULL	OPTIONAL

--

-- If smsf-supportIndicator was not included in the request, in RoutingInfoForSM-Arg,

-- then smsf-3gpp Number/DiameterAddress, smsf-non-3gpp Number/DiameterAddress and

-- smsf-address-indicator and smsf-non-3gpp-address-indicator shall be absent.

-- The HLR may use the networkNode-Number/networkNodeDiameterAddress to indicate the

-- 3GPP-SMSF address as applicable.

-- The HLR may use the additional-Number, additionalNetworkNodeDiameterAddress to

-- indicate the Non-3GPP-SMSF address as applicable

--

-- If smsf-3gpp-address-indicator is present, it indicates that the networkNode-Number

-- (and networkNodeDiameterAddress, if present) contains the address of an SMSF for

-- 3GPP access.

--

-- If smsf-non-3gpp-address-indicator is present, it indicates that the

-- networkNode-Number (and networkNodeDiameterAddress, if present) contains the

-- address of an SMSF for non 3GPP access.

--

-- At most one of gprsNodeIndicator, imsNodeIndicator, smsf-3gpp-address-indicator

-- and smsf-non-3gpp-address-indicator shall be present. Absence of all these

-- indicators indicate that the networkNode-Number (and networkNodeDiameterAddress,

-- if present) contains the address of an MSC/MME.

}

Additional-Number ::= CHOICE {

msc-Number	[0] ISDN-AddressString,

sgsn-Number	[1] ISDN-AddressString}

-- msc-number can be the MSC number or

-- the SMS Router number or the MME number for MT SMS

-- sgsn-number can be the SGSN number or the SMS Router number

MO-ForwardSM-Arg ::= SEQUENCE {

sm-RP-DA	SM-RP-DA,

sm-RP-OA	SM-RP-OA,

sm-RP-UI	SignalInfo,

extensionContainer	ExtensionContainer	OPTIONAL,

... ,

imsi	IMSI	OPTIONAL,

correlationID	[0] CorrelationID	OPTIONAL,

sm-DeliveryOutcome	[1] SM-DeliveryOutcome	OPTIONAL

}

MO-ForwardSM-Res ::= SEQUENCE {

sm-RP-UI	SignalInfo	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

MT-ForwardSM-Arg ::= SEQUENCE {

sm-RP-DA	SM-RP-DA,

sm-RP-OA	SM-RP-OA,

sm-RP-UI	SignalInfo,

moreMessagesToSend	NULL	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

smDeliveryTimer	SM-DeliveryTimerValue	OPTIONAL,

smDeliveryStartTime	Time	OPTIONAL,

smsOverIP-OnlyIndicator	[0] NULL	OPTIONAL,

correlationID	[1] CorrelationID	OPTIONAL,

maximumRetransmissionTime	[2] Time	OPTIONAL,

smsGmscAddress	[3] ISDN-AddressString	OPTIONAL,

smsGmscDiameterAddress	[4] NetworkNodeDiameterAddress	OPTIONAL }

-- SM-DeliveryTimerValue contains the value used by the SMS-GMSC

CorrelationID ::= SEQUENCE {

hlr-id	[0] HLR-Id	OPTIONAL,

sip-uri-A	[1] SIP-URI	OPTIONAL,

sip-uri-B	[2] SIP-URI}

SIP-URI ::= OCTET STRING

-- octets are coded as defined in IETF RFC 3261

MT-ForwardSM-Res ::= SEQUENCE {

sm-RP-UI	SignalInfo	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

... }

SM-RP-DA ::= CHOICE {

imsi	[0] IMSI,

lmsi	[1] LMSI,

serviceCentreAddressDA	[4] AddressString,

noSM-RP-DA	[5] NULL}

SM-RP-OA ::= CHOICE {

msisdn	[2] ISDN-AddressString,

serviceCentreAddressOA	[4] AddressString,

noSM-RP-OA	[5] NULL}

SM-DeliveryTimerValue ::= INTEGER (30..600)

ReportSM-DeliveryStatusArg ::= SEQUENCE {

msisdn	ISDN-AddressString,

serviceCentreAddress	AddressString,

sm-DeliveryOutcome	SM-DeliveryOutcome,

absentSubscriberDiagnosticSM	[0] AbsentSubscriberDiagnosticSM

OPTIONAL,

extensionContainer	[1] ExtensionContainer	OPTIONAL,

...,

gprsSupportIndicator	[2]	NULL	OPTIONAL,

-- gprsSupportIndicator is set only if the SMS-GMSC supports

-- handling of two delivery outcomes

deliveryOutcomeIndicator	[3]	NULL	OPTIONAL,

-- DeliveryOutcomeIndicator is set when the SM-DeliveryOutcome

-- is for GPRS

additionalSM-DeliveryOutcome	[4]	SM-DeliveryOutcome	OPTIONAL,

-- If received, additionalSM-DeliveryOutcome is for GPRS

-- If DeliveryOutcomeIndicator is set, then AdditionalSM-DeliveryOutcome shall be absent

additionalAbsentSubscriberDiagnosticSM	[5]	AbsentSubscriberDiagnosticSM OPTIONAL,

-- If received additionalAbsentSubscriberDiagnosticSM is for GPRS

-- If DeliveryOutcomeIndicator is set, then AdditionalAbsentSubscriberDiagnosticSM

-- shall be absent

ip-sm-gw-Indicator	[6]	NULL	OPTIONAL,

-- the ip-sm-gw indicator indicates by its presence that sm-deliveryOutcome

-- is for delivery via IMS

-- If present, deliveryOutcomeIndicator shall be absent.

ip-sm-gw-sm-deliveryOutcome	[7]	SM-DeliveryOutcome	OPTIONAL,

-- If received ip-sm-gw-sm-deliveryOutcome is for delivery via IMS

-- If ip-sm-gw-Indicator is set, then ip-sm-gw-sm-deliveryOutcome shall be absent

ip-sm-gw-absentSubscriberDiagnosticSM	[8]	AbsentSubscriberDiagnosticSM	OPTIONAL,

-- If received ip-sm-gw-sm-absentSubscriberDiagnosticSM is for delivery via IMS

-- If ip-sm-gw-Indicator is set, then ip-sm-gw-sm-absentSubscriberDiagnosticSM

-- shall be absent

imsi	[9] IMSI	OPTIONAL,

singleAttemptDelivery	[10] NULL	OPTIONAL,

correlationID	[11]	CorrelationID	OPTIONAL,

smsf-3gpp-deliveryOutcomeIndicator	[12]	NULL	OPTIONAL,

-- smsf-3gpp-deliveryOutcome is set when the SM-DeliveryOutcome

-- is for 3GPP-SMSF

smsf-3gpp-deliveryOutcome	[13]	SM-DeliveryOutcome	OPTIONAL,

-- If smsf-3gpp-deliveryOutcomeIndicator is set, then smsf-3gpp-deliveryOutcome

-- shall be absent

smsf-3gpp-absentSubscriberDiagSM	[14]	AbsentSubscriberDiagnosticSM	OPTIONAL,

-- If smsf-3gpp-deliveryOutcomeIndicator is set, then

-- smsf-3gpp-absentSubscriberDiagSM shall be absent

smsf-non-3gpp-deliveryOutcomeIndicator	[15]	NULL	OPTIONAL,

-- smsf-non-3gpp-deliveryOutcomeIndicator is set when the SM-DeliveryOutcome

-- is for non-3GPP-SMSF

smsf-non-3gpp-deliveryOutcome	[16]	SM-DeliveryOutcome	OPTIONAL,

-- If smsf-non-3gpp-deliveryOutcomeIndicator is set, then smsf-non-3gpp-deliveryOutcome

-- shall be absent

smsf-non-3gpp-absentSubscriberDiagSM	[17]	AbsentSubscriberDiagnosticSM	OPTIONAL,

-- If smsf-non-3gpp-deliveryOutcomeIndicator is set, then

-- smsf-non-3gpp-absentSubscriberDiagSM shall be absent

failedSMServingNodes	[18]	SMServingNodeAddressList	OPTIONAL

}

SM-DeliveryOutcome ::= ENUMERATED {

memoryCapacityExceeded  (0),

absentSubscriber  (1),

successfulTransfer  (2)}

ReportSM-DeliveryStatusRes ::= SEQUENCE {

storedMSISDN	ISDN-AddressString	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

registeredSMServingNodes	[0] SMServingNodeAddressList	OPTIONAL

}

SMServingNodeAddressList ::=

SEQUENCE SIZE (1..maxNumOfSMServingNodeAddresses) OF

SMServingNodeAddress

SMServingNodeAddress ::= CHOICE {

networkNode-Number	[0] ISDN-AddressString,

diameterAddress	[1] NetworkNodeDiameterAddress

}

maxNumOfSMServingNodeAddresses  INTEGER ::= 5

AlertServiceCentreArg ::= SEQUENCE {

msisdn	ISDN-AddressString,

serviceCentreAddress	AddressString,

...,

imsi	IMSI	OPTIONAL,

correlationID	CorrelationID	OPTIONAL,

maximumUeAvailabilityTime	[0] Time	OPTIONAL,

smsGmscAlertEvent	[1] SmsGmsc-Alert-Event	OPTIONAL,

smsGmscDiameterAddress	[2] NetworkNodeDiameterAddress	OPTIONAL,

newSGSNNumber	[3] ISDN-AddressString	OPTIONAL,

newSGSNDiameterAddress	[4] NetworkNodeDiameterAddress	OPTIONAL,

newMMENumber	[5] ISDN-AddressString	OPTIONAL,

newMMEDiameterAddress	[6] NetworkNodeDiameterAddress	OPTIONAL,

newMSCNumber	[7] ISDN-AddressString	OPTIONAL }

SmsGmsc-Alert-Event ::= ENUMERATED {

msAvailableForMtSms  (0),

msUnderNewServingNode  (1)  }

InformServiceCentreArg ::= SEQUENCE {

storedMSISDN	ISDN-AddressString	OPTIONAL,

mw-Status	MW-Status	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

... ,

absentSubscriberDiagnosticSM	AbsentSubscriberDiagnosticSM	OPTIONAL,

additionalAbsentSubscriberDiagnosticSM	[0]	AbsentSubscriberDiagnosticSM	OPTIONAL,

-- additionalAbsentSubscriberDiagnosticSM may be present only if

-- absentSubscriberDiagnosticSM is present.

-- if included, additionalAbsentSubscriberDiagnosticSM is for GPRS and

-- absentSubscriberDiagnosticSM is for non-GPRS

smsf3gppAbsentSubscriberDiagnosticSM	[1] AbsentSubscriberDiagnosticSM	OPTIONAL,

smsfNon3gppAbsentSubscriberDiagnosticSM	[2] AbsentSubscriberDiagnosticSM	OPTIONAL }

MW-Status ::= BIT STRING {

sc-AddressNotIncluded  (0),

mnrf-Set  (1),

mcef-Set  (2) ,

mnrg-Set	(3),

mnr5g-Set (4),

mnr5gn3g-Set (5)} (SIZE (6..16))

-- exception handling:

-- bits 6 to 15 shall be ignored if received and not understood

ReadyForSM-Arg ::= SEQUENCE {

imsi	[0] IMSI,

alertReason	AlertReason,

alertReasonIndicator	NULL	OPTIONAL,

-- alertReasonIndicator is set only when the alertReason

-- sent to HLR is for GPRS

extensionContainer	ExtensionContainer	OPTIONAL,

...,

additionalAlertReasonIndicator	[1] NULL	OPTIONAL,

-- additionalAlertReasonIndicator is set only when the alertReason

-- sent to HLR is for IP-SM-GW

maximumUeAvailabilityTime	Time	OPTIONAL }

ReadyForSM-Res ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

AlertReason ::= ENUMERATED {

ms-Present  (0),

memoryAvailable  (1)}

MT-ForwardSM-VGCS-Arg ::= SEQUENCE {

asciCallReference	ASCI-CallReference,

sm-RP-OA	SM-RP-OA,

sm-RP-UI	SignalInfo,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

MT-ForwardSM-VGCS-Res ::= SEQUENCE {

sm-RP-UI	[0] SignalInfo	OPTIONAL,

dispatcherList	[1] DispatcherList	OPTIONAL,

ongoingCall	NULL	OPTIONAL,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

...,

additionalDispatcherList	[3] AdditionalDispatcherList	OPTIONAL }

-- additionalDispatcherList shall be absent if dispatcherList is absent or

-- contains less than 5 ISDN-AddressStrings

DispatcherList ::=

SEQUENCE SIZE (1..maxNumOfDispatchers) OF

ISDN-AddressString

maxNumOfDispatchers  INTEGER ::= 5

AdditionalDispatcherList ::=

SEQUENCE SIZE (1..maxNumOfAdditionalDispatchers) OF

ISDN-AddressString

maxNumOfAdditionalDispatchers  INTEGER ::= 15

.#END

* * * Next Change * * * *

### 17.7.7	Error data types

.$MAP-ER-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ER-DataTypes (17) version22 (22)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

RoamingNotAllowedParam,

CallBarredParam,

CUG-RejectParam,

SS-IncompatibilityCause,

PW-RegistrationFailureCause,

SM-DeliveryFailureCause,

SystemFailureParam,

DataMissingParam,

UnexpectedDataParam,

FacilityNotSupParam,

OR-NotAllowedParam,

UnknownSubscriberParam,

NumberChangedParam,

UnidentifiedSubParam,

IllegalSubscriberParam,

IllegalEquipmentParam,

BearerServNotProvParam,

TeleservNotProvParam,

TracingBufferFullParam,

NoRoamingNbParam,

AbsentSubscriberParam,

BusySubscriberParam,

NoSubscriberReplyParam,

ForwardingViolationParam,

ForwardingFailedParam,

ATI-NotAllowedParam,

SubBusyForMT-SMS-Param,

MessageWaitListFullParam,

AbsentSubscriberSM-Param,

AbsentSubscriberDiagnosticSM,

ResourceLimitationParam,

NoGroupCallNbParam,

IncompatibleTerminalParam,

ShortTermDenialParam,

LongTermDenialParam,

UnauthorizedRequestingNetwork-Param,

UnauthorizedLCSClient-Param,

PositionMethodFailure-Param,

UnknownOrUnreachableLCSClient-Param,

# MM-EventNotSupported-Param,

ATSI-NotAllowedParam,

ATM-NotAllowedParam,

IllegalSS-OperationParam,

SS-NotAvailableParam,

SS-SubscriptionViolationParam,

InformationNotAvailableParam,

TargetCellOutsideGCA-Param,

OngoingGroupCallParam,

PositionMethodFailure-Diagnostic,

UnauthorizedLCSClient-Diagnostic

;

IMPORTS

SS-Status

FROM MAP-SS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-DataTypes (14) version22 (22)}

SignalInfo,

BasicServiceCode,

NetworkResource,

AdditionalNetworkResource,

IMSI,

Time

FROM MAP-CommonDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version22 (22)}

SS-Code

FROM MAP-SS-Code {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-Code (15) version22 (22)}

ExtensionContainer

FROM MAP-ExtensionDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ExtensionDataTypes (21) version22 (22)}

;

RoamingNotAllowedParam ::= SEQUENCE {

roamingNotAllowedCause	RoamingNotAllowedCause,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

additionalRoamingNotAllowedCause	[0] AdditionalRoamingNotAllowedCause OPTIONAL }

--	if the additionalRoamingNotallowedCause is received by the MSC/VLR or SGSN then the

--	roamingNotAllowedCause shall be discarded.

AdditionalRoamingNotAllowedCause ::= ENUMERATED {

supportedRAT-TypesNotAllowed (0),

...}

RoamingNotAllowedCause ::= ENUMERATED {

plmnRoamingNotAllowed  (0),

operatorDeterminedBarring  (3)}

CallBarredParam ::= CHOICE {

callBarringCause	CallBarringCause,

-- call BarringCause must not be used in version 3 and higher

extensibleCallBarredParam	ExtensibleCallBarredParam

-- extensibleCallBarredParam must not be used in version <3

}

CallBarringCause ::= ENUMERATED {

barringServiceActive  (0),

operatorBarring  (1)}

ExtensibleCallBarredParam ::= SEQUENCE {

callBarringCause	CallBarringCause	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

... ,

unauthorisedMessageOriginator	[1] NULL	OPTIONAL,

anonymousCallRejection	[2] NULL	OPTIONAL }

-- unauthorisedMessageOriginator and anonymousCallRejection shall be mutually exclusive.

CUG-RejectParam ::= SEQUENCE {

cug-RejectCause	CUG-RejectCause	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

CUG-RejectCause ::= ENUMERATED {

incomingCallsBarredWithinCUG  (0),

subscriberNotMemberOfCUG  (1),

requestedBasicServiceViolatesCUG-Constraints  (5),

calledPartySS-InteractionViolation  (7)}

SS-IncompatibilityCause ::= SEQUENCE {

ss-Code	[1] SS-Code	OPTIONAL,

basicService	BasicServiceCode	OPTIONAL,

ss-Status	[4] SS-Status	OPTIONAL,

...}

PW-RegistrationFailureCause ::= ENUMERATED {

undetermined  (0),

invalidFormat  (1),

newPasswordsMismatch  (2)}

SM-EnumeratedDeliveryFailureCause ::= ENUMERATED {

memoryCapacityExceeded  (0),

equipmentProtocolError  (1),

equipmentNotSM-Equipped  (2),

unknownServiceCentre  (3),

sc-Congestion  (4),

invalidSME-Address  (5),

subscriberNotSC-Subscriber  (6)}

SM-DeliveryFailureCause ::= SEQUENCE {

sm-EnumeratedDeliveryFailureCause	SM-EnumeratedDeliveryFailureCause,

diagnosticInfo	SignalInfo	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

AbsentSubscriberSM-Param ::= SEQUENCE {

absentSubscriberDiagnosticSM	AbsentSubscriberDiagnosticSM	OPTIONAL,

-- AbsentSubscriberDiagnosticSM can be either for non-GPRS

-- or for GPRS

extensionContainer	ExtensionContainer	OPTIONAL,

...,

additionalAbsentSubscriberDiagnosticSM 	[0]	AbsentSubscriberDiagnosticSM	OPTIONAL,

-- if received, additionalAbsentSubscriberDiagnosticSM

-- is for GPRS and absentSubscriberDiagnosticSM is

-- for non-GPRS

imsi	[1] IMSI	OPTIONAL,

-- when sent from HLR to IP-SM-GW, IMSI shall be present if UNRI is not set

-- to indicate that the absent condition is met for CS and PS but not for IMS.

requestedRetransmissionTime	[2] Time	OPTIONAL,

userIdentifierAlert	[3] IMSI	OPTIONAL }

AbsentSubscriberDiagnosticSM ::= INTEGER (0..255)

-- AbsentSubscriberDiagnosticSM values are defined in 3GPP TS 23.040

SystemFailureParam ::= CHOICE {

networkResource	NetworkResource,

-- networkResource must not be used in version 3

extensibleSystemFailureParam	ExtensibleSystemFailureParam

-- extensibleSystemFailureParam must not be used in version <3

}

ExtensibleSystemFailureParam ::= SEQUENCE {

networkResource	NetworkResource	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

additionalNetworkResource	[0] AdditionalNetworkResource	OPTIONAL,

failureCauseParam	[1] FailureCauseParam	OPTIONAL }

FailureCauseParam ::= ENUMERATED {

limitReachedOnNumberOfConcurrentLocationRequests (0),

... }

-- if unknown value is received in FailureCauseParam it shall be ignored

DataMissingParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

UnexpectedDataParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...,

unexpectedSubscriber [0]	NULL	OPTIONAL}

-- the unexpectedSubscriber indication in the unexpectedDataValue error shall not be used

-- for operations that allow the unidentifiedSubscriber error.

FacilityNotSupParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...,

shapeOfLocationEstimateNotSupported [0]	NULL	OPTIONAL,

neededLcsCapabilityNotSupportedInServingNode [1] NULL	OPTIONAL }

OR-NotAllowedParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

UnknownSubscriberParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...,

unknownSubscriberDiagnostic	UnknownSubscriberDiagnostic	OPTIONAL}

UnknownSubscriberDiagnostic ::= ENUMERATED {

imsiUnknown  (0),

gprs-eps-SubscriptionUnknown  (1),

...,

npdbMismatch  (2)}

-- if unknown values are received in

-- UnknownSubscriberDiagnostic they shall be discarded

NumberChangedParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

UnidentifiedSubParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

IllegalSubscriberParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

IllegalEquipmentParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

BearerServNotProvParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

TeleservNotProvParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

TracingBufferFullParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

NoRoamingNbParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

AbsentSubscriberParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...,

absentSubscriberReason	[0] AbsentSubscriberReason	OPTIONAL}

AbsentSubscriberReason ::= ENUMERATED {

imsiDetach (0),

restrictedArea (1),

noPageResponse (2),

... ,

purgedMS (3),

mtRoamingRetry (4),

busySubscriber (5)}

-- exception handling: at reception of other values than the ones listed the

-- AbsentSubscriberReason shall be ignored.

-- The AbsentSubscriberReason: purgedMS is defined for the Super-Charger feature

-- (see TS 23.116). If this value is received in a Provide Roaming Number response

-- it shall be mapped to the AbsentSubscriberReason: imsiDetach in the Send Routeing

-- Information response

-- The AbsentSubscriberReason: mtRoamingRetry is used during MT Roaming Retry,

-- see 3GPP TS 23.018[97].

-- The AbsentSubscriberReason: busySubscriber is used during MT Roaming Forwarding,

-- see 3GPP TS 23.018[97].

BusySubscriberParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...,

ccbs-Possible	[0] NULL	OPTIONAL,

ccbs-Busy	[1] NULL	OPTIONAL}

NoSubscriberReplyParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

ForwardingViolationParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

ForwardingFailedParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

ATI-NotAllowedParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

# ATSI-NotAllowedParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

# ATM-NotAllowedParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

IllegalSS-OperationParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

SS-NotAvailableParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

SS-SubscriptionViolationParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

InformationNotAvailableParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

SubBusyForMT-SMS-Param ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

... ,

gprsConnectionSuspended	NULL	OPTIONAL }

-- If GprsConnectionSuspended is not understood it shall

-- be discarded

MessageWaitListFullParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

ResourceLimitationParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

NoGroupCallNbParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

IncompatibleTerminalParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

ShortTermDenialParam ::= SEQUENCE {

...}

LongTermDenialParam ::= SEQUENCE {

...}

UnauthorizedRequestingNetwork-Param ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

UnauthorizedLCSClient-Param ::= SEQUENCE {

unauthorizedLCSClient-Diagnostic	[0] UnauthorizedLCSClient-Diagnostic	OPTIONAL,

extensionContainer	[1] ExtensionContainer	OPTIONAL,

... }

UnauthorizedLCSClient-Diagnostic ::= ENUMERATED {

noAdditionalInformation (0),

clientNotInMSPrivacyExceptionList (1),

callToClientNotSetup (2),

privacyOverrideNotApplicable (3),

disallowedByLocalRegulatoryRequirements (4),

...,

unauthorizedPrivacyClass (5),

unauthorizedCallSessionUnrelatedExternalClient (6),

unauthorizedCallSessionRelatedExternalClient (7) }

--	exception handling:

--	any unrecognized value shall be ignored

PositionMethodFailure-Param ::= SEQUENCE {

positionMethodFailure-Diagnostic	[0] PositionMethodFailure-Diagnostic	OPTIONAL,

extensionContainer	[1] ExtensionContainer	OPTIONAL,

... }

PositionMethodFailure-Diagnostic ::= ENUMERATED {

congestion  (0),

insufficientResources  (1),

insufficientMeasurementData  (2),

inconsistentMeasurementData  (3),

locationProcedureNotCompleted  (4),

locationProcedureNotSupportedByTargetMS  (5),

qoSNotAttainable  (6),

positionMethodNotAvailableInNetwork	(7),

positionMethodNotAvailableInLocationArea	(8),

... }

--	exception handling:

--	any unrecognized value shall be ignored

UnknownOrUnreachableLCSClient-Param ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

# MM-EventNotSupported-Param ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

TargetCellOutsideGCA-Param ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

OngoingGroupCallParam ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

.#END

* * * Next Change * * * *

### 17.7.8	Common data types

.$MAP-CommonDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version22 (22)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

-- general data types and values

AddressString,

ISDN-AddressString,

maxISDN-AddressLength,

FTN-AddressString,

ISDN-SubaddressString,

ExternalSignalInfo,

Ext-ExternalSignalInfo,

AccessNetworkSignalInfo,

SignalInfo,

maxSignalInfoLength,

AlertingPattern,

TBCD-STRING,

DiameterIdentity,

Time,

HLR-Id,

-- data types for numbering and identification

IMSI,

TMSI,

Identity,

SubscriberId,

IMEI,

HLR-List,

LMSI,

GlobalCellId,

NetworkResource,

AdditionalNetworkResource,

NAEA-PreferredCI,

NAEA-CIC,

ASCI-CallReference,

SubscriberIdentity,

PLMN-Id,

E-UTRAN-CGI,

NR-CGI,

TA-Id,

NR-TA-Id,

RAIdentity,

NetworkNodeDiameterAddress,

-- data types for CAMEL

CellGlobalIdOrServiceAreaIdOrLAI,

CellGlobalIdOrServiceAreaIdFixedLength,

LAIFixedLength,

-- data types for subscriber management

BasicServiceCode,

Ext-BasicServiceCode,

EMLPP-Info,

EMLPP-Priority,

MC-SS-Info,

MaxMC-Bearers,

MC-Bearers,

Ext-SS-Status,

-- data types for geographic location

AgeOfLocationInformation,

LCSClientExternalID,

LCSClientInternalID,

LCSServiceTypeID,

-- gprs location registration types

GSN-Address

;

IMPORTS

TeleserviceCode,

Ext-TeleserviceCode

FROM MAP-TS-Code {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-TS-Code (19) version22 (22)}

BearerServiceCode,

Ext-BearerServiceCode

FROM MAP-BS-Code {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-BS-Code (20) version22 (22)}

SS-Code

FROM MAP-SS-Code {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-Code (15) version22 (22)}

ExtensionContainer

FROM MAP-ExtensionDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ExtensionDataTypes (21) version22 (22)}

;

-- general data types

TBCD-STRING ::= OCTET STRING

-- This type (Telephony Binary Coded Decimal String) is used to

-- represent several digits from 0 through 9, *, #, a, b, c, two

-- digits per octet, each digit encoded 0000 to 1001 (0 to 9),

-- 1010 (*), 1011 (#), 1100 (a), 1101 (b) or 1110 (c); 1111 used

-- as filler when there is an odd number of digits.

-- bits 8765 of octet n encoding digit 2n

-- bits 4321 of octet n encoding digit 2(n-1) +1

DiameterIdentity ::= OCTET STRING (SIZE(9..255))

-- content of DiameterIdentity is defined in IETF RFC 3588 [139]

AddressString ::= OCTET STRING (SIZE (1..maxAddressLength))

-- This type is used to represent a number for addressing

-- purposes. It is composed of

--	a)	one octet for nature of address, and numbering plan

--	indicator.

--	b)	digits of an address encoded as TBCD-String.

-- a)	The first octet includes a one bit extension indicator, a

--	3 bits nature of address indicator and a 4 bits numbering

--	plan indicator, encoded as follows:

-- bit 8: 1  (no extension)

-- bits 765: nature of address indicator

--	000  unknown

--	001  international number

--	010  national significant number

--	011  network specific number

--	100  subscriber number

--	101  reserved

--	110  abbreviated number

--	111  reserved for extension

-- bits 4321: numbering plan indicator

--	0000  unknown

--	0001  ISDN/Telephony Numbering Plan (Rec ITU-T E.164)

--	0010  spare

--	0011  data numbering plan (ITU-T Rec X.121)

--	0100  telex numbering plan (ITU-T Rec F.69)

--	0101  spare

--	0110  land mobile numbering plan (ITU-T Rec E.212)

--	0111  spare

--	1000  national numbering plan

--	1001  private numbering plan

--	1111  reserved for extension

--	all other values are reserved.

-- b)	The following octets representing digits of an address

--	encoded as a TBCD-STRING.

maxAddressLength  INTEGER ::= 20

ISDN-AddressString ::=

AddressString (SIZE (1..maxISDN-AddressLength))

-- This type is used to represent ISDN numbers.

maxISDN-AddressLength  INTEGER ::= 9

FTN-AddressString ::=

AddressString (SIZE (1..maxFTN-AddressLength))

-- This type is used to represent forwarded-to numbers.

-- If NAI = international the first digits represent the country code (CC)

-- and the network destination code (NDC) as for E.164.

maxFTN-AddressLength  INTEGER ::= 15

ISDN-SubaddressString ::=

OCTET STRING (SIZE (1..maxISDN-SubaddressLength))

-- This type is used to represent ISDN subaddresses.

-- It is composed of

--	a)	one octet for type of subaddress and odd/even indicator.

--	b)	20 octets for subaddress information.

--	a)	The first octet includes a one bit extension indicator, a

--	3 bits type of subaddress and a one bit odd/even indicator,

--	encoded as follows:

--	bit 8: 1  (no extension)

--	bits 765: type of subaddress

--	000  NSAP (X.213/ISO 8348 AD2)

--	010  User Specified

--	All other values are reserved

--	bit 4: odd/even indicator

--	0  even number of address signals

--	1  odd number of address signals

--	The odd/even indicator is used when the type of subaddress

--	is "user specified" and the coding is BCD.

--	bits 321: 000 (unused)

--	b) Subaddress information.

--	The NSAP X.213/ISO8348AD2 address shall be formatted as specified

--	by octet 4 which contains the Authority and Format Identifier

--	(AFI). The encoding is made according to the "preferred binary

--	encoding" as defined in X.213/ISO834AD2. For the definition

--	of this type of subaddress, see ITU-T Rec I.334.

--	For User-specific subaddress, this field is encoded according

--	to the user specification, subject to a maximum length of 20

--	octets. When interworking with X.25 networks BCD coding should

--	be applied.

maxISDN-SubaddressLength  INTEGER ::= 21

ExternalSignalInfo ::= SEQUENCE {

protocolId	ProtocolId,

signalInfo	SignalInfo,

-- Information about the internal structure is given in

-- clause 7.6.9.

extensionContainer	ExtensionContainer	OPTIONAL,

-- extensionContainer must not be used in version 2

...}

SignalInfo ::= OCTET STRING (SIZE (1..maxSignalInfoLength))

maxSignalInfoLength  INTEGER ::= 200

-- This NamedValue represents the theoretical maximum number of octets which is

-- available to carry a single instance of the SignalInfo data type,

-- without requiring segmentation to cope with the network layer service.

-- However, the actual maximum size available for an instance of the data

-- type may be lower, especially when other information elements

-- have to be included in the same component.

ProtocolId ::= ENUMERATED {

gsm-0408  (1),

gsm-0806  (2),

gsm-BSSMAP  (3),

-- Value 3 is reserved and must not be used

ets-300102-1  (4)}

Ext-ExternalSignalInfo ::= SEQUENCE {

ext-ProtocolId	Ext-ProtocolId,

signalInfo	SignalInfo,

-- Information about the internal structure is given in

-- clause 7.6.9.10

extensionContainer	ExtensionContainer	OPTIONAL,

...}

Ext-ProtocolId ::= ENUMERATED {

ets-300356  (1),

...

}

-- exception handling:

-- For Ext-ExternalSignalInfo sequences containing this parameter with any

-- other value than the ones listed the receiver shall ignore the whole

-- Ext-ExternalSignalInfo sequence.

AccessNetworkSignalInfo ::= SEQUENCE {

accessNetworkProtocolId	AccessNetworkProtocolId,

signalInfo	LongSignalInfo,

-- Information about the internal structure is given in clause 7.6.9.1

extensionContainer	ExtensionContainer	OPTIONAL,

...}

LongSignalInfo ::= OCTET STRING (SIZE (1..maxLongSignalInfoLength))

maxLongSignalInfoLength  INTEGER ::= 2560

-- This Named Value represents the maximum number of octets which is available

-- to carry a single instance of the LongSignalInfo data type using

-- White Book SCCP with the maximum number of segments.

-- It takes account of the octets used by the lower layers of the protocol, and

-- other information elements which may be included in the same component.

AccessNetworkProtocolId ::= ENUMERATED {

ts3G-48006   (1),

ts3G-25413 (2),

...}

-- exception handling:

-- For AccessNetworkSignalInfo sequences containing this parameter with any

-- other value than the ones listed the receiver shall ignore the whole

-- AccessNetworkSignalInfo sequence.

AlertingPattern ::= OCTET STRING (SIZE (1) )

-- This type is used to represent Alerting Pattern

--	bits 8765 : 0000 (unused)

--	bits 43 : type of Pattern

--	00 level

--	01 category

--	10 category

--	all other values are reserved.

--	bits 21 : type of alerting

alertingLevel-0   AlertingPattern ::= '00000000'B

alertingLevel-1   AlertingPattern ::= '00000001'B

alertingLevel-2   AlertingPattern ::= '00000010'B

-- all other values of Alerting level are reserved

-- Alerting Levels are defined in GSM 02.07

alertingCategory-1   AlertingPattern ::= '00000100'B

alertingCategory-2   AlertingPattern ::= '00000101'B

alertingCategory-3   AlertingPattern ::= '00000110'B

alertingCategory-4   AlertingPattern ::= '00000111'B

alertingCategory-5   AlertingPattern ::= '00001000'B

-- all other values of Alerting Category are reserved

-- Alerting categories are defined in GSM 02.07

GSN-Address ::= OCTET STRING (SIZE (5..17))

-- Octets are coded according to TS 3GPP TS 23.003 [17]

Time ::= OCTET STRING (SIZE (4))

-- Octets are coded according to IETF RFC 3588 [139]

-- data types for numbering and identification

IMSI ::= TBCD-STRING (SIZE (3..8))

-- digits of MCC, MNC, MSIN are concatenated in this order.

Identity ::= CHOICE {

imsi	IMSI,

imsi-WithLMSI	IMSI-WithLMSI}

IMSI-WithLMSI ::= SEQUENCE {

imsi	IMSI,

lmsi	LMSI,

-- a special value 00000000 indicates that the LMSI is not in use

...}

ASCI-CallReference ::= TBCD-STRING (SIZE (1..8))

-- digits of VGCS/VBS-area,Group-ID are concatenated in this order if there is a

-- VGCS/VBS-area.

TMSI ::= OCTET STRING (SIZE (1..4))

SubscriberId ::= CHOICE {

imsi	[0] IMSI,

tmsi	[1] TMSI}

IMEI ::= TBCD-STRING (SIZE (8))

--	Refers to International Mobile Station Equipment Identity

--	and Software Version Number (SVN) defined in TS 3GPP TS 23.003 [17].

--	If the SVN is not present the last octet shall contain the

--	digit 0 and a filler.

--	If present the SVN shall be included in the last octet.

HLR-Id ::= IMSI

-- leading digits of IMSI, i.e. (MCC, MNC, leading digits of

-- MSIN) forming HLR Id defined in TS 3GPP TS 23.003 [17].

HLR-List ::= SEQUENCE SIZE (1..maxNumOfHLR-Id) OF

HLR-Id

maxNumOfHLR-Id  INTEGER ::= 50

LMSI ::= OCTET STRING (SIZE (4))

GlobalCellId ::= OCTET STRING (SIZE (5..7))

-- Refers to Cell Global Identification defined in TS 3GPP TS 23.003 [17].

-- The internal structure is defined as follows:

-- octet 1 bits 4321	Mobile Country Code 1st digit

--         bits 8765	Mobile Country Code 2nd digit

-- octet 2 bits 4321	Mobile Country Code 3rd digit

--         bits 8765	Mobile Network Code 3rd digit

--	or filler (1111) for 2 digit MNCs

-- octet 3 bits 4321	Mobile Network Code 1st digit

--         bits 8765	Mobile Network Code 2nd digit

-- octets 4 and 5	Location Area Code according to TS 3GPP TS 24.008 [35]

-- octets 6 and 7	Cell Identity (CI) according to TS 3GPP TS 24.008 [35]

NetworkResource ::= ENUMERATED {

plmn  (0),

hlr  (1),

vlr  (2),

pvlr  (3),

controllingMSC  (4),

vmsc  (5),

eir  (6),

rss  (7)}

AdditionalNetworkResource ::= ENUMERATED {

sgsn (0),

ggsn (1),

gmlc (2),

gsmSCF (3),

nplr (4),

auc (5),

... ,

ue (6),

mme (7)}

-- if unknown value is received in AdditionalNetworkResource

-- it shall be ignored.

NAEA-PreferredCI ::= SEQUENCE {

naea-PreferredCIC	[0] NAEA-CIC,

extensionContainer	[1] ExtensionContainer	OPTIONAL,

...}

NAEA-CIC ::= OCTET STRING (SIZE (3))

-- The internal structure is defined by the Carrier Identification

-- parameter in ANSI T1.113.3. Carrier codes between "000" and "999" may

-- be encoded as 3 digits using "000" to "999" or as 4 digits using

-- "0000" to "0999". Carrier codes between "1000" and "9999" are encoded

-- using 4 digits.

SubscriberIdentity ::= CHOICE {

imsi	[0] IMSI,

msisdn	[1] ISDN-AddressString

}

LCSClientExternalID ::= SEQUENCE {

externalAddress	[0] ISDN-AddressString	OPTIONAL,

extensionContainer	[1] ExtensionContainer	OPTIONAL,

... }

LCSClientInternalID ::= ENUMERATED {

broadcastService	(0),

o-andM-HPLMN	(1),

o-andM-VPLMN	(2),

anonymousLocation	(3),

targetMSsubscribedService	(4),

... }

-- for a CAMEL phase 3 PLMN operator client, the value targetMSsubscribedService shall be used

LCSServiceTypeID ::= INTEGER (0..127)

-- the integer values 0-63 are reserved for Standard LCS service types

-- the integer values 64-127 are reserved for Non Standard LCS service types

-- Standard LCS Service Types

emergencyServices	LCSServiceTypeID ::= 0

emergencyAlertServices	LCSServiceTypeID ::= 1

personTracking	LCSServiceTypeID ::= 2

fleetManagement	LCSServiceTypeID ::= 3

assetManagement	LCSServiceTypeID ::= 4

trafficCongestionReporting	LCSServiceTypeID ::= 5

roadsideAssistance	LCSServiceTypeID ::= 6

routingToNearestCommercialEnterprise	LCSServiceTypeID ::= 7

navigation	LCSServiceTypeID ::= 8

--this service type is reserved for use in previous releases

citySightseeing	LCSServiceTypeID ::= 9

localizedAdvertising	LCSServiceTypeID ::= 10

mobileYellowPages	LCSServiceTypeID ::= 11

trafficAndPublicTransportationInfo	LCSServiceTypeID ::= 12

weather	LCSServiceTypeID ::= 13

assetAndServiceFinding	LCSServiceTypeID ::= 14

gaming	LCSServiceTypeID ::= 15

findYourFriend	LCSServiceTypeID ::= 16

dating	LCSServiceTypeID ::= 17

chatting	LCSServiceTypeID ::= 18

routeFinding	LCSServiceTypeID ::= 19

whereAmI	LCSServiceTypeID ::= 20

-- The values of LCSServiceTypeID are defined according to 3GPP TS 22.071.

-- Non Standard LCS Service Types

serv64	LCSServiceTypeID ::= 64

serv65	LCSServiceTypeID ::= 65

serv66	LCSServiceTypeID ::= 66

serv67	LCSServiceTypeID ::= 67

serv68	LCSServiceTypeID ::= 68

serv69	LCSServiceTypeID ::= 69

serv70	LCSServiceTypeID ::= 70

serv71	LCSServiceTypeID ::= 71

serv72	LCSServiceTypeID ::= 72

serv73	LCSServiceTypeID ::= 73

serv74	LCSServiceTypeID ::= 74

serv75	LCSServiceTypeID ::= 75

serv76	LCSServiceTypeID ::= 76

serv77	LCSServiceTypeID ::= 77

serv78	LCSServiceTypeID ::= 78

serv79	LCSServiceTypeID ::= 79

serv80	LCSServiceTypeID ::= 80

serv81	LCSServiceTypeID ::= 81

serv82	LCSServiceTypeID ::= 82

serv83	LCSServiceTypeID ::= 83

serv84	LCSServiceTypeID ::= 84

serv85	LCSServiceTypeID ::= 85

serv86	LCSServiceTypeID ::= 86

serv87	LCSServiceTypeID ::= 87

serv88	LCSServiceTypeID ::= 88

serv89	LCSServiceTypeID ::= 89

serv90	LCSServiceTypeID ::= 90

serv91	LCSServiceTypeID ::= 91

serv92	LCSServiceTypeID ::= 92

serv93	LCSServiceTypeID ::= 93

serv94	LCSServiceTypeID ::= 94

serv95	LCSServiceTypeID ::= 95

serv96	LCSServiceTypeID ::= 96

serv97	LCSServiceTypeID ::= 97

serv98	LCSServiceTypeID ::= 98

serv99	LCSServiceTypeID ::= 99

serv100	LCSServiceTypeID ::= 100

serv101	LCSServiceTypeID ::= 101

serv102	LCSServiceTypeID ::= 102

serv103	LCSServiceTypeID ::= 103

serv104	LCSServiceTypeID ::= 104

serv105	LCSServiceTypeID ::= 105

serv106	LCSServiceTypeID ::= 106

serv107	LCSServiceTypeID ::= 107

serv108	LCSServiceTypeID ::= 108

serv109	LCSServiceTypeID ::= 109

serv110	LCSServiceTypeID ::= 110

serv111	LCSServiceTypeID ::= 111

serv112	LCSServiceTypeID ::= 112

serv113	LCSServiceTypeID ::= 113

serv114	LCSServiceTypeID ::= 114

serv115	LCSServiceTypeID ::= 115

serv116	LCSServiceTypeID ::= 116

serv117	LCSServiceTypeID ::= 117

serv118	LCSServiceTypeID ::= 118

serv119	LCSServiceTypeID ::= 119

serv120	LCSServiceTypeID ::= 120

serv121	LCSServiceTypeID ::= 121

serv122	LCSServiceTypeID ::= 122

serv123	LCSServiceTypeID ::= 123

serv124	LCSServiceTypeID ::= 124

serv125	LCSServiceTypeID ::= 125

serv126	LCSServiceTypeID ::= 126

serv127	LCSServiceTypeID ::= 127

PLMN-Id ::= OCTET STRING (SIZE (3))

-- The internal structure is defined as follows:

-- octet 1 bits 4321	Mobile Country Code 1st digit

--         bits 8765	Mobile Country Code 2nd digit

-- octet 2 bits 4321	Mobile Country Code 3rd digit

--         bits 8765	Mobile Network Code 3rd digit

--	or filler (1111) for 2 digit MNCs

-- octet 3 bits 4321	Mobile Network Code 1st digit

--         bits 8765	Mobile Network Code 2nd digit

E-UTRAN-CGI ::= OCTET STRING (SIZE (7))

-- Octets are coded as described in 3GPP TS 29.118 [152].

NR-CGI ::= OCTET STRING (SIZE (8))

-- Octets are coded as described in 3GPP TS 38.413 [153].

TA-Id ::= OCTET STRING (SIZE (5))

-- Octets are coded as described in 3GPP TS 29.118 [152].

NR-TA-Id ::= OCTET STRING (SIZE (6))

-- Octets are coded as described in 3GPP TS 38.413 [153].

RAIdentity ::= OCTET STRING (SIZE (6))

-- Routing Area Identity is coded in accordance with 3GPP TS 29.060 [105].

-- It shall contain the value part defined in 3GPP TS 29.060 only. I.e. the 3GPP TS 29.060

-- type identifier octet shall not be included.

NetworkNodeDiameterAddress::= SEQUENCE {

diameter-Name	[0] DiameterIdentity,

diameter-Realm	[1] DiameterIdentity }

-- data types for CAMEL

CellGlobalIdOrServiceAreaIdOrLAI ::= CHOICE {

cellGlobalIdOrServiceAreaIdFixedLength	[0] CellGlobalIdOrServiceAreaIdFixedLength,

laiFixedLength	[1] LAIFixedLength}

CellGlobalIdOrServiceAreaIdFixedLength ::= OCTET STRING (SIZE (7))

-- Refers to Cell Global Identification or Service Are Identification

-- defined in 3GPP TS 23.003.

-- The internal structure is defined as follows:

-- octet 1 bits 4321	Mobile Country Code 1st digit

--         bits 8765	Mobile Country Code 2nd digit

-- octet 2 bits 4321	Mobile Country Code 3rd digit

--         bits 8765	Mobile Network Code 3rd digit

--	or filler (1111) for 2 digit MNCs

-- octet 3 bits 4321	Mobile Network Code 1st digit

--         bits 8765	Mobile Network Code 2nd digit

-- octets 4 and 5	Location Area Code according to 3GPP TS 24.008

-- octets 6 and 7	Cell Identity (CI) value or

--	Service Area Code (SAC) value

--	according to 3GPP TS 23.003

LAIFixedLength ::= OCTET STRING (SIZE (5))

-- Refers to Location Area Identification defined in 3GPP TS 23.003 [17].

-- The internal structure is defined as follows:

-- octet 1 bits 4321	Mobile Country Code 1st digit

--         bits 8765	Mobile Country Code 2nd digit

-- octet 2 bits 4321	Mobile Country Code 3rd digit

--         bits 8765	Mobile Network Code 3rd digit

--	or filler (1111) for 2 digit MNCs

-- octet 3 bits 4321	Mobile Network Code 1st digit

--         bits 8765	Mobile Network Code 2nd digit

-- octets 4 and 5	Location Area Code according to 3GPP TS 24.008 [35]

-- data types for subscriber management

BasicServiceCode ::= CHOICE {

bearerService	[2] BearerServiceCode,

teleservice	[3] TeleserviceCode}

Ext-BasicServiceCode ::= CHOICE {

ext-BearerService	[2] Ext-BearerServiceCode,

ext-Teleservice	[3] Ext-TeleserviceCode}

EMLPP-Info ::= SEQUENCE {

maximumentitledPriority	EMLPP-Priority,

defaultPriority	EMLPP-Priority,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

EMLPP-Priority ::= INTEGER (0..15)

-- The mapping from the values A,B,0,1,2,3,4 to the integer-value is

-- specified as follows where A is the highest and 4 is the lowest

-- priority level

-- the integer values 7-15 are spare and shall be mapped to value 4

priorityLevelA	EMLPP-Priority ::= 6

priorityLevelB	EMLPP-Priority ::= 5

priorityLevel0	EMLPP-Priority ::= 0

priorityLevel1	EMLPP-Priority ::= 1

priorityLevel2	EMLPP-Priority ::= 2

priorityLevel3	EMLPP-Priority ::= 3

priorityLevel4	EMLPP-Priority ::= 4

MC-SS-Info ::= SEQUENCE {

ss-Code	[0] SS-Code,

ss-Status	[1] Ext-SS-Status,

nbrSB	[2] MaxMC-Bearers,

nbrUser	[3] MC-Bearers,

extensionContainer	[4] ExtensionContainer	OPTIONAL,

...}

MaxMC-Bearers ::= INTEGER (2..maxNumOfMC-Bearers)

MC-Bearers ::= INTEGER (1..maxNumOfMC-Bearers)

maxNumOfMC-Bearers  INTEGER ::= 7

Ext-SS-Status ::= OCTET STRING (SIZE (1..5))

-- OCTET 1:

--

-- bits 8765: 0000 (unused)

-- bits 4321: Used to convey the "P bit","R bit","A bit" and "Q bit",

--	 representing supplementary service state information

--	 as defined in TS 3GPP TS 23.011 [22]

-- bit 4: "Q bit"

-- bit 3: "P bit"

-- bit 2: "R bit"

-- bit 1: "A bit"

-- OCTETS 2-5: reserved for future use. They shall be discarded if

-- received and not understood.

-- data types for geographic location

AgeOfLocationInformation ::= INTEGER (0..32767)

-- the value represents the elapsed time in minutes since the last

-- network contact of the mobile station (i.e. the actuality of the

-- location information).

-- value "0" indicates that the MS is currently in contact with the

--           network

-- value "32767" indicates that the location information is at least

--               32767 minutes old

.#END

* * * Next Change * * * *

### 17.7.9	Teleservice Codes

.$MAP-TS-Code {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-TS-Code (19) version22 (22)}

DEFINITIONS

::=

BEGIN

TeleserviceCode ::= OCTET STRING (SIZE (1))

-- This type is used to represent the code identifying a single

-- teleservice, a group of teleservices, or all teleservices. The

-- services are defined in TS GSM 22.003 [4].

-- The internal structure is defined as follows:

-- bits 87654321: group (bits 8765) and specific service

-- (bits 4321)

Ext-TeleserviceCode ::= OCTET STRING (SIZE (1..5))

-- This type is used to represent the code identifying a single

-- teleservice, a group of teleservices, or all teleservices. The

-- services are defined in TS GSM 22.003 [4].

-- The internal structure is defined as follows:

-- OCTET 1:

-- bits 87654321: group (bits 8765) and specific service

-- (bits 4321)

-- OCTETS 2-5: reserved for future use. If received the

-- Ext-TeleserviceCode shall be

-- treated according to the exception handling defined for the

-- operation that uses this type.

-- Ext-TeleserviceCode includes all values defined for TeleserviceCode.

allTeleservices	TeleserviceCode ::= '00000000'B

allSpeechTransmissionServices	TeleserviceCode ::= '00010000'B

telephony	TeleserviceCode ::= '00010001'B

emergencyCalls	TeleserviceCode ::= '00010010'B

allShortMessageServices	TeleserviceCode ::= '00100000'B

shortMessageMT-PP	TeleserviceCode ::= '00100001'B

shortMessageMO-PP	TeleserviceCode ::= '00100010'B

allFacsimileTransmissionServices	TeleserviceCode ::= '01100000'B

facsimileGroup3AndAlterSpeech	TeleserviceCode ::= '01100001'B

automaticFacsimileGroup3	TeleserviceCode ::= '01100010'B

facsimileGroup4	TeleserviceCode ::= '01100011'B

-- The following non-hierarchical Compound Teleservice Groups

-- are defined in TS 3GPP TS 22.030:

allDataTeleservices	TeleserviceCode ::= '01110000'B

-- covers Teleservice Groups 'allFacsimileTransmissionServices'

-- and 'allShortMessageServices'

allTeleservices-ExeptSMS	TeleserviceCode ::= '10000000'B

-- covers Teleservice Groups 'allSpeechTransmissionServices' and

-- 'allFacsimileTransmissionServices'

--

-- Compound Teleservice Group Codes are only used in call

-- independent supplementary service operations, i.e. they

-- are not used in InsertSubscriberData or in

-- DeleteSubscriberData messages.

allVoiceGroupCallServices	TeleserviceCode ::= '10010000'B

voiceGroupCall	TeleserviceCode ::= '10010001'B

voiceBroadcastCall	TeleserviceCode ::= '10010010'B

allPLMN-specificTS	TeleserviceCode ::= '11010000'B

plmn-specificTS-1	TeleserviceCode ::= '11010001'B

plmn-specificTS-2	TeleserviceCode ::= '11010010'B

plmn-specificTS-3	TeleserviceCode ::= '11010011'B

plmn-specificTS-4	TeleserviceCode ::= '11010100'B

plmn-specificTS-5	TeleserviceCode ::= '11010101'B

plmn-specificTS-6	TeleserviceCode ::= '11010110'B

plmn-specificTS-7	TeleserviceCode ::= '11010111'B

plmn-specificTS-8	TeleserviceCode ::= '11011000'B

plmn-specificTS-9	TeleserviceCode ::= '11011001'B

plmn-specificTS-A	TeleserviceCode ::= '11011010'B

plmn-specificTS-B	TeleserviceCode ::= '11011011'B

plmn-specificTS-C	TeleserviceCode ::= '11011100'B

plmn-specificTS-D	TeleserviceCode ::= '11011101'B

plmn-specificTS-E	TeleserviceCode ::= '11011110'B

plmn-specificTS-F	TeleserviceCode ::= '11011111'B

.#END

* * * Next Change * * * *

### 17.7.10	Bearer Service Codes

.$MAP-BS-Code {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-BS-Code (20) version22 (22)}

DEFINITIONS

::=

BEGIN

BearerServiceCode ::= OCTET STRING (SIZE (1))

-- This type is used to represent the code identifying a single

-- bearer service, a group of bearer services, or all bearer

-- services. The services are defined in TS 3GPP TS 22.002 [3].

-- The internal structure is defined as follows:

--

-- plmn-specific bearer services:

-- bits 87654321: defined by the HPLMN operator

-- rest of bearer services:

-- bit 8: 0 (unused)

-- bits 7654321: group (bits 7654), and rate, if applicable

-- (bits 321)

Ext-BearerServiceCode ::= OCTET STRING (SIZE (1..5))

-- This type is used to represent the code identifying a single

-- bearer service, a group of bearer services, or all bearer

-- services. The services are defined in TS 3GPP TS 22.002 [3].

-- The internal structure is defined as follows:

--

-- OCTET 1:

-- plmn-specific bearer services:

-- bits 87654321: defined by the HPLMN operator

--

-- rest of bearer services:

-- bit 8: 0 (unused)

-- bits 7654321: group (bits 7654), and rate, if applicable

-- (bits 321)

-- OCTETS 2-5: reserved for future use. If received the

-- Ext-TeleserviceCode shall be

-- treated according to the exception handling defined for the

-- operation that uses this type.

-- Ext-BearerServiceCode includes all values defined for BearerServiceCode.

allBearerServices	BearerServiceCode ::= '00000000'B

allDataCDA-Services	BearerServiceCode ::= '00010000'B

dataCDA-300bps	BearerServiceCode ::= '00010001'B

dataCDA-1200bps	BearerServiceCode ::= '00010010'B

dataCDA-1200-75bps	BearerServiceCode ::= '00010011'B

dataCDA-2400bps	BearerServiceCode ::= '00010100'B

dataCDA-4800bps	BearerServiceCode ::= '00010101'B

dataCDA-9600bps	BearerServiceCode ::= '00010110'B

general-dataCDA	BearerServiceCode ::= '00010111'B

allDataCDS-Services	BearerServiceCode ::= '00011000'B

dataCDS-1200bps	BearerServiceCode ::= '00011010'B

dataCDS-2400bps	BearerServiceCode ::= '00011100'B

dataCDS-4800bps	BearerServiceCode ::= '00011101'B

dataCDS-9600bps	BearerServiceCode ::= '00011110'B

general-dataCDS	BearerServiceCode ::= '00011111'B

allPadAccessCA-Services	BearerServiceCode ::= '00100000'B

padAccessCA-300bps	BearerServiceCode ::= '00100001'B

padAccessCA-1200bps	BearerServiceCode ::= '00100010'B

padAccessCA-1200-75bps	BearerServiceCode ::= '00100011'B

padAccessCA-2400bps	BearerServiceCode ::= '00100100'B

padAccessCA-4800bps	BearerServiceCode ::= '00100101'B

padAccessCA-9600bps	BearerServiceCode ::= '00100110'B

general-padAccessCA	BearerServiceCode ::= '00100111'B

allDataPDS-Services	BearerServiceCode ::= '00101000'B

dataPDS-2400bps	BearerServiceCode ::= '00101100'B

dataPDS-4800bps	BearerServiceCode ::= '00101101'B

dataPDS-9600bps	BearerServiceCode ::= '00101110'B

general-dataPDS	BearerServiceCode ::= '00101111'B

allAlternateSpeech-DataCDA	BearerServiceCode ::= '00110000'B

allAlternateSpeech-DataCDS	BearerServiceCode ::= '00111000'B

allSpeechFollowedByDataCDA	BearerServiceCode ::= '01000000'B

allSpeechFollowedByDataCDS	BearerServiceCode ::= '01001000'B

-- The following non-hierarchical Compound Bearer Service

-- Groups are defined in TS 3GPP TS 22.030:

allDataCircuitAsynchronous	BearerServiceCode ::= '01010000'B

-- covers "allDataCDA-Services", "allAlternateSpeech-DataCDA" and

-- "allSpeechFollowedByDataCDA"

allAsynchronousServices	BearerServiceCode ::= '01100000'B

-- covers "allDataCDA-Services", "allAlternateSpeech-DataCDA",

-- "allSpeechFollowedByDataCDA" and "allPadAccessCDA-Services"

allDataCircuitSynchronous	BearerServiceCode ::= '01011000'B

-- covers "allDataCDS-Services", "allAlternateSpeech-DataCDS" and

-- "allSpeechFollowedByDataCDS"

allSynchronousServices	BearerServiceCode ::= '01101000'B

-- covers "allDataCDS-Services", "allAlternateSpeech-DataCDS",

-- "allSpeechFollowedByDataCDS" and "allDataPDS-Services"

--

-- Compound Bearer Service Group Codes are only used in call

-- independent supplementary service operations, i.e. they

-- are not used in InsertSubscriberData or in

-- DeleteSubscriberData messages.

allPLMN-specificBS	BearerServiceCode ::= '11010000'B

plmn-specificBS-1	BearerServiceCode ::= '11010001'B

plmn-specificBS-2	BearerServiceCode ::= '11010010'B

plmn-specificBS-3	BearerServiceCode ::= '11010011'B

plmn-specificBS-4	BearerServiceCode ::= '11010100'B

plmn-specificBS-5	BearerServiceCode ::= '11010101'B

plmn-specificBS-6	BearerServiceCode ::= '11010110'B

plmn-specificBS-7	BearerServiceCode ::= '11010111'B

plmn-specificBS-8	BearerServiceCode ::= '11011000'B

plmn-specificBS-9	BearerServiceCode ::= '11011001'B

plmn-specificBS-A	BearerServiceCode ::= '11011010'B

plmn-specificBS-B	BearerServiceCode ::= '11011011'B

plmn-specificBS-C	BearerServiceCode ::= '11011100'B

plmn-specificBS-D	BearerServiceCode ::= '11011101'B

plmn-specificBS-E	BearerServiceCode ::= '11011110'B

plmn-specificBS-F	BearerServiceCode ::= '11011111'B

.#END

* * * Next Change * * * *

### 17.7.11	Extension data types

.$MAP-ExtensionDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ExtensionDataTypes (21) version22 (22)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

PrivateExtension,

ExtensionContainer,

SLR-ArgExtensionContainer;

-- IOC for private MAP extensions

MAP-EXTENSION  ::= CLASS {

&ExtensionType	OPTIONAL,

&extensionId	OBJECT IDENTIFIER }

-- The length of the Object Identifier shall not exceed 16 octets and the

-- number of components of the Object Identifier shall not exceed 16

-- data types

ExtensionContainer ::= SEQUENCE {

privateExtensionList	[0]PrivateExtensionList	OPTIONAL,

pcs-Extensions	[1]PCS-Extensions	OPTIONAL,

...}

SLR-ArgExtensionContainer ::= SEQUENCE {

privateExtensionList	[0]PrivateExtensionList	OPTIONAL,

slr-Arg-PCS-Extensions	[1]SLR-Arg-PCS-Extensions	OPTIONAL,

...}

PrivateExtensionList ::= SEQUENCE SIZE (1..maxNumOfPrivateExtensions) OF

PrivateExtension

PrivateExtension ::= SEQUENCE {

extId	MAP-EXTENSION.&extensionId

({ExtensionSet}),

extType	MAP-EXTENSION.&ExtensionType

({ExtensionSet}{@extId})	OPTIONAL}

maxNumOfPrivateExtensions  INTEGER ::= 10

ExtensionSet	MAP-EXTENSION ::=

{...

-- ExtensionSet is the set of all defined private extensions

}

-- Unsupported private extensions shall be discarded if received.

PCS-Extensions ::= SEQUENCE {

...}

SLR-Arg-PCS-Extensions ::= SEQUENCE {

...,

na-ESRK-Request	[0]	NULL	OPTIONAL }

.#END

* * * Next Change * * * *

### 17.7.12	Group Call data types

.$MAP-GR-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-GR-DataTypes (23) version22 (22)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

PrepareGroupCallArg,

PrepareGroupCallRes,

SendGroupCallEndSignalArg,

SendGroupCallEndSignalRes,

ForwardGroupCallSignallingArg,

ProcessGroupCallSignallingArg,

SendGroupCallInfoArg,

SendGroupCallInfoRes

;

IMPORTS

ISDN-AddressString,

IMSI,

TMSI,

EMLPP-Priority,

ASCI-CallReference,

SignalInfo,

GlobalCellId,

AccessNetworkSignalInfo

FROM MAP-CommonDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version22 (22)}

Ext-TeleserviceCode

FROM MAP-TS-Code {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-TS-Code (19) version22 (22)}

Kc,

AdditionalInfo,

GroupId,

Long-GroupId,

AdditionalSubscriptions,

Cksn

FROM MAP-MS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version22 (22)}

ExtensionContainer

FROM MAP-ExtensionDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ExtensionDataTypes (21) version22 (22)}

;

PrepareGroupCallArg ::= SEQUENCE {

teleservice	Ext-TeleserviceCode,

asciCallReference	ASCI-CallReference,

codec-Info	CODEC-Info,

cipheringAlgorithm	CipheringAlgorithm,

groupKeyNumber-Vk-Id	[0] GroupKeyNumber	OPTIONAL,

groupKey	[1] Kc	OPTIONAL,

-- this parameter shall not be sent and shall be discarded if received

priority	[2] EMLPP-Priority	OPTIONAL,

uplinkFree	[3] NULL	OPTIONAL,

extensionContainer	[4] ExtensionContainer	OPTIONAL,

...,

vstk	[5] VSTK	OPTIONAL,

vstk-rand	[6] VSTK-RAND	OPTIONAL,

talkerChannelParameter	[7] NULL	OPTIONAL,

uplinkReplyIndicator	[8] NULL	OPTIONAL}

VSTK ::= OCTET STRING (SIZE (16))

VSTK-RAND ::= OCTET STRING (SIZE (5))

-- The 36 bit value is carried in bit 7 of octet 1 to bit 4 of octet 5

-- bits 3, 2, 1, and 0 of octet 5 are padded with zeros.

PrepareGroupCallRes ::= SEQUENCE {

groupCallNumber	ISDN-AddressString,

extensionContainer	ExtensionContainer	OPTIONAL,

...}

SendGroupCallEndSignalArg ::= SEQUENCE {

imsi	IMSI	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

talkerPriority	[0]TalkerPriority	OPTIONAL,

additionalInfo	[1]AdditionalInfo	OPTIONAL }

TalkerPriority ::= ENUMERATED {

normal  (0),

privileged  (1),

emergency  (2)}

SendGroupCallEndSignalRes ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...}

ForwardGroupCallSignallingArg ::= SEQUENCE {

imsi	IMSI	OPTIONAL,

uplinkRequestAck	[0] NULL	OPTIONAL,

uplinkReleaseIndication	[1] NULL	OPTIONAL,

uplinkRejectCommand	[2] NULL	OPTIONAL,

uplinkSeizedCommand	[3] NULL	OPTIONAL,

uplinkReleaseCommand	[4] NULL	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

stateAttributes	[5] StateAttributes	OPTIONAL,

talkerPriority	[6] TalkerPriority	OPTIONAL,

additionalInfo	[7] AdditionalInfo	OPTIONAL,

emergencyModeResetCommandFlag	[8] NULL	OPTIONAL,

sm-RP-UI	[9] SignalInfo	OPTIONAL,

an-APDU	[10] AccessNetworkSignalInfo	OPTIONAL

}

ProcessGroupCallSignallingArg ::= SEQUENCE {

uplinkRequest	[0] NULL	OPTIONAL,

uplinkReleaseIndication	[1] NULL	OPTIONAL,

releaseGroupCall	[2] NULL	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

...,

talkerPriority	[3] TalkerPriority	OPTIONAL,

additionalInfo	[4] AdditionalInfo	OPTIONAL,

emergencyModeResetCommandFlag	[5] NULL	OPTIONAL,

an-APDU	[6] AccessNetworkSignalInfo	OPTIONAL }

GroupKeyNumber ::= INTEGER (0..15)

CODEC-Info ::= OCTET STRING (SIZE (5..10))

-- Refers to channel type

-- coded according to 3GPP TS 48.008 [49] and including Element identifier and Length

CipheringAlgorithm ::= OCTET STRING (SIZE (1))

-- Refers to 'permitted algorithms' in 'encryption information'

-- coded according to 3GPP TS 48.008 [49]:

-- Bits 8-1

-- 8765 4321

-- 0000 0001	No encryption

-- 0000 0010	GSM A5/1

-- 0000 0100	GSM A5/2

-- 0000 1000	GSM A5/3

-- 0001 0000	GSM A5/4

-- 0010 0000	GSM A5/5

-- 0100 0000	GSM A5/6

-- 1000 0000	GSM A5/7

StateAttributes ::= SEQUENCE {

downlinkAttached	[5] NULL	OPTIONAL,

uplinkAttached	[6] NULL	OPTIONAL,

dualCommunication	[7] NULL	OPTIONAL,

callOriginator	[8] NULL	OPTIONAL }

-- Refers to 3GPP TS 44.068 for definitions of StateAttributes fields.

SendGroupCallInfoArg ::= SEQUENCE {

requestedInfo	RequestedInfo,

groupId	Long-GroupId,

teleservice	Ext-TeleserviceCode,

cellId	[0] GlobalCellId	OPTIONAL,

imsi	[1] IMSI	OPTIONAL,

tmsi	[2] TMSI	OPTIONAL,

additionalInfo	[3] AdditionalInfo	OPTIONAL,

talkerPriority	[4] TalkerPriority	OPTIONAL,

cksn	[5] Cksn	OPTIONAL,

extensionContainer	[6] ExtensionContainer	OPTIONAL,

... }

RequestedInfo ::= ENUMERATED {

anchorMSC-AddressAndASCI-CallReference	(0),

imsiAndAdditionalInfoAndAdditionalSubscription	(1),

... }

--	exception handling:

--	an unrecognized value shall be rejected by the receiver with a return error cause of

--	unexpected data value

SendGroupCallInfoRes ::= SEQUENCE {

anchorMSC-Address	[0] ISDN-AddressString	OPTIONAL,

asciCallReference	[1] ASCI-CallReference	OPTIONAL,

imsi	[2] IMSI	OPTIONAL,

additionalInfo	[3] AdditionalInfo	OPTIONAL,

additionalSubscriptions	[4] AdditionalSubscriptions	OPTIONAL,

kc	[5] Kc	OPTIONAL,

extensionContainer	[6] ExtensionContainer	OPTIONAL,

... }

.#END

* * * Next Change * * * *

### 17.7.13	Location service data types

.$MAP-LCS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-LCS-DataTypes (25) version22 (22)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

RoutingInfoForLCS-Arg,

RoutingInfoForLCS-Res,

ProvideSubscriberLocation-Arg,

ProvideSubscriberLocation-Res,

SubscriberLocationReport-Arg,

SubscriberLocationReport-Res,

LocationType,

DeferredLocationEventType,

LCSClientName,

LCS-QoS,

Horizontal-Accuracy,

ResponseTime,

Ext-GeographicalInformation,

VelocityEstimate,

SupportedGADShapes,

Add-GeographicalInformation,

LCSRequestorID,

LCS-ReferenceNumber,

LCSCodeword,

AreaEventInfo,

ReportingPLMNList,

PeriodicLDRInfo,

SequenceNumber,

LCSClientType,

LCS-Priority,

OccurrenceInfo,

IntervalTime

;

IMPORTS

AddressString,

ISDN-AddressString,

IMEI,

IMSI,

LMSI,

SubscriberIdentity,

AgeOfLocationInformation,

LCSClientExternalID,

LCSClientInternalID,

LCSServiceTypeID,

CellGlobalIdOrServiceAreaIdOrLAI,

PLMN-Id,

GSN-Address,

DiameterIdentity

FROM MAP-CommonDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version22 (22)}

ExtensionContainer,

SLR-ArgExtensionContainer

FROM MAP-ExtensionDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ExtensionDataTypes (21) version22 (22)}

USSD-DataCodingScheme,

USSD-String

FROM MAP-SS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3)

map-SS-DataTypes (14) version22 (22)}

APN,

SupportedLCS-CapabilitySets

FROM MAP-MS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version22 (22)}

Additional-Number

FROM MAP-SM-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SM-DataTypes (16) version22 (22)}

;

RoutingInfoForLCS-Arg ::= SEQUENCE {

mlcNumber	[0] ISDN-AddressString,

targetMS	[1] SubscriberIdentity,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

...}

RoutingInfoForLCS-Res ::= SEQUENCE {

targetMS	[0] SubscriberIdentity,

lcsLocationInfo	[1] LCSLocationInfo,

extensionContainer	[2] ExtensionContainer	OPTIONAL,

...,

v-gmlc-Address	[3]	GSN-Address	OPTIONAL,

h-gmlc-Address	[4]	GSN-Address	OPTIONAL,

ppr-Address	[5]	GSN-Address	OPTIONAL,

additional-v-gmlc-Address	[6]	GSN-Address	OPTIONAL }

LCSLocationInfo ::= SEQUENCE {

networkNode-Number	ISDN-AddressString,

-- NetworkNode-number can be msc-number, sgsn-number or a dummy value of "0"

lmsi	[0] LMSI	OPTIONAL,

extensionContainer	[1] ExtensionContainer	OPTIONAL,

... ,

gprsNodeIndicator	[2] NULL	OPTIONAL,

-- gprsNodeIndicator is set only if the SGSN number is sent as the Network Node Number

additional-Number	[3] Additional-Number	OPTIONAL,

supportedLCS-CapabilitySets	[4]	SupportedLCS-CapabilitySets	OPTIONAL,

additional-LCS-CapabilitySets	[5]	SupportedLCS-CapabilitySets	OPTIONAL,

mme-Name	[6]	DiameterIdentity	OPTIONAL,

aaa-Server-Name	[8]	DiameterIdentity	OPTIONAL,

sgsn-Name	[9]	DiameterIdentity	OPTIONAL,

sgsn-Realm	[10]	DiameterIdentity	OPTIONAL

}

ProvideSubscriberLocation-Arg ::= SEQUENCE {

locationType	LocationType,

mlc-Number	ISDN-AddressString,

lcs-ClientID	[0] LCS-ClientID	OPTIONAL,

privacyOverride	[1] NULL	OPTIONAL,

imsi	[2] IMSI	OPTIONAL,

msisdn	[3] ISDN-AddressString	OPTIONAL,

lmsi	[4] LMSI	OPTIONAL,

imei	[5] IMEI	OPTIONAL,

lcs-Priority	[6] LCS-Priority	OPTIONAL,

lcs-QoS	[7] LCS-QoS	OPTIONAL,

extensionContainer	[8] ExtensionContainer	OPTIONAL,

... ,

supportedGADShapes	[9]	SupportedGADShapes	OPTIONAL,

lcs-ReferenceNumber	[10]	LCS-ReferenceNumber	OPTIONAL,

lcsServiceTypeID	[11]	LCSServiceTypeID	OPTIONAL,

lcsCodeword	[12]	LCSCodeword	OPTIONAL,

lcs-PrivacyCheck	[13]	LCS-PrivacyCheck	OPTIONAL,

areaEventInfo	[14]	AreaEventInfo	OPTIONAL,

h-gmlc-Address	[15]	GSN-Address	OPTIONAL,

mo-lrShortCircuitIndicator	[16] NULL	OPTIONAL,

periodicLDRInfo	[17] PeriodicLDRInfo	OPTIONAL,

reportingPLMNList	[18] ReportingPLMNList	OPTIONAL }

-- one of imsi or msisdn is mandatory

-- If a location estimate type indicates activate deferred location or cancel deferred

-- location, a lcs-Reference number shall be included.

LocationType ::= SEQUENCE {

locationEstimateType	[0] LocationEstimateType,

...,

deferredLocationEventType	[1] DeferredLocationEventType	OPTIONAL }

LocationEstimateType ::= ENUMERATED {

currentLocation	(0),

currentOrLastKnownLocation	(1),

initialLocation	(2),

...,

activateDeferredLocation	(3),

cancelDeferredLocation	(4) ,

notificationVerificationOnly	(5) }

--	exception handling:

--	a ProvideSubscriberLocation-Arg containing an unrecognized LocationEstimateType

--	shall be rejected by the receiver with a return error cause of unexpected data value

DeferredLocationEventType ::= BIT STRING {

msAvailable	(0) ,

enteringIntoArea	(1),

leavingFromArea	(2),

beingInsideArea	(3) ,

periodicLDR	(4)  } (SIZE (1..16))

-- beingInsideArea is always treated as oneTimeEvent regardless of the possible value

-- of occurrenceInfo inside areaEventInfo.

-- exception handling:

-- a ProvideSubscriberLocation-Arg containing other values than listed above in

-- DeferredLocationEventType shall be rejected by the receiver with a return error cause of

-- unexpected data value.

LCS-ClientID ::= SEQUENCE {

lcsClientType	[0] LCSClientType,

lcsClientExternalID	[1] LCSClientExternalID	OPTIONAL,

lcsClientDialedByMS	[2] AddressString	OPTIONAL,

lcsClientInternalID	[3] LCSClientInternalID	OPTIONAL,

lcsClientName	[4] LCSClientName	OPTIONAL,

...,

lcsAPN	[5] APN	OPTIONAL,

lcsRequestorID	[6] LCSRequestorID	OPTIONAL }

LCSClientType ::= ENUMERATED {

emergencyServices	(0),

valueAddedServices	(1),

plmnOperatorServices	(2),

lawfulInterceptServices	(3),

... }

--	exception handling:

--	unrecognized values may be ignored if the LCS client uses the privacy override

--	otherwise, an unrecognized value shall be treated as unexpected data by a receiver

--	a return error shall then be returned if received in a MAP invoke

LCSClientName ::= SEQUENCE {

dataCodingScheme	[0] USSD-DataCodingScheme,

nameString	[2] NameString,

...,

lcs-FormatIndicator	[3] LCS-FormatIndicator	OPTIONAL }

-- The USSD-DataCodingScheme shall indicate use of the default alphabet through the

-- following encoding

--	bit	7 6 5 4 3 2 1 0

--	0 0 0 0 1 1 1 1

NameString ::= USSD-String (SIZE (1..maxNameStringLength))

maxNameStringLength  INTEGER ::= 63

LCSRequestorID ::= SEQUENCE {

dataCodingScheme	[0] USSD-DataCodingScheme,

requestorIDString	[1] RequestorIDString,

...,

lcs-FormatIndicator	[2] LCS-FormatIndicator	OPTIONAL }

RequestorIDString ::= USSD-String (SIZE (1..maxRequestorIDStringLength))

maxRequestorIDStringLength  INTEGER ::= 63

LCS-FormatIndicator ::= ENUMERATED {

logicalName	(0),

e-mailAddress	(1),

msisdn	(2),

url	(3),

sipUrl	(4),

... }

LCS-Priority ::= OCTET STRING (SIZE (1))

-- 0 = highest priority

-- 1 = normal priority

-- all other values treated as 1

LCS-QoS ::= SEQUENCE {

horizontal-accuracy	[0] Horizontal-Accuracy	OPTIONAL,

verticalCoordinateRequest	[1] NULL	OPTIONAL,

vertical-accuracy	[2] Vertical-Accuracy	OPTIONAL,	responseTime	[3] ResponseTime	OPTIONAL,

extensionContainer	[4] ExtensionContainer	OPTIONAL,

...,

velocityRequest	[5] NULL	OPTIONAL,

lcs-qos-class	[6] LCS-QoS-Class	OPTIONAL

}

-- lcs-qos-class may only be included in MO-LR request sent by the UE to the network.

Horizontal-Accuracy ::= OCTET STRING (SIZE (1))

-- bit 8 = 0

-- bits 7-1 = 7 bit Uncertainty Code defined in 3GPP TS 23.032. The horizontal location

-- error should be less than the error indicated by the uncertainty code with 67%

-- confidence.

Vertical-Accuracy ::= OCTET STRING (SIZE (1))

-- bit 8 = 0

-- bits 7-1 = 7 bit Vertical Uncertainty Code defined in 3GPP TS 23.032.

-- The vertical location error should be less than the error indicated

-- by the uncertainty code with 67% confidence.

ResponseTime ::= SEQUENCE {

responseTimeCategory	ResponseTimeCategory,

...}

--	note: an expandable SEQUENCE simplifies later addition of a numeric response time.

ResponseTimeCategory ::= ENUMERATED {

lowdelay  (0),

delaytolerant  (1),

... }

--	exception handling:

--	an unrecognized value shall be treated the same as value 1 (delaytolerant)

LCS-QoS-Class ::= ENUMERATED {

bestEffort  (0),

assured  (1),

... }

--	exception handling:

--	an unrecognized value shall be treated the same as value 0 (bestEffort)

SupportedGADShapes ::= BIT STRING {

ellipsoidPoint  (0),

ellipsoidPointWithUncertaintyCircle (1),

ellipsoidPointWithUncertaintyEllipse (2),

polygon (3),

ellipsoidPointWithAltitude (4),

ellipsoidPointWithAltitudeAndUncertaintyElipsoid (5),

ellipsoidArc  (6) } (SIZE (7..16))

-- A node shall mark in the BIT STRING all Shapes defined in 3GPP TS 23.032 it supports.

-- exception handling: bits 7 to 15 shall be ignored if received.

LCS-ReferenceNumber::= OCTET STRING (SIZE(1))

LCSCodeword ::= SEQUENCE {

dataCodingScheme	[0] USSD-DataCodingScheme,

lcsCodewordString	[1] LCSCodewordString,

...}

LCSCodewordString ::= USSD-String (SIZE (1..maxLCSCodewordStringLength))

maxLCSCodewordStringLength  INTEGER ::= 20

LCS-PrivacyCheck ::= SEQUENCE {

callSessionUnrelated	[0] PrivacyCheckRelatedAction,

callSessionRelated	[1] PrivacyCheckRelatedAction	OPTIONAL,

...}

PrivacyCheckRelatedAction ::= ENUMERATED {

allowedWithoutNotification (0),

allowedWithNotification (1),

allowedIfNoResponse (2),

restrictedIfNoResponse (3),

notAllowed (4),

...}

--	exception handling:

--	a ProvideSubscriberLocation-Arg containing an unrecognized PrivacyCheckRelatedAction

--	shall be rejected by the receiver with a return error cause of unexpected data value

AreaEventInfo ::= SEQUENCE {

areaDefinition	[0]	AreaDefinition,

occurrenceInfo	[1]	OccurrenceInfo	OPTIONAL,

intervalTime	[2]	IntervalTime	OPTIONAL,

...}

AreaDefinition ::= SEQUENCE {

areaList	[0]	AreaList,

...}

AreaList ::= SEQUENCE SIZE (1..maxNumOfAreas) OF Area

maxNumOfAreas  INTEGER ::= 10

Area ::= SEQUENCE {

areaType	[0]	AreaType,

areaIdentification	[1]	AreaIdentification,

...}

AreaType ::= ENUMERATED {

countryCode	(0),

plmnId	(1),

locationAreaId	(2),

routingAreaId	(3),

cellGlobalId	(4),

...,

utranCellId	(5) }

AreaIdentification ::= OCTET STRING (SIZE (2..7))

-- The internal structure is defined as follows:

-- octet 1 bits 4321	Mobile Country Code 1st digit

--         bits 8765	Mobile Country Code 2nd digit

-- octet 2 bits 4321	Mobile Country Code 3rd digit

--         bits 8765	Mobile Network Code 3rd digit if 3 digit MNC included

--	or filler (1111)

-- octet 3 bits 4321	Mobile Network Code 1st digit

--         bits 8765	Mobile Network Code 2nd digit

-- octets 4 and 5	Location Area Code (LAC) for Local Area Id,

--	Routing Area Id and Cell Global Id

-- octet 6	Routing Area Code (RAC) for Routing Area Id

-- octets 6 and 7	Cell Identity (CI) for Cell Global Id

-- octets 4 until 7	Utran Cell Identity (UC-Id) for Utran Cell Id

OccurrenceInfo ::= ENUMERATED {

oneTimeEvent	(0),

multipleTimeEvent	(1),

...}

IntervalTime ::= INTEGER (1..32767)

-- minimum interval time between area reports in seconds

PeriodicLDRInfo ::= SEQUENCE {

reportingAmount	ReportingAmount,

reportingInterval	ReportingInterval,

...,

reportingOptionMilliseconds	[0]	ReportingOptionMilliseconds	OPTIONAL

}

-- reportingInterval x reportingAmount shall not exceed 8639999 (99 days, 23 hours,

-- 59 minutes and 59 seconds) for compatibility with OMA MLP and RLP

-- When reportingOptionMilliseconds is provided and supported, reportingInterval &

-- reportingAmount shall be ignored.

ReportingAmount ::= INTEGER (1..maxReportingAmount)

maxReportingAmount INTEGER ::= 8639999

ReportingInterval ::= INTEGER (1..maxReportingInterval)

-- ReportingInterval is in seconds

maxReportingInterval INTEGER ::= 8639999

ReportingOptionMilliseconds ::= SEQUENCE {

reportingAmountMilliseconds	ReportingAmountMilliseconds,

reportingIntervalMilliseconds	ReportingIntervalMilliseconds,

...}

-- reportingAmountMilliseconds x reportingIntervalMilliseconds shall not exceed 8639999000

-- (99 days, 23 hours, 59 minutes and 59 seconds) for compatibility with OMA MLP and RLP

ReportingAmountMilliseconds ::= INTEGER (1..maxReportingAmountMilliseconds)

maxReportingAmountMilliseconds INTEGER ::= 8639999000

ReportingIntervalMilliseconds ::= INTEGER (1..maxReportingIntervalMilliseconds)

maxReportingIntervalMilliseconds INTEGER ::= 999

ReportingPLMNList::= SEQUENCE {

plmn-ListPrioritized	[0] NULL		OPTIONAL,

plmn-List		[1] PLMNList,

...}

# PLMNList::= SEQUENCE SIZE (1..maxNumOfReportingPLMN) OF

# ReportingPLMN

maxNumOfReportingPLMN INTEGER ::= 20

ReportingPLMN::= SEQUENCE {

plmn-Id		[0] PLMN-Id,

ran-Technology		[1] RAN-Technology	OPTIONAL,

ran-PeriodicLocationSupport	[2] NULL		OPTIONAL,

...}

# RAN-Technology ::= ENUMERATED {

gsm	(0),

umts	(1),

...}

ProvideSubscriberLocation-Res ::= SEQUENCE {

locationEstimate	Ext-GeographicalInformation,

ageOfLocationEstimate	[0] AgeOfLocationInformation	OPTIONAL,

extensionContainer	[1] ExtensionContainer	OPTIONAL,

... ,

add-LocationEstimate	[2] Add-GeographicalInformation	OPTIONAL,

deferredmt-lrResponseIndicator	[3] NULL	OPTIONAL,

geranPositioningData	[4] PositioningDataInformation	OPTIONAL,

utranPositioningData	[5] UtranPositioningDataInfo	OPTIONAL,

cellIdOrSai	[6] CellGlobalIdOrServiceAreaIdOrLAI	OPTIONAL,

sai-Present	[7] NULL	OPTIONAL,

accuracyFulfilmentIndicator	[8] AccuracyFulfilmentIndicator	OPTIONAL,

velocityEstimate	[9] VelocityEstimate	OPTIONAL,

mo-lrShortCircuitIndicator	[10] NULL	OPTIONAL,

geranGANSSpositioningData	[11] GeranGANSSpositioningData	OPTIONAL,

utranGANSSpositioningData	[12] UtranGANSSpositioningData	OPTIONAL,	targetServingNodeForHandover	[13] ServingNodeAddress	OPTIONAL,

utranAdditionalPositioningData	[14] UtranAdditionalPositioningData	OPTIONAL,

utranBaroPressureMeas	[15] UtranBaroPressureMeas	OPTIONAL,

utranCivicAddress	[16] UtranCivicAddress	OPTIONAL }

--	if deferredmt-lrResponseIndicator is set, locationEstimate is ignored.

-- the add-LocationEstimate parameter shall not be sent to a node that did not indicate the

-- geographic shapes supported in the ProvideSubscriberLocation-Arg

-- The locationEstimate and the add-locationEstimate parameters shall not be sent if

-- the supportedGADShapes parameter has been received in ProvideSubscriberLocation-Arg

-- and the shape encoded in locationEstimate or add-LocationEstimate is not marked

-- as supported in supportedGADShapes. In such a case ProvideSubscriberLocation

-- shall be rejected with error FacilityNotSupported with additional indication

-- shapeOfLocationEstimateNotSupported.

-- sai-Present indicates that the cellIdOrSai parameter contains a Service Area Identity.

AccuracyFulfilmentIndicator ::= ENUMERATED {

requestedAccuracyFulfilled  (0),

requestedAccuracyNotFulfilled  (1),

...	}

Ext-GeographicalInformation ::= OCTET STRING (SIZE (1..maxExt-GeographicalInformation))

-- Refers to geographical Information defined in 3GPP TS 23.032.

-- This is composed of 1 or more octets with an internal structure according to

-- 3GPP TS 23.032

-- Octet 1: Type of shape, only the following shapes in 3GPP TS 23.032 are allowed:

--	(a) Ellipsoid point with uncertainty circle

--	(b) Ellipsoid point with uncertainty ellipse

--	(c) Ellipsoid point with altitude and uncertainty ellipsoid

--	(d) Ellipsoid Arc

--	(e) Ellipsoid Point

-- Any other value in octet 1 shall be treated as invalid

-- Octets 2 to 8 for case (a) – Ellipsoid point with uncertainty circle

--	Degrees of Latitude	3 octets

--	Degrees of Longitude	3 octets

--	Uncertainty code	1 octet

-- Octets 2 to 11 for case (b) – Ellipsoid point with uncertainty ellipse:

--	Degrees of Latitude	3 octets

--	Degrees of Longitude	3 octets

--	Uncertainty semi-major axis	1 octet

--	Uncertainty semi-minor axis	1 octet

--	Angle of major axis	1 octet

--	Confidence	1 octet

-- Octets 2 to 14 for case (c) – Ellipsoid point with altitude and uncertainty ellipsoid

--	Degrees of Latitude	3 octets

--	Degrees of Longitude	3 octets

--	Altitude	2 octets

--	Uncertainty semi-major axis	1 octet

--	Uncertainty semi-minor axis	1 octet

--	Angle of major axis	1 octet

--	Uncertainty altitude	1 octet

--	Confidence	1 octet

-- Octets 2 to 13 for case (d) – Ellipsoid Arc

--	Degrees of Latitude	3 octets

--	Degrees of Longitude	3 octets

--	Inner radius	2 octets

--	Uncertainty radius	1 octet

--	Offset angle	1 octet

--	Included angle	1 octet

--	Confidence	1 octet

-- Octets 2 to 7 for case (e) – Ellipsoid Point

--	Degrees of Latitude	3 octets

--	Degrees of Longitude	3 octets

--

-- An Ext-GeographicalInformation parameter comprising more than one octet and

-- containing any other shape or an incorrect number of octets or coding according

-- to 3GPP TS 23.032 shall be treated as invalid data by a receiver.

--

-- An Ext-GeographicalInformation parameter comprising one octet shall be discarded

-- by the receiver if an Add-GeographicalInformation parameter is received

-- in the same message.

--

-- An Ext-GeographicalInformation parameter comprising one octet shall be treated as

-- invalid data by the receiver if an Add-GeographicalInformation parameter is not

-- received in the same message.

maxExt-GeographicalInformation  INTEGER ::= 20

-- the maximum length allows for further shapes in 3GPP TS 23.032 to be included in later

-- versions of 3GPP TS 29.002

VelocityEstimate ::= OCTET STRING (SIZE (4..7))

-- Refers to Velocity description defined in 3GPP TS 23.032.

-- This is composed of 4 or more octets with an internal structure according to

-- 3GPP TS 23.032

-- Octet 1: Type of velocity, only the following types in 3GPP TS 23.032 are allowed:

--	(a) Horizontal Velocity

--	(b) Horizontal with Vertical Velocity

--	(c) Horizontal Velocity with Uncertainty

--	(d) Horizontal with Vertical Velocity and Uncertainty

-- For types Horizontal with Vertical Velocity and Horizontal with Vertical Velocity

-- and Uncertainty, the direction of the Vertical Speed is also included in Octet 1

-- Any other value in octet 1 shall be treated as invalid

-- Octets 2 to 4 for case (a) Horizontal velocity:

--	Bearing	1 octet

--	Horizontal Speed	2 octets

-- Octets 2 to 5 for case (b) – Horizontal with Vertical Velocity:

--	Bearing	1 octet

--	Horizontal Speed	2 octets

--	Vertical Speed	1 octet

-- Octets 2 to 5 for case (c) – Horizontal velocity with Uncertainty:

--	Bearing	1 octet

--	Horizontal Speed	2 octets

--	Uncertainty Speed	1 octet

-- Octets 2 to 7 for case (d) – Horizontal with Vertical Velocity and Uncertainty:

--	Bearing	1 octet

--	Horizontal Speed	2 octets

--	Vertical Speed	1 octet

--	Horizontal Uncertainty Speed	1 octet

--	Vertical Uncertainty Speed	1 octet

PositioningDataInformation ::= OCTET STRING (SIZE (2..maxPositioningDataInformation))

-- Refers to the Positioning Data defined in 3GPP TS 49.031.

-- This is composed of 2 or more octets with an internal structure according to

-- 3GPP TS 49.031.

maxPositioningDataInformation INTEGER ::= 10

--

UtranPositioningDataInfo ::= OCTET STRING (SIZE (3..maxUtranPositioningDataInfo))

-- Refers to the Position Data defined in 3GPP TS 25.413.

-- This is composed of the positioningDataDiscriminator and the positioningDataSet

-- included in positionData as defined in 3GPP TS 25.413.

maxUtranPositioningDataInfo INTEGER ::= 11

--

GeranGANSSpositioningData ::= OCTET STRING (SIZE (2..maxGeranGANSSpositioningData))

-- Refers to the GANSS Positioning Data defined in 3GPP TS 49.031.

-- This is composed of 2 or more octets with an internal structure according to

-- 3GPP TS 49.031.

maxGeranGANSSpositioningData INTEGER ::= 10

--

UtranGANSSpositioningData ::= OCTET STRING (SIZE (1..maxUtranGANSSpositioningData))

-- Refers to the Position Data defined in 3GPP TS 25.413.

-- This is composed of the GANSS-PositioningDataSet only, included in PositionData

-- as defined in 3GPP TS 25.413.

maxUtranGANSSpositioningData INTEGER ::= 9

--

UtranAdditionalPositioningData ::= OCTET STRING (SIZE (1..maxUtranAdditionalPositioningData))

-- Refers to the Position Data defined in 3GPP TS 25.413.

-- This is composed of the Additional-PositioningDataSet only, included in PositionData

-- as defined in 3GPP TS 25.413.

maxUtranAdditionalPositioningData INTEGER ::= 8

--

UtranBaroPressureMeas ::= INTEGER (30000..115000)

-- Refers to the barometric pressure measurement defined in 3GPP TS 25.413.

-- This is composed of the BarometricPressureMeasurement only as defined in 3GPP TS

-- 25.413.

UtranCivicAddress ::= OCTET STRING

-- Refers to the civic address defined in 3GPP TS 25.413.

-- This is composed of the CivicAddress only as defined in 3GPP TS 25.413.

Add-GeographicalInformation ::= OCTET STRING (SIZE (1..maxAdd-GeographicalInformation))

-- Refers to geographical Information defined in 3GPP TS 23.032.

-- This is composed of 1 or more octets with an internal structure according to

-- 3GPP TS 23.032

-- Octet 1: Type of shape, all the shapes defined in 3GPP TS 23.032 are allowed:

-- Octets 2 to n (where n is the total number of octets necessary to encode the shape

-- according to 3GPP TS 23.032) are used to encode the shape itself in accordance with the

-- encoding defined in 3GPP TS 23.032

--

-- An Add-GeographicalInformation parameter, whether valid or invalid, received

-- together with a valid Ext-GeographicalInformation parameter in the same message

-- shall be discarded.

--

-- An Add-GeographicalInformation parameter containing any shape not defined in

-- 3GPP TS 23.032 or an incorrect number of octets or coding according to

-- 3GPP TS 23.032 shall be treated as invalid data by a receiver if not received

-- together with a valid Ext-GeographicalInformation parameter in the same message.

maxAdd-GeographicalInformation  INTEGER ::= 91

-- the maximum length allows support for all the shapes currently defined in 3GPP TS 23.032

SubscriberLocationReport-Arg ::= SEQUENCE {

lcs-Event	LCS-Event,

lcs-ClientID	LCS-ClientID,

lcsLocationInfo	LCSLocationInfo,

msisdn	[0] ISDN-AddressString	OPTIONAL,

imsi	[1] IMSI	OPTIONAL,

imei	[2] IMEI	OPTIONAL,

na-ESRD	[3] ISDN-AddressString	OPTIONAL,

na-ESRK	[4] ISDN-AddressString	OPTIONAL,

locationEstimate	[5] Ext-GeographicalInformation	OPTIONAL,

ageOfLocationEstimate	[6] AgeOfLocationInformation	OPTIONAL,

slr-ArgExtensionContainer	[7] SLR-ArgExtensionContainer	OPTIONAL,

... ,

add-LocationEstimate	[8] Add-GeographicalInformation	OPTIONAL,

deferredmt-lrData	[9] Deferredmt-lrData	OPTIONAL,

lcs-ReferenceNumber	[10] LCS-ReferenceNumber	OPTIONAL,

geranPositioningData	[11] PositioningDataInformation	OPTIONAL,

utranPositioningData	[12] UtranPositioningDataInfo	OPTIONAL,

cellIdOrSai	[13]	CellGlobalIdOrServiceAreaIdOrLAI	OPTIONAL,

h-gmlc-Address	[14]	GSN-Address	OPTIONAL,

lcsServiceTypeID	[15]	LCSServiceTypeID	OPTIONAL,

sai-Present	[17] NULL	OPTIONAL,

pseudonymIndicator	[18] NULL	OPTIONAL,

accuracyFulfilmentIndicator	[19] AccuracyFulfilmentIndicator	OPTIONAL,

velocityEstimate	[20] VelocityEstimate	OPTIONAL,

sequenceNumber	[21] SequenceNumber	OPTIONAL,

periodicLDRInfo	[22] PeriodicLDRInfo	OPTIONAL,

mo-lrShortCircuitIndicator	[23] NULL	OPTIONAL,

geranGANSSpositioningData	[24] GeranGANSSpositioningData	OPTIONAL,

utranGANSSpositioningData	[25] UtranGANSSpositioningData	OPTIONAL,

targetServingNodeForHandover	[26] ServingNodeAddress	OPTIONAL,

utranAdditionalPositioningData	[27] UtranAdditionalPositioningData	OPTIONAL,

utranBaroPressureMeas	[28] UtranBaroPressureMeas	OPTIONAL,

utranCivicAddress	[29] UtranCivicAddress	OPTIONAL }

-- one of msisdn or imsi is mandatory

-- a location estimate that is valid for the locationEstimate parameter should

-- be transferred in this parameter in preference to the add-LocationEstimate.

-- the deferredmt-lrData parameter shall be included if and only if the lcs-Event

-- indicates a deferredmt-lrResponse.

-- if the lcs-Event indicates a deferredmt-lrResponse then the locationEstimate

-- and the add-locationEstimate parameters shall not be sent if the

-- supportedGADShapes parameter had been received in ProvideSubscriberLocation-Arg

-- and the shape encoded in locationEstimate or add-LocationEstimate was not marked

-- as supported in supportedGADShapes. In such a case terminationCause

-- in deferredmt-lrData shall be present with value

-- shapeOfLocationEstimateNotSupported.

-- If a lcs event indicates deferred mt-lr response, the lcs-Reference number shall be

-- included.

-- sai-Present indicates that the cellIdOrSai parameter contains a Service Area Identity.

Deferredmt-lrData ::= SEQUENCE {

deferredLocationEventType	DeferredLocationEventType,

terminationCause	[0] TerminationCause	OPTIONAL,

lcsLocationInfo	[1] LCSLocationInfo	OPTIONAL,

...}

-- lcsLocationInfo may be included only if a terminationCause is present

-- indicating mt-lrRestart.

LCS-Event ::= ENUMERATED {

emergencyCallOrigination  (0),

emergencyCallRelease  (1),

mo-lr  (2),

...,

deferredmt-lrResponse  (3) ,

deferredmo-lrTTTPInitiation  (4),

emergencyCallHandover (5)  }

--	deferredmt-lrResponse is applicable to the delivery of a location estimate

--	for an LDR initiated earlier by either the network (via an MT-LR activate deferred

--	location) or the UE (via a deferred MO-LR TTTP initiation)

--	exception handling:

--	a SubscriberLocationReport-Arg containing an unrecognized LCS-Event

--	shall be rejected by a receiver with a return error cause of unexpected data value

TerminationCause ::= ENUMERATED {

normal  (0),

errorundefined  (1),

internalTimeout  (2),

congestion  (3),

mt-lrRestart  (4),

privacyViolation  (5),

...,

shapeOfLocationEstimateNotSupported (6) ,

subscriberTermination (7),

uETermination (8),

networkTermination (9)  }

-- mt-lrRestart shall be used to trigger the GMLC to restart the location procedure,

-- either because the sending node knows that the terminal has moved under coverage

-- of another MSC or SGSN (e.g. Send Identification received), or because the subscriber

-- has been deregistered due to a Cancel Location received from HLR.

--

-- exception handling

-- an unrecognized value shall be treated the same as value 1 (errorundefined)

SequenceNumber ::= INTEGER (1..maxReportingAmount)

ServingNodeAddress ::= CHOICE {

msc-Number	[0] ISDN-AddressString,

sgsn-Number	[1] ISDN-AddressString,

mme-Number	[2] DiameterIdentity }

SubscriberLocationReport-Res ::= SEQUENCE {

extensionContainer	ExtensionContainer	OPTIONAL,

...,

na-ESRK	[0] ISDN-AddressString	OPTIONAL,

na-ESRD	[1] ISDN-AddressString	OPTIONAL,

h-gmlc-Address	[2]	GSN-Address	OPTIONAL,

mo-lrShortCircuitIndicator	[3] NULL	OPTIONAL,

reportingPLMNList	[4] ReportingPLMNList	OPTIONAL,

lcs-ReferenceNumber	[5]	LCS-ReferenceNumber	OPTIONAL }

-- na-ESRK and na-ESRD are mutually exclusive

--

-- exception handling

-- receipt of both na-ESRK and na-ESRD shall be treated the same as a return error

.#END

* * * End of Change * * * *