3GPP TSG-CT WG4 Meeting #131	C4-254108

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | 1000 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Link-specific and MPQUIC Proxy IP addresses for IPv4v6 PDU sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, MASSS, ATSSS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CR 23.501 #6366 (approved at SA#109) clarified that: In case PDU Session is of type IPv4v6, both IPv4 and IPv6 "MPQUIC link-specific multipath" addresses/prefixes are allocated for each access, i.e. four IP addresses/prefixes may be allocated to the UE. For the MPQUIC-E steering functionality, TS 29.244 explicitly states that the UPF should allocate two MPQUIC Proxy IP addresses (IPv4 and IPv6) and four Link-Specific Multipath Addresses for 3GPP Access Non-3GPP accesses. Similar clarifications are necessary for the handling of MA PDU sessions of type IPv4v6. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For MA PDU sessions of type IPv4v6, the UPF shall allocate 4 Link-specific IP addresses and 2 MPQUIC Proxy IP addresses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Failure to establish MA PDU sessions of type IPv4v6 if the UPF does not return all the required IP addresses. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.20.2A.2, 5.20.2A.3, 5.20.2A.4, 8.2.158, 8.2.226 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 5.20.2A.2	Activate MPQUIC-based steering functionalities and Exchange MPQUIC Parameters

If the MPQUIC-UDP, MPQUIC-IP and/or MPQUIC-E steering functionality is required for a Multi-Access PDU session and if the UPF indicates support of the respective MPQUIC-UDP, MPQUIC-IP and/or MPQUIC-E features (see clause 8.2.25), the SMF shall send MPQUIC Control information to the UPF in PFCP Session Establishment Request, to instruct the UPF to activate one or more MPQUIC-based steering functionalities for this PFCP session. The SMF may also request to activate the PMF functionality for this PFCP session by sending PMF Control Information to the UPF.

As a result, the UPF(PSA) shall allocate necessary resources for the requested MPQUIC-based steering functionalities and PMF and return the corresponding MPQUIC Parameters (e.g. MPQUIC Proxy IP addresses and ports, MPQUIC link-specific multipath IP addresses) and PMF Parameters (e.g. PMF IP address and port, etc.) to the SMF in PFCP Session Establishment Response. The SMF may also activate the MPTCP functionality for this PFCP session as specified in clause 5.20.2.2.

For an SMF and UPF that support both MPQUIC-IP and MPQUIC-UDP, when both MPQUIC-IP and MPQUIC-UDP are used simultaneously for different packet flows of the PDU session, the UPF may (based on implementation) provide a common MPQUIC IP Address information to be used for both MPQUIC-IP and MPQUIC-UDP, or different MPQUIC IP Address Information respectively for MPQUIC-IP and MPQUIC-UDP. In the former case, the UPF shall return one single MPQUIC IP Address Information IE to the SMF, with the MPQUIC Proxy Type indicating "both MPQUIC-IP and MPQUIC-UDP". In the latter case, the UPF shall return two MPQUIC IP Address Information IEs to the SMF, wherein each MPQUIC IP Address Information IE shall indicate (via the MPQUIC Proxy Type) whether the information applies to MPQUIC-IP or MPQUIC-UDP. The same pair of MPQUIC link-specific multipath addresses shall apply to both MPQUIC-IP and MPQUIC-UDP.

When the MPQUIC-UDP or MPQUIC-IP steering functionality is required for a MA-PDU session of type IPv4IPv6, the UPF shall allocate:

-	both the MPQUIC Proxy IPv4 and the MPQUIC Proxy IPv6 addresses in the MPQUIC Address Information IE; and

-	both the Link-Specific Multipath IPv4 Address for 3GPP Access and the Link-Specific Multipath IPv6 Address for 3GPP Access, and both the Link-Specific Multipath IPv4 Address for Non-3GPP Access and the Link-Specific Multipath IPv6 Adress for Non-3GPP Access in the Link-Specific Multipath IP Address IE.

NOTE X:	Allocating both pairs of IPv4 and IPv6 MPQUIC Proxy and Link-Specific Multipath addresses enables the UE to select IPv4 or IPv6 addresses to establish QUIC connection(s) based on whether IPv4 or IPv6 traffic is to be sent.

When the MPQUIC-E steering functionality is required for an Ethernet MA-PDU session and if the UPF supports MPQUIC-E feature, the UPF should allocate:

-	both the MPQUIC Proxy IPv4 and the MPQUIC Proxy IPv6 addresses in the MPQUIC Address Information IE; and

-	both the Link-Specific Multipath IPv4 Address for 3GPP Access and the Link-Specific Multipath IPv6 Address for 3GPP Access, and both the Link-Specific Multipath IPv4 Address for Non-3GPP Access and the Link-Specific Multipath IPv6 Adress for Non-3GPP Access in the Link-Specific Multipath IP Address IE.

NOTE Y:	Allocating both pair of IPv4 and IPv6 MPQUIC Proxy and Link-Specific Multipath addresses enables the UE to select either IPv4 or IPv6 address to establish QUIC connection(s) based on its own preference.

The SMF may send MPQUIC Control Information and/or PMF Control Information (and/or MPTCP Control Information if the MPTCP functionality is also activated) to the UPF in PFCP Session Modification Request, with updated value, e.g. if the SMF receives updated ATSSS rules form the PCF.

NOTE Z: The MPQUIC Parameters received by the SMF are sent to the UE together with the ATSSS rules, as specified in 3GPP TS 24.193 [59].

When provisioning an UL PDR for user plane traffic for which MPQUIC is applicable, the SMF shall include a Multipath Application Indication IE with the MQAI flag set to "1" in the Create PDR IE.

NOTE W: This indication can be used by the UPF to prepare itself fo the reception of MPQUIC traffic for the PDU session.

* * * Next Change * * *

#### 5.20.2A.3	Control of Multipath QUIC Connection Establishment by MPQUIC Proxy

When the MPQUIC-UDP, MPQUIC-IP or MPQUIC-E steering functionality is utilized, the UDP, IP or Ethernet connection establishment and data exchange between the UE and a remote host shall be proxied by the MPQUIC Proxy in the UPF(PSA), as specified in clause 5.32.6.2.2 of 3GPP TS 23.501 [28] and clause 6.1.4.1.0 of 3GPP TS 24.193 [59].

Once a multipath QUIC connection is set up, the MPQUIC Proxy shall store the MPQUIC session entry which includes the following information:

-	MPQUIC link-specific multipath IP addresses for both 3GPP and non-3GPP and the associated UDP ports (as the source UDP port) for establishing the QUIC connection;

-	MPQUIC Proxy IP address and the port which were allocated by the UPF;

-	UE's MA-PDU session IP address (if it is routable over N6) or a N6 routable IP address together with the non-routable UE's MA-PDU session IP address, e.g. a NATed IP address, and the UDP port which are used as the source IP address and port when opening a UDP socket to the requested remote host, for an IP MA-PDU session if MPQUIC-UDP is used;

-	the remote host IP address and its UDP port received as part of the HTTP Connect request message, for an IP MA-PDU session if MPQUIC-UDP is used; and

-	the QFI associated with the multipath QUIC connection.

NOTE 1:	The UE establishes as many multipath QUIC connections with the MPQUIC Proxy in the UPF(PSA) as the number of QoS flows of the MA PDU session, and each multipath QUIC connection carries the UDP, IP or Ethernet traffic mapped to a single QoS flow, as specified in clause 5.32.6.2.2 of 3GPP TS 23.501 [28] and clause 6.1.4.1.0 of 3GPP TS 24.193 [59]. For a MA PDU session of type IPv4v6, the UE establishes a given multipath QUIC connection using either the MPQUIC link-specific multipath IPv4 and MPQUIC Proxy IPv4 addresses, or the MPQUIC link-specific multipath IPv6 and MPQUIC Proxy IPv6 addresses, depending on whether IPv4 or IPv6 traffic is to be sent on the multipath QUIC connection.

NOTE 2:	All the multipath QUIC connections established with the MPQUIC-UDP and MPQUIC-IP Proxy for the same PDU session share the same MPQUIC link-specific multipath IP addresses and associated UDP ports, and the same UE's MA-PDU session IP address and UDP port (for an IP MA-PDU session).

NOTE 3:	When the MPQUIC-E steering functionality is used, or when the MPQUIC-UDP or MPQUIC-IP steering functionality is used for a MA PDU session of type IPv4v6, the MPQUIC link-specific multipath IP addresses (for both 3gpp and non-3gpp access) can contain both IPv4 and IPv6 addresses. See also clause 5.20.2A.2.

The stored MPQUIC session entry shall be used by the MPQUIC Proxy for subsequent packets forwarding when receiving uplink or sending downlink MPQUIC traffic.

A UPF implementation may use N6 routable IP address instead of UE's MA-PDU session IP address for IP translation.

NOTE 4:	The DL PDR from the SMF for MPQUIC-UDP or MPQUIC-IP traffic carries the UE's MA-PDU session IP address. If the UPF uses a different N6 routable IP address, it is UPF implementation specific how to match DL PDRs for the downlink MPQUIC traffic received with the destination address set to the N6 routable IP address.

* * * Next Change * * *

#### 5.20.2A.4	Traffic forwarding by MPQUIC Proxy

For an IP MA-PDU session using the MPQUIC-UDP steering functionality, the MPQUIC-UDP Proxy in the UPF receives HTTP datagrams encapsulating a UDP payload sent from the link-specific multipath IP addresses which were allocated for the PDU session, where the target URI contains the target host IP address and target port number. The MPQUIC-UDP proxy retrieves the UDP payload, adds the UDP/IP header by using the PDU session address as the source IP address and a port number as source port number and setting the target host IP address as the destination IP address and the target port number as the destination port number, and then sends the IP packets over N6. The MPQUIC-UDP proxy in the UPF does the reverse process when it receives UDP/IP payload packets from N6 matching DL PDRs associated with the MAR where the MPQUIC-UDP steering functionality is required. See also IETF RFC 9298 [96]. For an MA PDU session of type IPv4v6, the UPF shall use the PDU session's IPv4 or IPv6 address when adding the UDP/IP header as mentioned above, depending on whether the multipath QUIC connection used for transferring the UDP payload was established with a MPQUIC link-specific multipath IPv4 address and a MPQUIC Proxy IPv4 address, or with a MPQUIC link-specific multipath IPv6 address and a MPQUIC Proxy IPv6 address.

For an IP MA-PDU session using the MPQUIC-IP steering functionality, the MPQUIC-IP Proxy in the UPF receives HTTP datagrams encapsulating an IP packet sent from the link-specific multipath IP addresses which were allocated for the PDU session. The MPQUIC-IP proxy retrieves the IP packet and then sends it over N6. The MPQUIC-IP proxy in the UPF does the reverse process when it receives IP packets from N6 matching the DL PDRs associated with the MAR where the MPQUIC-IP steering functionality is required. See also IETF RFC 9484 [103].

For an Ethernet MA-PDU session using the MPQUIC-E steering functionality, the MPQUIC-E Proxy in the UPF receives HTTP datagrams encapsulating an Ethernet frame sent from the link-specific multipath IP addresses which were allocated for the PDU session. The MPQUIC-E proxy retrieves the Ethernet frame and then sends it over N6. The MPQUIC-E proxy in the UPF does the reverse process when it receives Ethernet frames from N6 matching DL PDRs associated with the MAR where the MPQUIC-E steering functionality is required. See also IETF draft-ietf-masque-connect-ethernet [104].

The MPQUIC Proxy shall send the DL traffic of a QoS flow using the multipath QUIC connection associated with the corresponding QoS flow.

The UPF may detect the uplink IP packets for which an MPQUIC-based steering functionality is applicable by checking whether the UL PDR matching the user plane traffic is set with a Multipath Applicable Indication, or (per UPF implementation choice) by checking whether the source or destination IP address of the user plane packets (after removing the GTP-U header) corresponds to MPQUIC link-specific multipath IP address(es) and the MPQUIC proxy IP address.

The UPF may detect the downlink IP packets for which an MPQUIC-based steering functionality is applicable, by checking the information in the associated MAR (e.g. checking if the steering functionality is set to MPQUIC-UDP, MPQUIC-IP or MPQUIC-E) or by checking the destination IP address, e.g. the MA PDU session IP address.

* * * Next Change * * *

### 8.2.158	Link-Specific Multipath IP Address

The Link-Specific Multipath IP Address IE shall carry link-specific IP address used for MPTCP, MPQUIC-UDP, MPQUIC-IP or MPQUIC-E steering function. It shall be encoded as shown in Figure 8.2.158-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 229 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare |  |  |  | NV6 | NV4 | V6 | V4 |  |
|  | p to (p+3) | Link-Specific Multipath IPv4 Address for 3GPP Access |  |  |  |  |  |  |  |  |
|  | q to (q+15) | Link-Specific Multipath IPv6 Address for 3GPP Access |  |  |  |  |  |  |  |  |
|  | r to (r+3) | Link-Specific Multipath IPv4 Address for Non-3GPP Access |  |  |  |  |  |  |  |  |
|  | s to (s+3) | Link-Specific Multipath IPv6 Adress for Non-3GPP Access |  |  |  |  |  |  |  |  |
|  | m to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.158-1: Link-Specific Multipath IP Address

The following flags are coded within Octet 5:

-	Bit 1 – V4: If this bit is set to "1", then the Link-Specific Multipath IPv4 Address for 3GPP Access shall be present in the Link-Specific Multipath IP Address IE.

-	Bit 2 – V6: If this bit is set to "1", then the Link-Specific Multipath IPv6 Address for 3GPP Access shall be present in the Link-Specific Multipath IP Address IE.

-	Bit 3 – NV4: If this bit is set to "1", then the Link-Specific Multipath IPv4 Address for Non-3GPP Access shall be present in the Link-Specific Multipath IP Address IE.

-	Bit 4 – NV6: If this bit is set to "1", then the Link-Specific Multipath IPv6 Address for Non-3GPP Access shall be present in the Link-Specific Multipath IP Address IE.

-	Bit 5 to 8 Spare, for future use and set to "0".

Octets "p to (p+3)" or "q to (q+15)" (IPv4 address / IPv6 address fields), if present, shall contain the value for Link-Specific Multipath IP Address for 3GPP access.

Octets "r to (r+3)" or "s to (s+15)" (IPv4 address / IPv6 address fields), if present, shall contain the value for Link-Specific Multipath IP Address for Non-3GPP access.

If the PDU session is of type IPv4v6, both Link-Specific Multipath IPv4 Addresses and Link-Specific Multipath IPv6 Adresses for both 3GPP Access and non-3GPP Access shall be included when the MPQUIC-UDP or MPQUIC-IP steering functionality is required for the PFCP session. See also clause 5.20.2A.2.

Both Link-Specific Multipath IPv4 Addresses and Link-Specific Multipath IPv6 Adresses for both 3GPP Access and non-3GPP Access should be included when the MPQUIC-E steering functionality is required for the PFCP session. See also clause 5.20.2A.2.

* * * Next Change * * *

### 8.2.226	MPQUIC Address Information

The MPQUIC Address Information IE shall carry the address information of MPQUIC proxy in the UPF, when using the MPQUIC-UDP, MPQUIC-IP or MPQUIC-E steering functionality. It shall be encoded as shown in Figure 8.2.226-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 332 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare |  |  |  |  |  | V6 | V4 |  |
|  | 6 | MPQUIC Proxy Type |  |  |  |  |  |  |  |  |
|  | 7 to 8 | MPQUIC Proxy Port |  |  |  |  |  |  |  |  |
|  | p to (p+3) | MPQUIC Proxy IPv4 Address |  |  |  |  |  |  |  |  |
|  | q to (q+15) | MPQUIC Proxy IPv6 Address |  |  |  |  |  |  |  |  |
|  | m to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.226-1: MPQUIC Address Information

The following flags are coded within Octet 5:

-	Bit 1 – V4: If this bit is set to "1", then the MPQUIC Proxy IPv4 Address field shall be present in the MPQUIC Address Information IE.

-	Bit 2 – V6: If this bit is set to "1", then the MPQUIC IPv6 Address field shall be present in the MPQUIC Address Information IE.

-	Bit 3 to 8 Spare, for future use and set to "0".

Octets 6 shall indicate the MPQUIC Proxy Type, with the value specified in clause 6.1.4 of 3GPP TS 24.193 [59].

Octets 7 to 8 shall indicate the allocated UDP port number of the MPQUIC Proxy.

Octets "p to (p+3)" or "q to (q+15)" (IPv4 address / IPv6 address fields), if present, shall contain the address value.

If the PDU session is of type IPv4v6, both MPQUIC Proxy IPv4 Address and MPQUIC Proxy IPv6 Adress shall be included when the MPQUIC-UDP or MPQUIC-IP steering functionality is required for the PFCP session. See also clause 5.20.2A.2.

Both MPQUIC Proxy IPv4 Address and MPQUIC Proxy IPv6 Adress should be included when the MPQUIC-E steering functionality is required for the PFCP session. See also clause 5.20.2A.2.

* * * End of Changes * * *