3GPP TSG-CT WG4 Meeting #131	C4-254291

# Sophia Antipolis, France; 13th – 17th October 2025						revision of C4-254071

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 1244 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of CHF group ID description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, CHFSeg |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 6.2.6.2 of TS 23.501, it is stated that the NF profile of NF instance maintained in an NRF may include Group ID for UDM, UDR, AUSF, PCF and CHF. CHF group ID is defined as: This refers to one or more CHF instances managing a specific set of SUPIs. This is similar to the corresponding definition for UDM, UDR, AUSF and PCF. However in stage-3, the description of Group ID for CHF is not aligned with the description of Group ID for UDM, UDR, AUSF and PCF. In all of these cases, A number of NF instances can be considered to conform a group when they serve the same set of users. Similar criteria is applicable to CHF and therefore, this CR proposes to add the missing description to CHF group ID. It is also proposed to correct the existing definition of CHF group ID by adding the association with the subscription. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description CHF group ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between stage-2 and stage-3. CHF group ID definition in stage-3 remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.32 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact any yaml file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - First part of the change reverted. - The added sentence moved into a separate paragraph. |  |  |  |  |  |  |  |  |

* * *1st Change * * * *

##### 6.1.6.2.32	Type: ChfInfo

Table 6.1.6.2.32-1: Definition of type ChfInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supiRangeList | array(SupiRange) | O | 1..N | List of ranges of SUPIs that can be served by the CHF instance. (NOTE 1) |
| gpsiRangeList | array(IdentityRange) | O | 1..N | List of ranges of GPSI that can be served by the CHF instance. (NOTE 1) |
| plmnRangeList | array(PlmnRange) | O | 1..N | List of ranges of PLMNs (including the PLMN IDs of the CHF instance) that can be served by the CHF instance. If not provided, the CHF can serve any PLMN. |
| groupId | NfGroupId | O | 0..1 | Identity of the CHF group that is served by the CHF instance. A number of CHF instances can be considered to conform a CHF group when they serve the same set of users (identified by supiRanges and gpsiRanges), even if they are not identified by a CHF Group ID. (NOTE 1) |
| primaryChfInstance | NfInstanceId | C | 0..1 | This IE shall be present if the CHF instance serves as a secondary CHF instance of another primary CHF instance. When present, it shall be set to the NF Instance Id of the primary CHF instance. This IE shall be absent if the secondaryChfInstance is present. (NOTE 2, NOTE 3) |
| secondaryChfInstance | NfInstanceId | C | 0..1 | This IE shall be present if the CHF instance serves as a primary CHF instance of another secondary CHF instance. When present, it shall be set to the NF Instance Id of the secondary CHF instance. This IE shall be absent if the primaryChfInstance is present. (NOTE 2, NOTE 3) |
| NOTE 1: If none of these parameters are provided, the CHF can serve any SUPI or GPSI managed by the PLMN of the CHF instance. If "supiRangeList" and "gpsiRangeList" attributes are absent, and "groupId" is present, the SUPIs / GPSIs served by this CHF instance is determined by the NRF (see 3GPP TS 23.501 [2], clause 6.2.6.2). NOTE 2: The NF Service Consumer of the CHF may use these attributes as primary/secondary redundancy mechanism, or alternatively, it may also rely on the availability of an NF Set (or NF Service Set) of CHF Instances (or CHF Service Instances) for the same purpose. NOTE 3: If the CHF does not provide NF set ID or NF Service Set ID in NFProfile, it shall provide one of these attributes. These attributes may be present if the CHF registers an NF set ID or NF service set ID. |  |  |  |  |

* * * End of Changes * * * *