3GPP TSG-CT WG4 Meeting #131	C4-254221

Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.564 | CR | 0153 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Minor corrections |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  |  | 2025-10-01 |  |
| Category: | F |  |  |  |  |  |  | Release: |  |  |  | Rel-19 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |  |
| Reason for change: | There are some typos and reference need to be corrected. |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: | Correct typos and the reference. |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: | Incomplete specification may lead to the confusion of the implementations. |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: | 5.2.2.3.3, 6.1.6.2.21 |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | N |  |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: | This CR doesn't introduce any impact to openAPI file. |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

##### 5.2.2.3.3	UPF handling of Skip Reporting Instruction in the subscription

The procedure specified in this clause is optional to support for the UPF and the NF Service Consumer. It may be used if both the UPF and the NF Service Consumer support the Skip Event Exposure Reporting (SEER) feature (see clause 6.1.8).

If the UPF supports the SEER feature, the NF service consumer may request the UPF to skip reporting event reports by including the Skip Reporting Instruction in the subscription request, for one or more of the conditions specified below:

1)	if the volume and throughput measurement is null;

NOTE 1:	The UPF can consider that a measurement is null if the measurement is zero or below an operator-configurable value in the UPF.

2)	if the volume and/or throughput measurements are lower than threshold(s) provided as part of the subscription parameters (i.e. thresholdTrafficVolume and/or thresholdThroughput); if both thresholdTrafficVolume and thresholdThroughput are provided, the UPF shall skip sending event reports only when both volume and throughput measurements are lower than the respective thresholds; and/or

NOTE 2:	The UPF considers the volume measurement is lower than the threshold only if all three measurements on total, UL and DL volume are lower than the provisioned threshold. Likewise, it is the same for determining if the throughput measurement is lower than the provisioned threshold.

3)	for time periods outside of measurement validity times; in this case, the UPF shall skip performing measurements outside of the measurement validity times, i.e. the UPF shall perform measurements only during the measurement validity time periods and send corresponding event reports (during or possibly outside the measurement validity times).

Conditions 1) and 2) shall be exclusive from each other, i.e. either condition 1) or 2) may be requested.

Upon being instructed to skip sending event reports to the NF service consumer for one or more of the above conditions, the UPF shall skip sending event reports to the NF service consumer when at least one condition is met.

When the UPF sends the next notification request containing an event report after the UPF has skipped some event reports as described above, the UPF shall include a skippedReportInd in the notification to indicate to the NF service consumer that some event reports were skipped previously.

# *** Next Change ***

##### 6.1.6.2.21	Type: SkipReportingInstruction

Table 6.1.6.2.21-1: Definition of type SkipReportingInstruction

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| skipReportCond | array(SkipReportingCondition) | M | 1..N | Indicates one or more conditions under which event reports shall be skipped. |
| validityTimes | array(RecurTime) | C | 1..N | This IE shall be included if SKIP_TIME_DURATION is included as one of skipReportCond. When present, it shall indicate a list of measurement validity times during which the UPF shall perform the measurements and for which the UPF shall generate relevant event reports as specified in clause 5.2.2.3.3. |
| thresholdCond | ThresholdCond | C | 0..1 | This IE shall be included if SKIP_BELOW_THRESHOLD_REPORT is included as one of skipReportCond. When present, it shall indicate a set of ThresholdCond, so that the UPF shall perform the measurement and generate relevant event reports only when the measurement result meets the threshold conditions as specified in clause 5.2.2.3.3. |

# *** End of Changes ***