3GPP TSG-CT WG4 Meeting #131	C4-254077

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.673 | CR | 0065 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cleanup on references numbers and data type RacFormat |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Reference number is missed in several places of the specification. 3GPP is misspelled as 3GPPP. Data type name RacFormat is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections on the reference number, data type name RacFormat and also the template N<NF>. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect reference number, data type name left in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1, 5.2.2.3, 6.1.6.1, 6.1.6.2.2, 6.1.6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution does not change the OpenAPI. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.2.1	Service Description

The Nucmf_UECapabilityManagement service operates on the dictionary entries for the mapping between UE Radio Capability ID and UE Radio Access Capability Information. The service operations exposed by this service allow service consumer NF, e.g. an AMF:

-	to retrieve UE radio access capability information with a UE Radio Capability ID (either Manufacturer-assigned or PLMN-assigned);

-	to obtain a PLMN-assigned UE Radio Capability ID for a specific UE radio access capability information;

-	to subscribe or unsubscribe for notifications of UCMF dictionary entries;

-	to be notified about creation and deletion of UCMF dictionary entries;

The Nucmf_UECapabilityManagement service supports the following service operations;

Table 5.2.1-1: Service operations supported by the Nucmf_UECapabilityManagement service

| Service Operations | Description | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Resolve | Retrieve UE radio access capability information from the dictionary entry identified by the UE Radio Capability ID (either Manufacturer-assigned or PLMN-assigned) | Request/Response | AMF, MME |
| Assign | Request to assign a UE Radio Capability ID by providing the UE Radio Capability Information. See 3GPP TS 23.502 [3], clause 5.2.18.3.2. | Request/Response | AMF, MME |
| Subscribe | Subscribe for notifications of UCMF dictionary entries. | Subscribe/Notify | AMF, MME |
| Unsubscribe | Unsubscribe for notifications of UCMF dictionary entries. | Subscribe/Notify | AMF, MME |
| Notify | To be notified about creation and deletion of UCMF dictionary entries. | Subscribe/Notify | AMF, MME |

* * * * Next Change * * * *

5.2.2.3	Service Operation Assign

The Assign service operation shall be used by the service consumer NF (e.g. AMF) to obtain a PLMN-assigned UE Radio Capability ID from the UCMF for a specific UE radio access capability information. The UCMF shall create a new dictionary entry and assign a PLMN-assigned UE Radio Capability ID unless such dictionary entry already exists and a PLMN-assigned UE Radio Capability ID has already been assigned to the given UE radio access capability information in the UCMF. The NF service consumer, e.g. an AMF, may consume the service by providing the UE Radio Access Capability Information retrieved from the UE, and Type Allocation Code of PEI of the UE, e.g. when it hasn't received any UE Radio Capability ID.

The NF Service Consumer (e.g. AMF) shall obtain the PLMN Assigned UE Radio Capability ID by using the HTTP POST method as shown in Figure 5.2.2.3.1-1.

Figure 5.2.2.3-1 Create a dictionary entry

1.	The NF Service Consumer shall send a POST request to the resource representing the Dictionary Entries collection resource of the UCMF. The content of the POST request shall contain:

-	the UE Radio Access Capability Information for the current radio configuration of the UE in 5GS format, or EPS format, or both the formats;

-	the Type Allocation Code of the PEI of the UE.

2a.	On success, the UCMF shall check whether for the provided input a dictionary entry already exists and a PLMN Assigned UE Radio Capability ID has already been assigned. If so "201 Created" shall be returned, the content of the POST response shall contain already assigned PLMN Assigned UE Radio Capability ID included in DicEntryCreatedData and the "Location" header shall be present and shall contain the URI of the already existing resource. Otherwise a new dictionary entry shall be created and "201 Created" shall be returned, the content of the POST response shall contain the newly assigned PLMN Assigned UE Radio Capability ID included in DicEntryCreatedData and the "Location" header shall be present and shall contain the URI of the newly created resource.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.2.3.2-3 shall be returned.  For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.3.3.2-2, where applicable. A UCMF configured to operate in Mode of Operation A (3GPP TS 23.501 [2], Clause 5.4.4.1a) shall reject the operation if the request does not contain UE Radio Access Capability Information in both the formats and the UCMF is not able to find match of the received UE Radio Access Capability Information in its database.

* * * * Next Change * * * *

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nucmf_UECapabilityManagement service based interface protocol.

Table 6.1.6.1-1: Nucmf_UECapabilityManagement specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| DicEntryData | 6.1.6.2.2 | A dictionary entry for a UE radio capability ID in the UCMF |  |
| DicEntryCreateData | 6.1.6.2.3 | Data for a creating a dictionary entry request |  |
| DicEntryCreatedData | 6.1.6.2.4 | Data for a creating a dictionary entry response |  |
| UeRadioCapaId | 6.1.6.2.5 | UE Radio Capability ID |  |
| CreateSubscription | 6.1.6.2.6 | Data for a creating a subscription request |  |
| CreatedSubscription | 6.1.6.2.7 | Data for a creating a subscription response |  |
| UcmfNotification | 6.1.6.2.8 | Data for a notification request from a UCMF |  |
| ManAssOpRequestlist | 6.1.6.2.9 | Manufacturer Assigned operation requested list |  |
| EventType | 6.1.6.3.3 |  |  |
| RacFormat | 6.1.6.3.4 | The encoding type of the UE's Radio Access Capability Information |  |

Table 6.1.6.1-2 specifies data types re-used by the Nucmf_UECapabilityManagement service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nucmf_UECapabilityManagement service based interface.

Table 6.1.6.1-2: Nucmf_UECapabilityManagement re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| PlmnAssiUeRadioCapId | 3GPP TS 29.571 [14] | PLMN Assigned UE Radio Capability ID |  |
| ManAssiUeRadioCapId | 3GPP TS 29.571 [14] | Manufacturer Assigned UE Radio Capability ID |  |
| TypeAllocationCode | 3GPP TS 29.571 [14] | Type Allocation Code |  |
| NfInstanceId | 3GPP TS 29.571 [14] | NF Instance ID |  |
| DateTime | 3GPP TS 29.571 [14] | A date-time format defined in OpenAPI Specification [6] |  |
| SupportedFeatures | 3GPP TS 29.571 [14] | Supported Features |  |
| ProblemDetails | 3GPP TS 29.571 [14] | Detailed problems in failure case |  |
| RedirectResponse | 3GPP TS 29.571 [14] | Response body of the redirect response message. |  |

* * * * Next Change * * * *

6.1.6.2.2	Type: DicEntryData

Table 6.1.6.2.2-1: Definition of type DicEntryData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dicEntryId | DicEntryId | C | 0..1 | Identifier of the Dictionary Entry |
| plmnAssiUeRadioCapId | PlmnAssiUeRadioCapId | C | 0..1 | This IE shall include a PLMN Assigned UE Radio Capability ID if allocated in the dictionary entry. |
| manAssiUeRadioCapId | ManAssiUeRadioCapId | C | 0..1 | This IE shall include a Manufacturer Assigned UE Radio Capability ID if available in the dictionary entry. |
| typeAllocationCode | TypeAllocationCode | M | 1 | This IE shall contain the Type Allocation Code in corresponding to the UE Radio Access Capability in the dictionary entry. |
| ueRadioCapability5GS | RefToBinaryData | C | 0..1 | This IE shall be included to contain the UE Radio Access Capability Information encoded either as OCTET STRING of UE Radio Capability IE as specified in clause 9.3.1.74 of 3GPP TS 38.413 [15], when the message is sent to AMF, otherwise it may be included. |
| ueRadioCapabilityEPS | RefToBinaryData | C | 0..1 | This IE shall be included to contain the UE Radio Access Capability Information encoded as OCTET STRING of UE Radio Capability IE as specified in clause 9.2.1.27 of 3GPP TS 36.413 [17], when the message is sent to MME, otherwise it may be included. |
| ueRadioCap5GSForPaging | RefToBinaryData | O | 0..1 | May be present if ueRadioCapability5GS is present. If present it shall contain the UE Radio Access Capability Information for paging encoded as OCTET STRING of UE Radio Capability for Paging IE as specified in clause 9.3.1.68 of 3GPP TS 38.413 [15], when the message is sent to AMF |
| ueRadioCapEPSForPaging | RefToBinaryData | O | 0..1 | May be present if ueRadioCapabilityEPS is present. If present it shall contain the UE Radio Access Capability Information for paging encoded as OCTET STRING of UE Radio Capability for Paging IE as specified in clause 9.2.1.98 of 3GPP TS 36.413 [17], when the message is sent to MME, otherwise it may be included. |
| NOTE: The information in the dictionary entry which is included as the query parameter(s) or URI variable in the request message shall not be present. |  |  |  |  |

* * * * Next Change * * * *

6.1.6.3.4	Enumeration: RacFormat

The enumeration RacFormat represents the encoding type of the UE's Radio Access Capability Information.

Table 6.1.6.3.4-1: Enumeration RacFormat

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "5GS" | This value indicates UE Radio Access Capability Information shall be encoded as OCTET STRING of UE Radio Capability IE specified in clause 9.3.1.74 of 3GPP TS 38.413 [15]. |  |
| "EPS" | This value indicates UE Radio Access Capability Information shall be encoded as OCTET STRING of UE Radio Capability IE specified in clause 9.2.1.27 of 3GPP TS 36.413 [17]. |  |

* * * * End of Changes * * * *