3GPP TSG-CT WG4 Meeting #131	C4-254312

# Sophia Antipolis, France; 13th – 17th October 2025							   Revision of C4-254109

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0903 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Synchronizing status of QoS Flow(s) between anchor SMF and UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2025-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CR 23.502 #4501 (S2-2506050, approved at SA#108) clarifies how the anchor SMF deletes QoS flows in the UE when a previous PDU Session Modification Command requesting the deletion of QoS flows could not be sent to the UE (e.g. due to no paging response). In such a case, at the next UP activation without V-SMF change, the V-SMF notifies the H-SMF that UP is activated and the H-SMF triggers a new PDU Session Modification procedure to remove the QoS flows that are marked to be synchronized with the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The I-SMF/V-SMF sends an Update Request to the anchor SMF to signal that the UP resource of the PDU session has been established, during a service request procedure for a PDU session with an I-SMF/V-SMF, if the I-SMF/V-SMF has marked that the status of one or more QoS Flow(s) is to be deleted in the 5GC and synchronized with the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage 2. Stale QoS flows may remain active in the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.8.2.23, 6.1.6.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR does not cause any OpenAPI change. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1: text changed to " if the I-SMF/V-SMF has marked that the status of one or more QoS Flow(s) is to be deleted in the 5GC and synchronized with the UE." |  |  |  |  |  |  |  |  |

* * * 1st Change * * *

###### 5.2.2.8.2.23	Service Request without I-SMF/V-SMF Change or with I-SMF/V-SMF Change or with I-SMF Insertion

During a Service Request without I-SMF/V-SMF Change or with I-SMF/V-SMF Change or with I-SMF Insertion, the NF Service Consumer (i.e. the V-SMF for a HR PDU session, or the I-SMF for a PDU session with an I-SMF) shall update the PDU session in the H-SMF or SMF as follows:

Figure 5.2.2.8.2.23-1: PDU session update towards H-SMF or SMF during Service Request

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall additionally contain:

-	the upCnxState IE set to ACTIVATING if the User Plane resource for the PDU Session is going to be established by the I-SMF/V-SMF, or set to ACTIVATED if the User Plane resource for the PDU session has been established and the I-SMF/V-SMF has marked that the status of one or more QoS Flow(s) is to be deleted in the 5GC and synchronized with the UE (see clause 4.2.3.2 of 3GPP TS 23.502 [3]).

2a. Same as step 2a of Figure 5.2.2.8.2-1, with the following modifications.

The SMF shall behave as specified in clause as specified in clauses 4.3.3.3, 4.23.4.2 (step 7a) and 4.23.4.3 (step 8a) of 3GPP TS 23.502 [3].

2b.	Same as step 2b of Figure 5.2.2.8.2-1.

3.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall additionally contain:

-	the upCnxState IE set to ACTIVATED when User Plane resource has been established, if the upCnxState IE with the value ACTIVATING was previously provided to the SMF in the procedure, via a previous Update operation (step 1) or via Create operation for I-SMF/V-SMF insertion (see clause 5.2.2.7.6).

4a.	Same as step 2a of Figure 5.2.2.8.2-1, with the following modifications.

The SMF shall behave as specified in clause as specified in clause 4.23.4.2 (step 16) and clause 4.23.4.3 (step 20a) of 3GPP TS 23.502 [3].

4b.	Same as step 2b of Figure 5.2.2.8.2-1.

NOTE:	The upCnxState IE set to ACTIVATING or ACTIVATED implements the "Operation Type" parameter set to "UP Activate" or "UP Activated" (respectively) specified in clauses 4.2.3.2, 4.23.4.2 (step 7a, 16) and4.23.4.3 (step 8a, 20a) of 3GPP TS 23.502 [3].

* * * Next Change * * *

##### 6.1.6.2.11	Type: HsmfUpdateData

Table 6.1.6.2.11-1: Definition of type HsmfUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestIndication | RequestIndication | M | 1 | This IE shall indicate the request type. |  |
| *** skipped for clarity *** |  |  |  |  |  |
| upCnxState | UpCnxState | C | 0..1 | This IE shall be present to indicate the User Plane resource establishment status in the I-SMF/V-SMF, if an Update or Create request is required to be sent to the (H-)SMF before the User Plane resource for the PDU session is established during a service request procedure for a PDU session with an I-SMF/V-SMF (see clause 4.23.4.2 and clause 4.23.4.3 of 3GPP TS 23.502 [3]). This IE shall also be present to indicate the User Plane is activated in the I-SMF/V-SMF, during a service request procedure for a PDU session with an I-SMF/V-SMF, if the I-SMF/V-SMF has marked that the status of one or more QoS Flow(s) is to be deleted in the 5GC and synchronized with the UE (see clause 4.2.3.2 of 3GPP TS 23.502 [3]). When present, this IE shall be set as specified in clause 5.2.2.8.2.23. |  |
| *** skipped for clarity *** |  |  |  |  |  |
| NOTE 1: In shared networks, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator. NOTE 2: An SMF complying with this version of the specification should report secondary RAT usage using the secondaryRatUsageInfo attribute that replaces the secondaryRatUsageReport attribute. NOTE 3: If the smPolicyNotifyInd IE is received with the value "true", the SMF shall forward the callback information of the PCF for the UE to the PCF for SM Policy if exists via SM Policy Association Modification. See clause 4.3.3.2 of 3GPP TS 23.502 [3]. NOTE 4: The attribute name does not follow the naming conventions specified in 3GPP TS 29.501 [5]. The attribute name is kept though as defined in the current specification for backward compatibility reason. NOTE 5: The maxIntegrityProtectedDataRateUl and maxIntegrityProtectedDataRateDl IEs shall be ignored by the H-SMF or SMF if the Integrity protection maximum data rate in n1SmInfoFromUe IE is received. NOTE 6: The H-SMF shall determine that the UE is not registered for Disaster Roaming service if the disasterRoamingInd is not present during a V-SMF change procedure. NOTE 7: The (H-)SMF shall consider the PDU Set based handling is not supported if the pduSetSupportInd IE is absent during a V-SMF change. |  |  |  |  |  |

* * * End of Changes * * *