3GPP TSG-CT WG4 Meeting #131	C4-254324

# Sophia Antipolis, France; 13th – 17th October 2025						revision of C4-254065

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.571 | CR | 0684 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of presence conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There is a NOTE under Table 5.4.4.71-1: NOTE: Either ipv4Addr, or ipv6Addr, or ipv6Prefix shall be present. This is incorrect, because "either" is used only when referring to two alternatives. So in the above sentence, the wording leads to a confusion about which two items "either" is referring to. - Similarly the presence condition of measurementPeriodLte in Type: MdtConfiguration is unclear: |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the table note in Table 5.4.4.71-1. Correct the description of measurementPeriodLte in Table 5.6.4.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect text remains in the specification and the presence condition of IEs will remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4.71, 5.6.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact any yaml file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: further correction in the proposed text. |  |  |  |  |  |  |  |  |

* * *1st Change * * * *

#### 5.4.4.71	Type: IpAddr

Table 5.4.4.71-1: Definition of type IpAddr

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4Addr | C | 0..1 | When present, it shall contain the IPv4 address. |
| ipv6Addr | Ipv6Addr | C | 0..1 | When present, it shall contain the IPv6 address. |
| ipv6Prefix | Ipv6Prefix | C | 0..1 | When present, it shall contain the IPv6 Prefix. |
| NOTE: One and only one of ipv4Addr, ipv6Addr and ipv6Prefix shall be present. |  |  |  |  |

* * * 2nd Change * * * *

#### 5.6.4.2	Type: MdtConfiguration

Table 5.6.4.2-1: Definition of type MdtConfiguration

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| jobType | JobType | M | 1 | This IE shall indicate the Job type for MDT, see 3GPP TS 32.422 [19]. This IE shall be set to one of the following values: "IMMEDIATE_MDT_ONLY" "LOGGED_MDT_ONLY" "IMMEDIATE_MDT_AND_TRACE" "LOGGED_MBSFN_MDT" "IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS" "TRACE_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS" |
| reportType | ReportTypeMdt | C | 0..1 | This IE shall be present for logged MDT. When present, this IE shall indicate the report type for logged MDT, see 3GPP TS 32.422 [19]. |
| areaScope | AreaScope | O | 0..1 | When present, this IE shall contain the area in Cells or Tracking Areas where the MDT data collection shall take place, see 3GPP TS 32.422 [19]. |
| measurementLteList | array(MeasurementLteForMdt) | C | 1..N | This IE shall be present if the Job type is configured for Immediate MDT or combined Immediate MDT and Trace. When present, this IE shall contain a list of the measurements that shall be collected for LTE. |
| measurementNrList | array(MeasurementNrForMdt) | C | 1..N | This IE shall be present if the Job type is configured for Immediate MDT, combined Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements. When present, this IE shall contain a list of the measurements that shall be collected for NR. |
| sensorMeasurementList | array(SensorMeasurement) | O | 1..N | When present, this IE shall include a list of the sensor measurements to be collected for UE in NR if they are available. |
| sensorMeasurementLteList | array(SensorMeasurement) | O | 1..N | When present, this IE shall include a list of the sensor measurements to be collected for UE in LTE if they are available. |
| reportingTriggerList | array(ReportingTrigger) | C | 1..N | This IE shall be present if MeasurementList is configured for UE side measurements (such as M1 measurement in LTE) and the jobType is configured for Immediate MDT, combined Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements. When present, this IE shall contain a list of the reporting triggers. For LTE and NR, this IE shall not have the combination of periodical, event based and event based periodic reporting at the same time. |
| reportInterval | ReportIntervalMdt | C | 0..1 | This IE shall be present if the reportingTriggerList is configured for Periodic UE side measurements (such as M1 measurement in LTE) and the jobType is configured for Immediate MDT or combined Immediate MDT and Trace. When present, this IE shall indicate the interval between the periodical measurements to be taken when UE is in connected in LTE. |
| reportIntervalNr | ReportIntervaLNrMdt | C | 0..1 | This IE shall be present if the reportingTriggerList is configured for Periodic UE side measurements (such as M1 measurement in NR) and the jobType is configured for Immediate MDT, combined Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements. When present, this IE shall indicate the interval between the periodical measurements to be taken when UE is in connected in NR. |
| reportAmount | ReportAmountMdt | C | 0..1 | This IE shall be present if the reportingTriggerList is configured for Periodic UE side measurements (such as M1 measurement in LTE) and the jobType is configured for Immediate MDT, combined Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements. When present, this IE shall indicate the overall number of measurement reports that shall be taken for periodical reporting while UE is in connected. |
| reportAmountPerMeasurementLte | map(ReportAmountMdt) | C | 1..N | A map (list of key-value pairs where MeasurementLteForMdt serves as key; see clause 5.6.3.5) of ReportAmountMdt). May be present if reportAmount is present. When present, this IE shall indicate the number of measurement reports per measurement that shall be taken for periodical reporting while UE is in connected in LTE. |
| reportAmountPerMeasurementNR | map(ReportAmountMdt) | C | 1..N | A map (list of key-value pairs where MeasurementNrForMdt serves as key; see clause 5.6.3.6) of ReportAmountMdt). May be present if reportAmount is present. When present, this IE shall indicate the number of measurement reports per measurement that shall be taken for periodical reporting while UE is in connected in NR. |
| eventThresholdRsrp | integer | C | 0..1 | This IE shall be present if the report trigger parameter is configured for A2 event reporting or A2 event triggered periodic reporting and the job type parameter is configured for Immediate MDT or combined Immediate MDT and Trace in LTE. When present, this IE shall indicate the Event Threshold for RSRP, and the value shall be between 0-97. |
| eventThresholdRsrpNr | integer | C | 0..1 | This IE shall be present if the report trigger parameter is configured for A2 event reporting or A2 event triggered periodic reporting and the job type parameter is configured for Immediate MDT, combined Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements in NR. When present, this IE shall indicate the Event Threshold for RSRP, and the value shall be between 0-127. |
| eventThresholdRsrq | integer | C | 0..1 | This IE shall be present if the report trigger parameter is configured for A2 event reporting or A2 event triggered periodic reporting and the job type parameter is configured for Immediate MDT or combined Immediate MDT and Trace in LTE. When present, this IE shall indicate the Event Threshold for RSRQ, and the value shall be between 0-34. |
| eventThresholdRsrqNr | integer | C | 0..1 | This IE shall be present if the report trigger parameter is configured for A2 event reporting or A2 event triggered periodic reporting and the job type parameter is configured for Immediate MDT, combined Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements in NR. When present, this IE shall indicate the Event Threshold for RSRQ, and the value shall be between 0-127. |
| eventList | array(EventForMdt) | C | 1..N | This IE shall be present for event triggered measurement in the case of logged MDT. When present, this IE shall contain a list of events triggered measurement in NR. |
| loggingInterval | LoggingIntervalMdt | C | 0..1 | This IE shall be present if the job type is configured for Logged MDT or Logged MBSFN MDT in LTE. When present, this IE shall contain the periodicity for logging MDT measurement results for periodic downlink pilot strength measurement in LTE when UE is in Idle. |
| loggingIntervalNr | LoggingIntervalNrMdt | C | 0..1 | This IE shall be present if the job type is configured for Logged MDT or Logged MBSFN MDT in NR. When present, this IE shall contain the periodicity for logging MDT measurement results for periodic downlink pilot strength measurement in NR when UE is in Idle. |
| loggingDuration | LoggingDurationMdt | O | 0..1 | This IE shall be present if the job type parameter is configured for Logged MDT or Logged MBSFN MDT. When present, this IE shall indicate the validity time of MDT logged configuration for IDLE in LTE |
| loggingDurationNr | LoggingDurationNrMdt | O | 0..1 | This IE shall be present if the job type parameter is configured for Logged MDT or Logged MBSFN MDT. When present, this IE shall indicate the validity time of MDT logged configuration for IDLE in NR. |
| positioningMethod | PositioningMethodMdt | O | 0..1 | This IE may be present if the job type is set to Immediate MDT, Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements. When present, it shall indicate the positioning method that shall be used for the MDT job. For LTE the value "GNSS" may be selected only if the M1 measurement is selected in measurementList. |
| addPositioningMethodList | array(PositioningMethodMdt) | O | 1..N | This IE may be present if positioningMethod is present. When present, it shall indicate a list of the additional positioning methods that shall be used for the MDT job. For LTE, the value "GNSS" may be selected only if the M1 measurement is selected in measurementList. |
| collectionPeriodRmmLte | CollectionPeriodRmmLteMdt | C | 0..1 | This IE shall be present if the job type is set to Immediate MDT or Immediate MDT and Trace and any of the "M2" or "M3" is contained in measurementList attribute in LTE. When present, it shall contain the collection period that should be used to collect available measurement samples in case of RRM configured measurements. The same collection period should be used for all such measurements that are requested in the same MDT or combined Trace and MDT job. |
| collectionPeriodRmmNr | CollectionPeriodRmmNrMdt | C | 0..1 | This IE shall be present if the job type is set to Immediate MDT, Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements and any of the "M4" or "M5" is contained in measurementList attribute in NR. When present, it shall contain the collection period that should be used to collect available measurement samples in case of RRM configured measurements. The same collection period should be used for all such measurements that are requested in the same MDT or combined Trace and MDT job. |
| measurementPeriodLte | MeasurementPeriodLteMdt | C | 0..1 | This IE shall be present if: - the job type is set to Immediate MDT or combined Immediate MDT and Trace; and, - at least one of the values "M4_DL", "M4_UL", "M5_DL" and "M5_UL" is contained in measurementList attribute in LTE. When present, it shall contain the collection period that should be used for the Data Volume and Scheduled IP Throughput measurements made by the eNB. The same measurement period should be used for the UL and DL. |
| mdtAllowedPlmnIdList | array(PlmnId) | O | 1..N | When present, this IE shall contain the PLMNs where measurement collection, status indication and log reporting is allowed. E.g. the UE performs these actions for Logged MDT when the RPLMN is part of this set of PLMNs. Maximum of 16 PLMNs can be contained. |
| mbsfnAreaList | array(MbsfnArea) | O | 1..N | When present, this IE shall contain MBSFN Area(s) for MBSFN measurement logging. Maximum of 8 MBSFN area(s) can be contained. This parameter is applicable only if the job type is Logged MBSFN MDT and for eUTRAN only. |
| interFreqTargetList | array(InterFreqTargetInfo) | O | 1..8 | When present, this IE shall indicate Inter Frequency Target(s) for which the UE is requested to perform measurement logging. |
| mnOnlyInd | boolean | O | 0..1 | When present, this IE shall indicate whether the MDT Configuration is only applicable for the master node or not if the UE is configured with MR-DC. This IE shall be set as follows: - true: MDT Configuration is for master node only - false (default): MDT Configuration is for both master node and secondary node. |
| eventThresholdSinrNr | integer | C | 0..1 | This IE shall be present if the report trigger parameter is configured for A2 event reporting or A2 event triggered periodic reporting and the job type parameter is configured for Immediate MDT or combined Immediate MDT and Trace in NR. When present, this IE shall indicate the Event Threshold for SINR, and the value shall be between 0-127. |
| bluetoothMeasurementNr | BluetoothMeasurement | O | 0..1 | When present, this IE shall include the bluetooth measurements to be collected for UE in NR if they are available. |
| bluetoothMeasurementLte | BluetoothMeasurement | O | 0..1 | When present, this IE shall include the bluetooth measurements to be collected for UE in LTE if they are available. |
| wlanMeasurementNr | WlanMeasurement | O | 0..1 | When present, this IE shall include the WLAN Measurements to be collected for UE in NR if they are available. |
| wlanMeasurementLte | WlanMeasurement | O | 0..1 | When present, this IE shall include the WLAN Measurements to be collected for UE in LTE if they are available. |
| sliceAreaScope | array(SliceScopePerPlmn) | O | 1..16 | When present, this IE shall contain the list of network slices for MDT. |

* * * End of Changes * * * *