3GPP TSG-CT WG4 Meeting #131	C4-254325

# Sophia Antipolis, France; 13th – 17th October 2025							   Revision of C4-254110

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.571 | CR | 0687 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR NTN Intended Service Area |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 5.9.2 contains the following editor's note: Editor's note: the encoding of the IntendedServiceArea needs to be confirmed/aligned on TS 38.413. TS 38.413 v19.0.0 specifies: |  |  |  |  |  |  |  |  |
| Summary of change: |  | A reference is added to clause 9.3.1.273 of TS 38.413 for the encoding of the IntendedServiceArea. The editor's note is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not cause any OpenAPI change. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1: Huawei and Ericsson added as co-sources |  |  |  |  |  |  |  |  |

* * * 1st Change * * *

### 5.9.2	Simple Data Types

This clause specifies common simple data types.

Table 5.9.2-1: Simple Data Types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| AreaSessionId | Uint16 | Area Session Identifier used for MBS session with location dependent content. When present, the Area Session ID together with the TMGI uniquely identifies the MBS session in a specific MBS service area. |
| AreaSessionPolicyId | Uint16 | Area Session Policy ID used for MBS session with location dependent content. |
| MbsFsaId | string | MBS Frequency Selection Area ID, for a broadcast MBS session The value of the MbsFsaId shall be encoded in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the MBS FSA Id shall appear first in the string, and the character representing the 4 least significant bit of the MBS FSA Id shall appear last in the string. Pattern: '^[A-Fa-f0-9]{6}$' |
| IntendedServiceArea | Bytes | String with format "byte" as defined in OpenAPI Specification [3], i.e. base64-encoded characters, encoding the value of the Intended Service Area Coordinates IE defined in clause 9.3.1.273 of 3GPP TS 38.413 [11]. |

* * * End of Changes * * *