3GPP TSG-CT WG4 Meeting #132	C4-255318

Dallas; 17th – 21th Novermber 2025	was C4-255129

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0391 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the Condition for Conditional IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1/ CT4#131 has agreed on the statement for the condition of conditional IEs: "It has been discussed about the understanding of “Conditional” IE in SBI specs, the conclusion is: when an IE is conditional it SHALL be present when the condition is met. The rapporteurs of the SBI specs are tasked to come up with Rel-19 CRs making the corrections if current spec is not inline with aforementioned understanding." - For integrity result, the LMF shall provide the integrity result when it is requested by the NF consumer. The description is chagned from should to shall. - For minimumInterval, maximumInterval, there is no determined condition to mandate for the presence of the IEs. The presence is changed to optional. 2/ Condition of a conditional IE is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ Correct the condition of integrity result IE to align the agreed conclusion. 2/ Update the presence to from conditional to optional for IEs for minimumInterval, maximumInterval 3/ Add a missing condition for a conditional IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Condition of the conditional IEs are unclear in the specification which leads to possible interwork issues in the network deployments. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.3, 6.1.6.2.25, 6.1.6.2.27, 6.1.6.2.34 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact any yaml file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Align the condition description with SA2 - Update reason for change - Correct WI code to TEI19 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.2.3	Type: LocationData

Table 6.1.6.2.3-1: Definition of type LocationData

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| locationEstimate |  | GeographicArea |  | M |  | 1 |  | For a request for triggered location where location estimates are not required, the location estimate can be based on current serving cell. |  |  |  |
| accuracyFulfilmentIndicator |  | AccuracyFulfilmentIndicator |  | O |  | 0..1 |  | When present, this IE shall indicate fulfilment of required accuracy. |  |  |  |
| ageOfLocationEstimate |  | AgeOfLocationEstimate |  | O |  | 0..1 |  | When present, this IE shall indicate age of the location estimate. |  |  |  |
| timestampOfLocationEstimate |  | DateTime |  | O |  | 0..1 |  | When present, this IE shall indicate the estimated UTC time when the location estimate corresponded to the UE location (i.e. when the location estimate and the actual UE location was the same). |  |  |  |
| velocityEstimate |  | VelocityEstimate |  | O |  | 0..1 |  | When present, this IE shall indicate velocity estimate. |  |  |  |
| civicAddress |  | CivicAddress |  | O |  | 0..1 |  | When present, this IE shall indicate a civic address. |  |  |  |
| localLocationEstimate |  | LocalArea |  | O |  | 0..1 |  | When present, this IE shall indicate a local area in reference system. |  |  |  |
| positioningDataList |  | array(PositioningMethodAndUsage) |  | O |  | 1..N |  | When present, this IE shall include a list of data related to positioning methods. |  |  |  |
| gnssPositioningDataList |  | array(GnssPositioningMethodAndUsage) |  | O |  | 1..N |  | When present, this IE shall include a list of data related to GNSS positioning methods. |  |  |  |
| ecgi |  | Ecgi |  | O |  | 0..1 |  | When present, this IE shall indicate the ID of the E-UTRAN cell serving the UE. |  |  |  |
| ncgi |  | Ncgi |  | O |  | 0..1 |  | When present, this IE shall indicate the ID of the NR cell serving the target UE, or the NR cell serving the relay UE if remoteUeInd IE is present with the value true. |  |  |  |
| remoteUeInd |  | boolean |  | C |  | 0..1 |  | This IE shall be present with the value true when the UE is a Remote UE. Presence of this IE with the value false shall be prohibited. |  |  |  |
| altitude |  | Altitude |  | O |  | 0..1 |  | Altitude of the positioning estimate. When the shape used in "locationEstimate" supports conveying the altitude parameter, this IE shall be absent. |  |  |  |
| barometricPressure |  | BarometricPressure |  | O |  | 0..1 |  | If present, this IE contains the barometric pressure measurement as reported by the target UE. |  |  |  |
| servingLMFidentification |  | LMFIdentification |  | O |  | 0..1 |  | When present, this IE shall indicate the identity of the serving LMF |  |  |  |
| uePositioningCap |  | UePositioningCapabilities |  | O |  | 0..1 |  | When present, this IE shall indicate the positioning capabilities supported by the UE. |  |  |  |
| supportedFeatures |  | SupportedFeatures |  | C |  | 0..1 |  | This IE shall be present if at least one feature defined in clause 6.1.9 is supported. |  |  |  |
| ueAreaInd |  | UeAreaIndication |  | O |  | 0..1 |  | When present, this IE shall contain a country, area in a country or international area indication where UE is located. If UE is outside of the area of any known country, i.e. international area, then the LMF shall provide the International Area Indication without a country. |  | SAT |  |
| achievedQos |  | MinorLocationQoS |  | O |  | 0..1 |  | When present, this IE shall contain the achieved Location QoS Accuracy of the estimated location. This IE may be present if the LCS QoS Class required in the request message is "MULTIPLE_QOS". If it's absent, LCS QoS Class required in the request message is "MULTIPLE_QOS" and AccuracyFulfilmentIndicator is "REQUESTED_ACCURACY_FULFILLED", it indicates that the location QoS in the highest priority is achieved. |  | MUTIQOS |  |
| directReportInd |  | boolean |  | C |  | 0..1 |  | This IE shall be present, if the H-GMLC contact address and the Notification Correlation ID are received in the request, i.e. the location estimates will be sent directly to the H-GMLC. When present, this IE shall be set for the following value: - true: location determination will be sent by LMF to GMLC directly - false (default): location determination will not be sent by LMF to GMLC directly |  |  |  |
| indoorOutdoorInd |  | IndoorOutdoorInd |  | O |  | 0..1 |  | When present, this IE shall indicate whether the UE is indoor or outdoor. |  |  |  |
| acceptedPeriodicEventInfo |  | PeriodicEventInfo |  | C |  | 0..1 |  | This IE shall be present if PeriodicEventInfo was received in the request which includes reportingAmountInf IE and/or reportingIntervalMs IE. When present, this IE shall provide the accepted periodic event reporting information. |  |  |  |
| haGnssMetrics |  | HighAccuracyGnssMetrics |  | O |  | 0..1 |  | When present, this IE shall indicate the high accuracy GNSS metrics received from the device in the LPP HA-GNSS-Metrics-r17 IE as specified in 3GPP TS 37.355 [21]. |  |  |  |
| losNlosMeasureInd |  | LosNlosMeasureInd |  | O |  | 0..1 |  | When present, this IE shall indicate whether LOS measurement or NLOS measurement is used. |  |  |  |
| rangingSlCapability |  | RangingSlCapability |  | O |  | 0..1 |  | When present, this IE shall indicate that the UE supports Ranging/Sidelink Positioning Capability. |  |  | Ranging_SL |  |
| relatedApplicationlayerId |  | ApplicationlayerId |  | O |  | 0..1 |  | Identifies the application layer ID of the related UE for ranging and sidelink positioning, such as located UE, reference UE, etc. |  |  | Ranging_SL |  |
| distanceDirection |  | RangeDirection |  | O |  | 0..1 |  | When present, this IE identifies a straight line distance (range) and direction from a point A to a point B, comprising a straight line distance (range) from point A to point B, an azimuth direction from point A to point B and an elevation direction from point A to point B. |  |  | Ranging_SL |  |
| 2dRelativeLocation |  | 2DRelativeLocation |  | O |  | 0..1 |  | When present, this IE identifies a relative 2D location with uncertainty ellipse, characterised by a point described in 2D local co-ordinates with origin corresponding to another known point, distances r1 and r2 and an angle of orientation A. |  |  | Ranging_SL |  |
| 3dRelativeLocation |  | 3DRelativeLocation |  | O |  | 0..1 |  | When present, this IE identifies a relative 3D location with uncertainty ellipsoid, characterised by a point described in 3D local co-ordinates with origin corresponding to another known point, distances r1 (the "semi-major uncertainty"), r2 (the "semi-minor uncertainty") and r3 (the "vertical uncertainty") and an angle of orientation A (the "angle of the major axis"). |  |  | Ranging_SL |  |
| relativeVelocity |  | VelocityEstimate |  | O |  | 0..1 |  | When present, this IE identifies UE velocity relative to the UE identified with relatedApplicationlayerId. |  |  | Ranging_SL |  |
| integrityResult |  | IntegrityResult |  | C |  | 0..1 |  | This IE shall be present when the integrity requirements are present in the request. When present, this IE shall indicate the integrity result. |  |  | INTRES |  |
| nrppaPeriodicInd |  | boolean |  | C |  | 0..1 |  | This IE shall be present with the value true, if the LMF has determined to generate Periodic Location Estimates based on NRPPa Periodic Measurement. Presence of this IE with the value false shall be prohibited. |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.25	Type: AreaEventInfo

Table 6.1.6.2.25-1: Definition of type AreaEventInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| areaDefinition | array(ReportingArea) | M | 1..250 | One or more reporting areas |
| occurrenceInfo | OccurrenceInfo | O | 0..1 | One time only report indication |
| minimumInterval | MinimumInterval | O | 0..1 | Minimum interval between event reports. This IE shall not be included if occurrenceInfo is present and set to one time event. |
| maximumInterval | MaximumInterval | O | 0..1 | Maximum interval between event reports. This IE shall not be included if occurrenceInfo is present and set to one time event. |
| samplingInterval | SamplingInterval | O | 0..1 | Maximum time interval between consecutive evaluations by a UE of a trigger event. |
| reportingDuration | ReportingDuration | O | 0..1 | Maximum duration of event reporting. |
| reportingLocationReq | boolean | C | 0..1 | This IE shall be present and set to true if a location estimate is required for each event report. When present, it shall be set as follows: - true: location report is required. - false: location report is not required. |

* * * Next Change * * * *

##### 6.1.6.2.27	Type: MotionEventInfo

Table 6.1.6.2.27-1: Definition of type MotionEventInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| linearDistance | LinearDistance | M | 1 | Minimum linear (straight line) distance for motion event reports. |
| occurrenceInfo | OccurrenceInfo | O | 0..1 | One time only report indication |
| minimumInterval | MinimumInterval | O | 0..1 | Minimum interval between event reports. This IE shall not be included if occurrenceInfo is present and set to one time event. |
| maximumInterval | MaximumInterval | O | 0..1 | Maximum interval between event reports. This IE shall not be included if occurrenceInfo is present and set to one time event. |
| samplingInterval | SamplingInterval | O | 0..1 | Maximum time interval between consecutive evaluations by a UE of a trigger event. |
| reportingDuration | ReportingDuration | O | 0..1 | Maximum duration of event reporting. |
| reportingLocationReq | boolean | C | 0..1 | This IE shall be present and set to true if a location estimate is required for each event report. When present, it shall be set as follows: - true: location report is required. - false: location report is not required. |

* * * Next Change * * * *

##### 6.1.6.2.34	Type: EventNotifyData

Table 6.1.6.2.34-1: Definition of type EventNotifyData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reportedEventType | ReportedEventType | M | 1 | This IE shall contain the type of event being reported. |  |
| supi | Supi | C | 0..1 | This IE shall contain the SUPI if available. |  |
| gpsi | Gpsi | C | 0..1 | This IE shall contain the GPSI if available. |  |
| hgmlcCallBackURI | Uri | C | 0..1 | Callback URI of the H-GMLC (NOTE 1) |  |
| ldrReference | LdrReference | M | 1 | LDR Reference When the ReportedEventType is "INTERMEDIATE_EVENT", the LMF shall set this IE to the value "NULL" and shall be ignored by the receiver. |  |
| lirReference | LirRefernece | C | 0..1 | This IE shall be present when the ReportedEventType is set to the value "INTERMEDIATE_EVENT". When present, this IE shall include the LIR Reference number received in the 5GC-MR-LR multiple location request. |  |
| locationEstimate | GeographicArea | O | 0..1 | If present, this IE shall contain an estimate of the location of the UE in universal coordinates and the accuracy of the estimate. |  |
| ageOfLocationEstimate | AgeOfLocationEstimate | O | 0..1 | If present, this IE shall contain an indication of how long ago the location estimate was obtained. |  |
| timestampOfLocationEstimate | DateTime | O | 0..1 | When present, this IE shall indicate the estimated UTC time when the location estimate corresponded to the UE location (i.e. when the location estimate and the actual UE location was the same). |  |
| civicAddress | CivicAddress | O | 0..1 | If present, this IE shall contain a civic address. |  |
| localLocationEstimate | LocalArea | O | 0..1 | When present, this IE shall indicate a local area in reference system. |  |
| positioningDataList | array(PositioningMethodAndUsage) | O | 1..N | If present, this IE shall indicate the usage of each non-GANSS positioning method that was attempted to determine the location estimate, either successfully or unsuccessfully. |  |
| gnssPositioningDataList | array(GnssPositioningMethodAndUsage) | O | 1..N | If present, this IE shall indicate the usage of each GANSS positioning method that was attempted to determine the location estimate, either successfully or unsuccessfully. |  |
| servingLMFIdentification | LMFIdentification | C | 0..1 | This IE shall be included to identify an LMF which acts as a serving LMF if a serving LMF is used. |  |
| terminationCause | TerminationCause | C | 0..1 | This IE shall be included if event reporting has been terminated |  |
| velocityEstimate | VelocityEstimate | O | 0..1 | If present, this IE shall contain an estimate of the velocity of the target UE, composed by horizontal speed, vertical speed, and their respective uncertainty. |  |
| altitude | Altitude | O | 0..1 | If present, this IE indicates the altitude of the positioning estimate. When the shape used in "locationEstimate" supports conveying the altitude parameter, this IE shall be absent. |  |
| achievedQos | MinorLocationQoS | O | 0..1 | When present, this IE shall contain the achieved Location QoS Accuracy of the estimated location. This IE may be present if the required LCS QoS Class in the location request procedure is "MULTIPLE_QOS". If it's absent, the required LCS QoS Class in the location request procedure is "MULTIPLE_QOS" and AccuracyFulfilmentIndicator is "REQUESTED_ACCURACY_FULFILLED", it indicates that the location QoS in the highest priority is achieved. | MUTIQOS |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.9 is supported. |  |
| indoorOutdoorInd | IndoorOutdoorInd | O | 0..1 | When present, this IE shall indicate whether the UE is indoor or outdoor. |  |
| haGnssMetrics | HighAccuracyGnssMetrics | O | 0..1 | When present, this IE shall indicate the high accuracy GNSS metrics received from the device in the LPP HA-GNSS-Metrics-r17 IE as specified in 3GPP TS 37.355 [21]. |  |
| losNlosMeasureInd | LosNlosMeasureInd | O | 0..1 | When present, this IE shall indicate whether LOS measurement or NLOS measurement is used. |  |
| upLocRepStatAf | integer | O | 0..1 | When present, this IE contains the number of event reports have transferred over user plane. If the cumulative event report has been sent previously, this IE contains the number of event reports have transferred over user plane since the last cumulative event report was sent. |  |
| relatedApplicationlayerId | ApplicationlayerId | O | 0..1 | Identifies the application layer ID of the related UE for ranging and sidelink positioning, such as located UE, reference UE, etc. | Ranging_SL |
| distanceDirection | RangeDirection | O | 0..1 | When present, this IE identifies a straight line distance (range) and direction from a point A to a point B, comprising a straight line distance (range) from point A to point B, an azimuth direction from point A to point B and an elevation direction from point A to point B. | Ranging_SL |
| 2dRelativeLocation | 2DRelativeLocation | O | 0..1 | When present, this IE identifies a relative 2D location with uncertainty ellipse, characterised by a point described in 2D local co-ordinates with origin corresponding to another known point, distances r1 and r2 and an angle of orientation A. | Ranging_SL |
| 3dRelativeLocation | 3DRelativeLocation | O | 0..1 | When present, this IE identifies a relative 3D location with uncertainty ellipsoid, characterised by a point described in 3D local co-ordinates with origin corresponding to another known point, distances r1 (the "semi-major uncertainty"), r2 (the "semi-minor uncertainty") and r3 (the "vertical uncertainty") and an angle of orientation A (the "angle of the major axis"). | Ranging_SL |
| relativeVelocity | VelocityEstimate | O | 0..1 | When present, this IE identifies UE velocity relative to the UE identified with relatedApplicationlayerId. | Ranging_SL |
| integrityResult | IntegrityResult | C | 0..1 | This IE shall be present when the integrity requirements are present in the request. When present, this IE shall indicate the integrity result. | INTRES |
| NOTE 1: If the reportedEventType is not "INTERMEDIATE_EVENT", the hgmlcCallBackURI shall be included when the consumer NF is not the H-GMLC. |  |  |  |  |  |

* * * End of Changes * * * *