3GPP TSG CT Meeting #110	CP-253183

# Baltimore, United States, 8 - 9 December, 2025							revision of C4-255384

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.570 | CR | 0003 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of SCP event report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, Nokia, vivo |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | S2-2509569 (TS 23.288 CR 1505) has made corrections to Table 6.22.2-8: SCP Signalling Infomation. According to this table, the SCP may provide some statistical information per NF instance and/or per NF service instance. According the stage-2 the following statistics may be provided: - Number of requests sent by an NF instance - Number of times SCP reselect NF instance that is originally to that NF instance - Reason of SCP reselection of NF instance - Status of the connection to the NF instance - Average time duration from sending request to the NF instance to receiving response from the NF instance. However in stage-3 currently some of the statistics are missing and some descriptions deviate from SA2 definitions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In ScpSignallingInfo, align the description to stage-2 definitions. Add missing report attributes: - Number of requests sent by an NF instance - Number of times SCP reselect NF instance that is originally to that NF instance |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage-2 definition and mechanism. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.8, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.288 CR 1505 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following APIs: TS29570_Nscp_EventExposure.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Co-source companies added (merged CRs). - Descriptions and new IEs aligned with merged CRs. Rev2: Description of avgResponseTimeToNF further updated. Rev3: Correction of a missing change in Rev2: in Table 6.1.6.2.8-1, the description of rcvRequestCount was updated without revision marks. In Rev3, this is corrected. |  |  |  |  |  |  |  |  |

* * *1st Change * * * *

##### 6.1.6.2.8	Type: ScpSignallingInfo

Table 6.1.6.2.8-1: Definition of type ScpSignallingInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serviceInstanceId | string | C | 0..1 | This IE shall be present if available. When present it shall identify the NF service instance for which the statistics are reported. (NOTE) |  |
| nfInstanceId | NfInstanceId | C | 0..1 | NF Instance ID hosting the NF service instance. The IE shall be present when available. (NOTE) |  |
| serviceName | ServiceName | O | 0..1 | Identifies the name of the NF service (e.g., Namf-Comm, Nudm-SDM) for which the statistics apply. (NOTE) |  |
| nfType | NFType | O | 0..1 | Identifies the NF type of The NF instance for which the statistics are reported. (NOTE) |  |
| nfSetId | NfSetId | O | 0..1 | Identifies the NF set of the NF instance for which the statistics are reported. (NOTE) |  |
| rcvRequestCount | Uinteger | O | 0..1 | Number of requests received by SCP from the NF service instance. |  |
| sentRequestCount | Uinteger | O | 0..1 | Number of requests sent by the SCP to the NF service instance. |  |
| successfulResponseCount | Uinteger | O | 0..1 | Number of successful responses received from this NF service instance. |  |
| failureResponseCount | Uinteger | O | 0..1 | Total number of failed responses received from this NF service instance. |  |
| failureCauseStats | array(FailureCauseOccurrence) | O | 1..N | Breakdown of failure causes for this NF service instance. |  |
| scpFailureCount | Uinteger | O | 0..1 | Number of failures generated by the SCP before reaching the NF (e.g. unreachable NF). |  |
| scpFailureCauseStats | array(FailureCauseOccurrence) | O | 0..1 | Breakdown of SCP-side failure causes targeting this NF service instance. |  |
| reselectionCount | Uinteger | O | 0..1 | Number of times SCP reselect NF instance |  |
| reselectionStatList | array(ReselectionStat) | O | 1..N | List of reasons of SCP reselecting NF instance producer with their occurrence counts. Each entry pairs a ReselectionReason with the corresponding number of reselections performed by the SCP. |  |
| connectionStatus | ConnectionStatus | O | 0..1 | Connection status between SCP and the NF service instance (e.g., ACTIVE, INACTIVE). |  |
| avgResponseTimeToNF | Uinteger | O | 0..1 | Average time in units of milliseconds between SCP sending the request to the NF instance and SCP receiving the response from the NF instance. The requests for which no response from the NF instance is received are ignored for calculating the average time in this IE. |  |
| overloadControlInfo | OverloadControlInfo | O | 0..1 | Contains overload control metrics (e.g., reduction percentage, validity period) for the reported NF service instance, as defined in clause 6.4 and 5.2.3.2.9 of 3GPP TS 29.500 [4]. The SCP may send this information based on the 3gpp-Sbi-Oci header when overload mitigation is active towards the NF (service) instance. |  |
| NOTE: At least one of the following identifiers serviceInstanceId, nfInstanceId, serviceName, nfType or nfSetId shall be present. |  |  |  |  |  |

* * * 2nd Change * * * *

# A.2	Nscp_EventExposure API

openapi: 3.0.0

info:

title: Nscp_EventExposure

version: 1.0.0-alpha.5

description: |

SCP Event Exposure Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.570 V19.0.0; Service Communication Proxy Services.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.570/

[non changing parts skipped for clarity]

ScpSignallingInfo:

description:

Per service instance statistics including SCP and NF statistics behavior and overload.

type: object

properties:

serviceInstanceId:

type: string

nfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

serviceName:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ServiceName'

nfType:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

nfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

rcvRequestCount:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

sentRequestCount:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

successfulResponseCount:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

failureResponseCount:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

failureCauseStats:

type: array

items:

$ref: '#/components/schemas/FailureCauseOccurrence'

minItems: 1

scpFailureCount:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

scpFailureCauseStats:

type: array

items:

$ref: '#/components/schemas/FailureCauseOccurrence'

minItems: 1

reselectionCount:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

reselectionStatList:

type: array

items:

$ref: '#/components/schemas/ReselectionStat'

minItems: 1

connectionStatus:

$ref: '#/components/schemas/ConnectionStatus'

avgResponseTimeToNF:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

overloadControlInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/OverloadControlInfo'

[non changing parts skipped for clarity]

* * * End of Changes * * * *