3GPP TSG-CT WG4 Meeting #131	C4-254359

Sophia-Antipolis; 13th – 17th October 2025	was C4-254336, C4-254178

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0379 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UP Configuration for existing LCS-UPP connection after AMF Change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc19 |  |  |  |  |  | Date: |  |  | 2025-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in 23.273, for LCS-UPP modification procedures, when the source LMF cannot continue serving the UE (e.g. due to UE mobility to a new area), the source LMF triggers the LCS-UPP modification by sending UP Notification towards the target AMF using the callback URI previously provided by the AMF. However, as specified by TS 23.273, after AMF change, the target AMF may inform the LMF using UP Config. If the target AMF chooses not to inform the LMF, the source LMF may possibly not know the LCS-UPP modification is needed. Even worse, if the LMF determined that LCS-UPP modification is needed, the source LMF cannot deliver the UP Notify to the target AMF because the callback URI of UP Notification is not available (the LMF will most likely deliver the UP notify to the old AMF with the stored callback URI). This CR proposes that the new AMF always informs the corresponding serving LMF immediately after the AMF change and provides the callback URI for existing LCS-UPP connection context in the UE context. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description to mandate the AMF to inform the LMF for AMF change with UP config after AMF changed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | After AMF change during UE mobility, the LMF cannot know that the AMF has changed and cannot deliver the UP notification to the target AMF. Source LMF triggered LCS-UP modification cannot work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact any yaml file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Update the change to mandate AMF inform LMF with UP config after AMF change. Rev2: - Add Nokia as co-source - Editorial corrections |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.2.9.2	Configure LCS-UP connection

This procedure allows a consumer NF (i.e. AMF) to set up, modify or terminate a secure LCS-UP connection for a target UE and to get the notification of LCS-UPP connection status.

Additionally, if AMF change happens during UE mobility, the target AMF shall also invoke the service operation to inform the serving LMF, which has established LCS-UPP connection with the UE, that the serving AMF has changed.

Figure 5.2.2.9.2-1: LCS-UP connection

1.	The NF service consumer shall send an HTTP POST request to the resource URI associated with the "configure-up" custom operation to set up, modify or terminate LCS-UP connection. The request content shall include a UP notify callback URI, a notification correlation ID, and the UE identity.

2a.	On success, "204 No Content" shall be returned.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.4.7.2-2 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.4.7.2-2.

* * * End of Changes * * * *