3GPP TSG-CT WG1 Meeting #157	C1-256044

Sophia Antipolis, France, 13-17 October 2025

Source:	vivo

Title:	Resolve EN on AIoT device identity

Spec:	3GPP TS 24.369-100

Agenda item:	19.70

Document for:	Decision

1. Introduction

<Introduction part (optional)>

2. Reason for Change

For the following EN:

Editor's note:	Details of the AIoT device identity IE is FFS and depends on SA3 progress.

According to clause 5.4 of TS 33.369:

-	if privacy protection is not used, AIoT device permanent identifier is included;

-	if privacy protection is used, the AIoT device does not need to include any "device ID".

For the format of AIoT device permanent ID, though defined in TS 23.003, the NID is 44 bit and not byte-aligned. Hence, the coding of NID should refer to TS 24.502, and the detailed coding format of AIoT permanent ID is defined in 7.2.6.

The minimal size of AIoT device identity IE = IEI (1 octet) + len (1 octet) + AIoT device permanent ID (1 octet ID type + 0 octet domain info + 12 octets identification information) = 15 octets

(clause 5.4 of TS 33.369 is copied as follows)

## 5.4 Protection of AIoT device identifier privacy ### 5.4.1 General … When privacy protection is not used during the inventory procedure, the AIoT device includes its AIoT device permanent identifier as a device identification information in the procedure specified in clause 5.2.2. 5.4.2 The AIoT device identifier protection for inventory with filtering information For the protection of AIoT device permanent ID during the inventory procedure described in clause 5.2.2, the following change shall apply: … - In step 5 and 6, a device identification information is not included in the D2R message and Inventory Report message. - In step 7, filtering information is used as a device identification information if the AIOTF received it in step 0. NOTE 2: The AIOTF identifies the AIoT device by checking the received RESAIoT. Therefore, device identification information is not needed in the D2R and Inventory Report message. 5.4.3 Procedure for AIoT Device identifier protection with Temp ID update during Individual inventory For the protection of AIoT device permanent identifier during the inventory procedure with AIoT device identifier described in clause 5.2.2, the following changes shall apply: … - In step 5 and 6, a device identification information is not included in the D2R message and Inventory Report message. - In step 7, the AIoT device permanent identifier is used as a device identification information. AIOTF requests the ADM to derive a new T-ID as specified in Annex B.1 and to store it in the AIoT Device profile. NOTE 1: The AIOTF identifies the AIoT device by checking the received RESAIoT parameter. Therefore, the device identification information is not needed in the D2R message and Inventory Report message.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 V1.0.0.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[3]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[4]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[5]	3GPP TS 23.003: "Numbering, addressing and identification".

[6]	3GPP TS 33.369: "Security aspects of ambient IoT services in 5G".

[7]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[8]	3GPP TS 38.391: "Ambient IoT Medium Access Control Protocol specification".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[10]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[X]	3GPP TS 24.502: "Access to the 3GPP 5G System (5GS) via non-3GPP access networks; Stage 3".

* * * Next Change * * * *

5.2.3	Inventory procedure completion

The AIoT device determines to respond to the AIoT paging:

a)	based on the indication from lower layer as specified in 3GPP TS 38.391 [8]; or

b)	based on whether the AIoT identification information received from the lower layer is matched or not.

If the AIoT device determines to respond to the AIoT paging, the AIoT device shall send an INVENTORY REPORT message to the AIOTF.

The AIoT device shall derive a RANDAIOT_d value as specified in 3GPP TS 33.369 [6] and shall include the derived RANDAIOT_d  value in the in the RANDAIOT_d IE in the INVENTORY REPORT message.

The AIoT device shall derive a RESAIOT value, using the long-term configured KAIoT and the value of the RANDAIOT_n included in the paging message provided by lower layers as specified in 3GPP TS 33.369 [6]. The derived RESAIOT value shall be included in the in the RESAIOT IE in the INVENTORY REPORT message.

The AIoT device shall include the AIoT device identity IE in the INVENTORY REPORT message if the privacy protection is not used as specified in 3GPP TS 33.369 [6].

The AIoT device determines whether the AIoT identification information is matched or not as follows:

Editor's note:	It is FFS how to determine the component of AIoT identification information for AIoT device, and this may need coordination with RAN2/3.

a)	if the AIoT identification information includes an AIoT device identifier:

Editor's note:	Details of AIoT device identifier should be aligned with security consideration from SA3. How to match the AIoT device identifier depends on the AIoT device identifier included.

b)	if the AIoT identification information includes filtering information (see clause 31.3 of 3GPP TS 23.003 [5]):

1)	based on ID type field included in the filtering information, the AIoT device determines the AIoT identification information is matched if:

A)	the PLMN ID field is not included in the filtering information, or the PLMN ID field is included in the filtering information and:

i)	the PLMN ID field is included in the AIoT device permanent identifier; and

ii)	the PLMN ID field in the AIoT device permanent identifier is the same as the PLMN ID field within the filtering information;

B)	the NID field is not included in the filtering information, or the NID field is included in the filtering information and:

i)	the NID field is included in the AIoT device permanent identifier; and

ii)	the NID field in the AIoT device permanent identifier is the same as the NID field within the filtering information;

C)	the third party ID field is not included in the filtering information, or the third party ID field is included in the filtering information and:

i)	the third party ID field is included in the AIoT device permanent identifier; and

ii)	the third party ID field in the AIoT device permanent identifier is the same as the third party ID field within the filtering information; and

D)	the identification information filters field is not included in the filtering information, or the identification information filters field is included in the filtering information and for each identification information filter, the component value within the identification information filter is the same as the corresponding bitstring from the AIoT device permanent identifier based on offset field and length field; or

2)	otherwise, the AIoT device determines the AIoT identification information is not matched.

NOTE 1:	The AIoT identification information corresponds to paging ID, as defined in 3GPP TS 38.391 [8]).

Editor's note:	The reference to filtering information (e.g. IE name) will be updated according to CT4 and RAN2 progress.

Based on whether the AIoT identification information is matched:

a)	if the AIoT identification information is matched, the AIoT device shall indicate to the lower layer that the AIoT identification information is matched, and determine to respond to the AIoT paging; or

b)	otherwise, the AIoT device shall ignore the AIoT identification information, indicate to the lower layer that the filtering information is not matched, and determine not to respond to the AIoT paging.

Upon all the selected NG-RAN indicating the inventory procedure has been completed, the inventory procedure is considered as completed by the AIOTF as specified in 3GPP TS 23.369 [2].

NOTE:	How NG-RAN indicates the inventory procedure has been completed is defined in 3GPP TS 38.300 [7].

Editor's note:	The reference to TS 38.300 may be updated according to RAN2 progress.

Editor's note:	AIoT .device authentication of the network is FFS.

* * * Next Change * * * *

7.1.2	Inventory report

7.1.2.1	Message definition

The INVENTORY REPORT message is sent by the AIoT device to the AIOTF to respond to the AIoT paging.

See table 7.1.2.1-1.

Message type:	INVENTORY REPORT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.2.1-1: INVENTORY REPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Message type | Message type 7.2.2 | M | V | 1 |
|  | Security parameters | Security parameters 7.2.3 | M | V | n |
|  | RANDAIOT_d | RAND 7.2.10 | M | V | TBD |
|  | RESAIOT | Authentication response parameter 7.2.7 | M | V | TBD |
| TBD | AIoT device identity | AIoT device identity 7.2.6 | O | TLV | 15-n |

* * * Next Change * * * *

7.2.6	AIoT device identity

The purpose of the AIoT device identity information element is to indicate the AIoT device permanent identifier to the AIOTF if the privacy protection is not used as specified in 3GPP TS 33.369 [6].

The AIoT device identity is a type 4 information element with a minimum length of 15 octets.

The AIoT device identity information element is coded as shown in figure 7.2.6-1, figure 7.2.6-2, and table 7.2.6-1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AIoT device identity IEI |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of AIoT device identity contents |  |  |  |  |  |  |  |  | octet 2 |  |
| AIoT device permanent identifier |  |  |  |  |  |  |  |  | octet 3 octet n |  |

Figure 7.2.6-1: AIoT device identity information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID type |  |  |  |  |  |  |  |  | octet 3 |  |
| Domain information |  |  |  |  |  |  |  |  | octet 4* octet k* |  |
| Identification information |  |  |  |  |  |  |  |  | octet k+1 (NOTE) octet n |  |

Figure 7.2.6-2: AIoT device permanent identifier

NOTE:	The field is placed immediately after the last present preceding field.

Table 7.2.6-1: AIoT device identity information element

ID Type (octets 3):

The ID type field contains the value of "ID Type" as defined in clause 31.2 of 3GPP TS 23.003 [5])

Domain information (octets 4 to k): The ID type field contains the value of "Domain Information" as defined in clause 31.2 of 3GPP TS 23.003 [5]). In case of the "Domain Information" contains the network identifier (NID), the NID field is coded as figure 9.2.7-2 and table 9.2.7-2 in clause 9.2.7 of 3GPP TS 24.502 [x]. Identification information (octets k+1 to n): The ID type field contains the value of "Identification Information" as defined in clause 31.2 of 3GPP TS 23.003 [5].

* * * End of Changes * * * *