3GPP TSG-CT WG1 Meeting #157	C1-256110

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0831 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding the list of layer-2 IDs in the multi-hop path information of the direct link establishment procedure for multi-hop UE-to-network relay based on model B discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the reply LS S2-2508102 from SA2 and the corresponding stage-2 agreed CR S2-2508008, there is a need to add the list of Layer-2 IDs (associated with the corresponding User info IDs) in the Path information of the multi-hop U2N relay discovery and communication procedures based on model B discovery. This CR implements the corresponding changes in stage-3 spec for the 5G ProSe direct link establishment procedure for multi-hop UE-to-network relay based on model B discovery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the list of Layer-2 IDs and the associated User info IDs in the Path information of the multi-hop U2N relay discovery and communication procedures based on model B discovery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5G ProSe direct link establishment procedure for multi-hop UE-to-network relay based on model B discovery doesn't work properly due to missing Layer-2 IDs from the Multi-hop path information, and Stage-2 requirements are not implemented in stage-3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8b.2.2.2.3, 10.3.1.1, 10.3.1.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

##### 8b.2.2.2.3	5G ProSe direct link establishment procedure for multi-hop UE-to-network relay based on model B discovery

The 5G ProSe direct link establishment procedure for multi-hop UE-to-network relay based on model B discovery shall be initiated by the 5G ProSe multi-hop remote UE towards the first 5G ProSe intermediate UE-to-network relay UE, after the 5G ProSe multi-hop remote UE completes the multi-hop UE-to-network relay discovery over PC5 interface with model B.

The 5G ProSe direct link establishment procedure for multi-hop UE-to-network relay based on model B discovery reuses the procedure for 5G ProSe direct link establishment procedure for UE-to-network relay communication described in clause 7.2.2 but with the following modifications:

a)	for the first hop, i.e. the hop between the 5G ProSe multi-hop remote UE and the first 5G ProSe intermediate UE-to-network relay UE, the PROSE DIRECT LINK ESTABLISHMENT REQUEST message created by the 5G ProSe multi-hop remote UE:

1)	shall additionally include the Multi-hop U2N path info IE set to the list of the user info IDs and the corresponding layer-2 IDs of the 5G ProSe intermediate UE-to-network relay UE(s), if any, and the 5G ProSe multi-hop UE-to-network relay UE selected by the 5G ProSe multi-hop remote UE based on the path information provided to the 5G ProSe multi-hop remote UE during the multi-hop UE-to-network relay discovery over PC5 interface with model B. The user info IDs and the corresponding layer-2 IDs in the list shall be ordered starting from the first 5G ProSe intermediate UE-to-network relay UE till the 5G ProSe multi-hop UE-to-network relay UE; and

2)	may additionally include the End-to-end QoS flow descriptions IE to indicate the end to end QoS Information;

b)	Further, any 5G ProSe intermediate UE-to-network relay UE, after receiving and accepting the PROSE DIRECT LINK ESTABLISHMENT REQUEST message from its 5G ProSe child UE-to-network relay UE or the 5G ProSe multi-hop remote UE, shall use the Multi-hop path info IE to determine its 5G ProSe parent UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE. The 5G ProSe intermediate UE-to-network relay UE shall then initiate the 5G ProSe direct link establishment procedure towards its 5G ProSe parent UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE before proceeding with establishing the secured PC5 link with its 5G ProSe child UE-to-network relay UE or the 5G ProSe multi-hop remote UE. The PROSE DIRECT LINK ESTABLISHMENT REQUEST message created by the 5G ProSe intermediate UE-to-network relay UE towards its 5G ProSe multi-hop parent UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE:

1)	shall additionally include the Multi-hop U2N path info IE set to the same list of the user info IDs and the corresponding layer-2 IDs received from the 5G ProSe multi-hop child UE-to-network relay UE or the 5G ProSe multi-hop remote UE,

2)	may additionally include the End-to-end QoS flow descriptions IE to indicate the end to end QoS Information; and

c)	any 5G ProSe intermediate UE-to-network relay UE, after receiving and accepting the PROSE DIRECT LINK ESTABLISHMENT ACCEPT message from its 5G ProSe multi-hop parent UE-to-network relay UE or its 5G ProSe multi-hop UE-to-network relay UE, shall proceed with establishing the secured PC5 link with its 5G ProSe multi-hop child UE-to-network relay UE or the 5G ProSe multi-hop remote UE.

***** Next change *****

#### 10.3.1.1	Message definition

This message is sent by a UE to another peer UE to establish a direct link. See table 10.3.1.1.1.

Message type:	PROSE DIRECT LINK ESTABLISHMENT REQUEST

Significance:	dual

Direction:	UE to peer UE

Table 10.3.1.1.1: PROSE DIRECT LINK ESTABLISHMENT REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK ESTABLISHMENT REQUEST message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | Source user info | Application layer ID 11.3.4 | M | LV | 2-256 |
|  | UE security capabilities | UE security capabilities 11.3.11 | M | LV | 3-9 |
|  | UE PC5 unicast signalling security policy | UE PC5 unicast signalling security policy 11.3.12 | M | V | 1 |
| 4F | MIC | MIC 11.3.38 | O | TV | 5 |
| 7B | ProSe identifiers | ProSe identifier 11.3.3 | O | TLV-E | 21-65538 |
| 74 | Key establishment information container | Key establishment information container 11.3.9 | O | TLV-E | 4-65538 |
| 56 | Nonce_1 | Nonce 11.3.10 | O | TV | 17 |
| 5C | MSB of KNRP-sess ID | MSB of KNRP-sess ID 11.3.13 | O | TLV | 3 |
| 28 | Target user info | Application layer ID 11.3.4 | O | TLV | 3-257 |
| 58 | KNRP ID | KNRP ID 11.3.14 | O | TV | 5 |
| 54 | Relay service code | Relay service code 11.3.26 | O | TV | 4 |
| 7D | UE identity | 5GS mobile identity 11.3.30 | O | TLV-E | 4-n |
| 64 | User security key ID | User security key ID 11.3.32 | O | TLV | 3-n |
| 2A | HPLMN ID | PLMN ID 11.3.33 | O | TLV | 5 |
| 2B | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | O | TV | 2 |
| 65 | UE-to-UE relay UE user info | User info ID 2 11.3.39 | O | TLV | 8 |
| 2D | Target end UE layer-2 ID | Layer-2 ID 11.3.25 | O | TV | 4 |
| 29 | MSB of KSLP-sess ID | MSB of KSLP-sess ID 11.3.54 | O | TLV | 3 |
| 2C | KSLP ID | KSLP ID 11.3.55 | O | TLV | 6 |
| 8- | Relay indication | Relay indication 11.3.56 | O | TV | 1 |
| 10 | Multi-hop U2N path info | List of user info IDs and layer-2 IDs 11.3.48 | O | TLV | 12-257 |
| 9- |  | Hop limit | Hop limit 11.3.59 | O | TV | 1 |
| 11 |  | End-to-end QoS flow descriptions | PC5 QoS flow descriptions 11.3.5 | O | TLV-E | 6-65538 |

***** Next change *****

#### 10.3.1.19	Multi-hop U2N path info

The UE shall include this IE if:

- the UE is acting as a 5G ProSe multi-hop remote UE or a 5G ProSe intermediate UE-to-network relay UE to indicate the list of the user info IDs and layer-2 IDs of the 5G ProSe intermediate UE-to-network relay UE(s), if any, and the 5G ProSe multi-hop UE-to-network relay UE selected during the multi-hop UE-to-network relay discovery over PC5 interface with model B; or

- the UE is acting as a source 5G ProSe multi-hop end UE towards a 5G ProSe multi-hop UE-to-UE relay UE, or acting as a 5G ProSe multi-hop UE-to-UE relay UE towards another 5G ProSe multi-hop UE-to-UE relay UE, to indicate the list of the user info IDs and layer-2 IDs of the 5G ProSe multi-hop UE-to-UE relay UE(s) selected during the multi-hop layer-3 UE-to-UE relay discovery for Ethernet and Unstructured data unit type with model A or model B.

***** End of changes *****