3GPP TSG-CT WG1 Meeting #157	C1-256156

# Sophia-Antipolis, France, 13-17 October 2025

Source:	InterDigital Inc.

Title:	Updates to the Privacy of AIoT device identifiers

Spec:	3GPP TS 24.369 V1.0.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

<Introduction part (optional)>

2. Reason for Change

The change request introduces updates with respect to the privacy protections for AIoT device identifiers in alignment with SA3 TS 33.369 v19.0.0 on security aspects for Ambient IoT (AIoT) services for isolated private networks. This includes introducing Temporary Identifiers (T-IDs), removing permanent device identifiers from INVENTORY REPORT messages, and ensuring AIoT device identification is based on RESAIOT verification.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 V1.0.0

* * * First Change * * * *

### 4.2.6	Privacy of AIoT device identifiers

The AIoT NAS protocol shall support privacy protection of AIoT device identifiers by use of Temporary Identifiers (T-IDs) as specified in 3GPP TS 33.369 [6] during the inventory procedure. The AIoT device and the AIOTF shall generate the T-IDs using the long-term configured KAIOT_root in the AIoT device and ADM, and the AIoT device identifier shall not be included in AIoT NAS messages (e.g. INVENTORY REPORT). The AIOTF identifies the AIoT device by verifying the RESAIOT parameter included in the INVENTORY REPORT message. The generation of the T-ID is specified in 3GPP TS 33.369 Annex B.

* * * End of Changes * * * *