3GPP TSG-CT WG1 Meeting #157	C1-256236

Sophia-Antipolis, France 13 – 17 October 2025

Source:	Nokia

Title:	Pseudo-CR on update to Security aspects of AIoT NAS

Spec:	3GPP TS 24.369 V1.0.0

Agenda item:	19.70

Document for:	Approval

1. Introduction

Normative work progressed in SA3 on security aspects of AIoT feature. Therefore, this pCR proposes to update respective NAS security specific aspects along with removing the corresponding ENs.

2. Reason for Change

This pCR specifies updates to the NAS security aspects of Ambient IoT.

3. Conclusions

<Conclusion part (optional)>.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 V0.2.0.

* * * First Change * * * *

### 4.2.2	Authentication

Authentication of the AIoT device is done in the inventory procedure. A network generated RANDAIOT_n is provided by the AIOTF and received by the AIoT device in the paging request. The AIoT device which is target of the paging request (see clause 5.2.3), generate RANDAIOT_d,uses the RANDAIOT_n and its long-term KAIOT_root to derive a RESAIOT. and includes the RESAIOT and RANDAIOT_d in the INVENTORY REPORT message to the AIOTF. The network uses the RESAIOT and generate RANDAIOT_d to authenticate the AIoT device.

The requirements for the AIoT device to derive RESAIOT at receiving RANDAIOT_n from lower layers are described in 3GPP TS 33.369 [6].

The requirements for parameters and algorithms used at AIoT authentication are specified in 3GPP TS 33.369 [6].

* * * Next Change * * * *

### 4.2.3	Replay protection of AIoT NAS signalling messages

The AIoT device shall support replay protection of AIoT NAS messages between the AIoT device and the AIOTF.

* * * Next Change * * * *

### 4.2.4	Integrity protection of AIoT NAS signalling messages

The AIoT device shall support integrity protection of AIoT NAS messages between the AIoT device and the AIOTF. Details of integrity protection algorithm is described in annex D of 3GPP TS 33.501 [9].

* * * Next Change * * * *

### 4.2.5	Ciphering of AIoT NAS signalling messages

The use of ciphering in a network is an operator option subject to AIOTF configuration. When operation of the network without ciphering is configured, the AIOTF shall indicate the use of "null ciphering algorithm" NEA0.

All AIoT NAS messages, except the INVENTORY REPORT message, shall be sent ciphered. If the "null ciphering algorithm" NEA0 has been selected as a ciphering algorithm, the AIoT NAS message is regarded as ciphered.

NOTE : If confidentiality algorithm is not null-scheme, the AIOT NAS Command Request message is partly ciphered with the exception that the selected protection algorithms are in clear text.

The algorithm to calculate the ciphering of AIoT NAS messages is described in annex D.2 in 3GPP TS 33.501 [9]. In addition to the data that is to be ciphered, the constant BEARER, DIRECTION bit, COUNT, KEY and LENGTH are input to the ciphering algorithm. These parameters are described in 3GPP TS 33.369 [6].

* * * Next Change * * * *

### 4.2.6	Privacy of AIoT device identifiers

AIoT device identifier may be privacy protected during inventory procedure. Temporary ID, generated using KAIoT_root is used to protect privacy of the AIoT device identifier.

If privacy protection is not applied, AIoT device permanent identifier is used as device identification information in the inventory procedure.

* * * End of Changes * * * *