3GPP TSG-CT WG1 Meeting #157	C1-256315

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.283 | CR | 0002 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections to MCLoc TS 24.283 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMCLoc |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The CR propose a number of miscellaneous corrections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | First change: Corrects an implementation mistake of the agreed pCR C1-254710. Second change: Change “send” to “publish”, which is more correct in this operation. Third change: Changing MC service group to MC service group ID. Forth change: Adding a requirement about updating location reporting configuration when subscribing to location information. Fifth change: Removal of an Editor’s note. This Editor’s note was deleted in the TS in CT1#156 (C1-254713), however it was copied into this clause by mistake since the content was based on an earlier agreed version. Sixth change: Correction of resource URI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1, 6.2.2.8.1, 6.3.2.2.2, 6.3.2.3.2, 6.3.2.7.2, 7.1.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.2.2.1	Introduction

The service operation defined for LMS_Reporting service is shown in the table 6.2.2.1-1.

Table 6.2.2.1-1: Operations of the LMS_Reporting service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| RegistrationRequest | This service operation is used by the LMC to register in the LMS. | LMC |
| RegistrationUpdate | This service operation is used by the LMC to update a registration in the LMS. | LMC |
| RegistrationDelete | This service operation is used by the LMC to deregister in the LMS. | LMC |
| ConfigurationNotify | This service operation is a callback request used by the LMS to send location configuration to the LMC. | LMS |
| ConfigurationRequest | This service operation is used by the LMC to request the location configuration. | LMC |
| LocationReportRequest | This service operation is a callback request used by the LMS to send a location information request to the LMS. | LMS |
| LocationReportPublish | This service operation is used by the LMC to report the location information of the MC UE to the LMS. | LMC |

* * * Second Change * * * *

##### 6.2.2.8.1	General

This service operation is used by LMC to publish a location report to a given LMS.

If a location reporting trigger fires the LMC checks if the minimumReportInterval timer is running. If the timer is running the LMC waits until the timer expires. When the minimumReportInterval timer fires, the LMC:

1)	shall, if any of the reporting triggers are still true, send a location report as specified in clause 6.2.2.8.2.

If the LMC receives a LocationReportRequest as specified in clause 7.1.5.3, the LMC shall publish a location report as specified in clause 6.2.2.8.2.

* * * Third Change * * * *

##### 6.3.2.2.2	Client request location information to LMS using LocationRequest operation

For a client to request location information from the LMS, the client shall send an HTTP POST message to the LMS on the LMS_Information collection resource (i.e., Client location information request) to create the resource associated to the request for location information.

The request URI shall be set according to clause 7.2.1.

If the client is an LMC, the HTTP request shall include an Authorization header field with the "Bearer" authentication scheme set to all access tokens of the "bearer" token type as specified in IETF RFC 6750 [19].

NOTE:	The LocationRequest is relevant for all valid MC service IDs, hence the request includes all access tokens.

If the client is a network entity (e.g. an IWF, an MC service server or another LMS) then the client shall insert an X-3GPP-Asserted-Identity header field as specified in 3GPP TS 24.482 [10] clause A.2.1.3. The identity in the 3GPP-Asserted-Identity header field shall be set to the public service identity of the network entity.

Other HTTP headers shall be set according to clause 7.2.2.2.

The body of the HTTP POST message shall include the ClientOriginatingLocationRequest as specified in clause 7.2.6.2.2. The ClientOriginatingLocationRequest datatype shall include:

1)	a mclocClientId attribute identifying the instance of the LMC;

2)	a locationNotificationUri attribute specifying the URI of the LMC to which location information notification shall be sent to;

3)	at least one of the following:

a)	an array of MC service IDs of MC users for which location information are requested;

b)	an array of activated functional alias IDs used to determine MC service IDs of MC users for which location information are requested; and

c)	an array of MC service group IDs used to determine the MC service groups' affiliated MC service IDs of MC users for which location information are requested;

4)	may include a refresh attribute to indicate that the location information shall be updated immediately from the requested LMC;

5)	may include the requestedLocationHistory attribute including details of what location information history events that shall be included in the location report; and

6)	may include the requestedLocationStatus attribute indicating that the location history status should be included in the location report.

Editor's note: The usage of an LocationFilter attribute according to TS 24.379 is FFS.

Upon receiving the HTTP POST message from the LMC, the LMS shall:

1)	shall validate the access tokens as specified in 3GPP TS 33.180 [17];

2)	consider the MC service IDs derived from the bearer access tokens as the identities of the sender of the Locationrequest;

3)	if any access token is not validated successfully, the LMS shall reject the request with a HTTP 401 (Unauthorized) response and not continue with the remaining steps;

4)	if the array of MC service IDs, the array of functional alias IDs and the array of MC service group IDs are all empty then the LMS shall respond with a HTTP 403 (Forbidden) response including warning text set to "Invalid location request target client list" in a Warning header field, and shall not continue with the rest of the steps; and

5)	if the access token validation is successful, then:

a)	the LMS shall create a new resource with the client location information request as specified in clause 7.2.3.2.3; and

b)	shall send a HTTP 201 Created response message to the LMC;

6)	determine that the initiating user is authorized to request group based location information for each MC service group ID in the array provided in the attribute mcServiceGroupIds according to the following:

a)	The initiating user's location user configuration data document according to 3GPP TS 24.484 [12] shall include a <rule> element that includes:

i)	a <condition> element containing an <identity> element that matches the MC service group ID; and

ii)	an <action> element containing an <allow-request-location-of-groups> element with the value set to "true"; and

b)	if the item a) above is not fulfilled the MC service group ID shall be ignored in the remaining steps;

7)	subscribe to group dynamic data to for each MC service group ID that passes the authorization check in step 6) according to the following:

a)	if the MC service group is a MCPTT group send a SIP SUBSCRIBE request as specified in 3GPP TS 24.379 [9] clause 9.2.2.2.16 where the LMS takes the role of an MCPTT server serving the initiating user;

b)	if the MC service group is a MCVideo group send a SIP SUBSCRIBE request as specified in 3GPP TS 24.281 [7] clause 8.2.2.2.16 where the LMS takes the role of an MCVideo server serving the initiating user; and

c)	if the MC service group is a MCData group send a SIP SUBSCRIBE request as specified in 3GPP TS 24.282 [8] clause 8.3.3.8 where the LMS takes the role of an MCData server serving the initiating user;

8)	identify all MC service IDs for which location information is requested, based on the following content of the incoming request:

a)	the MC service IDs from the array provided in the attribute mcServiceIDs;

b)	the MC service IDs of the MC users that have activated the functional alias IDs from the array provided in the attribute functionalAliasIds

NOTE 1:	One way to do that is to use the functional alias status determination procedure defined in 3GPP TS 24.379 [9] clause 9A.2.1.3.

c)	the MC service IDs of each MC client that have affiliated to the MC service groups based on the notifications from the group dynamic data subscriptions;

9)	verify that the initiating user is authorized to request the location of the MC service IDs identified in step 6, according to local policies or to the following:

a)	The initiating user's location user configuration data document according to 3GPP TS 24.484 [12] shall include a <rule> element that include:

i)	a <condition> element containing an <identity> element that match each of identified MC service ID; and

ii)	an <action> element containing an <allow-request-location-of-users> element with the value set to "true"; and

b)	if the item i) above is not fulfilled the MC service ID shall be ignored in the remaining steps;

10)	If any MC Service IDs identified in the steps above are not owned by the LMS, the LMS shall, in accordance with the procedure in this clause, generate a new HTTP request to the LMS that owns those MC Service IDs;

NOTE 3:	How the LMS determines the HTTP request URI of the LMS that owns those MC Service IDs is out of scope of this specification.

11)	verify that each target user identified from step 7 is authorized to share the location information with the initiating MC user, according to local policies or to the following:

a)	Each target user's location user configuration data document according to 3GPP TS 24.484 [12] shall include a <rule> element that include:

i)	a <condition> element containing an <identity> element that match the identity of the initiating user MC service ID; and

ii)	an <action> element containing an <allow-to-share-location-information> element with the value set to "true"; and

b)	if the item i) above is not fulfilled the MC service ID shall be ignored in the remaining steps;

12)	if the refresh attribute is present and set to true, initiate LocationReportRequest operations (see clause 6.2.2.7.2) to each LMC identified in step 8).

The LMS shall create one or multiple location information notification and send the notifications to the requesting LMC according to the procedure in clause 6.3.2.4. Each location information notification message may contain location information from one or several MC UEs.

The location information notification message shall include the MC UE location details.

* * * Forth Change * * * *

##### 6.3.2.3.2	Client request subscription using LocationInfoSubscribe operation

To request a subscription for location information, the client shall send an HTTP POST message to the LMS on LMS_Information collection resource (i.e, Location information subscription) to create the resources associated with the subscription in the LMS.

The request URI shall be set according to clause 7.2.1.

If the client is an LMC, The HTTP request shall include an Authorization header field with the "Bearer" authentication scheme set to all access tokens of the "bearer" token type as specified in IETF RFC 6750 [19].

NOTE 1:	The LocationInfoSubscribe operation is relevant for all valid MC service IDs, hence the request includes all access tokens.

If the client is a network entity (e.g. an IWF, an MC service server or another LMS) then the client shall insert an X-3GPP-Asserted-Identity header field as specified in 3GPP TS 24.482 [10] clause A.2.1.3. The identity in the 3GPP-Asserted-Identity header field shall be set to the public service identity of the network entity.

Other HTTP headers shall be set according to clause 7.2.2.2.

The body of the HTTP POST message shall include the LocationSubscriptionRequest as specified in clause 7.2.6.2.5.

The LocationSubscriptionRequest data object shall include:

1)	a mclocClientId attribute identifying the instance of the LMC;

2)	a locationNotificationUri attribute specifying the URI of the LMC to which location notifications shall be sent to;

3)	at least one of the following:

a)	an array of MC service IDs of MC users for which location information are requested;

b)	an array of activated functional alias IDs used to determine MC service IDs of MC users for which location information are requested; or

c)	an array of MC service group IDs used to determine the MC service groups' affiliated MC service IDs of MC users for which location information are requested; and

4)	an expTime attribute containing the time and date for the subscription expiry;

Upon receiving the HTTP POST message from the LMC, the LMS shall:

1)	validate the access tokens as specified in 3GPP TS 33.180 [17];

2)	consider the MC service IDs derived from the bearer access tokens as the identities of the sender of the LMS_LocationInformation_request;

3)	if any access token is not validated successfully, the LMS shall reject the request with a HTTP 401 (Unauthorized) response and not continue with the remaining steps;

Editor's note:	How to handle the 3GPP-Asserted-Identity header is FFS.

4)	if the array of MC service IDs, the array of functional alias IDs and the array of MC service group IDs are all empty then the LMS shall respond with a HTTP 403 (Forbidden) response including warning text set to "Invalid subscription request target client list" in a Warning header field, and shall not continue with the rest of the steps;

5)	if the MC user's location user configuration data document does not include a <rule> element including a <action> element including a <allow-subscription-on-location-information> element set to true the LMS shall respond with a HTTP 403 (Forbidden) response including warning text set to "Unauthorized for location subscription" in a Warning header field, and shall not continue with the rest of the steps;

6)	generate a unique subscription identifier;

7)	create a new resource with the location subscription request as specified in clause 7.2.3.3.3; and

8)	send a HTTP 201 Created response message to the LMC and include the subscription identifier in the Location header;

The LMS shall identify all MC service IDs for which the subscription request applies according to steps 6-11 in the procedure for a one-time location information request as specified in clause 6.3.2.2.2.

The LMS shall create location information notifications and send the notifications to the requesting LMC according to the procedure in clause 6.3.2.4. Each location information notification message shall contain location information for the MC UEs of the identified MC service IDs.

The identified MC service IDs may change during the subscription time if the MC service IDs are determine based on affiliated group members or activated functional aliases.

The LMS may need to update the location reporting configuration for the subscribed LMCs according to clause 6.2.2.6.

The notifications shall be sent repeatedly until the subscription expires based on the attribute expTime or until an unsubscribe message is received.

* * * Fifth Change * * * *

##### 6.3.2.7.2	Client request location configuration change for another client from LMS using LocationInfoConfigRequest operation

For a client to request location configuration change for another client via the LMS, the client shall send an HTTP POST message to the LMS on the LMS_LocationInformation collection resource (i.e., Location Info Config request) to create the resource associated to the request for configuration change.

The request URI shall be set according to clause 7.2.2.

If the client is an LMC, the HTTP request shall include an Authorization header field with the "Bearer" authentication scheme set to all access tokens of the "bearer" token type as specified in IETF RFC 6750 [19].

NOTE:	The LocationInfoConfigRequest is relevant for all valid MC service IDs, hence the request includes all access tokens.

If the client is a network entity (e.g. an IWF, an MC service server or another LMS) then the client shall insert an X-3GPP-Asserted-Identity header field as specified in 3GPP TS 24.482 [10] clause A.2.1.3. The identity in the 3GPP-Asserted-Identity header field shall be set to the public service identity of the network entity.

Other HTTP headers shall be set according to clause 7.2.2.2.

The body of the HTTP POST message shall include the LocationConfiguration as specified in clause 7.1.6.2.3. The LocationConfiguration datatype shall include:

1)	shall include the triggerId attribute, with its already assigned value, if existent and valid, or with a newly generated unique value, if not, where defined for the sub-elements defining the trigger criterion;

2)	shall include the mcServiceId attribute set to the identity of the target user for the location information configuration update; and

3)	may include the locationHistory attribute set to true to indicate that location information shall be stored in the LMC during the state when the LMC is not able to report location information. If the attribute is not included or set to false location information may not be stored in the LMC if the LMC fails to report location information.

Upon receiving the HTTP POST message from the LMC, the LMS shall:

1)	shall validate the access tokens as specified in 3GPP TS 33.180 [17];

2)	consider the MC service IDs derived from the bearer access tokens as the identities of the sender of the LocationInfoConfigRequest;

3)	if any access token is not validated successfully, the LMS shall reject the request with a HTTP 401 (Unauthorized) response and not continue with the remaining steps;

4)	if the access token validation is successful, then:

a)	shall send a HTTP 200 OK response message to the LMC;

5)	identify MC service ID for which location information configuration update is requested, based on content of the attribute mcServiceId;

6)	verify that the initiating user is authorized to update the location information configuration of the MC service ID identified in step 5, according to local policies or to the following:

a)	The initiating user's location user configuration data document according to 3GPP TS 24.484 [12] shall include a <rule> element that include:

i)	a <condition> element containing an <identity> element that matches identified MC service ID; and

ii)	an <action> element containing an <allow-change-location-configuration-of-users> element with the value set to "true"; and

b)	if the item a) above is not fulfilled the MC service ID shall be ignored in the remaining steps;

7)	Location information configuration for the target MC Service Id shall be updated with information LMS received in LocationConfiguration element:

a)	Replace value of mcServiceId attribute in the LocationConfiguration element with the MC Service ID value derived from the bearer access tokens as the identity of the requestor of location information configuration update; and

8)	Upon successful completion of the previous steps, the LMS shall:

a)	Replace value of mcServiceId attribute in the LocationConfiguration element with the MC Service ID value derived from the bearer access tokens as the identity of the requestor of location information configuration update; and

b)	create ConfigurationNotify message using updated configuration information, also taking into account existing LMS policies.

LMS shall send ConfigurationNotify message to the target LMC according to the procedure in clause 6.2.2.5.

* * * Sixth Change * * * *

##### 7.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/lms-reporting//<apiVersion>/registrations/{registrationId}

This resource shall support the resource URI variables defined in table 7.1.3.3.2-1.

Table 7.1.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.1.1 |
| registrationId | string | The Identifier of a specific LMC registration. |

* * * End of Changes * * * *