3GPP TSG-CT WG1 Meeting #156	C1-256326

# Goteburg, SE, 25– 29 August 2025

Source:	Lenovo

Title:	Pseudo-CR on HFL training service API

Spec:	3GPP TS 24.560V1.1.0

Agenda item:	19.41

Document for:	Approval

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Stage 2 text exists for this feature in TS 23.482 and therefore stage 3 is needed.

The HFL training is a very iterative process and is performed over many training rounds and many AIMLE clients. Therefore, the AIMLE server is required to have not only the capability to subscribe to the HFL training event for the training schedules of the selected AIMLE clients, but also the capabilities to modify, update, and terminate the configuration of the training schedules of the selected AIMLE clients.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.560V1.1.0.

* * * First Change * * * *

## 6.2	Aimlec_HFLTraining API

### 6.2.1	Introduction

The Horizontal Federated Learning (HFL) training service shall use the Aimlec_HFLTraining API.

The API URI of the Aimlec_HFLTraining API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [5].

-	The <apiName> shall be "aimlec-hfl-trng".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.2.3.

### 6.2.2	Usage of HTTP and common API related aspects

The provisions of clause 5.2 of 3GPP TS 29.122 [5] shall apply for the Aimlec_HFLTraining API.

### 6.2.3	Resources

#### 6.2.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.2.3.1-1 depicts the resource URIs structure for the Aimlec_HFLTraining API.

Figure 6.2.3.1-1: Resource URI structure of the Aimlec_HFLTraining API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| HFL training subscription | /subscriptions | POST | Request the creation of the HFL training subscription resources. |
| Individual HFL training subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual HFL training subscription" resource. |
|  |  | PUT | Request the full replacement of an existing "Individual HFL training subscription" resource. |
|  |  | PATCH | Request the partial replacement of an existing "Individual HFL training subscription" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual HFL training subscription" resource. |

#### 6.2.3.2	Resource: HFL training subscription

##### 6.2.3.2.1	Description

This resource represents the AIMLE server subscription request to the HFL training event with the AIMLE client.

##### 6.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/aimlec-hfl-trng/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.2.3.2.2-1.

Table 6.2.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1 |

##### 6.2.3.2.3	Resource Standard Methods

6.2.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.2.3.2.3.1-1.

Table 6.2.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| HflTrngSub | M | 1 | Represents the " HFL training subscription" resource. |

Table 6.2.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| HflTrngSub | M | 1 | 201 Created | Successful case. The creation of an Individual HFL training subscription resource is confirmed, and a representation of that resource is returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
|  |  |  | 4XX (Client Error) | Failure causes are described in clause 6.2.7 |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [5] also apply. |  |  |  |  |

Table 6.2.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/aimlec-hfl-trng/<apiVersion>/subscriptions{subscriptionId} |

##### 6.2.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.2.3.3	Resource: Individual HFL training subscription

##### 6.2.3.3.1	Description

This resource represents an individual HFL training subscription resource at a given AIMLE client.

##### 6.2.3.3.2	Resource Definition

Resource URI: {apiRoot}/aimles-hfl-trng/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1 |
| subscriptioinId | string | Represents the identifier of an "Individual HFL training subscription" resource. |

##### 6.2.3.3.3	Resource Standard Methods

6.2.3.3.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.3.3.1-1.

Table 6.2.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.1-2 and the response data structures and response codes specified in table 6.2.3.3.3.1-3.

Table 6.2.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| HflTrngSub | M | 1 | 200 OK | Successful case. The requested "Individual HFL training subscription" resource is successfully retrieved and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE client. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE client. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
|  |  |  | 4XX (Client Error) | Failure causes are described in clause 6.2.7 |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [5] also apply. |  |  |  |  |

Table 6.2.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE client. |

Table 6.2.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE client. |

6.2.3.3.3.2	PUT

This method shall support the URI query parameters specified in table 6.2.3.3.3.2-1.

Table 6.2.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.2-2 and the response data structures and response codes specified in table 6.2.3.3.3.2-3.

Table 6.2.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| HflTrngSub | M | 1 | Represents the complete update of the representation of the existing "Individual HFL training subscription" resource. |

Table 6.2.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| HflTrngSub | M | 1 | 200 OK | Successful case. The requested "Individual HFL training subscription" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual HFL training subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE client. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE client. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
|  |  |  | 4XX (Client Error) | Failure causes are described in clause 6.2.7 |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [5] also apply. |  |  |  |  |

Table 6.2.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE client. |

Table 6.2.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE client. |

6.2.3.3.3.3	PATCH

This method shall support the URI query parameters specified in table 6.2.3.3.3.3-1.

Table 6.2.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.3-2 and the response data structures and response codes specified in table 6.2.3.3.3.3-3.

Table 6.2.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| HflTrngSubPatch | M | 1 | Represents the parameters to request the modification of the "Individual HFL training subscription" resource. |

Table 6.2.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| HflTrngSub | M | 1 | 200 OK | Successful case. The "Individual HFL training subscription" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual HFL training subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE client. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE client. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
|  |  |  | 4XX (Client Error) | Failure causes are described in clause 6.2.7 |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [5] also apply. |  |  |  |  |

Table 6.2.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE client. |

Table 6.2.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE client. |

6.2.3.3.3.4	DELETE

This method shall support the URI query parameters specified in table 6.2.3.3.3.4-1.

Table 6.2.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.4-2 and the response data structures and response codes specified in table 6.2.3.3.3.4-3.

Table 6.2.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual HFL training subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE client. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE client. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
|  |  |  | 4XX (Client Error) | Failure causes are described in clause 6.2.7 |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [5] also apply. |  |  |  |  |

Table 6.2.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE client. |

Table 6.2.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE client. |

##### 6.3.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 6.2.4	Custom Operations without associated resources

#### 6.2.4.1	Overview

There are no custom operations without associated resources defined for this resource in this release of the specification.

### 6.2.5	Notifications

#### 6.2.5.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [5].

Table 6.2.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| HFL training event notification | {notifUri} | POST | This service operation enables the AIMLE client to notify the AIMLE server on HFL training event. |

#### 6.2.5.2	HFL training event notification

##### 6.2.5.2.1	Description

The HFL training event notification is used by the AIMLE client to report HFL training event to AIMLE server that has subscribed to such an event notification.

##### 6.2.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.2.5.2.2-1.

Table 6.2.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | The notification URI is assigned within the Individual HFL training subscription and described by the HflTrngSub structure data as specified in clause 6.2.6.2.2. |

##### 6.2.5.2.3	Standard Methods

6.2.5.2.3.1	POST

This method shall support the request data structures specified in table 6.2.5.2.3.1-1 and the response data structures and response codes specified in table 6.2.5.2.3.1-1.

Table 6.2.5.2.3.1-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| HflTrngNotify | M | 1 | Represents the HFL training event notification. |

Table 6.2.5.2.3.1-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The HFL training event notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
|  |  |  | 5XX (Server Error) | Failure causes are described in clause 6.2.7 |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [5] also apply. |  |  |  |  |

### 6.2.6	Data Model

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Aimlec_HFLTraining API.

Table 6.2.6.1-1: Aimlec_HFLTraining API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| HflTrngSub | 6.2.6.2.2 | Represents the information for: - creation of an HFL training subscription resource; or - patial update or complete update of an existing HFL training subscription resource. |  |
| HflTrngSubPatch | 6.2.6.2.3 | Represents the information for partial uppdate of an existing HFL training subscription resource. |  |
| HflTrngNotify | 6.2.6.2.4 | Represent the information from an HFL training event notification. |  |

Table 6.2.6.1-2 specifies data types re-used by the Aimlec_HFLTraining API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the Aimlec_HFLTraining API.

Table 6.2.6.1-2: Aimlec_HFLTraining API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Uri | 3GPP TS 29.122 [5] | Represents a URI. |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [5] | Represents an offered scheduled communication time. |  |
| ReportingRequirements | 3GPP TS 29.549 [8] | Identifies the requirements for reporting. |  |
| SupportedFeatures | 3GPP TS 29.571 [9] | Used to negotiate the supported optional features of the API. |  |
| DateTime | 3GPP TS 29.122 [5] | Used to represent the date and time. |  |

#### 6.2.6.2	Structured data types

##### 6.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.2.6.2.2	Type: HflTrngSub

Table 6.2.6.2.2-1: Definition of type HflTrngSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | M | 1 | Identifies the URI towards which the notification should be delivered. |  |
| aimlMdlInfo | TBD | M | 1 | Identifies the information about the AIML model and the model parameters which are to be used for the HFL training. This attribute shall be provided in the HTTP request. |  |
| dataId | string | M | 1 | Identifies the dataset which is to be used for the HFL training. This attribute shall be provided in the HTTP request. |  |
| noDataSamp | integer | M | 1 | Identifies the required number of samples for a round of the HFL training. This attribute shall be provided in the HTTP request. |  |
| operSched | ScheduledCommunicationTime | O | 0..1 | Identifies the schedule for when the HFL training is to occur. |  |
| notifReqs | ReportingRequirements | O | 0..1 | Identifies the requirements for the notifications. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of optional features. This attribute shall be present only if feature negotiation in clause 6.2.8 needs to take place. This attribute shall be provided in the HTTP request and in the HTTP response of successful resource creation. |  |
| vaSrvId | string | M | 1 | Identifies the VAL service for the AIMLE HFL training operation. This parameter shall be supplied by the AMILE client in the HTTP response. |  |
| subID | string | O | 0..1 | Identifies the subscription. |  |

Editor's note:	Definition for aimlMdlInfo attribute is FFS and will be defined in clause 6.1.8 of 3GPP TS 29.482.

##### 6.2.6.2.3	Type: HflTrngSubPatch

Table 6.2.6.2.3-1: Definition of type HflTrngSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 0..1 | Identifies the URI towards which the notification should be delivered. |  |
| aimlMdlInfo | TBD | O | 0..1 | Identifies the information about the AIML model and the model parameters which are to be used for the HFL training. |  |
| dataId | string | O | 0..1 | Identifies the dataset which is to be used for the HFL training. |  |
| noDataSamp | integer | O | 0..1 | Identifies the required number of samples for a round of the HFL training. |  |
| operSched | ScheduledCommunicationTime | O | 0..1 | Identifies the schedule for when the HFL training is to occur. |  |
| notifReqs | ReportingRequirements | O | 0..1 | Identifies the requirements for the notifications. |  |

Editor's note:	Definition for aimlMdlInfo attribute is FFS and will be defined in clause 6.1.8 of 3GPP TS 29.482.

##### 6.2.6.2.4	Type: HflTrngNotify

Table 6.2.6.2.4-1: Definition of type HflTrngNotify

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| vaSrvId | string | M | 1 | Identifies the VAL service for the AIMLE HFL training operation. |  |
| hflTrngOut | TBD | M | 1 | Identifies the ML model parameters from the HFL training. |  |
| hflTrngErr | TBD | C | 0..1 | Identifies the encountered error list from the HFL training. This attribute shall be included is there is an encountered error list. |  |
| timestamp | DateTime | O | 0..1 | Identifies timeStamp for the notification. |  |

Editor's note:	Definitions for hflTrngOut and hflTrngErr attributes are FFS and will be defined in clause 6.1.8 of 3GPP TS 29.482.

#### 6.2.6.3	Simple data types and enumerations

##### 6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.2.6.3.2	Simple data types

The simple data types defined in table 6.2.6.3.2-1 shall be supported.

Table 6.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 6.2.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 6.2.6.5	Binary data

##### 6.2.6.5.1	Binary Data Types

Table 6.2.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 6.2.7	Error Handling

#### 6.2.7.1	General

For the Aimlec_HFLTraining API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [5]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [5] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [5].

In addition, the requirements in the following clauses are applicable for the Aimlec_HFLTraining API.

#### 6.2.7.2	Protocol Errors

No specific procedures for the Aimlec_HFLTraining API are specified.

#### 6.2.7.3	Application Errors

The application errors defined for the Aimlec_HFLTraining API are listed in Table 6.2.7.3-1.

Table 6.2.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

### 6.2.8	Feature negotiation

The optional features in table 6.2.8-1 are defined for the Aimlec_HFLTraining API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [5].

Table 6.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.2.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [5] shall apply for the Aimlec_HFLTraining API.

* * * End of Change * * * *