3GPP TSG-CT WG1 Meeting #157	C1-256375

Sophia Antipolis, France, 13-17 October 2025

Source:	Samsung

Title:	Pseudo-CR on AIoT Integrity check failure (Option 2)

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Decision

1. Introduction

Security aspects for Command procedures have been agreed in SA3.

2. Reason for Change

SA3 has agreed on the integrity protection aspects of Command messages. The corresponding behavior of the AIoT device on detecting an integrity check failure of the AIoT NAS message needs to be defined in CT1 specification.

3. Conclusions

Propose subsequent behavior on integrity check failure of Command messages in the AIoT device. In this option, it is proposed that the AIoT device ignores the message on integrity check failure, and does not respond to the message. Further behavior of the AIOTF when there is no response from the command request is subject to CT1 discussions.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

#### 5.3.2.3	Read command procedure accepted by the AIoT device

Upon receiving the READ COMMAND message, if the AIoT device supports read command and the integrity check is successful, the AIoT device:

a)	shall validate the offset in the Read offset IE and the length in the Read length IE;

NOTE:	The validation of the offset and the length is implementation-specific to the AIoT device.

b)	shall retrieve the AIoT data based on the Read offset IE and the Read length IE which are validated in a); and

c)	shall send a READ COMPLETE message to the AIOTF. In the READ COMPLETE message, the AIoT device shall include the AIoT data IE set to the retrieved AIoT data described in b).

* * * Next Change * * * *

#### 5.3.2.5	Read command procedure not accepted by AIoT device

Upon receiving a READ COMMAND message, if read command is not supported by the AIoT device, the AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

Editor's note:	Whether to use the READ COMMAND REJECT message to reject the READ COMMAND message when the read command not supported is FFS.

If the integrity check of the READ COMMAND message is not successful the AIoT device shall ignore the READ COMMAND message.

Editor's note:	The behavior of the AIOTF when there is no response to a Command request due to integrity check failure is FFS.

Upon receiving a READ COMMAND message, if the command type specific parameters (e.g. the offset to read application data from the AIoT Device user memory, the length of the application data to read.) is invalid, the AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

* * * Next Change * * * *

#### 5.3.3.3	Write command procedure accepted by AIoT device

Upon receiving the WRITE COMMAND message, if the AIoT device supports write command and the integrity check is successful, the AIoT device:

a)	shall validate the offset in the Write offset IE and the length of AIoT data contents in the AIoT data IE;

NOTE:	The validation of the offset and the length is implementation-specific to the AIoT device.

b)	shall write the AIoT data contents in the received AIoT data IE to the AIoT device's user memory at the offset which is validated in a); and

c)	shall send a WRITE COMPLETE message to the AIOTF.

* * * Next Change * * * *

#### 5.3.3.5	Write command procedure not accepted by AIoT device

Upon receiving a WRITE COMMAND message, if write command is not supported by the AIoT device, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

Editor's note:	Whether to use the WRITE COMMAND REJECT message to reject the WRITE COMMAND message when the write command not supported is FFS.

If the integrity check of the WRITE COMMAND message is not successful the AIoT device shall ignore the WRITE COMMAND message.

Editor's note:	The behavior of the AIOTF when there is no response to a Command request due to integrity check failure is FFS.

Upon receiving a WRITE COMMAND message, if the command type specific parameters (e.g. the offset where to write the application data, the length of the application data to write) is invalid, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

Upon receiving a WRITE COMMAND message, if the write command procedure cannot be implemented because the energy will be run out, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

* * * Next Change * * * *

#### 5.3.4.3	Permanent disable command procedure accepted by AIoT device

Upon receiving the PERMANENT DISABLE COMMAND message, if the integrity check of the message is successful the AIoT device shall send a PERMANENT DISABLE COMPLETE message to the AIOTF and disable the communication capability of the AIoT device.

* * * Next Change * * * *

#### 5.3.4.5	Permanent disable command procedure not accepted by AIoT device

Upon receiving the PERMANENT DISABLE COMMAND message if the integrity check of the message is not successful, the AIoT device shall ignore the PERMANENT DISABLE COMMAND message.

Editor's note:	The behavior of the AIOTF when there is no response to a Command request due to integrity check failure is FFS.

* * * End of Changes * * * *