3GPP TSG-CT WG1 Meeting #157	C1-256474

# Sophia Antipolis, France, 13-17 October 2025

Source:	Lenovo

Title:	Pseudo-CR on AIoT NAS Integrity protection

Spec:	3GPP TS 24.369 V1.0.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

This PCR introduces integrity protection for AIoT NAS command messages.

2. Reason for Change

The current version of TS 24.369 does not support integrity protection for AIoT NAS command messages. This PCR adds text to clause 4.2.4 to specify integrity protection for AIoT NAS command messages as defined in 3GPP TS 33.369 and 3GPP TS 22.369.

3. Conclusions

This PCR adds the necessary text to support integrity protection of AIoT NAS command messages.

4. Proposal

It is proposed to adopt the following changes to 3GPP TS 24.369 V1.0.0.

# *** First Change ***

### 4.2.4	Integrity protection of AIoT NAS signalling messages

Integrity protection of AIoT NAS signalling messages shall follow the security requirements and procedures specified in 3GPP TS 33.369.

For the AIoT device and the network, integrity protection is mandatory for all AIoT NAS command messages (see clause 5.3). Integrity protection shall not be applied to the INVENTORY REPORT message.

The AIoT NAS signalling message is integrity protected by calculating the message authentication code (MAC) using the 128-NIA2 algorithm using KCommand_int derived from KAIOTF as specified in 3GPP TS 33.369 [6]. The MAC shall be calculated over all octets of the AIoT NAS message, excluding the octets of the MAC field. The calculated MAC shall be included in the MAC field (see clause 7.2.3).

If integrity verification fails, the message shall be discarded without further processing.

# *** End of Changes ***