3GPP TSG-CT WG1 Meeting #157	C1-256610

Sophia-Antipolis, France 13-17 October 2025                                                   was C1-256178

# Source:	InterDigital Inc.

# Title:	New SID on Network Selection for 6G System

# Document for:	Approval

# Agenda Item:	20.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on PLMN Selection for 6G System

Acronym:	FS_6G_PLMNSel

Unique identifier:	TBD

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes | x | x |  | x |  |
| No |  |  |  |  |  |
| Don't know |  |  |  |  | x |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| x | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

## 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 1050110 | Study on 6G Use Cases and Service Requirements | Study on Stage 1 requirements for terminal or terminal-network interaction are relevant. |
| 1080057 | Study on Architecture for 6G System | Study on Stage 2 requirements on the architecture, procedures and NAS mechanism are relevant. |
| TBD | Study on Security for the 6G System | Study on Security aspect on NAS security and authentication/authorization procedures are relevant. |

# 3	Justification

PLMN and cell selection procedures in 5G (TS 23.122) have evolved with incremental additions since 2G, creating a broad and flexible framework. However, this evolution has also introduced a wide range of options that are not always well aligned, for example as UEs support an expanding set of RATs and policies (USIM-based, CP-SoR, network signalled). As networks move toward 6G, there is an opportunity to unify and streamline Network and cell selection mechanisms. The introduction of NTN access, localized services, slice-aware roaming, service-aware extensions, and AI-driven optimizations provide the chance to design an enhanced or restructured Network and cell selection framework that ensures predictable outcomes, seamless service continuity, and improved efficiency.

In 6G, Network and cell selection evolves from a legacy idle-mode procedure into a foundational enabler for both NAS connectivity and delivery of new operator services.

To realize 6G’s modular service vision, Network and cell selection must be enhanced to:

- Define a baseline Network and cell selection framework mandatory for all devices, ensuring strong and reliable support of NAS connectivity services.

- Study a service-aware selection designed to prioritize PLMNs based on the need for operator services, NTN integration, Localized services in 6G, network slice-aware extensions, and AI-driven optimization.

Therefore, it is proposed to study Network and cell selection for 6G in CT1. This study will identify key opportunities and develop solutions to unify, simplify, and modernize PLMN and cell selection, ensuring robust support for NAS connectivity services.

# 4	Objective

This CT1 study aims to identify the key issues and propose solutions for Network Selection for the 6G System to meet 6G service and architecture requirements. The key issues to be studied may include:

WT#1:	 Study the baseline framework for NAS connectivity services:

-	Whether and how to define a network and cell selection framework for all UE categories, incorporating the capabilities of new 6G RATs.

WT#2:	 Study service-aware extensions:

-	Whether and how to identify and select PLMNs offering 6G operator services (e.g., AI/ML inference, compute orchestration, sensing).

-	Whether and how to ensure service modularity so that optional operator services do not disturb the baseline PLMN and cell selection framework.

WT#3:	 Study slice-aware PLMN selection mechanisms:

-	Whether and how to extend PLMN and cell selection to support slice-aware roaming, including slice-aware SoR delivery, preferred PLMN handling per S-NSSAI, and UE behaviour for multi-slice conflicts and reselection.

The conclusions of this study will form the basis for the normative work and for any further study.

NOTE: The progress and results of 3GPP TR 22.870 (SA1 study) and TR 23.801-1 (SA2 study) shall be taken into account and may result in new work tasks/key issues.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal | TR 24.XXX | Study on PLMN Selection for 6G System | TSG#114 (Dec 2026) | TSG#XXX (TBD) |  |

NOTE: 	The timeline for the study will be decided at CT#110 (Dec 2025)

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

CT1

# 8	Aspects that involve other WGs

Potential NAS parameter storage aspects to be covered by CT6.

Potential RAN impact to be covered by RAN WGs.

# 9	Supporting Individual Members

Supporting IM name

InterDigital