3GPP TSG-CT WG1 Meeting #157	C1-256895

Sophia Antipolis, France, 13-17 October 2025

Source:	Huawei, HiSilicon, Lenovo, Samsung

Title:	Pseudo-CR on Integrity protection, general clause

Spec:	3GPP TS 24.369

Agenda item:	19.70

Document for:	Decision

1. Introduction

Work has started in CT1 on AmbientIoT-CT and a new TS for AIoT NAS between AIoT device and AIOTF is to be developed.

2. Reason for Change

In CT1#156 the general clause for security aspects was addressed, but the subclause in Integrity protection could not be concluded. It is proposed to add content aligned to level of specification of the other subclauses, and also take into account progress in SA3.

3. Conclusions

Following 33.369 v1.0.0 content of Integrity protection clause is proposed to be added

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0

* * * First Change * * *

### 4.2.4	Integrity protection of AIoT NAS signalling messages

For the AIoT device and AIOTF, integrity protected signalling is applied for AIoT NAS messages. Integrity protection of all AIoT NAS signalling messages, except the INVENTORY REPORT message, is mandatory and the responsibility of the AIoT NAS.

The algorithm to calculate the integrity protection information is described in annex D.3 in 3GPP TS 33.501 [9], and the integrity protection shall include octets 1 and 6 to n of the security protected AIoT NAS messages. The AIoT NAS signalling message is integrity protected by calculating the message authentication code (MAC) using KCommand_int derived from KAIOTF as specified in 3GPP TS 33.369 [6]. The calculated MAC shall be included in the MAC field (see clause 7.2.3).

* * * End of Changes * * *