3GPP TSG-CT1 Meeting #157	C1-256716

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.186 | CR | 0101 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS AS procedures for Standalone IMS DC sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 8.2.2 covers only the case in which the received INVITE request contains the combined SDP offers for bootstrap DC and Application DC and the user is authorized to use a standalone DC. In this TDoc, we propose to add IMS AS procedure for the case when the received INVITE request contains only bootstrap DC offer also. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: Added IMS AS procedures when the received INVITE request contains only bootstrap DC offer and the UE is authorized to use standalone data channel. Change 2: Clauses 9.3.2.1.6.4 and 9.3.3.1.6.3 are pointing to the UE procedures which is incorrect. Hence, corrected the clause numbers to refer to the detailed IMS AS procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect clause references; IMS AS procedures in standalone DC sessions when the received INVITE request contains only bootstrap DC offer will be unspecified in the overview. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Was C1-256707 was C1-256210 |  |  |  |  |  |  |  |  |

# *** First Change ***

### 8.2.2	Support of Standalone IMS Data Channel session

The application data channel can be established along with the bootstrap data channel. If the IMS AS receives the INVITE request to establish the bootstrap data channel and the application data channel, it shall check whether the user is authorized to use standalone data channel. If the user is authorized to use standalone data channel, the IMS AS shall notify the DCSF and reserve the data channel media as specified in clause 9.3.2.2.4.4 and 9.3.3.2.4.3.

If the IMS AS receives the INVITE request to establish only the bootstrap data channel, it shall check whether the user is authorized to use standalone data channel. If the user is authorized to use standalone data channel, the IMS AS shall notify the DCSF and reserve the data channel media as specified in clause 9.3.2.2.4.3A and 9.3.3.2.4.1A.

# *** End of Changes ***