3GPP TSG-CT WG1 Meeting #157	C1-256818

Sophia Antipolis, France, 13th – 17th October 2025

Source:	InterDigital

Title:	Defining update and delete operations for Split AIML operation pipeline service

Spec:	3GPP TS 24.560 v1.0.0

Agenda item:	19.41

Document for:	Agreement

1. Introduction

<Introduction part (optional)>

2. Reason for Change

The pCR adds Aimles_SplitOpPipeline_Update and Aimles_SplitOpPipeline_Delete operations definition to Split AIML operation pipeline service, as defined in stage-2 TS 23.482

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.560 v1.0.0.

* * * First Change * * * *

#### 5.6.2.4	Aimles_SplitOpPipeline_Update

##### 5.6.2.4.1	General

This service operation is used by AIMLE client to update instance(s) of split AI/ML operation pipeline or processing nodes from the AIMLE server.

##### 5.6.2.4.2	 AIML operation for pipeline update

To update instance(s) of split AIML operation pipeline or processing nodes, the AIMLE client shall send an HTTP PATCH request (for partial update) or HTTP PUT request (for full replacement) to AIMLE server as specified in clause TBD. The body of the PATCH or PUT message shall include the SplitOpPipelinePatch data structure as specified in clause TBD.

Upon receipt of the HTTP PATCH or PUT request from AIMLE client:

a)	the AIMLE server shall verify the identity of the AIMLE client and determine if the AIMLE client is authorized for the request; and

b)	if the AIMLE client:

1)	is not authorized to request split operation pipeline update, the AIMLE server shall respond to the AIMLE client with an appropriate error status code; or

2)	is authorized to request split operation pipeline update;

i)	has updated the instance, then in the response the AIMLE server shall include an indication of success, and may include an expiration time; or

ii)	has not updated the instance, then in the response the AIMLE server shall include an indication of failure and may include an appropriate error response as specified in clause TBD.

If the AIMLE server determined the received HTTP PATCH or PUT request needs to be redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the HTTP PATCH or PUT request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

#### 5.6.2.5	Aimles_SplitOpPipeline_Delete

##### 5.6.2.5.1	General

This service operation is used by AIMLE client to delete an instance of split AIML operation pipeline at the AIMLE server.

##### 5.6.2.5.2	 AIML operation for pipeline deletion

To delete an instance of split AIML operation pipeline, the AIMLE client shall send an HTTP DELETE request to AIMLE server as specified in clause TBD. The body of the POST message shall include the SplitOpPipelineDelReq data structure as specified in clause TBD.

Upon receipt of the HTTP DELETE request from AIMLE client:

a)	the AIMLE server shall verify the identity of the AIMLE client and determine if the AIMLE client is authorized for the request;

b)	if the AIMLE client:

1)	is not authorized to delete instance(s) of split AI/ML operation pipeline, the AIMLE server shall respond to the AIMLE client with an appropriate error status code; or

2)	is authorized to delete instance(s) of split AI/ML operation pipeline, the AIMLE server may delete the instance(s). If the AIML server:

i)	has deleted an instance of split AI/ML operation pipeline, then in the response the AIMLE server shall include an indication of success; or

ii)	has not deleted an instance of split AI/ML operation pipeline, then in the response the AIMLE server shall include an indication of failure and may include an appropriate error response as specified in clause TBD.

If the AIMLE server determined the received HTTP DELETE request needs to be redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the HTTP DELETE request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

* * * End of Change * * * *