3GPP TSG CT WG3 Meeting #143	C3-254039

# Sophia-Antipolis, FR, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.548 | CR | 0059 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to the definition of the UE-to-UE policy of the Multi-modal SEALDD policy |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | For the UEToUEPolicy data structure, since the ProximityThresholds and QoSThresholds data structures used to encode its attributes may include a minimum, maximum and average threshold value, there is a need to separate the thresholds for entering the UE-to-UE direct communication mode from the ones for leaving the UE-to-UE direct communication mode, so as to, avoid confusion and different interpretations and allow for implementations that desire to avoid too frequent transitions between entering and leaving the UE-to-UE direct communication mode. Otherwise, there could be several interpretations of how the minimum/maximum/average threshold values for both entering and leaving, e.g., whether the distance between the UEs has to be above/below the threshold minimum distance value for entering and below/above the threshold maximum distance value for leaving. If we have separate thresholds, then it is straightforward how to interpret the threshold minimum/maximum/average separately for entering and leaving. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Address the above-detailed necessary updates/corrections to complete the definition of the UE-to-UE Policy. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - The above-detailed necessary updates/corrections are not addressed and the UE-to-UE Policy definition is not complete in stage 3. |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.6.2.16, A.6 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR introduces a backwards compatible new feature to the OpenAPI descriptions of the following APIs: - TS29548_SDD_PolicyConfiguration.yaml |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

##### 6.5.6.2.16	Type: UEToUEPolicy

Table 6.5.6.2.16-1: Definition of type UEToUEPolicy

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| entProxThresh | ProximityThresholds | C | 0..1 | Contains the proximity threshold(s) for entering the UE-to-UE direct communication mode, i.e., the threshold proximity between the UEs used to determine that the UE-to-UE direct communication mode should be entered (i.e., used). (NOTE 1, NOTE 2) |  |
| leavProxThresh | ProximityThresholds | C | 0..1 | Contains the proximity threshold(s) for leaving the UE-to-UE direct communication mode, i.e., the threshold proximity between the UEs used to determine that the UE-to-UE direct communication mode should be left (i.e., not used anymore). (NOTE 1, NOTE 3) |  |
| entQoSThresh | QoSThresholds | C | 0..1 | Contains the QoS threshold(s) for entering the UE-to-UE direct communication mode, i.e., the threshold QoS value(s) used to determine that the UE-to-UE direct communication mode should be entered (i.e., used). (NOTE 1, NOTE 2) |  |
| leavQoSThresh | QoSThresholds | C | 0..1 | Contains the QoS threshold(s) for leaving the UE-to-UE direct communication mode, i.e., the threshold QoS value(s) used to determine that the UE-to-UE direct communication mode should be left (i.e., not used anymore). (NOTE 1, NOTE 3) |  |
| entQoEThresh | QoEThresholds | C | 0..1 | Contains the QoE threshold(s) for entering the UE-to-UE direct communication mode, i.e., the threshold QoE value(s) used to determine that the UE-to-UE direct communication mode should be entered (i.e., used). (NOTE 1) |  |
| leavQoEThresh | QoEThresholds | C | 0..1 | Contains the QoE threshold(s) for leaving the UE-to-UE direct communication mode, i.e., the threshold QoE value(s) used to determine that the UE-to-UE direct communication mode should be left (i.e., not used anymore). (NOTE 1) |  |
| NOTE 1: At least one of these attributes shall be present. NOTE 2: When these attributes contain both a minimum and a maximum threshold value, then the minimum threshold represents the threshold below which the UE-to-UE direct communication mode should be entered (i.e., used), and the maximum threshold represents the threshold above which the UE-to-UE direct communication mode should be entered (i.e., used). NOTE 3: When these attributes contain both a minimum and a maximum threshold value, then the minimum threshold represents the threshold above which the UE-to-UE direct communication mode should be left (i.e., not used anymore), and the maximum threshold represents the threshold below which the UE-to-UE direct communication mode should be left (i.e., not used anymore). |  |  |  |  |  |

* * * * Next changes * * * *

# A.6	SDD_PolicyConfiguration API

openapi: 3.0.0

info:

title: SEALDD Server Policy Configuration Service

version: 1.1.0-alpha.3

description: |

SEALDD Server Policy Configuration Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.548 V19.4.0; Service Enabler Architecture Layer for Verticals (SEAL);

SEAL Data Delivery (SEALDD) Server Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.548/

servers:

- url: '{apiRoot}/sdd-pc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/configurations:

post:

summary: Request the creation of a Policy Configuration.

operationId: CreatePolicyConfig

tags:

- Policy Configurations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

responses:

'201':

description: >

Created. The Policy Configuration is successfully created and a representation of

the created Individual Policy Configuration resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

headers:

Location:

description: >

Contains the URI of the created Individual Policy Configuration resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/configurations/{configId}:

parameters:

- name: configId

in: path

description: >

Represents the identifier of the Individual Policy Configuration resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Policy Configuration resource.

operationId: GetIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

responses:

'200':

description: >

OK. The requested Individual Policy Configuration resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Policy Configuration resource.

operationId: UpdateIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

responses:

'200':

description: >

OK. The Individual Policy Configuration resource is successfully updated and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

'204':

description: >

No Content. The Individual Policy Configuration resource is successfully updated

and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Policy Configuration resource.

operationId: ModifyIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/PolicyConfigPatch'

responses:

'200':

description: >

OK. The Individual Policy Configuration resource is successfully modified and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

'204':

description: >

No Content. The Individual Policy Configuration resource is successfully modified and

no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Policy Configuration resource.

operationId: DeleteIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

responses:

'204':

description: >

No Content. The Individual Policy Configuration resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

PolicyConfig:

description: >

Represents a SEALDD Policy Configuration.

type: object

properties:

appTrafficIds:

type: array

items:

type: string

minItems: 1

valTargetId:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

sliceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

sealddPol:

$ref: '#/components/schemas/SealddPolicy'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTimeRo'

multimodsddPol:

$ref: '#/components/schemas/MultiModalSealddPolicy'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- appTrafficIds

anyOf:

- required: [sealddPol]

- required: [multimodsddPol]

PolicyConfigPatch:

description: >

Represents the requested modifications to a SEALDD Policy Configuration.

type: object

properties:

sealddPol:

$ref: '#/components/schemas/SealddPolicy'

multimodsddPol:

$ref: '#/components/schemas/MultiModalSealddPolicy'

SealddPolicy:

description: >

Represents a SEALDD Policy.

type: object

properties:

qualGuarPol:

$ref: '#/components/schemas/QualGuarPolicy'

qualOptPol:

$ref: '#/components/schemas/QualGuarPolicy'

bdwCtrlSets:

type: array

items:

$ref: '#/components/schemas/BdwCtrlPolicy'

minItems: 1

geoPol:

type: array

items:

$ref: '#/components/schemas/GeofencingPolicy'

minItems: 1

tempPol:

$ref: '#/components/schemas/TempPolicy'

non3gppAccMeasPol:

$ref: '#/components/schemas/Non3gppAccessMeasPol'

anyOf:

- required: [qualGuarPol]

- required: [qualOptPol]

- required: [bdwCtrlSets]

- required: [geoPol]

- required: [tempPol]

- required: [non3gppAccMeasPol]

QualGuarPolicy:

description: >

Represents the quality guarantee policy.

type: object

properties:

thresholds:

$ref: '#/components/schemas/QualGuarThresh'

required:

- thresholds

QualGuarThresh:

description: >

Represents the quality guarantee related thresholds.

type: object

properties:

measId:

type: array

items:

$ref: 'TS29548_SDD_TransmissionQualityMeasurement.yaml#/components/schemas/MeasurementId'

minItems: 1

measThesh:

$ref: 'TS29548_SDD_TransmissionQualityMeasurement.yaml#/components/schemas/TransQualMeasCriteria'

required:

- measId

- measThesh

GeofencingPolicy:

description: Represents a geofencing policy.

type: object

properties:

valServiceId:

type: string

area:

$ref: '#/components/schemas/GeofencingArea'

action:

$ref: '#/components/schemas/GeoPolAction'

required:

- valServiceId

- area

- action

GeofencingArea:

description: Represents a geofencing area.

type: object

properties:

geoAreas:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

minItems: 1

anyOf:

- required: [geoAreas]

MultiModalSealddPolicy:

description: >

Represents a Multi-modal SEALDD Policy.

type: object

properties:

syncPol:

type: array

items:

$ref: '#/components/schemas/SyncPolicy'

minItems: 1

description: >

Contains the synchronization policy.

The key of the map shall be set to the multi-modal flow type provided within the

"flowType" attribute of the corresponding map value encoded using the SyncPolicy data

structure.

alignmentPol:

$ref: '#/components/schemas/AlignmentPolicy'

ueToUePol:

$ref: '#/components/schemas/UEToUEPolicy'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTimeRo'

anyOf:

- required: [syncPol]

- required: [alignmentPol]

- required: [ueToUePol]

TempPolicy:

description: >

Represents a temporal policy.

type: object

properties:

schedule:

type: array

items:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

timeWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

oneOf:

- required: [schedule]

- required: [timeWindow]

Non3gppAccessMeasPol:

description: >

Represents the non-3GPP access measurement policy.

type: object

properties:

accessIds:

type: array

items:

type: string

minItems: 1

thresholds:

type: array

items:

$ref: '#/components/schemas/SignalStrengthThreshold'

minItems: 1

thrHandling:

$ref: 'TS29549_SS_NetworkResourceMonitoring.yaml#/components/schemas/ThresholdHandlingMode'

timeThr:

$ref: '#/components/schemas/TempPolicy'

anyOf:

- required: [accessIds]

- required: [thresholds]

- required: [timeThr]

SignalStrength:

description: >

Represents the signal strength value.

type: object

properties:

rssi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

anyOf:

- required: [rssi]

SignalStrengthThreshold:

description: >

Represents the signal strength threshold.

type: object

properties:

value:

$ref: '#/components/schemas/SignalStrength'

direction:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/MatchingDirection'

required:

- value

- direction

SyncPolicy:

description: >

Represents the synchronization policy.

type: object

properties:

syncThreshold:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/DurationMilliSec'

required:

- syncThreshold

AlignmentPolicy:

description: >

Represents the multi-modal flows alignment policy.

type: object

properties:

multiModalId:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/MultiModalId'

maxAlignTime:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/DurationMilliSec'

anyOf:

- required: [multiModalId]

- required: [maxAlignTime]

UEToUEPolicy:

description: >

Represents the multi-modal UE-to-UE policy.

type: object

properties:

entProxThresh:

$ref: '#/components/schemas/ProximityThresholds'

leavProxThresh:

$ref: '#/components/schemas/ProximityThresholds'

entQoSThresh:

$ref: '#/components/schemas/QoSThresholds'

leavQoSThresh:

$ref: '#/components/schemas/QoSThresholds'

entQQoEThresh:

$ref: '#/components/schemas/QoEThresholds'

leavQoEThresh:

$ref: '#/components/schemas/QoEThresholds'

anyOf:

- required: [entProxThresh]

- required: [leavProxThresh]

- required: [entQoSThresh]

- required: [leavQoSThresh]

- required: [entQoEThresh]

- required: [leavQoEThresh]

ProximityThresholds:

description: >

Represents the proximity thresholds for entering/leaving the UE-to-UE direct communication

mode.

type: object

properties:

minUeToUedist:

type: number

format: double

minimum: 0

avgUeToUedist:

type: number

format: double

minimum: 0

maxUeToUedist:

type: number

format: double

minimum: 0

anyOf:

- required: [minUeToUedist]

- required: [avgUeToUedist]

- required: [maxUeToUedist]

QoSThresholds:

description: >

Represents the QoS thresholds for entering/leaving the UE-to-UE direct communication mode.

type: object

properties:

minLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avgLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

maxLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

avgBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

minPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

avgPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

maxPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

minPackErrRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

avgPackErrRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

maxPackErrRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

minJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

avgJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

maxJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

anyOf:

- required: [minLatency]

- required: [avgLatency]

- required: [maxLatency]

- required: [minBitRate]

- required: [avgBitRate]

- required: [maxBitRate]

- required: [minPackLossRate]

- required: [avgPackLossRate]

- required: [maxPackLossRate]

- required: [minPackErrRate]

- required: [avgPackErrRate]

- required: [maxPackErrRate]

- required: [minJitter]

- required: [avgJitter]

- required: [maxJitter]

QoEThresholds:

description: >

Represents the QoE thresholds for entering/leaving the UE-to-UE direct communication mode.

type: object

properties:

mos:

type: integer

minimum: 1

maximum: 5

anyOf:

- required: [mos]

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

BdwCtrlPolicy:

anyOf:

- type: string

enum:

- REALLOCATE_DL

- REALLOCATE_UL

- NOT_REALLOCATE_DL

- NOT_REALLOCATE_UL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the bandwidth control policy.

Possible values are:

- REALLOCATE_DL: Indicates that the bandwidth control action is to reallocate the

bandwidth limit between different VAL users for DL traffic.

- REALLOCATE_UL: Indicates that the bandwidth control action is to reallocate the

bandwidth limit between different VAL users for UL traffic.

- NOT_REALLOCATE_DL: Indicates that the bandwidth control action is to not reallocate

the bandwidth limit between different VAL users for DL traffic.

- NOT_REALLOCATE_UL: Indicates that the bandwidth control action is to not reallocate

the bandwidth limit between different VAL users for UL traffic.

GeoPolAction:

anyOf:

- type: string

enum:

- ALLOW

- BLOCK

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the geofencing policy action.

Possible values are:

- ALLOW: Indicates that the geofencing policy is to allow.

- BLOCK: Indicates that the geofencing policy is to block.

# Data types describing alternative data types or combinations of data types:

#

* * * * End of changes * * * *