3GPP TSG-CT WG3 Meeting #143	C3-254112

# Sophia-Antipolis, France, 13 - 17 October 2025

Source:	Nokia

Title:	Spatial Anchor Management Pose handling

Spec:	3GPP TS 29.437 and 1.1.0

Agenda item:	19.42

Document for:	Approval

1. Introduction

This pCR proposes to Spatial Anchor management Pose handling and its corresponding data types.

2. Reason for Change

For Spatial Anchor management, the pose data type is introduced as per TR 23.700-21 (Metaverse_App study) mentions as follows:

7.1.3.2 Information flows Table 7.1.3.2-1 shows the request sent by a MMEC to a MMES for the discovery of spatial anchor(s). Table 7.1.3.2-1: Spatial anchor discovery request Information element; Status; Description. Requestor identity; M; The identity of the requestor (e.g., MMEC, UE, VAL server). Requestor security credentials; M; The security credentials of the requestor.. Location of interest; O; Location of spatial anchors the requestor is interest in.. Spatial anchor discovery filters; O; Set of characteristics to determine matching spatial anchors (e.g., spatial anchor location, VAL client type, universal filter).. Current UE location; O; Current location of the UE. Current UE pose; O; Current pose of the user (as defined in clause 6.2.2 of 3GPP TS 26.119 [11]). SA Group ID; O; Indicates the identifier of the requested group of SA.

3. Conclusions

Pose Datatype and the corresponding update in the spatial anchor management update is done.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.437 v1.1.0.

# * * * First Change * * * *

##### 6.1.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.1.6.1-1 specifies the data types defined for the SS_SAnManagement API.

Table 6.1.1.6.1-1: SS_SAnManagement API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AllowedEntity | 6.1.1.6.2.9 | Represents the allowed application entity. |  |
| AllowedRules | 6.1.1.6.2.8 | Represents the allowed entities rules. |  |
| AnchorEvent | 6.1.1.6.3.4 | Represents the spatial anchor event type. |  |
| AnchorsEventsSub | 6.1.1.6.2.16 | Represents the spatial anchor event subscription. |  |
| AnchorFilter | 6.1.1.6.2.21 | Represents the spatial anchors specific filters. |  |
| AnchorManagementEvent | 6.1.1.6.3.5 | Represents the spatial anchor management events. |  |
| AnchorsNotification | 6.1.1.6.2.14 | Represents the Spatial Anchors notification. |  |
| Angle | 6.1.1.6.3.2 | Represents the angle value |  |
| DetectedEvent | 6.1.1.6.3.6 | Represents the triggered spatial anchor event for the notification. |  |
| DiscoveryFilter | 6.1.1.6.2.18 | Represents the discovery event filter. |  |
| EntityType | 6.1.1.6.3.3 | Represents the type of entity. |  |
| EventFilter | 6.1.1.6.2.17 | Represents the spatial anchors event subscription filters. |  |
| Fov | 6.1.1.6.2.27 | Represents the field of view |  |
| LocationInfo | 6.1.1.6.2.23 | Represents the Localization information for the Spatial Anchor. |  |
| Orientation | 6.1.1.6.2.26 | Represents the Orientation |  |
| Pose | 6.1.1.6.2.25 | Represents the current pose. |  |
| PositionInfo | 6.1.1.6.2.10 | Represents the positioning information of the Spatial Anchor and the related Spatial Map. |  |
| RangeFilter | 6.1.1.6.2.20 | Represents the range event filter. |  |
| ServConnectInfo | 6.1.1.6.2.5 | Represents the connectivity information related to the application service. |  |
| SpatialAnchor | 6.1.1.6.2.6 | Represents the Spatial Anchor profile information. |  |
| SpatialAnchorRm | 6.1.1.6.2.7 | Represents the same as the SpatialAnchor data structure but with the OpenAPI "nullable" property set to "true". |  |
| SpatialAnchorId | 6.1.1.6.3.2 | Represents the identifier of a Spatial Anchors List. |  |
| SpatialAnchorsList | 6.1.1.6.2.2 | Represents the Spatial Anchors List. |  |
| SpatialAnchorsListPatch | 6.1.1.6.2.3 | Represents the requested modifications to a Spatial Anchors List. |  |
| SpatialAnchorMapPosition | 6.1.1.6.2.22 | Represents the position information of a spatial anchor in a given spatial map. |  |
| SpatialAnchorsNotif | 6.1.1.6.2.13 | Represents the spatial anchor event notification message. |  |
| SpatialAnchorProfile | 6.1.1.6.2.24 | Represents the spatial anchor profile information. |  |
| SpatialAnchorsSub | 6.1.1.6.2.11 | Represents a Spatial Anchor subscription. |  |
| SpatialAnchorsSubPatch | 6.1.1.6.2.12 | Represents the requested modifications to a Spatial Anchor Subscription. |  |
| UEAnchorInterest | 6.1.1.6.2.15 | Represents the UE information with spatial anchor interest. |  |
| UeFilter | 6.1.1.6.2.19 | Represent the event filter for UE with spatial anchor interest. event |  |
| VALServiceInfo | 6.1.1.6.2.4 | Represents the VAL service information. |  |

Table 6.1.1.6.1-2 specifies data types re-used by the SS_SAnManagement API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SS_SAnManagement API.

Table 6.1.1.6.1-2: SS_SAnManagement API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Bytes | 3GPP TS 29.122 [2] | Represents a string formatted as a sequence of bytes. |  |
| DateTime | 3GPP TS 29.122 [2] | Represents a date and a time. |  |
| Dnai | 3GPP TS 29.571 [15] | Represents a DNAI. |  |
| Dnn | 3GPP TS 29.571 [15] | Represents a DNN. |  |
| EndPoint | 3GPP TS 29.558 [18] | Represents the end point information. |  |
| Float | 3GPP TS 29.571 [15] | Represents a float. |  |
| Local3dPointUncertaintyEllipsoid | 3GPP TS 29.572 [16] | Represents a 3D point. |  |
| LocationInfo | 3GPP TS 29.122 [2] | Represents the location information and direction. |  |
| ReportingInformation | 3GPP TS 29.523 [19] | Represents the reporting requirements information. |  |
| ServiceArea | 3GPP TS 29.558 [18] | Represents the service area information. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Uint32 | 3GPP TS 29.571 [15] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.122 [2] | Represent a URI. |  |

# * * * * Next changes * * * *

6.1.1.6.2.20	Type: RangeFilter

Table 6.1.1.6.2.20-1: Definition of type RangeFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| perSrch | boolean | O | 0..1 | Contains the persistent search preference. - "true" indicates to enable persistent search. - "false" indicates not to enable persistent search. - The default value is "false" if omitted. |  |
| usrLoc | LocationInfo | O | 0..1 | Current location and direction of the user. |  |
| pose | Pose | O | 0..1 | Contains the current pose of the user. |  |
| appRng | Float | M | 1 | Contains the range within which the spatial anchors are monitored to be in range or out of range of the subscriber’s field of view, expressed in centimetres. Minimum = 0 |  |

# * * * * Next changes * * * *

6.1.1.6.2.25	Type: Pose

Table 6.1.1.6.2.25-1: Definition of type Pose

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| orientation | Orientation | M | 1 | Representation the orientation of the Pose. |  |
| position | Local3dPointUncertaintyEllipsoid | O | 0..1 | Represents the position of the pose relative to the spatial Map. |  |
| fov | Fov | O | 0..1 | Represents the four sides of the field of view used for the projection of the corresponding Spatial map. |  |

# * * * * Next changes * * * *

6.1.1.6.2.26	Type: Orientation

Table 6.1.1.6.2.26-1: Definition of type Orientation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| x | Uint32 | M | 1 | Represents the X coordinate of the quaternion. |  |
| y | Uint32 | M | 1 | Represents the Y coordinate of the quaternion. |  |
| z | Uint32 | M | 1 | Represents the Z coordinate of the quaternion. |  |
| w | Uint32 | M | 1 | Represents the W coordinate of the quaternion. |  |

# * * * * Next changes * * * *

6.1.1.6.2.27	Type: Fov

Table 6.1.1.6.2.27-1: Definition of type Fov

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| angleLeft | Angle | M | 1 | Represents the angle in radians of the left side of the field of view. For a symmetric field of view this value is negative. |  |
| angleRight | Angle | M | 1 | Represents the angle in radians of the right side of the field of view. |  |
| angleUp | Angle | M | 1 | Represents the angle in radians of the top part of the field of view. |  |
| angleDown | Angle | M | 1 | Represents the angle in radians of the bottom part of the field of view. For a symmetric field of view this value is negative. |  |

# * * * * Next changes * * * *

6.1.1.6.3.2	Simple data types

The simple data types defined in table 6.1.1.6.3.2-1 shall be supported.

Table 6.1.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| SpatialAnchorId | Uint32 | Represents the unique identifier of a Spatial Anchor. |  |
| Angle | integer | Represents the value of the angles including negative one. Minimum = -360 Maximum = 360 |  |

# * * * * Next changes * * * *

# A.2	SS_SAnManagement API

openapi: 3.0.0

info:

title: SAn Server Spatial Anchors Management Service

version: 1.0.0-alpha.4

description: |

API for Spatial Anchor Management Service.

©2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.437 V1.1.0; Service Enabler Architecture Layer for Verticals (SEAL);

Metaverse Enablement Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.437/

servers:

- url: "{apiRoot}/san_sanm/v1"

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/anchors-lists:

post:

summary: Create a new Spatial Anchors List.

operationId: CreateSpatialAnchorsList

tags:

- Spatial Anchors List (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsList'

responses:

'201':

description: >

Created. The Spatial Anchors List is successfully created and a representation of the

created Individual Spatial Anchors List resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsList'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/anchors-lists/{anchorsListId}:

parameters:

- name: anchorsListId

in: path

description: Represents the identifier of the Individual Spatial Anchors List resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Spatial Anchors List.

operationId: GetIndSpatialAnchorsList

tags:

- Individual Spatial Anchors List (Document)

responses:

'200':

description: >

OK. The requested Individual Spatial Anchors List resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsList'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual Spatial Anchors list.

operationId: UpdateIndSpatialAnchorsList

tags:

- Individual Spatial Anchors List (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsList'

responses:

'200':

description: >

OK. The Individual Spatial Anchors List resource is successfully updated and

a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsList'

'204':

description: >

No Content. The Individual Spatial Anchors List resource is successfully updated and no

content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an existing Individual Spatial Anchors List.

operationId: ModifyIndSpatialAnchorsList

tags:

- Individual Spatial Anchors List (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsListPatch'

responses:

'200':

description: >

OK. The Individual Spatial Anchors List resource is successfully modified and

a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsList'

'204':

description: >

No Content. The Individual Spatial Anchors List resource is successfully modified and no

content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual Spatial Anchors List.

operationId: DeleteIndSpatialAnchorsList

tags:

- Individual Spatial Anchors List

responses:

'204':

description: >

No Content. The 'Individual Spatial Anchors List' resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions:

post:

summary: Create a new Spatial Anchors Subscription.

operationId: CreateSpatialAnchorsSubsc

tags:

- Spatial Anchors Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsSub'

responses:

'201':

description: >

Created. The Spatial Anchors Subscription is successfully created and a

representation of the created Individual Spatial Anchors Subscription

resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

SpatialAnchorsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsNotif'

responses:

'204':

description: >

No Content. The Spatial Anchors Notification is successfully

received and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of the Spatial Anchors Subscription.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Spatial Anchors Subscription resource.

operationId: GetIndSpatialAnchorsSubsc

tags:

- Individual Spatial Anchors Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual Spatial Anchors Subscription

resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Spatial Anchors Subscription resource.

operationId: UpdateIndSpatialAnchorsSubsc

tags:

- Individual Spatial Anchors Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsSub'

responses:

'200':

description: >

OK. The Individual Spatial Anchors Subscription resource is successfully

updated and a representation of the updated resource shall be returned in the response

body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsSub'

'204':

description: >

No Content. The Individual Spatial Anchors Subscription resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Spatial Anchors Subscription resource.

operationId: ModifyIndSpatialAnchorsSubsc

tags:

- Individual Spatial Anchors Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/SpatialAnchorsSubPatch'

responses:

'200':

description: >

OK. The Individual Spatial Anchors Subscription resource is successfully

modified and a representation of the updated resource shall be returned in the response

body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsSub'

'204':

description: >

No Content. The Individual Spatial Anchors Subscription resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Spatial Anchors Subscription resource.

operationId: RemoveIndSpatialAnchorsSubsc

tags:

- Individual Spatial Anchors Subscription (Document)

responses:

'204':

description: >

The Individual Spatial Anchors Subscription resource

is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: "{tokenUrl}"

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

SpatialAnchorsList:

description: >

Represents the Spatial Anchors List.

type: object

properties:

valServInfo:

$ref: '#/components/schemas/VALServiceInfo'

anchors:

type: object

additionalProperties:

$ref: '#/components/schemas/SpatialAnchor'

minProperties: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- valServInfo

- anchors

SpatialAnchorsListPatch:

description: >

Represents the patch of Spatial Anchors List.

type: object

properties:

valServInfo:

$ref: '#/components/schemas/VALServiceInfo'

anchors:

type: object

additionalProperties:

$ref: '#/components/schemas/SpatialAnchorRm'

minProperties: 1

VALServiceInfo:

description: >

Represents the VAL service information.

type: object

properties:

serviceId:

type: string

endPt:

$ref: 'TS29548_SDD_Transmission.yaml#/components/schemas/ConnInfo'

servConInfo:

$ref: "#/components/schemas/ServConnectInfo"

required:

- serviceId

- endPt

ServConnectInfo:

description: >

Represents the connectivity information related to the application service.

type: object

properties:

dnais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

anyOf:

- required: [dnais]

- required: [dnn]

SpatialAnchor:

description: >

Represents the Spatial Anchor profile information.

type: object

properties:

anchorId:

$ref: '#/components/schemas/SpatialAnchorId'

posInfo:

$ref: '#/components/schemas/PositionInfo'

specInfo:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Bytes'

svcArea:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

rules:

$ref: '#/components/schemas/AllowedRules'

required:

- posInfo

SpatialAnchorRm:

description: >

Represents the Spatial Anchor profile information.

type: object

properties:

anchorId:

$ref: '#/components/schemas/SpatialAnchorId'

posInfo:

$ref: '#/components/schemas/PositionInfo'

specInfo:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Bytes'

svcArea:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

rules:

$ref: '#/components/schemas/AllowedRules'

required:

- posInfo

nullable: true

AllowedRules:

description: >

Represents the access control rules.

type: object

properties:

allowedEntities:

type: array

items:

$ref: '#/components/schemas/AllowedEntity'

minItems: 1

valServiceList:

type: array

items:

type: string

minItems: 1

anyOf:

- required: [allowedEntities]

- required: [valServiceList]

AllowedEntity:

description: >

Represents the allowed application entity.

type: object

properties:

type:

$ref: '#/components/schemas/EntityType'

identifier:

type: string

anyOf:

- required: [type]

- required: [identifier]

PositionInfo:

description: >

Represents the positioning information of the Spatial Anchor and the related Spatial Map.

type: object

properties:

pos:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Local3dPointUncertaintyEllipsoid'

spaMapPos:

$ref: '#/components/schemas/SpatialAnchorMapPosition'

anyOf:

- required: [pos]

- required: [spaMapPos]

SpatialAnchorMapPosition:

description: >

Represents the Spatial map positioning of the Spatial Anchor.

type: object

properties:

spaMapId:

type: string

description: Contains the Spatial Map Identifier.

spaMapLayerId:

type: array

items:

type: string

minItems: 1

description: Contains the identifier(s) of Spatial Map layer(s).

localInfo:

$ref: '#/components/schemas/LocalizationInfo'

required:

- spaMapId

LocalizationInfo:

description: >

Represents the localization information of the Spatial Anchor.

type: object

properties:

localInfo:

type: string

description: Contains the localization information.

SpatialAnchorsSub:

description: >

Represents the Spatial Anchors subscription.

type: object

properties:

eventSubs:

type: array

items:

$ref: '#/components/schemas/AnchorsEventsSub'

minItems: 1

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

report:

$ref: '#/components/schemas/SpatialAnchorsNotif'

reportReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- eventSubs

- notifUri

SpatialAnchorsSubPatch:

description: >

Represents the partial Spatial Anchors subscription.

type: object

properties:

eventSubs:

type: array

items:

$ref: '#/components/schemas/AnchorsEventsSub'

minItems: 1

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

reportReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

SpatialAnchorsNotif:

description: >

Represents the Spatial Anchors Notification.

type: object

properties:

subId:

type: string

reports:

type: array

items:

$ref: '#/components/schemas/AnchorsReport'

minItems: 1

required:

- subId

- reports

AnchorsReport:

description: >

Represents the Spatial Anchors report.

type: object

properties:

event:

$ref: '#/components/schemas/AnchorEvent'

detEvent:

$ref: '#/components/schemas/DetectedEvent'

achrList:

type: array

items:

$ref: '#/components/schemas/SpatialAnchorProfile'

minItems: 1

ueAnchrInt:

type: array

items:

$ref: '#/components/schemas/UEAnchorInterest'

minItems: 1

required:

- event

UEAnchorInterest:

description: >

Represents the UE with Spatial Anchors interest.

type: object

properties:

ueId:

type: string

anchrId:

$ref: '#/components/schemas/SpatialAnchorId'

anchrRng:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

anyOf:

- required: [ueId]

- required: [anchrId]

- required: [anchrRng]

AnchorsEventsSub:

description: >

Represents the Spatial Anchors subscription event and filters.

type: object

properties:

event:

$ref: '#/components/schemas/AnchorEvent'

evntFltr:

$ref: '#/components/schemas/EventFilter'

required:

- event

- evntFltr

EventFilter:

description: >

Represents the Spatial Anchors Event filters.

type: object

properties:

discFltr:

$ref: '#/components/schemas/DiscoveryFilter'

ueFltr:

$ref: '#/components/schemas/UeFilter'

rngFltr:

$ref: '#/components/schemas/RangeFilter'

anchrFltr:

$ref: '#/components/schemas/AnchorFilter'

DiscoveryFilter:

description: >

Represents the Spatial Anchors discovery event filters.

type: object

properties:

mgntEvts:

type: array

items:

$ref: '#/components/schemas/AnchorManagementEvent'

minItems: 1

required:

- mgntEvts

UeFilter:

description: >

Represents the UE with spatial anchors event filters.

type: object

properties:

rngVal:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

disc:

type: boolean

description: >

Indicates whether the UEs that have discovered spatial anchors are to be

included in the report. When present, it shall be set as follows:

true: include UEs that discovered the spatial anchors.

false: not include UEs that discovered the spatial anchors.

Default value when omitted is "false".

required:

- rngVal

RangeFilter:

description: >

Represents the Spatial Anchors in range event filters.

type: object

properties:

perSrch:

type: boolean

description: >

Contains the persistent search preference. When present, it shall be set as follows:

true: enable persistent search.

false: not enable persistent search.

Default value when omitted is "false".

usrLoc:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'
        pose:

type: string

$ref: '#/components/schemas/Pose'

appRng:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

required:

- appRng

AnchorFilter:

description: >

Represents the applicable spatial anchors filters.

type: object

properties:

anchIds:

type: array

items:

$ref: '#/components/schemas/SpatialAnchorId'

minItems: 1

svcIds:

type: array

items:

type: string

minItems: 1

areaInt:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

anyOf:

- required: [anchIds]

- required: [svcIds]

- required: [areaInt]

SpatialAnchorProfile:

description: >

Represents the spatial anchor profile.

type: object

properties:

anchInfo:

$ref: '#/components/schemas/SpatialAnchor'

valSvcInf:

$ref: '#/components/schemas/VALServiceInfo'

required:

- anchInfo

- valSvcInf

Pose:

description: >

Represents the Pose.

type: object

properties:

orientation:

$ref: '#/components/schemas/Orientation'

position:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Local3dPointUncertaintyEllipsoid'

fov:

$ref: '#/components/schemas/Fov'

required:

- orientation

Orientation:

description: >

Represents the Orientation.

type: object

properties:

x:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

y:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

z:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

w:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

Fov:

description: >

Represents the field of view.

type: object

properties:

angleLeft:

$ref: '#/components/schemas/Angle'

angleRight:

$ref: '#/components/schemas/Angle'

angleUp:

$ref: '#/components/schemas/Angle'

angleDown:

$ref: '#/components/schemas/Angle'

# SIMPLE DATA TYPES

#

SpatialAnchorId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

Angle:

description: Indicates value of angle.

type: integer

minimum: -360

maximum: 360

#

# ENUMERATIONS

#

EntityType:

anyOf:

- type: string

enum:

- VAL_SERVER

- VAL_CLIENT

- type: string

description: >

This string provides forward-compatibility with future extensions to the

enumeration but is not used to encode content defined in the present version of

this API.

description: |

Represents the type of entity.

Possible values are:

- VAL_SERVER: Indicates that the entity type is a VAL Server

- VAL_CLIENT: Indicates that the entity type is VAL Client.

AnchorEvent:

anyOf:

- type: string

enum:

- ANCHOR_DISCOVERY

- UE_SPATIAL_ANCHOR_INTEREST

- SPATIAL_ANCHOR_IN_RANGE

- type: string

description: >

This string provides forward-compatibility with future extensions to the

enumeration but is not used to encode content defined in the present version of

this API.

description: |

Represents the type of entity.

Possible values are:

- ANCHOR_DISCOVERY: Indicates that the Spatial Anchors event is spatial anchors dicovery.

- UE_SPATIAL_ANCHOR_INTEREST: Indicates that the Spatial Anchors event is the UE

with spatial anchors interest.

- SPATIAL_ANCHOR_IN_RANGE: Indicates that the Spatial Anchors event is the

spatial anchors in range.

AnchorManagementEvent:

anyOf:

- type: string

enum:

- ANCHOR_CREATION

- ANCHOR_UPDATE

- ANCHOR_DELETION

- type: string

description: >

This string provides forward-compatibility with future extensions to the

enumeration but is not used to encode content defined in the present version of

this API.

description: |

Represents the type of entity.

Possible values are:

- ANCHOR_CREATION: Indicates that the spatial anchor management event is

spatial anchor creation.

- ANCHOR_UPDATE: Indicates that the spatial anchor management event is

spatial anchor updation.

- ANCHOR_DELETION: Indicates that the spatial anchor management event is

spatial anchor deletion.

DetectedEvent:

anyOf:

- type: string

enum:

- ANCHOR_IN_RANGE_OF_UE

- ANCHOR_OUT_RANGE_OF_UE

- UE_IN_RANGE_OF_ANCHOR

- UE_OUT_RANGE_OF_ANCHOR

- type: string

description: >

This string provides forward-compatibility with future extensions to the

enumeration but is not used to encode content defined in the present version of

this API.

description: |

Represents the type of entity.

Possible values are:

- ANCHOR_IN_RANGE_OF_UE: Indicates that the triggered event is spatial anchor

in range of the UE.

- ANCHOR_OUT_RANGE_OF_UE: Indicates that the triggered event is spatial anchor

out of range of the UE.

- UE_IN_RANGE_OF_ANCHOR: Indicates that the triggered event is UE entering

in range of spatial anchor.

- UE_OUT_RANGE_OF_ANCHOR: Indicates that the triggered event is UE leaving

the range of spatial anchor.

# Data types describing alternative data types or combinations of data types:

#

# * * * * End of Changes * * * *