3GPP TSG CT WG3 Meeting #143	C3-254268

# Sophia-Antipolis, France, 13th – 17th October, 2025

Source:	Nokia

Title:	General corrections for the AF VFL APIs

Spec:	3GPP TS 29.530

Agenda item:	19.39 (AIML_CN)

Document for:	Agreement

1. Introduction

There are some mistakes in the high-level clauses of TS 29.530, including wrong API name, duplicate clause headers, and more.

2. Reason for Change

Correct the high-level clauses of TS 29.530.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.530 V 1.0.0.

* * * * First change * * * *

## 5.1	Introduction

The AF offers to other NFs the following services:

-	Naf_VFLTraining;

-	Naf_VFLInference;

-	Naf_Training;

-	Naf_Inference.

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Naf_VFLTraining | 5.2 | VFL Training Service | TS29530_Naf_VFLTraining.yaml | naf-vfl-train | A.2 |
| Naf_VFLInference | 5.3 | AF VFL Inference | TS29530_Naf_ VFLInference.yaml | naf-vflinference | A.3 |
| Naf_Training | 5.4 | Model training service | TS29530_Naf_Training.yaml | naf-train | A.4 |
| Naf_Inference | 5.5 | Naf_Inference API | TS29530_Naf_Inference.yaml | naf-inference | A.5 |

* * * * Next change * * * *

### 5.2.1	Service Description

#### 5.2.1.1	Overview

The Naf_VFLTraining service exposed by the AF acting as VFL client and enables an NF service consumer to:

-	request the creation/update of a VFL Training Subscription; and

-	receive VFL Training related event(s) reporting.

#### 5.2.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture is defined in 3GPP TS 23.288 [14]. The VFL signalling flows are defined in 3GPP TS 29.552 [15].

The Naf_VFLTraining service is part of the Naf service-based interface exhibited by the trusted Application Function (AF) or untrusted Application Function (AF).

Known consumers of the Naf_VFLTraining service are:

-	Network Data Analytics Function (NWDAF) when the AF is trusted.

-	Network Exposure Function (NEF) when the AF is untrusted.

Figure 5.2.1.2-1: Reference Architecture for the Naf_VFLTraining service; SBI representation

Figure 5.2.1.2-2: Reference Architecture for the Naf_VFLTraining service: reference point representation

#### 5.2.1.3	Network Functions

##### 5.2.1.3.1	Application Function (AF)

The Application Function (AF) acting as VFL client provides VFL training for different analytics events to NF service consumers.

The Application Function (AF) acting as VFL client allows NF service consumers to subscribe to and unsubscribe from VFL training event notifications.

##### 5.2.1.3.2	NF Service Consumers

The Network Data Analytics Function (NWDAF) and Network Exposure Function (NEF) support (un)subscription to the notification of different VFL training events.

* * * * Next change * * * *

### 5.4.1	Service Description

#### 5.4.1.1	Overview

The Naf_Training service exposed by the AF acting as VFL server and enables an NF service consumer to:

-	request the creation/update of a Training subscription; and

-	receive Training related event(s) reporting.

#### 5.4.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture is defined in 3GPP TS 23.288 [14]. The VFL signalling flows are defined in 3GPP TS 29.552 [15].

The Naf_Training service is part of the Naf service-based interface exhibited by the trusted Application Function (AF) or untrusted Application Function (AF).

Known consumers of the Naf_Training service are:

-	Network Data Analytics Function (NWDAF) when the AF is trusted.

-	Network Exposure Function (NEF) when the AF is untrusted.

Figure 5.4.1.2-1: Reference Architecture for the Naf_Training service; SBI representation

Figure 5.4.1.2-2: Reference Architecture for the Naf_Training service: reference point representation

#### 5.4.1.3	Network Functions

##### 5.4.1.3.1	Application Function (AF)

The Application Function (AF) acting as VFL server provides training for different analytics events to NF service consumers.

The Application Function (AF) acting as VFL server allows NF service consumers to subscribe to and unsubscribe from training event notifications.

##### 5.4.1.3.2	NF Service Consumers

The Network Data Analytics Function (NWDAF) and Network Exposure Function (NEF) support (un)subscription to the notification of different training events.

* * * * Next change * * * *

### 5.5.1	Service Description

#### 5.5.1.1	Overview

The Naf_Inference service as defined in 3GPP TS 23.288 [14], is provided by the Application Function (AF) acting as VFL server.

This service allows the NF service consumers to:

-	subscribe to and unsubscribe from different inference events;

-	modify inference subscriptions; and

-	be notified about events for corresponding inference subscriptions.

#### 5.5.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture is defined in 3GPP TS 23.288 [14]. The VFL signalling flows are defined in 3GPP TS 29.552 [15].

The Naf_Inference service is part of the Naf service-based interface exhibited by the trusted Application Function (AF) or untrusted Application Function (AF).

Known consumers of the Naf_Inference service are:

-	Network Data Analytics Function (NWDAF) when the AF is trusted.

-	Network Exposure Function (NEF) when the AF is untrusted.

Figure 5.5.1.2-1: Reference Architecture for the Naf_Inference service; SBI representation

Figure 5.5.1.2-2: Reference Architecture for the Naf_Inference service: reference point representation

#### 5.5.1.3	Network Functions

##### 5.5.1.3.1	Application Function (AF)

The Application Function (AF) acting as VFL server provides inference for different analytics events to NF service consumers.

The Application Function (AF) acting as VFL server allows NF service consumers to subscribe to and unsubscribe from inference events.

##### 5.5.1.3.2	NF Service Consumers

The Network Data Analytics Function (NWDAF) and Network Exposure Function (NEF) support (un)subscription to the notification of different inference events.

* * * * Next change * * * *

* * * * Next change * * * *

* * * * Next change * * * *

# B.3	Naf_VFLInference API

The API versions listed in table B.3-1 are withdrawn for the Naf_VFLInference API.

Table B.3-1: Withdrawn API versions of the Naf_VFLInference service

| API version number | Remarks |
| --- | --- |

* * * * End of changes * * * *