3GPP TSG-CT3 Meeting #143	C3-254287

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0440 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Open API Discovery Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following errors have been identified and need to be corrected: - The GET response body is using the wrong data type (DiscoveryResp). It should instead use OpenDiscoveryResp. - In the OpenAefProfile data type, the attributes aefId and versions should be mandatory, just like in AefProfile used in the discovery service. Also, the NOTE should point to OpenAefProfile instead of AefProfile. - The OpenAefProfile data structure in the OpenAPI definition needs to be updated so it matches the data model definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to correct the above identified errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation inconsistencies and incomplete OpenAPI definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.2.2.3.1, 8.11.5.2.4, A.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following API: TS29222_CAPIF_Open_Discover_Service_API.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st Change * * * *

###### 8.11.2.2.3.1	GET

The HTTP GET method enables to discover the Service APIs currently registered at the CCF.

Table 8.11.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| api-names | array(string) | O | 1..N | Contains the name(s) of the target Service API(s). Each Service API name shall be set to the value of the <apiName> placeholder of the Service API URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. |  |
| api-versions | map(array(string)) | O | 1..N(1..M) | Contains the major version(s) (e.g., v1) of the target Service API(s). Each Service API version shall be set to the value of the <apiVersion> placeholder of the Service API URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. The key of the map shall be set to the value of the Service API name (i.e., the value of the <apiName> placeholder of the Service API URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]) of the Service API to which the provided list of Service API version(s) provided within the map value applies. |  |
| comm-type | CommunicationType | O | 0..1 | Contains the communication type supported by the target Service API(s). |  |
| protocols | array(Protocol) | O | 1..N | Contains the protocol(s) supported by the target Service API(s). |  |
| data-format | DataFormat | O | 0..1 | Contains data format(s) supported by the target Service API(s). |  |
| api-cats | array(string) | O | 1..N | Contains the category(ies) of the target Service API(s). |  |
| preferred-aef-loc | AefLocation | O | 0..1 | Contains the preferred location information for AEF(s) exposing the target Service API(s). This query parameter is ignored by the CCF if there are no matching records at the CCF. |  |
| api-prov-names | array(string) | O | 1..N | Contains the name(s) of the provider(s) of the target Service API(s). |  |
| api-supported-features | map(SupportedFeatures) | O | 1..N | Contains the list of the feature(s) supported by the target Service API(s) indicated by the "api-names" query parameter. This query parameter may be present only if the "api-name" query parameter is also present. The key of the map shall be set to the value of the Service API name, among the ones provided within the "api-names" query parameter) of the Service API to which the provided list of supported feature(s) provided within the map value applies. |  |
| service-kpis | ServiceKpis | O | 0..1 | Contains information about the service characteristics provided by the target Service API(s). |  |
| api-ids | array(string) | O | 1..N | Contains the identifier(s) of the target Service APIs. |  |
| res-ops | array(ResOperInfo) | O | 1..N | Contains the list of supported API resource(s) and service operation(s). This query parameter may only be present if the "api-names" query parameter is present. |  |
| supported-features | SupportedFeatures | O | 0..1 | Contains the list of supported features among the ones defined in clause 8.11.6. This attribute shall be present only when feature negotiation is required. |  |
| NOTE: In addition to the above standardized query parameters, the service consumer may also provide vendor-specific query parameter(s) as specified in clause 5.2.13.3 of 3GPP TS 29.122 [14]. The CCF shall use any received vendor-specific query parameters in the filtering process of the results to be returned in the response in a similar way and in addition to the standardized query parameters defined in this table. |  |  |  |  |  |

Editor's note:	The access credentials and requirements for this service operation are FFS and pending progress in SA3.

This method shall support the request data structures specified in table 8.11.2.2.3.1-2 and the response data structures and response codes specified in table 8.11.2.2.3.1-3.

Table 8.11.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.11.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| OpenDiscoveryResp | M | 1 | 200 OK | Successful case. The result of the Service APIs discovery is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative CCF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative CCF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| ProblemDetails | O | 0..1 | 414 URI Too Long | Indicates that the server refuses to process the request because the request target is too long. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [14] shall also apply. |  |  |  |  |

Table 8.11.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative CCF. |

Table 8.11.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative CCF. |

# * * * * 2nd Change * * * *

##### 8.11.5.2.4	Type: OpenAefProfile

Table 8.11.5.2.4-1: Definition of type OpenAefProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aefId | string | M | 1 | Contains the AEF identifier. |  |
| versions | array(Version) | M | 1..N | Contains the supported Service API version(s). |  |
| protocol | Protocol | O | 0..1 | Contains the protocol used by the Service API. (NOTE) |  |
| dataFormat | DataFormat | O | 0..1 | Contains the data format used by the API (NOTE) |  |
| aefLocation | AefLocation | O | 0..1 | Contains the location information of the AEF exposing the Service API. |  |
| serviceKpis | ServiceKpis | O | 0..1 | Contains information about the service characteristics provided by the Service API. |  |
| NOTE: If the VendorExt feature is supported, vendor-specific extensions to the OpenAefProfile data structure, using the mechanism defined in clause 7.11, may be used to convey vendor-specific information. |  |  |  |  |  |

# * * * * 3rd Change * * * *

# A.13	CAPIF_Open_Discover_Service_API

openapi: 3.0.0

info:

title: CAPIF_Open_Discover_Service_API

version: 1.0.0-alpha.1

description: |

API for Routing information.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.222 V19.4.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/open-api-disc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222.

paths:

/service-apis:

get:

description: >

Enables Open discovery of the currently registered at the CCF and satisfying

a number of filter criteria.

parameters:

- name: api-names

in: query

description: >

Contains the name(s) of the target Service API(s).

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: api-versions

in: query

description: >

Contains the major version(s) (e.g., v1) of the target Service API(s).

schema:

type: object

additionalProperties:

type: array

items:

type: string

minItems: 1

minProperties: 1

- name: comm-type

in: query

description: >

Contains the communication type supported by the target Service API(s).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/CommunicationType'

- name: protocols

in: query

description: >

Contains the protocol(s) supported by the target Service API(s).

style: form

explode: false

schema:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Protocol'

minItems: 1

- name: data-format

in: query

description: >

Contains data format(s) supported by the target Service API(s).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/DataFormat'

- name: api-cats

in: query

description: >

Contains the category(ies) of the target Service API(s).

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: preferred-aef-loc

in: query

description: >

Contains the preferred location information for AEF(s) exposing the target

Service API(s).

This query parameter is ignored by the CCF if there are no matching records at the CCF.

content:

application/json:

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/AefLocation'

- name: api-prov-names

in: query

description: >

Contains the name(s) of the provider(s) of the target Service API(s).

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: api-supported-features

in: query

description: >

Contains the features supported by the discovered service API indicated by

api-name parameter. This may only be present if api-name query parameter is

present.

schema:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

minProperties: 1

- name: api-ids

in: query

description: >

Contains the identifier(s) of the targeted service APIs.

required: false

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: service-kpis

in: query

description: >

Contains iInformation about service characteristics provided by the targeted

service API(s).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceKpis'

- name: res-ops

in: query

description: >

Contains the list of supported API resource(s) and service operation(s).

style: form

explode: false

schema:

type: array

items:

$ref: 'TS29222_CAPIF_Discover_Service_API.yaml#/components/schemas/ResOperInfo'

minItems: 1

- name: supported-features

in: query

description: >

Contains a list of supported features among the ones defined in clause 8.1.6. This

attributed shall be present only when feature negotiation needs to take place.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

The response body contains the result of the search over the list of the registered

APIs.

content:

application/json:

schema:

$ref: '#/components/schemas/OpenDiscoveryResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

OpenDiscoveryResp:

type: object

description: >

Represents the Open Service API Discovery response..

properties:

discApis:

type: array

items:

$ref: '#/components/schemas/OpenAPIDetails'

minItems: 0

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- discApis

OpenAPIDetails:

type: object

description: >

Represents the Service API details provided within an Open Service API Discovery

response.

properties:

apiName:

type: string

apiId:

type: string

apiStatus:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ApiStatus'

description:

type: string

serviceAPICategory:

type: string

apiSuppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

apiProvName:

type: string

aefProfiles:

type: array

items:

$ref: '#/components/schemas/OpenAefProfile'

minItems: 1

required:

- apiName

OpenAefProfile:

type: object

description: Represents the AEF Profile details provided within an Open Service API Discovery response.

properties:

aefId:

type: string

versions:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Version'

minItems: 1

protocol:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Protocol'

dataFormat:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/DataFormat'

aefLocation:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/AefLocation'

serviceKpis:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceKpis'

required:

- aefId

- versions

# * * * * End of Changes * * * *