3GPP TSG-CT WG3 Meeting #143	C3-254308

# Sophia-Antipolis, FR, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0445 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of EN in CAPIF_Open_Discover_Service_API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 06-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the SID proposal (S3-252966) by SA3, it has been decided to study security aspects related to Open Discover API as part of CAPIF Phase 4. There is an editor’s note in TS29.222 regarding security aspects of CAPIF Open Discover that needs to be removed, as this is part of the SID proposal and will be addressed in Release 20. Also, the related EN is removed in stage-2 TS 23.222 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of below EN: “Editor's note: The access credentials and requirements for this service operation are FFS and pending progress in SA3.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 and stage 3 are not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI descriptions of the APIs. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

##### 8.11.2.2.3	Resource Standard Methods

###### 8.11.2.2.3.1	GET

The HTTP GET method enables to discover the Service APIs currently registered at the CCF.

Table 8.11.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| api-names | array(string) | O | 1..N | Contains the name(s) of the target Service API(s). Each Service API name shall be set to the value of the <apiName> placeholder of the Service API URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. |  |
| api-versions | map(array(string)) | O | 1..N(1..M) | Contains the major version(s) (e.g., v1) of the target Service API(s). Each Service API version shall be set to the value of the <apiVersion> placeholder of the Service API URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. The key of the map shall be set to the value of the Service API name (i.e., the value of the <apiName> placeholder of the Service API URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]) of the Service API to which the provided list of Service API version(s) provided within the map value applies. |  |
| comm-type | CommunicationType | O | 0..1 | Contains the communication type supported by the target Service API(s). |  |
| protocols | array(Protocol) | O | 1..N | Contains the protocol(s) supported by the target Service API(s). |  |
| data-format | DataFormat | O | 0..1 | Contains data format(s) supported by the target Service API(s). |  |
| api-cats | array(string) | O | 1..N | Contains the category(ies) of the target Service API(s). |  |
| preferred-aef-loc | AefLocation | O | 0..1 | Contains the preferred location information for AEF(s) exposing the target Service API(s). This query parameter is ignored by the CCF if there are no matching records at the CCF. |  |
| api-prov-names | array(string) | O | 1..N | Contains the name(s) of the provider(s) of the target Service API(s). |  |
| api-supported-features | map(SupportedFeatures) | O | 1..N | Contains the list of the feature(s) supported by the target Service API(s) indicated by the "api-names" query parameter. This query parameter may be present only if the "api-name" query parameter is also present. The key of the map shall be set to the value of the Service API name, among the ones provided within the "api-names" query parameter) of the Service API to which the provided list of supported feature(s) provided within the map value applies. |  |
| service-kpis | ServiceKpis | O | 0..1 | Contains information about the service characteristics provided by the target Service API(s). |  |
| api-ids | array(string) | O | 1..N | Contains the identifier(s) of the target Service APIs. |  |
| res-ops | array(ResOperInfo) | O | 1..N | Contains the list of supported API resource(s) and service operation(s). This query parameter may only be present if the "api-names" query parameter is present. |  |
| supported-features | SupportedFeatures | O | 0..1 | Contains the list of supported features among the ones defined in clause 8.11.6. This attribute shall be present only when feature negotiation is required. |  |
| NOTE: In addition to the above standardized query parameters, the service consumer may also provide vendor-specific query parameter(s) as specified in clause 5.2.13.3 of 3GPP TS 29.122 [14]. The CCF shall use any received vendor-specific query parameters in the filtering process of the results to be returned in the response in a similar way and in addition to the standardized query parameters defined in this table. |  |  |  |  |  |

This method shall support the request data structures specified in table 8.11.2.2.3.1-2 and the response data structures and response codes specified in table 8.11.2.2.3.1-3.

Table 8.11.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.11.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DiscoveryResp | M | 1 | 200 OK | Successful case. The result of the Service APIs discovery is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative CCF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative CCF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| ProblemDetails | O | 0..1 | 414 URI Too Long | Indicates that the server refuses to process the request because the request target is too long. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [14] shall also apply. |  |  |  |  |

Table 8.11.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative CCF. |

Table 8.11.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative CCF. |

* * * End of Changes * * *