3GPP TSG CT WG3 Meeting #143	C3-254327

Sophia-Antipolis, France, 13 – 17 October 2025	(Revision of C3-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0448 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Supported Features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NBI19, CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-09-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clauses 8.6.2.2.3.1, 8.8.2.2.3.1 and 8.10.2.2.3.1, the table descriptions for the "supported-features" query parameter are not complete and not exactly correct, and the property optional needs to be corrected as conditional. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected table descriptions for the "supported-features" query parameter with correct description, also corrected the optional property as conditional in the affected clauses. Also added the missing colume of applicability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not correct table descriptions and property for supported features in the impacted clauses will arouse wrong implementation issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.2.2.3.1, 8.8.2.2.3.1, 8.10.2.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

###### 8.6.2.2.3.1	GET

This method shall support the URI query parameters specified in table 8.6.2.2.3.1-1.

Table 8.6.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name |  | Data type |  |  | P | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| aef-id |  | string |  |  | M | 1 |  | AEF identifier |  |  |  |
| api-invoker-id |  | string |  |  | O | 1 |  | String identifying the API invoker |  |  |  |
| supported-features |  | SupportedFeatures |  |  | C | 0..1 |  | Contains the list of supported feature(s) among the ones defined in clause 8.6.6. This query parameter shall be present only when feature negotiation needs to take place. |  |  |  |

This method shall support the request data structures specified in table 8.6.2.2.3.1-2 and the response data structures and response codes specified in table 8.6.2.2.3.1-3.

Table 8.6.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.6.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AccessControlPolicyList | M | 1 | 200 OK | List of the access control policy applicable for the service API requested. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative CAPIF core function. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative CAPIF core function. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| ProblemDetails | O | 0..1 | 414 URI Too Long | Indicates that the server refuses to process the request because the request-target is too long. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [14] also apply. |  |  |  |  |

Table 8.6.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative CAPIF core function. |

Table 8.6.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative CAPIF core function. |

# *** 2nd Change ***

###### 8.8.2.2.3.1	GET

This method shall support the URI query parameters specified in table 8.8.2.2.3.1-1.

Table 8.8.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name |  | Data type |  |  | P | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| aef-id |  | string |  |  | O | 0..1 |  | String identifying the API exposing function |  |  |  |
| api-invoker-id |  | string |  |  | O | 0..1 |  | String identifying the API invoker which invoked the service API |  |  |  |
| time-range-start |  | DateTime |  |  | O | 0..1 |  | Start time of the invocation time range |  |  |  |
| time-range-end |  | DateTime |  |  | O | 0..1 |  | End time of the invocation time range |  |  |  |
| api-id |  | string |  |  | O | 0..1 |  | String identifying the API invoked. |  |  |  |
| api-name |  | string |  |  | O | 0..1 |  | Contains the API name set to the value of the "<apiName>" placeholder of the API URI as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. |  |  |  |
| api-version |  | string |  |  | O | 0..1 |  | Version of the API which was invoked |  |  |  |
| protocol |  | Protocol |  |  | O | 0..1 |  | Protocol invoked |  |  |  |
| operation |  | Operation |  |  | O | 0..1 |  | Operation that was invoked on the API |  |  |  |
| result |  | string |  |  | O | 0..1 |  | HTTP status code of the invocation |  |  |  |
| resource-name |  | string |  |  | O | 0..1 |  | Name of the specific resource invoked |  |  |  |
| src-interface |  | InterfaceDescription |  |  | O | 0..1 |  | Interface description of the API invoker. |  |  |  |
| dest-interface |  | InterfaceDescription |  |  | O | 0..1 |  | Interface description of the API invoked. |  |  |  |
| net-slice-info |  | array(NetSliceId) |  |  | O | 1..N |  | Contains the identifier(s) of the network slice(s) within which the API shall be available. |  | SliceBasedAPIExposure |  |
| supported-features |  | SupportedFeatures |  |  | C | 0..1 |  | Contains the list of supported feature(s) among the ones defined in in clause 8.8.6. This query parameter shall be present only when feature negotiation needs to take place. |  |  |  |

This method shall support the request data structures specified in table 8.8.2.2.3.1-2 and the response data structures and response codes specified in table 8.8.2.2.3.1-3.

Table 8.8.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.8.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| InvocationLogsRetrieveRes | M | 1 | 200 OK | Result of the query operation along with fetched service API invocation log data. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative CAPIF core function. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative CAPIF core function. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| ProblemDetails | O | 0..1 | 414 URI Too Long | Indicates that the server is refusing to service the request because the request-target is too long. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [14] shall also apply. |  |  |  |  |

Table 8.8.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative CAPIF core function. |

Table 8.8.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative CAPIF core function. |

# *** 3rd Change ***

###### 8.10.2.2.3.1	GET

This method shall support the URI query parameters specified in table 8.10.2.2.3.1-1.

Table 8.10.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name |  | Data type |  |  | P | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| aef-id |  | string |  |  | M | 1 |  | AEF identifier |  |  |  |
| supp-feat |  | SupportedFeatures |  |  | C | 0..1 |  | Contains the list of supported feature(s) among the ones defined in in clause 8.10.6. This query parameter shall be present only when feature negotiation needs to take place. |  |  |  |

This method shall support the request data structures specified in table 8.10.2.2.3.1-2 and the response data structures and response codes specified in table 8.10.2.2.3.1-3.

Table 8.10.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.10.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| RoutingInfo | M | 1 | 200 OK | The Routing information applicable for the service API requested. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative CAPIF core function. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative CAPIF core function. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| ProblemDetails | O | 0..1 | 414 URI Too Long | Indicates that the server refuses to process the request because the request-target is too long. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [14] also apply. |  |  |  |  |

Table 8.10.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative CAPIF core function. |

Table 8.10.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative CAPIF core function. |

*** End of Changes ***