3GPP TSG-CT3 Meeting #143	C3-254444

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025				(revision of C3-254297)

Source:	Nokia, Huawei, Ericsson, Samsung

Title:	Pseudo-CR on correction for AIMLES_ContextTransfer API

Spec:	3GPP TS 29.482 v 1.1.0

Agenda item:	19.41

Document for:	Agreement

1. Introduction

There is existing definition of AIMLES_ContextTransfer API in 29.482 under the AIML_App WI.

2. Reason for Change

Correct the definition of AIMLES_ContextTransfer API as defined in clause 8.24.2 in 3GPP TS 23.482.

The data type for the aimleServRes attributeis proposed to define as a string because as per description from the procedure in clause 8.24.2 the AIMLE service result depends on how the results are generated based on the model training output, inference result, split outputs. Hence the result (i.e. the data type of this attribute) may be status messages or model data. So, it is proposed to keep it as string.

The data type for aimleServApp attribute is defined as per Table 8.24.3.1-2 of 23.482.

The new information element related to ML context information is defined.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.482 V 1.1.0.

* * * First Change * * *

##### 6.1.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.1.6.1-1 specifies the data types defined for the AIMLES_ContextTransfer API.

Table 6.1.1.6.1-1: AIMLES_ContextTransfer API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AimleServApp | 6.1.1.6.2.4 | Represents the AIMLE service applicability information. |  |
| AimleServOpStatus | 6.1.1.6.3.3 | Represents the AIMLE service operation status information. |  |
| AimleServStatus | 6.1.1.6.2.3 | Represents the AIMLE service status information. |  |
| MLContextInformation | 6.1.1.6.2.5 | Represents the context information related to the ML operation. |  |
| TransReq | 6.1.1.6.2.2 | Represents the AIMLE context information transfer request. |  |

Table 6.1.1.6.1-2 specifies data types re-used by the AIMLES_ContextTransfer API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the AIMLES_ContextTransfer API.

Table 6.1.1.6.1-2: AIMLES_ContextTransfer API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AimlOperation | 3GPP TS 24.560 [16] | Represents the type of ML task. |  |
| DataSetRequirements | 3GPP TS 24.560 [16] | Represents the requirements on data set for FL training. |  |
| MlModelDetail | 6.1.13.6.2.7 | Represents the ML Model information. |  |
| LocationArea5G | 3GPP TS 29.122 [2] |  | Used to indicate a location area represented as list of geographic areas, civic addresses and network area. |  |  |  |
| SplitOpProfile | 6.1.12.6.2.9 | Represents the split operation profile that VAL server participates to. |  |

* * * 2nd Change * * *

6.1.1.6.2.2	Type: TransReq

Table 6.1.1.6.2.2-1: Definition of type TransReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aimleClientId | AimleClientId | M | 1 | Contains the identifier of the concerned AIMLE Client associated with the context. |  |
| aimleServerId | string | O | 0..1 | Contains the identifier of the concerned AIMLE Server associated with service area that the AIMLE Client is currently in. |  |
| aimleServerIds | array(string) | O | 1..N | Contains the list of identifier(s) of the AIMLE Server(s) maintained by the AIMLE Client as the list of source AIMLE Server(s) whenever the UE transitioned from a source edge area to a target edge area. |  |
| aimleServStatus | AimleServStatus | O | 0..1 | Contains the status of the AIML operations at the AIMLE Client. |  |
| aimleServRes | string | O | 0..1 | Contains the result of the AIML operations performed by the AIMLE Client. |  |
| aimleServApp | AimleServApp | O | 0..1 | Contains the AIMLE Client's applicability information for the performed AIML operations. |  |
| mlContextInfo | MLContextInformation | O | 0..1 | Contains the context information related to the ML operation that the AIMLE client is participating in or performing. |  |

* * * 3rd Change * * *

6.1.1.6.2.3	Type: AimleServStatus

Table 6.1.1.6.2.3-1: Definition of type AimleServStatus

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aimleServOpStatus | AimleServOpStatus | M | 1 | Identifies the AIMLE service status of AIML operation at the AIMLE Client. |  |
| aimleServOpPercent | integer | C | 0..1 | Contains the percentage of completion of the AIML operation at the AIMLE Client. Minimum = 0. Maximum = 100. This attribute shall be present only when the "aimleServOpStatus" attribute value is either "ACTIVE" or "PAUSED". |  |

* * * 4th Change * * *

6.1.1.6.2.4	Type: AimleServApp

Table 6.1.1.6.2.4-1: Definition of type AimleServApp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serviceArea | LocationArea5G | O | 0..1 | Contains the service area where the AIML operations are performed by the AIMLE client. |  |
| operationPipeline | string | O | 0..1 | Contains the split operation pipeline information where the AIML operations are performed by the AIMLE client. |  |

* * * 5th Change * * *

6.1.1.6.2.5	Type: MLContextInformation

Table 6.1.1.6.2.5-1: Definition of type MLContextInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServId | string | O | 0..1 | Identifies the VAL Service, for which the request applies. (NOTE) |  |
| mlModelInfo | MlModelDetail | M | 1 | Identifies the ML model, for which the request applies. |  |
| mlTask | AimlOperation | O | 0..1 | Identifies the ML task, for which the request applies. |  |
| dataSetInfo | DataSetRequirements | O | 0..1 | Identifies the data set information for training. This attribute may be present only when the mlTask attribute value is set to "MODEL_TRAINING". |  |
| trainOpStatus | AimleServOpStatus | O | 0..1 | Identifies the status of the training operation at AIMLE client. This attribute may be present only when the mlTask attribute value is set to "MODEL_TRAINING". |  |
| splitTaskInfo | SplitOpProfile | O | 0..1 | Identifies the split operation profile that VAL server participates to. This attribute may be present only when the mlTask attribute value is set to "MODEL_SPLIT". |  |

* * * 6th Change * * *

6.1.1.6.3.3	Enumeration: AimleServOpStatus

The enumeration AimleServOpStatus represents the AIMLE service operation status information. It shall comply with the provisions defined in table 6.1.1.6.3.3-1.

Table 6.1.1.6.3.3-1: Enumeration AimleServOpStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ACTIVE | Indicates that the status of the AIML operations at the AIMLE Client is active. |  |
| PAUSED | Indicates that the status of the AIML operations at the AIMLE Client is paused. |  |
| COMPLETED | Indicates that the status of the AIML operations at the AIMLE Client is completed. |  |

* * * End of Changes * * *