3GPP TSG-CT WG4 Meeting #131	C4-254242

Sophia Antipolis, France, 13-17 October 2025	(revision of CP-242250)

3GPP TSG-CT WG1 Meeting #157	C1-256481

Sophia Antipolis, France, 13-17 October 2025	(revision of CP-242250)

3GPP TSG-CT WG3 Meeting #143	C3-254365

Sophia Antipolis, France, 13-17 October 2025	(revision of CP-242250)

# Source:	China Mobile

# Title:	Revised WID on Next Generation Real time Communication services Phase 2

# Document for:	Approval

# Agenda Item:	19.3.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	CT aspects of Next Generation Real time Communication services

Acronym:	NG_RTC_Ph2

Unique identifier:	1050022

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes | X | X |  | X |  |
| No |  |  | X |  |  |
| Don't know |  |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| NG_RTC_Ph2 | SA2 | 1040026 | System architecture for Next Generation Real time Communication services Phase 2 |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 770003 | Study on enhancements to IMS for new real time communication services | Study Item of Stage 1 requirements |
| 790003 | Enhancements to IMS for new real time communication services | Work Item of Stage 1 requirements |
| 850042 | Study on evolution of IMS multimedia telephony service | Study Item of Stage 1 requirements |
| 920036 | Evolution of IMS Multimedia Telephony Service | Work Item of Stage 1 requirements |
| 940066 | Study on system architecture for next generation real time communication services | Study Item of Stage 2 architecture and procedures |
| 970014 | System architecture for Next Generation Real time Communication services | Work Item of Stage 2 architecture and procedures |
| 990023 | CT1 aspects of NG_RTC | Work Item of Stage 3 |
| 990086 | CT3 aspects of NG_RTC | Work Item of Stage 3 |
| 990087 | CT4 aspects of NG_RTC | Work Item of Stage 3 |
| 1030027 | CT6 aspects of NG_RTC | Work Item of Stage 3 |
| 850003 | Study on security support for Next Generation Real Time Communication services | Study Item of Stage 3 |
| 990049 | PS Data Off for IMS Data Channel Service | Work Item of Stage 1 requirements |
| 950005 | Study on Localized Mobile Metaverse Services | Study Item of Stage 1 requirements |
| 1010030 | Study on system architecture for next generation real time communication services phase 2 | Study Item of Stage 2 architecture and procedures |
| 1020038 | Study on the security support for the Next Generation Real Time Communication services phase 2 | Study item of stage2 security |

3	Justification

SA2 further studied the system architecture enhancements to support next generation real time communication services under SID FS_NG_RTC_Ph2 "Study on system architecture for next generation real time communication services phase 2" and has agreed conclusions for all the key issues in 3GPP TR 23.700-77. In addition, TSG-SA has approved work item "System architecture for Next Generation Real time Communication services Phase 2" in TSG SA Meeting #104 (June 2024) to capture the required stage 2 requirements and work.

Considering the above, impacts on protocols, services and interfaces under CT WGs' responsibilities are foreseen and the related work in CT WGs should be carried out within Rel-19.

4	Objective

The objective of the work item is to enhance IMS protocols and procedures, define and extend NF services, analyze interaction with existing services in specifications under remit of CT WGs for the stage 2 requirements agreed under the stage 2 work item NG_RTC_Ph2. Stage 3 work shall follow the normative stage-2 requirements. The following areas of work are expected to be covered (non-exhaustive):

CT1

- Enhancement of IMS data channel setup procedures to support IMS data channel interworking.

- Enhancement of IMS procedures and functionalities to support provision and authorization of third party specific identity.

- Specification of the procedures for:

  - 3GPP PS data off handling of IMS session with IMS data channel;

  - IMS session handling of standalone IMS data channel; and

  - IMS data channel multiplexing capability discovery and media handling.

- Specification of the procedures and functionalities for IMS Avatar communication and transition in media between Avatar communication and MMTel session.

- Extension of UE configuration to support 3GPP PS data off exemption for service over IMS data channel and potential extension to support standalone DC setup.

- Enhancement of IMS SDP to support IMS data channel multiplexing, metadata media and IMS Avatar representation animation negotiation.

- Potential interaction with existing supplementary services and third-party specific identity.

- Potential interaction with existing supplementary services on standalone IMS data channel and IMS data channel multiplexing

CT3

- Definition of new NEF services to support:

  - IMS event subscription/notification related to IMS data channel;

  - IMS network capability exposure for IMS data channel establishment/update/termination/interworking; and

  - Provisioning third party identity specific information in HSS.

- Extension of NNI interface to support:

  - DCMTSI client interworking with MTSI client;

  - IMS data channel multiplexing; and

  - Provision of IMS data channel operation initiator.

CT4

- Definition of new HSS services and IMS AS services to support:

  - IMS event subscription/notification related to IMS data channel; and

  - IMS network capability exposure for IMS data channel establishment/update/termination/interworking.

- Extension of IMS AS services and MF services to support:

  - media transcoding for IMS data channel interworking;

  - 3GPP PS data off status notification and 3GPP PS data off handling;

  - standalone IMS data channel;

  - IMS data channel multiplexing; and

  - IMS Avatar communication and transition in media between Avatar communication and MMTel session.

- Extension of HSS service to support the registration of the IMS AS instance in HSS over N71.

- Extension of repository data in HSS to support authorization of third-party identity, standalone IMS data channel subscription and IMS Avatar communication.

- Extension of NRF services to support:

  - discovery of IMS AS supporting IMS event exposure; and

  - discovery of MF supporting IMS data channel multiplexing and IMS Avatar communication.

- Enhancement of Sh to support functionality list in bullet 1).

CT6

- Extension of UICC configuration to support 3GPP PS data off exemption for service over IMS data channel.

- Potential extension of UICC configuration to support standalone data channel.

5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 24.186 | Specification of procedures and functionalities for IMS DC interworking, standalone DC, DC multiplexing capability discovery and media handling, 3GPP PS data off handling on DC, IMS Avatar communication and transition. Potential interaction with existing supplementary services on standalone DC and DC multiplexing. | TSG CT #109 (September 2025) | CT1 |
| 24.229 | Enhancement of SIP to support provision and authorization of third party specific identity, IMS data channel multiplexing capability discovery. Enhancement of SDP to support DC multiplexing, metadata media and IMS Avatar communication. | TSG CT #109 (September 2025) | CT1 |
| 24.275 | Extension of 3GPP PS data off node to support exemption for service over IMS data channel, and potential extension of IMS DC node to support standalone DC. | TSG CT #109 (September 2025) | CT1 |
| 24.607 | Potential enhancements of the interaction between OIR and RCD and addition of Call-Info header field with RCD info for the served user which activated the OIR service. | TSG CT #109 (September 2025) | CT1 |
| 29.522 | Definition of new NEF services to support IMS event subscription/notification, IMS network capability exposure and third party identity provision. | TSG CT #109 (September 2025) | CT3 |
| 29.165 | Extension of NNI interface to support DC multiplexing, DC interworking, as well as provision of IMS data channel operation initiator. | TSG CT #109 (September 2025) | CT3 |
| 29.175 | Definition of new IMS AS services to support subscriber specific and non-subscriber specific IMS event subscription/notification, IMS network capability exposure. Enhancement of IMS AS services to support new media instructions for DC interworking, 3GPP PS data off handling, standalone DC, DC de-multiplexing, IMS Avatar communication and transition, and notification of 3GPP PS data off status, IMS Avatar communication events. | TSG CT #109 (September 2025) | CT4 |
| 29.176 | Enhancement of MF services to support media process for DC interworking, PS data off handling, standalone DC, DC de-multiplexing, IMS Avatar communication and transition. | TSG CT #109 (September 2025) | CT4 |
| 29.562 | Definition of new HSS services to support subscriber specific IMS event subscription/notification, IMS network capability exposure. Enhancement of HSS services to support IMS AS registration. Extension of repository data in HSS to support authorization of third-party identity, standalone IMS DC subscription and IMS Avatar communication. | TSG CT #109 (September 2025) | CT4 |
| 29.364 | Potential update of IMS AS service data to support standalone IMS DC subscription. | TSG CT #109 (September 2025) | CT4 |
| 29.510 | Enhancement of NRF services to support discovery of IMS AS supporting IMS event exposure (e.g. by NEF/AS) and MF supporting DC multiplexing and IMS Avatar communication. | TSG CT #109 (September 2025) | CT4 |
| 29.571 | Potential enhancement on Common Data for NG_RTC Phase 2. | TSG CT #109 (September 2025) | CT4 |
| 29.328 | Enhancement of Sh to support IMS event subscription/notification. | TSG CT #109 (September 2025) | CT4 |
| 29.329 | Update of AVPs in Sh to support IMS event subscription/notification. | TSG CT #109 (September 2025) | CT4 |
| 29.230 | Update on commands and AVP codes to support IMS event subscription/notification. | TSG CT #109 (September 2025) | CT4 |
| 31.102 | Extension of 3GPP PS data off exemption configuration for service over IMS data channel. | TSG CT #109 (September 2025) | CT6 |
| 31.103 | Potential extension of IMS DC configuration to support standalone DC. | TSG CT #109 (September 2025) | CT6 |

6	Work item Rapporteur(s)

Chen Xu, China Mobile, chenxu@chinamobile.com

7	Work item leadership

CT1

8	Aspects that involve other WGs

SA4 for definition of new media, media handling and negotiation mechanism.

9	Supporting Individual Members

Supporting IM name

China Mobile

Huawei

HiSilicon

CATT

Deutsche Telekom

ZTE

Nokia

Ericsson

vivo

InterDigital

Xiaomi

AT&T

Telefonica