3GPP TSG-CT WG4 Meeting #131	C4-254266

# Sophia Antipolis, France; 13th – 17th October 2025							was C4-254050

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.080 | CR | 0129 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add ExtendedFacility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, the Facility defined in clause 3.6 is a type 4 information element with no upper length limit except that given by the maximum number of octets in a L3 message. However, the type 4 information element is no long than 255 octets as defined in TS 24.007 clause 11.2 below: A type 4 standard information element has format LV or TLV. Its LI has one octet and precedes the value part, which consists of zero, one, or up to 255 octets The component within the Facility information element might exceed than 255 octets, so the Facility could not fit the requirements for 5G. It proposes to define a new ExtendedFacility info to solve the above issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define ExtendedFacility and related Error. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Facility could not match the requirements for 5G, the UE could not send the related information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2.3, 2.4, 2.5, 3.5, 3.x(new), 4.2.2.xx(new), 4.4.2, 4.4.3.a (new), 4.4.3.b (new), 4.4.3.c (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 24.008 CR 3359 TS 24.571 CR 0104 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

2.3	Facility

This message is sent by the mobile station or the network to request or acknowledge a supplementary service. It is used when information is to be conveyed and the transaction already exists, but is not to be released. The supplementary service to be invoked, and its associated parameters, are specified in the Facility information element and ExtendedFacility information element (see table 2.2). When the length of the component within the Facility exceeds 255 octets, the ExtendedFacility shall be used, the Facility shall be ignored when ExtendedFacility is present. The ExtendedFacility shall be present when the extendedFacilityInd indicated the MS or LMF supports the Extended Facility.

Table 2.2: FACILITY message content

| IEI | Information element | Type / Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Supplementary service | Protocol discriminator | M | V | 1/2 |
|  | protocol discriminator | 3.2 |  |  |  |
|  | Transaction identifier | Transaction identifier | M | V | 1/2 |
|  |  | 3.3 |  |  |  |
|  | Facility | Message type | M | V | 1 |
|  | message type | 3.4 |  |  |  |
|  | Facility | Facility | M | LV | 2-? |
|  |  | 3.6 |  |  |  |
| F1 | ExtendedFacility | ExtendedFacility | O | TLV | 3-? |
|  |  | 3.x |  |  |  |

* * * Next Change * * * *

2.4	Register

2.4.1	Register (network to MS direction)

This message is sent by the network to the mobile station to assign a new transaction identifier for call independent supplementary service control and to request or acknowledge a supplementary service (see table 2.3). When the length of the component within the Facility exceeds 255 octets, the ExtendedFacility shall be used, the Facility shall be ignored when ExtendedFacility is present. The ExtendedFacility shall be present when the extendedFacilityInd indicated the MS or LMF supports the Extended Facility.

Table 2.3: REGISTER message content (network to MS direction)

| IEI | Information element | Type / Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Supplementary service | Protocol discriminator | M | V | 1/2 |
|  | protocol discriminator | 3.2 |  |  |  |
|  | Transaction identifier | Transaction identifier | M | V | 1/2 |
|  |  | 3.3 |  |  |  |
|  | Register | Message type | M | V | 1 |
|  | message type | 3.4 |  |  |  |
| 1C | Facility | Facility | M | TLV | 2-? |
|  |  | 3.6 |  |  |  |
| F1 | ExtendedFacility | ExtendedFacility | O | TLV | 3-? |
|  |  | 3.x |  |  |  |

2.4.2	Register (MS to network direction)

This message is sent by the mobile station to the network to assign a new transaction identifier for call independent supplementary service control and to request or acknowledge a supplementary service (see table 2.4). When the length of the component within the Facility exceeds 255 octets, the ExtendedFacility shall be used, the Facility shall be ignored when ExtendedFacility is present. The ExtendedFacility shall be present when the extendedFacilityInd indicated the MS or LMF supports the Extended Facility.

Table 2.4: REGISTER message content (MS to network direction)

| IEI | Information element | Type / Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Supplementary service | Protocol discriminator | M | V | 1/2 |
|  | protocol discriminator | 3.2 |  |  |  |
|  | Transaction identifier | Transaction identifier | M | V | 1/2 |
|  |  | 3.3 |  |  |  |
|  | Register | Message type | M | V | 1 |
|  | message type | 3.4 |  |  |  |
| 1C | Facility | Facility | M | TLV | 2-? |
|  |  | 3.6 |  |  |  |
| 7F | SS version | SS version indicator | O | TLV | 3 |
|  |  | 3.7.2 |  |  |  |
| F1 | ExtendedFacility | ExtendedFacility | O | TLV | 3-? |
|  |  | 3.x |  |  |  |

2.4.2.1	SS version

This information element shall be included if the supplementary service operation being invoked is implemented according to the phase 2 or higher protocol version.

* * * Next Change * * * *

2.5	Release complete

This message is sent by the mobile station or the network to release a transaction used for call independent supplementary service control. It may also request or acknowledge a supplementary service (see table 2.5).

Table 2.5: RELEASE COMPLETE message content

| IEI | Information element | Type / Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Supplementary service | Protocol discriminator | M | V | 1/2 |
|  | protocol discriminator | 3.2 |  |  |  |
|  | Transaction identifier | Transaction identifier | M | V | 1/2 |
|  |  | 3.3 |  |  |  |
|  | Release Complete | Message type | M | V | 1 |
|  | message type | 3.4 |  |  |  |
| 08 | Cause | Cause | O | TLV | 4-32 |
|  |  | TS 24.008 |  |  |  |
| 1C | Facility | Facility | O | TLV | 2-? |
|  |  | 3.6 |  |  |  |
| F1 | ExtendedFacility | ExtendedFacility | O | TLV | 3-? |
|  |  | 3.x |  |  |  |

2.5.1	Cause

This information element shall be included when the functional handling of the Cause IE is specified in the service description or TS 29.011. If the functional handling of the Cause IE is not specified, the receiving entity may ignore the IE.

2.5.2	Facility

This information element shall be included as required by the service description and the procedures defined in TS 24.010. When the length of the component within the Facility exceeds 255 octets, the ExtendedFacility shall be used, the Facility shall be ignored when ExtendedFacility is present. The ExtendedFacility shall be present when the extendedFacilityInd indicated the MS or LMF supports the Extended Facility.

For general rules, format and coding of transaction identifier values, see TS 24.008.

* * * Next Change * * * *

3.5	Other information elements

These information elements are coded according to the general coding rules as defined in TS 24.008.

Table 3.2 contains the code-points allocated to the information elements used in messages defined in this specification. All IEs are defined in TS 24.008, but the content of the Facility and SS version indicator IEs are defined within this specification.

Table 3.2: Information elements specific to call independent supplementary service control

| 8 7 6 5 4 3 2 1 |  | Reference (IE content) |
| --- | --- | --- |
| 0 . . . . . . . | Type 3 and 4 information elements |  |
| 0 0 0 1 0 0 0 | Cause | TS 24.008 |
| 0 0 1 1 1 0 0 | Facility | 3.6 |
| 1 1 1 1 1 1 1 | SS version indicator | 3.8.2 |
| 1 . . . . . . . | Type 6 information elements |  |
| 1 1 1 0 0 0 1 | ExtendedFacility | 3.x |

* * * Next Change * * * *

3.x	ExtendedFacility information element

The purpose of the ExtendedFacility information element is to extend the maximum length of Facilty. The ExtendedFacility shall be present when the extendedFacilityInd indicated the MS or LMF supports the Extended Facility.

The ExtendedFacility information element is coded as shown in figure 3.y and tables 3.3 to 3.17.

The ExtendedFacility is a type 6 information element with no upper length limit.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | octet 1 |
| ExtendedFacility IEI |  |  |  |  |  |  |  |  |
| Length of ExtendedFacility contents |  |  |  |  |  |  |  | octet 2 to 3 |
| Component(s) (note) |  |  |  |  |  |  |  | octet 4 etc. |
| NOTE: One or more components may be included depending on specific service requirements. |  |  |  |  |  |  |  |  |

Figure 3.y: ExtendedFacility information element

* * * Next Change * * * *

4.2	Operation types

Table 4.1 summarizes the operations defined for supplementary services in this specification and shows which of these operations are call related and call independent. The terms "call related" and "call independent" are defined in 3GPP TS 24.010 [6]. Refer to 3GPP TS 24.171 [15] for operations for E-UTRAN LCS, and 3GPP TS 24.571 [19] for operation for 5G LCS.

Table 4.1: Relevance of supplementary service operations

| Operation name | Call related SS | Call independent SS |
| --- | --- | --- |
| registerSS | - | + |
| eraseSS | - | + |
| activateSS | - | + |
| deactivateSS | - | + |
| interrogateSS | - | + |
| registerPassword | - | + |
| getPassword | - | + |
| processUnstructuredSS-Data | + | + |
| forwardCheckSS-Indication | - | + |
| processUnstructuredSS-Request | - | + |
| unstructuredSS-Request | - | + |
| unstructuredSS-Notify | - | + |
| forwardChargeAdvice | + | - |
| notifySS | + | - |
| forwardCUG-Info | + | - |
| buildMPTY | + | - |
| holdMPTY | + | - |
| retrieveMPTY | + | - |
| splitMPTY | + | - |
| explicitCT | + | - |
| accessRegisterCCEntry | + | - |
| eraseCCEntry | - | + |
| callDeflection | + | - |
| userUserService | + | - |
| lcs-LocationNotification | - | + |
| lcs-MOLR | - | + |
| lcs-SLMOLR | - | + |
| lcs-AreaEventRequest | - | + |
| lcs-AreaEventReport | - | + |
| lcs-AreaEventCancellation | - | + |
| lcs-PeriodicLocationRequest | - | + |
| lcs-LocationUpdate | - | + |
| lcs-PeriodicLocationCancellation lcs-PeriodicTriggeredInvoke | - - | + + |
| lcs-EventReport | - | + |
| lcs-CancelDeferredLocation | - | + |
| lcs-MSCancelDeferredLocation | - | + |
| lcs-LocationPrivacySetting lcs-PruAssociation lcs-PruDisassociation | - - - | + + + |
| lcs-SLMTLR | - | + |
| lcs-DLRSPPTransport | - | + |
| lcs-ULRSPPTransport capabilityInformation | - - | + + |
| NOTE: The processUnstructuredSS-Data operation may be used call related by a GSM Phase 1 MS. |  |  |

The following ASN.1 module defines operations by allocating them a local value. For the involved operations the same local values as in MAP are allocated.

.$SS-Operations {

itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3)

ss-Operations (0) version18 (18)}

DEFINITIONS ::=

BEGIN

EXPORTS

-- exports operations

-- operations defined in this specification

processUnstructuredSS-Data, notifySS, forwardChargeAdvice, forwardCUG-Info, buildMPTY, holdMPTY, retrieveMPTY, splitMPTY, explicitCT, accessRegisterCCEntry, callDeflection, userUserService,

lcs-LocationNotification, lcs-MOLR, lcs-SLMOLR, lcs-AreaEventRequest, lcs-AreaEventReport, lcs-AreaEventCancellation, lcs-PeriodicLocationRequest, lcs-LocationUpdate, lcs-PeriodicLocationCancellation,

lcs-PeriodicTriggeredInvoke, lcs-EventReport, lcs-CancelDeferredLocation, lcs-MSCancelDeferredLocation, lcs-LocationPrivacySetting, lcs-PruAssociation, lcs-PruDisassociation, lcs-SLMTLR, lcs-DLRSPPTransport, lcs-ULRSPPTransport;

IMPORTS

OPERATION FROM

Remote-Operations-Information-Objects {

joint-iso-itu-t remote-operations(4)

informationObjects(5) version1(0)}

-- The MAP operations:

-- registerSS, eraseSS, activateSS, deactivateSS, interrogateSS, registerPassword,

-- getPassword, processUnstructuredSS-Request, unstructuredSS-Request, unstructuredSS-Notify

-- forwardCheckSS-Indication

-- are imported from MAP-Operations in SS-Protocol module.

-- imports SS-data types

NotifySS-Arg,

ForwardChargeAdviceArg,

ForwardCUG-InfoArg,

SS-UserData,

AccessRegisterCCEntryArg,

CallDeflectionArg,

UserUserServiceArg,

LocationNotificationArg,

LocationNotificationRes,

LCS-MOLRArg,

LCS-MOLRRes,

LCS-SLMOLRArg,

LCS-SLMOLRRes,

LCS-AreaEventRequestArg,

LCS-AreaEventReportArg,

LCS-AreaEventCancellationArg,

LCS-PeriodicLocationRequestArg,

LCS-PeriodicLocationRequestRes,

LCS-LocationUpdateArg,

LCS-LocationUpdateRes,

LCS-PeriodicLocationCancellationArg,

LCS-PeriodicTriggeredInvokeArg,

LCS-PeriodicTriggeredInvokeRes,

LCS-EventReportArg,

LCS-EventReportRes,

LCS-CancelDeferredLocationArg,

LCS-LocationPrivacySettingArg,

LCS-PruAssociationArg,

LCS-PruAssociationRes,

LCS-PruDisassociationArg,

LCS-SLMTLRArg,

LCS-SLMTLRRes,

LCS-DLRSPPTransportArg,

LCS-DLRSPPTransportRes,

LCS-ULRSPPTransportArg,

LCS-ULRSPPTransportRes,

CapabilityInformationArg,

CapabilityInformationRes

FROM SS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3)

ss-DataTypes (2) version18 (18)}

-- imports MAP-SS-data types

RegisterCC-EntryRes

FROM MAP-SS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-DataTypes (14) version21 (21)}

-- imports MAP-errors

illegalSS-Operation, ss-ErrorStatus, ss-NotAvailable, ss-SubscriptionViolation,

ss-Incompatibility, systemFailure, facilityNotSupported, callBarred, unexpectedDataValue, shortTermDenial, longTermDenial, dataMissing, forwardingViolation, forwardingFailed, positionMethodFailure, resourceLimitation

FROM MAP-Errors {

itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3)

map-Errors (10) version21 (21)}

-- imports SS-Errors

resourcesNotAvailable, maxNumberOfMPTY-ParticipantsExceeded, deflectionToServedSubscriber, invalidDeflectedToNumber, specialServiceCode, rejectedByUser, rejectedByNetwork, lcs-invalidPru, lcs-PruAssociationRej

FROM SS-Errors {

itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3)

ss-Errors (1) version18 (18)}

;

-- operations definition

processUnstructuredSS-Data	OPERATION ::= { -- Timer T(PUSSD)= 15s to 30s

ARGUMENT	SS-UserData

RESULT	SS-UserData

-- optional

ERRORS	{

systemFailure |

unexpectedDataValue}

CODE	local:19 }

notifySS	OPERATION ::= {

ARGUMENT	NotifySS-Arg

CODE	local:16 }

forwardChargeAdvice	OPERATION ::= { -- Timer T(AoC)= 1s to 40s

ARGUMENT	ForwardChargeAdviceArg

RETURN RESULT TRUE

CODE	local:125 }

forwardCUG-Info	OPERATION ::= {

ARGUMENT	ForwardCUG-InfoArg

CODE	local:120 }

buildMPTY	OPERATION ::= { -- Timer T(BuildMPTY)= 5s to 30s

RETURN RESULT TRUE

ERRORS	{

illegalSS-Operation |

ss-ErrorStatus |

ss-NotAvailable |

ss-Incompatibility |

systemFailure |

resourcesNotAvailable |

maxNumberOfMPTY-ParticipantsExceeded}

CODE	local:124 }

holdMPTY	OPERATION ::= { -- Timer T(HoldMPTY)= 5s to 30s

RETURN RESULT TRUE

ERRORS	{

illegalSS-Operation |

ss-ErrorStatus |

ss-Incompatibility |

facilityNotSupported |

systemFailure}

CODE	local:123 }

retrieveMPTY	OPERATION ::= { -- Timer T(RetrieveMPTY)= 5s to 30s

RETURN RESULT TRUE

ERRORS	{

illegalSS-Operation |

ss-ErrorStatus |

ss-Incompatibility |

facilityNotSupported |

systemFailure}

CODE	local:122 }

splitMPTY	OPERATION ::= { -- Timer T(SplitMPTY)= 5s to 30s

RETURN RESULT TRUE

ERRORS	{

illegalSS-Operation |

ss-ErrorStatus |

ss-Incompatibility |

facilityNotSupported |

systemFailure}

CODE	local:121 }

explicitCT	OPERATION ::= { -- Timer T(ECT)= 5s to 15s

RETURN RESULT TRUE

ERRORS	{

illegalSS-Operation |

ss-ErrorStatus |

ss-NotAvailable |

ss-Incompatibility |

facilityNotSupported |

systemFailure |

resourcesNotAvailable |

callBarred}

CODE	local:126 }

accessRegisterCCEntry	OPERATION ::= { -- Timer T(AccRegCCEntry)= 30s

ARGUMENT	AccessRegisterCCEntryArg

RESULT	RegisterCC-EntryRes

ERRORS	{

systemFailure |

dataMissing |

unexpectedDataValue |

callBarred |

illegalSS-Operation |

ss-ErrorStatus |

ss-Incompatibility |

shortTermDenial |

longTermDenial |

facilityNotSupported}

CODE	local:119 }

-- the timer value is defined by T308, see also in TS 24.008 for definition of timer T308

callDeflection	OPERATION ::= { -- Timer T(CD)= 30s

ARGUMENT	CallDeflectionArg

RETURN RESULT TRUE

ERRORS	{

illegalSS-Operation |

ss-ErrorStatus |

ss-NotAvailable |

ss-Incompatibility |

facilityNotSupported |

systemFailure |

resourcesNotAvailable |

forwardingViolation |

callBarred |

deflectionToServedSubscriber |

invalidDeflectedToNumber |

specialServiceCode |

forwardingFailed}

CODE	local:117 }

-- the timer value is defined by T305, see also in TS 24.008 for definition of timer T305

-- extensionContainer shall not be used with this operation

userUserService	OPERATION ::= { -- Timer T(UUS3)= 10s

ARGUMENT	UserUserServiceArg

RETURN RESULT TRUE

ERRORS	{

illegalSS-Operation |

ss-ErrorStatus |

ss-NotAvailable |

ss-Incompatibility |

facilityNotSupported |

systemFailure |

resourcesNotAvailable |

rejectedByNetwork |

rejectedByUser}

CODE	local:118 }

-- The timer value for UUS3 is 10s; it is applicable only if UUS3 is activated by FACILITY

-- message. If UUS service (UUS1, UUS2 or UUS3) is activated by SETUP message, no timers are

-- needed. In those cases Return Result or Return Error must be received within certain call

-- control messages, see 3GPP TS 24.087.

-- extensionContainer shall not be used with this operation.

lcs-LocationNotification	OPERATION ::= { -- Timer T(LCSN)= 10s to 20s

ARGUMENT	LocationNotificationArg

RESULT	LocationNotificationRes

ERRORS	{

systemFailure |

unexpectedDataValue}

CODE	local:116 }

lcs-MOLR	OPERATION ::= { -- Timer T(LCSL)= 10s to 300s

ARGUMENT	LCS-MOLRArg

RESULT	LCS-MOLRRes

ERRORS	{

systemFailure |

unexpectedDataValue |

dataMissing |

facilityNotSupported |

ss-SubscriptionViolation |

positionMethodFailure}

CODE	local:115 }

lcs-AreaEventRequest	OPERATION ::= { -- Timer T(LCSN)= 10s to 20s

ARGUMENT	LCS-AreaEventRequestArg

RETURN RESULT TRUE

ERRORS	{

systemFailure |

facilityNotSupported |

unexpectedDataValue}

CODE	local:114 }

lcs-AreaEventReport	OPERATION ::= { -- Timer T(LCSL)= 10s to 300s

ARGUMENT	LCS-AreaEventReportArg

RETURN RESULT TRUE

ERRORS	{

systemFailure |

unexpectedDataValue |

facilityNotSupported}

CODE	local:113 }

lcs-AreaEventCancellation	OPERATION ::= { -- Timer T(LCSN)= 10s to 20s

ARGUMENT	LCS-AreaEventCancellationArg

RETURN RESULT TRUE

ERRORS	{

systemFailure |

facilityNotSupported |

unexpectedDataValue}

CODE	local:112 }

lcs-PeriodicLocationRequest	OPERATION ::= { -- Timer T(LCSN)= 10s to 20s

ARGUMENT	LCS-PeriodicLocationRequestArg

RESULT	LCS-PeriodicLocationRequestRes

ERRORS	{

systemFailure |

facilityNotSupported |

unexpectedDataValue |

dataMissing }

CODE	local: 111 }

lcs-LocationUpdate	OPERATION ::= { -- Timer T(LCSN)= 10s to 20s

ARGUMENT	LCS-LocationUpdateArg

RESULT	LCS-LocationUpdateRes

ERRORS	{

systemFailure |

unexpectedDataValue}

CODE	local: 110 }

lcs-PeriodicLocationCancellation	OPERATION ::= { -- Timer T(LCSN)= 10s to 20s

ARGUMENT	LCS-PeriodicLocationCancellationArg

RETURN RESULT TRUE

ERRORS	{

systemFailure |

unexpectedDataValue}

CODE	local: 109 }

lcs-PeriodicTriggeredInvoke	OPERATION ::= { -- Timer T(LCSN)= 10s to 20s

ARGUMENT	LCS-PeriodicTriggeredInvokeArg

RESULT	LCS-PeriodicTriggeredInvokeRes

ERRORS	{

systemFailure |

facilityNotSupported |

unexpectedDataValue |

dataMissing |

resourceLimitation }

CODE	local: 108 }

lcs-EventReport	OPERATION ::= { -- Timer T(LCSL)= 10s to 300s

ARGUMENT	LCS-EventReportArg

RESULT	LCS-EventReportRes

ERRORS	{

systemFailure |

facilityNotSupported |

ss-SubscriptionViolation |

unexpectedDataValue |

dataMissing |

resourceLimitation }

CODE	local: 107 }

lcs-CancelDeferredLocation	OPERATION ::= { -- Timer T(LCSN)= 10s to 20s

ARGUMENT	LCS-CancelDeferredLocationArg

RETURN RESULT TRUE

ERRORS	{

systemFailure |

unexpectedDataValue}

CODE	local: 106 }

lcs-MSCancelDeferredLocation	OPERATION ::= { -- Timer T(LCSN)= 10s to 20s

ARGUMENT	LCS-CancelDeferredLocationArg

RETURN RESULT TRUE

ERRORS	{

systemFailure |

unexpectedDataValue}

CODE	local: 105 }

lcs-LocationPrivacySetting	OPERATION ::= { -- Timer T(LCSN)= 10s to 20s

ARGUMENT	LCS-LocationPrivacySettingArg

RETURN RESULT TRUE

ERRORS	{

systemFailure |

unexpectedDataValue}

CODE	local: 104 }

lcs-PruAssociation	OPERATION ::= { -- Timer T(LCSN)= 10s to 20s

ARGUMENT	LCS-PruAssociationArg

RESULT	LCS-PruAssociationRes

ERRORS	{

lcs-invalidPru |

lcs-PruAssociationRej |

systemFailure |

unexpectedDataValue}

CODE	local: 103 }

lcs-PruDisassociation	OPERATION ::= { -- Timer T(LCSN)= 10s to 20s

ARGUMENT	LCS-PruDisassociationArg

RETURN RESULT TRUE

ERRORS	{

systemFailure |

unexpectedDataValue}

CODE	local: 102 }

lcs-SLMOLR	OPERATION ::= { -- Timer T(LCSL)= 10s to 300s

ARGUMENT	LCS-SLMOLRArg

RESULT	LCS-SLMOLRRes

ERRORS	{

systemFailure |

unexpectedDataValue |

dataMissing |

facilityNotSupported |

ss-SubscriptionViolation |

positionMethodFailure}

CODE	local:101 }

lcs-SLMTLR	OPERATION ::= { -- Timer T(LCSL)= 10s to 300s

ARGUMENT	LCS-SLMTLRArg

RESULT	LCS-SLMTLRRes

ERRORS	{

systemFailure |

unexpectedDataValue |

dataMissing |

facilityNotSupported }

CODE	local:100 }

lcs-DLRSPPTransport	OPERATION ::= { -- Timer T(LCSL)= 10s to 300s

ARGUMENT	LCS-DLRSPPTransportArg

RESULT	LCS-DLRSPPTransportRes

ERRORS	{

systemFailure |

facilityNotSupported }

CODE	local: 99 }

lcs-ULRSPPTransport	OPERATION ::= { -- Timer T(LCSL)= 10s to 300s

ARGUMENT	LCS-ULRSPPTransportArg

RESULT	LCS-ULRSPPTransportRes

ERRORS	{

systemFailure |

facilityNotSupported }

CODE	local: 98 }

capabilityInformation OPERATION ::= { -- Timer T(CAP)= 5s to 30s

ARGUMENT	CapabilityInformationArg

RESULT	CapabilityInformationRes

ERRORS	{

systemFailure |

unexpectedDataValue |

facilityNotSupported

CODE	local: x }

* * * Next Change * * * *

#### 4.2.2.xx	capabilityInformation (MS -->Network, Network-->MS)

This operation is invoked by the LMF (MS) to notify the MS (LMF) and request extended Facility capabilities, enabling the transmission of messages with content lengths exceeding 255 bytes. When the error or failure has been received by the LMF or MS, it indicates the MS or LMF does not support Extended Facility.

Refer to 3GPP TS 24.571 [19] for capabilityInformation operations for 5G LCS.

* * * Next Change * * * *

4.4.2	ASN.1 data types

This clause provides an ASN.1 module defining the abstract data types in operations and errors specification. Only data types which are specific for this specification are defined. All other data types are imported from MAP together with the import of operations and errors.

[…]

LCS-ULRSPPTransportRes::= SEQUENCE {

}

CapabilityInformationArg::= SEQUENCE {

extendedFacilityInd	[0]	IMPLICIT BOOLEAN,

...

}

CapabilityInformationRes::= SEQUENCE {

extendedFacilityInd	[0]	IMPLICIT BOOLEAN,

...

}

.#END

* * * Next Change * * * *

4.4.3.a	capabilityInformationArg

The capabilityInformationArg identifier refers to the message sent from the LMF (MS) to the MS (LMF) to request capability parameters.

4.4.3.b	capabilityInformationRes

The capabilityInformationRes identifier refers to the message sent from the MS (LMF) to the LMF (MS) of its capability parameters.

4.4.3.c	extendedFacilityInd

The extendedFacilityInd refers to whether the MS or LMF supports the Extended Facility.

* * * * End of Changes * * * *