3GPP TSG-CT WG4 Meeting #131	C4-254278

# Sophia Antipolis, France; 13th – 17th October 2025                                              was4100

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.175 | CR | 0097 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the MultiDcMediaSpecs attribute to MediaInfo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in clause AA.2.4.2.2 of TS 23.228, If the media type is data channel and multiple data channels are multiplexed in the same m line in the SDP offer or answer, multiple media specifications for all multiplexed data channels are included, each of which represents a data channel being multiplexed in the media item. Each media specification includes the following media description attributes: - Media Type: … So, this CR propose to add the MultiDcMediaSpecs attribute to represent the multiple data channel media specifications which is multiplexed in the same media item. The condition of the presence of dcMediaSpec is when the media information identified by the mediaId is not multiplexed. So, this CR proposed to update the description for the dcMediaSpec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the MultiDcMediaSpecs attribute; - Update the description for the dcMediaSpec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with SA2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.5, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following APIs: TS29175_Nimsas_SessionEventControl.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Clause 6.1.6.2.5, change the cardinality of multiDcMediaSpecs to 1..N. - Clause 6.1.6.2.5, update the attribute description. - A.2, add the minItems in the OpenAPI and update the description. |  |  |  |  |  |  |  |  |

* * *1st Change * * * *

6.1.6.2.5	Type: MediaInfo

Table 6.1.6.2.5-1: Definition of type MediaInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mediaId | MediaId | M | 1 | The media ID uniquely identifies this media item within the list. The identity is allocated by IMS AS. |
| mediaType | MediaType | M | 1 | The media type of the media for which the notification is generated. |
| dcMediaSpec | DcMediaSpec | C | 0..1 | The data channel media specification includes the relevant media attributes of interest to the consumer. It shall be included if the mediaType is set to "DC" and the media information identified by the mediaId is not multiplexed. |
| multiDcMediaSpecs | array(DcMediaSpec) | C | 1..N | The multiple data channel media specifications which is multiplexed in the same media item. It shall be included if the mediaType is set to "DC" and the media information identified by the mediaId is multiplexed. |
| mediaSuspended | boolean | C | 0..1 | This IE indicates whether the media is suspended or not. It shall be contained if the media is suspended or resumed as specified in 3GPP TS 24.610 [18] and 3GPP TS 24.186 [19]. - true: the media is suspended. - false: the media is resumed. |
| mediaRemovalInd | boolean | C | 0..1 | This IE indicates whether the media is to be removed or not. It shall be only present with true value if the media is to be removed, e.g. removal of bootstrap data channel media during the interaction with supplementary service, or removal of application data channel in other cases. Presence with false value shall be prohibited. |

* * * 2nd Change * * * *

A.2	Nimsas_SessionEventControl API

openapi: 3.0.0

info:

title: 'IMS AS SessionEventControl Service'

version: 1.1.0-alpha.4

description: |

Nimsas_SessionEventControl Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.175 V19.3.0; IP Multimedia Subsystem; IP Multimedia Subsystem (IMS) Application

Server (AS) Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.175/'

servers:

- url: '{apiRoot}/nimsas-sec/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

[…]

MediaInfo:

description: The media information related to the notification event.

type: object

required:

- mediaId

- mediaType

properties:

mediaId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaId'

mediaType:

$ref: '#/components/schemas/MediaType'

dcMediaSpec:

$ref: '#/components/schemas/DcMediaSpec'

multiDcMediaSpecs:

description: Includes a list of multiplexed data channel media specifications.

type: array

minItems: 1

items:

$ref: '#/components/schemas/DcMediaSpec'

mediaSuspended:

type: boolean

mediaRemovalInd:

type: boolean

enum:

- true

[…]

* * * End of Changes * * * *