3GPP TSG-CT WG4 Meeting #131	C4-254377

# Sophia Antipolis, France; 13rd – 17th October 2025					(Revision of C4-254345)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.328 | CR | 0661 | rev | 7 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add HSS subscription to IMS AS and event notification procedure via Sh interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-07-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to 3GPP TS 23.228, the IMS AS receives subscription requests from HSS related to supported subscriber specific IMS DC events via the N71 reference point using the Nimsas_ImsEE_Subscribe service operation, or via the Sh reference point. So, this CR proposes to add the procedure of IMS AS registration to HSS via Sh interface. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the HSS subscription to IMS AS and event notification procedure. - Add information element content related to the procedure, i.e., Any User Indication, Subscription Identity, Notification Address, Notification Correlation Identity, Event Reporting Options, Event List, Event Reports. - Add the Sh-ee message to Diameter command mapping. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HSS cannot subscribe to AS via Sh interface. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x, 6.x (new), 6.x.1 (new), 6.x.2 (new), 7.n1 (new), 7.n2 (new), 7.n3 (new), 7.n4 (new), 7.n5 (new), 7.n6 (new), 7.n7 (new), 7.n8 (new), 7.n9 (new), 7.n10 (new), 7.n11 (new), 7.n12 (new), 7.n13 (new), 7.n14 (new), 7.n15 (new), 7.n16 (new), Annex A, A.1, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: add the event notification command. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.x	Functional classification of Sh-ee procedures

Operations on the Diameter application of Sh-ee are classified in functional groups:

1	Subcription/notification procedure for IMS events

-	An HSS can subscribe to receive notifications from the AS of IMS event.

-	The AS can notify an HSS of IMS events for which the HSS previously had subscribed.

* * * Next Change * * * *

6.x	IMS events subscription/notification produres

6.x.1	Subscription to event notifications (Sh-Event-Subs)

This procedure is used between the AS and the HSS. The procedure is invoked by the HSS and is used:

-	To subscribe to notifications for when a specific IMS event for a specified IMS Public User Identity occurs at the AS.

This procedure is mapped to the commands Event-Subscribe-Request/Answer in the Diameter application of Sh-ee for IMS events subscription/notification specified in 3GPP TS 29.329 [5]. Tables 6.x.1-1 and 6.x.1-2 detail the information elements involved.

Table 6.x.1-1: Sh-Event-Subs

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| Subscriber Identity | Public-Identity | C | IMS Public User Identity for which notifications of IMS events are requested. See clause 7.1 for the content of this AVP. (NOTE) |
| Any User Indication (see 7.n1) | Any-User-Indication | C | Identifies whether the request applies to any UE. This attribute is set to "true" if applicable for any UE, otherwise, it is set to "false". (NOTE) |
| Subscription request type (See 7.7) | Subs-Req-Type | M | This information element indicates the action requested on subscription to notifications. |
| Send Data Indication (See 7.13) | Send-Data-Indication | O | This information element requests that immediate report is to be sent in the response. |
| Expiry Time (See 7.12) | Expiry-Time | O | Gives the absolute time requested at which the subscription expires. |
| Event Reporting Options (see 7.n5) | Event-Reporting-Options | M | Describes how the reports of the event to be generated. |
| Event List (See 7.n10) | Event-List | M | Describes the IMS events to be subscribed. |
| Notification Address (See 7.n3) | Notification-Address | M | Identifies the recipient of notifications sent by the HSS. When an HSS subscribes on behalf of another NF, the notification address identifies a resource under the authority of the other NF. |
| Notification Correlation Identity (See 7.n4) | Notification-Correlation-Identity | O | This attribute identifies the notification correlation identity shall be present by HSS in subscription. The value of this IE shall be unique per subscription for a given HSS. |
| NOTE: If the event subscription applies for a specific UE, Subscriber Identity shall be included; otherwise Any User Indication set to true shall be included. |  |  |  |

Table 6.x.1-2: Sh-Event-Subs Resp

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| Subscription Identity (See 7.n2) | Subscription-Identity | M | Identifies the subscription which is globally unique. |
| Ims Event Report (see 7.n14) | Ims-Event-Report | O | If immediate report is requested, this IE shall contain the associated status to the event(s) being requested if the status associated to at least one of the requested event(s) is available at the time of subscription. |
| Event List (See 7.n10) | Event-List | M | Describes the IMS events successfully subscribed. |
| Result (See 7.5) | Result-Code / Experimental-Result | M | Result of the request. Result-Code AVP shall be used for errors defined in the Diameter base protocol (see IETF RFC 6733 [44]). Experimental-Result AVP shall be used for Sh-ee errors. This is a grouped AVP which contains the 3GPP Vendor ID in the Vendor-Id AVP, and the error code in the Experimental-Result-Code AVP. |
| Expiry Time (See 7.12) | Expiry-Time | O | Acknowledges the absolute time at which the subscription expires. |

6.x.1.1	Detailed behaviour

Upon reception of the Sh-Event-Subs request, the AS shall, in the following order (if there is an error in any of the following steps the AS shall stop processing and return the corresponding error code, see 3GPP TS 29.329 [5] and 3GPP TS 29.229 [7]), process the hereafter steps that, unless otherwise stated, apply both for the Subscription request type information element indicating the request is to subscribe or to unsubscribe:

1.	The AS shall check that the HSS is allowed to subscribe to notifications (Sh-Event-Subs) for the IMS event by checking the identity of the subscriber.

-	If this HSS does not have Sh-Event-Subs permission to subscribe to IMS event notifications, Experimental-Result shall be set to DIAMETER_ERROR_OPERATION_NOT_ALLOWED in the Sh-Event-Subs Response.

2.	Upon the reception of the request, the AS shall supply the subscription identity and set the Result-Code to DIAMETER_SUCCESS in the Sh-Event-Subs response.

3.	The AS should include in the Sh-Event-Subs Response an Expiry Time IE with the absolute time at which the subscription expires in the case of a successful subscription. This time may be earlier than the requested expiry time. No notification shall be sent to the HSS after the expiration time. If the AS receives a Sh-Event-Subs Request without the Expiry Time IE, the AS should treat it as a request for an unlimited subscription.

If a subsequent request is received by the AS where the Expiry Time IE is present but different from what the AS had previously stored, the AS should replace the stored expiration time with what was received in the request.

4.	If the Sh-Event-Subs Request contains a One Time Report in Event Reporting Options, the HSS should include the Event Report in the Sh-Event-Subs response.

6.x.2	Event Notifications (Sh-Event-Notif)

This procedure is used between the HSS and the AS. The procedure is invoked by the AS and is used:

-	To inform the HSS of IMS event for which the HSS previously had subscribed to receive Notifications for, using Sh-Event-Notif.

This procedure is mapped to the commands Event-Notification-Request/Answer in the Diameter application for of Sh-ee for IMS events subscription/notification specified in 3GPP TS 29.329 [5]. Tables 6.x.2-1 and 6.x.2-2 detail the involved information elements.

Table 6.x.2-1: Sh-Event-Notif

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| Subscriber Identity | Public-Identity | M | IMS Public User Identity for which notifications of IMS evnts are requested. See clause 7.1 for the content of this AVP. |
| Ims Event (See 7.n11) | Ims-Event | M | Event that is notified. |
| Event Report (See 7.n13) | Ims-Event-Report | O | Report Information per event type. |
| Notification Correlation Identity (See 7.n4) | Notification-Correlation-Identity | M | String identifying a notification correlation identity received in the subscribe request. |
| Session Identity (See 7.n16) | Session-Identity | O | Identifies the session identity. |
| Timestamp (See 7.n16) | Timestamp | M | Time at which the event is observed. |

Table 6.x.2-2: Sh-Event-Notif Resp

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| Result (See 7.5) | Result-Code / Experimental-Result | M | Result of the request. Result-Code AVP shall be used for errors defined in the Diameter base protocol (see IETF RFC 6733 [44]). Experimental-Result AVP shall be used for Sh-ee errors. This is a grouped AVP which contains the 3GPP Vendor ID in the Vendor-Id AVP, and the error code in the Experimental-Result-Code AVP. |

6.x.2.1	Detailed behaviour

Upon reception of the Sh-Event-Notif request, the HSS shall, in the following order:

1.	The HSS shall check that the AS is allowed to notify (Sh-Event-Notif) the subscribed IMS event by checking the notification correlation identity.

-	If the HSS does not consider the notification correlation identity as a valid notification identity, Experimental-Result shall be set to DIAMETER_INVALID_AVP_VALUE in the Sh-Event-Notif Response.

* * * Next Change * * * *

7.n1	Any User Indication

This information element identifies whether the request applies to any UE. See 3GPP TS 29.329 [5] for the detailed definition of the AVP.

7.n2	Subscription Identity

This information element indicates the subscription identity. See 3GPP TS 29.329 [5] for the detailed definition of the AVP.

7.n3	Notification Address

This information element identifies the recipient of notifications sent by the HSS. See 3GPP TS 29.329 [5] for the detailed definition of the AVP.

7.n4	Notification Correlation Identity

This information element indicates the notification correlation identity shall be present by HSS in subscription. See 3GPP TS 29.329 [5] for the detailed definition of the AVP.

7.n5	Event Reporting Options

This information element indicates how the reports of the event are to be generated:

-	Report Mode (see 7.n6)

-	One Time Report (see 7.n7)

-	One Time Notification (see 7.16)

-	Max Number of Reports (see 7.n8)

-	Report Period (see 7.n9)

-	Expiry Time (see 7.12)

7.n6	Report Mode

This information element indicates the mode of report (e.g, periodic reporting along with periodicity, reporting based on event detection). See 3GPP TS 29.329 [5] for the detailed definition of the AVP.

7.n7	One Time Report

This information element indicates whether immediateReport is requested. See 3GPP TS 29.329 [5] for the detailed definition of the AVP.

7.n8	Max Number of Reports

This information element indicates maximum number of reports. See 3GPP TS 29.329 [5] for the detailed definition of the AVP.

7.n9	Report Period

This information element indicates the interval time between which the event notification is reported. See 3GPP TS 29.329 [5] for the detailed definition of the AVP.

7.n10	Event List

This information element contains the IMS events to be subscribed in a subscription request or the events successfully subscribed for this subscription in a subscription response:

-	Ims Event (see 7.n11)

-	Ims Event Filter (see 7.n12)

7.n11	Ims Event

This information element indicates the IMS event. See 3GPP TS 29.329 [5] for the detailed definition of the AVP.

7.n12	Ims Event Filter

This information element indicates list of IMS event filters. See 3GPP TS 29.329 [5] for the detailed definition of the AVP.

7.n13	Ims Event Report

This information element contains the associated status to the event(s) being requested if the status associated to at least one of the requested event(s) is available at the time of subscription:

-	Ims Event (see 7.n11)

-	Ims Event Report Information (see 7.n15)

-	Session Identity (see 7.n16)

7.n14	Ims Event Report Information

This information element indicates report information per event type. See 3GPP TS 29.329 [5] for the detailed definition of the AVP.

7.n15	Session Identity

This information element indicates the IMS session identity. See 3GPP TS 29.329 [5] for the detailed definition of the AVP.

7.n16	Timestamp

This information element indicates the time at which the event is observed. See 3GPP TS 29.329 [5] for the detailed definition of the AVP.

* * * Next Change * * * *

Annex A (normative):
Mapping of Sh and Sh-ee operations and terminology to Diameter

A.1	Introduction

This appendix gives mappings from Sh and Sh-ee to Diameter protocol elements. Diameter protocol elements are defined in 3GPP TS 29.329 [5].

* * * Next Change * * * *

A.3	Sh-ee message to Diameter command mapping

The following table defines the mapping between stage 2 operations and Diameter commands:

Table A.2.1: Sh-ee message to Diameter command mapping

| Sh-ee message | Source | Destination | Command-Name | Abbreviation |
| --- | --- | --- | --- | --- |
| Sh-Event-Subs | HSS | AS | Event-Subscribe-Request | ESR |
| Sh-Event-Subs Resp | AS | HSS | Event-Subscribe-Answer | ESA |
| Sh-Event-Notif | AS | HSS | Event-Notification-Request | ENR |
| Sh-Event-Notif Resp | HSS | AS | Event-Notification-Answer | ENA |

* * * End of Changes * * * *