3GPP TSG-CT WG4 Meeting #131	C4-254382

# Sophia Antipolis, France; 13th – 17th October 2025

# Title:	LS on LCS User Plane Connection Support

# Release:	Rel-18

# Work Item:	5G_eLCS_Ph3

Source:	CT4

To:	SA2

Cc:	TSG CT

Contact Person:

Name:	Jones Lu

Tel. Number:

E-mail Address:	jones.lu@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR0380 of TS 29.572, CR1266 of TS 29.518, CR0379 of TS 29.572

1. Overall Description:

During implementation of LCS User Plane support in stage 3, CT4 has observed some issues in stage 2 requirements as specified in TS 23.273:

ISSUE-1: The UE ID used for LCS-UPP connection support

To allow the LMF to use the LCS-UPP connection for subsequent location request for the same UE, TS 23.273 (in clause 6.1.2 & clause 6.2) has specified that the AMF shall provide the SUPI and/or GPSI to the LMF if user plane positioning is supported and subsequently shall always provide the consistent UE ID:

"11.	The AMF includes the scheduled location time in the Nlmf_Location_DetermineLocation service operation sent towards the LMF. If received in step 5, the AMF provides the LPHAP indication to the LMF.

If H-GMLC contact address is received in step 5, AMF also includes the H-GMLC contact address and the Notification Correlation ID in the Nlmf_Location_DetermineLocation service operation and sends towards the LMF.

Based on UE user plane positioning capabilities, if the target UE supports the UE user plane positioning capability for LCS-UPP, the AMF shall provide the SUPI and/or GPSI of the target UE (see TS 29.572 [12]) as UE identity in the Nlmf_Location_DetermineLocation Request.

NOTE 6:	In this case, the AMF ensures that a consistent UE identity is used for a particular LMF by implementation."

The requirement cannot resolve the scenario that one UE may have multiple GPSIs. In case the LCS-UPP connection is established with GPSI-1 but another AF subsequently performs a location request with GPSI-2, the AMF can only provide the GPSI-2 to the LMF (because AMF cannot provide multiple GPSIs to the LMF in a location request). As a result, this location request cannot use user plane positioning as expected because the LMF cannot determine that GPSI-1 and GPSI-2 belong to the same UE.

To resolve this issue, CT4 has recommended that the SUPI of the UE shall be used as the consistent UE ID if user plane positioning is supported. CT4#131 has agreed CR0380 of TS 29.572 and CR1266 of TS 29.518 which implement the above recommendation (attached).

ISSUE-2: UP Notification delivery after AMF change

TS 23.273 (clause 6.18.3) has specified that the AMF may inform the LMF with UP config after AMF is changed in step 0 and require the LMF to deliver the UP Notify message towards the AMF:

"0.	[Conditional] The LMF discovers a need to change LMF or decides to terminate the user plane connection, e.g., after receiving an event report via user plane from UE or after receiving an AMF relocation information from a target AMF (target AMF obtains from source AMF the UE LCS-UP context which indicates UE has maintained LCS-UP connection with the source LMF, the target AMF may inform the source LMF about the AMF change using the Nlmf_Locatoion_UPConfig request).

1a.	[Conditional] The LMF sends an Nlmf_Location_UPNotify message that includes an indication of connection change or termination and if connection change is requested then the message may include a target LMF identification. The address of the AMF was provided to LMF as a "Notification Target Address" in latest Nlmf_Location_UPConfig message or Nlmf_Location_UP Subscribe message. If the LMF is going to terminate the user plane connection, only step 1a and step 6 are performed and other steps are skipped."

If after AMF change, the new AMF does not inform the LMF and provide the callback URI for UP Notify in step 0, the LMF will deliver the UP Notify message using the stored callback URI of the old AMF.

CT4 has recommended that the new AMF shall always inform the serving LMF for existing LCS-UPP connection after AMF change with UP Config, thus the LMF can know the serving AMF is changed and also update the callback URI of the UP Notify towards the new AMF. CT4#131 has agreed CR0379 of TS 29.572 which implements the recommendation (attached).

2. Actions:

To SA2 group.

ACTION: 	CT4 kindly ask SA2 to review the above issues and confirm (before CT#110) whether the CT4 recommendations are the right way forward. If the CT4 recommendations are acceptable CT4 also kindly ask SA2 to align the stage 2 specification accordingly.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm