3GPP TSG RAN WG2 Meeting #132	R2-2508065

Dallas, USA, Nov. 17th – 21th, 2025

Agenda Item:	9.2.2

Source:	Xiaomi

Title:	Support of Topology 2 for A-IoT

Document for:	Discussion and Decision

# Introduction

This contribution discusses the support of Topology 2 for A-IoT, based on the related objective in Rel-20 A-IoT phase 2 WID [1].

# Impact on Uu RRC signalling by A-IoT

## Impact on RRC signalling content due to A-IoT NGAP information

Based on the below agreement from last meeting [2], which A-IoT NGAP information needs to signaled in Uu by the gNB to the UE reader needs to be evaluated.

Agreements

1.  gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN.  FFS what information

Specifically, this evaluation involves in two aspects: (1) which A-IoT NGAP IE received by the gNB from the A-IoT CN is relevant to the operations in the A-IoT radio interface (thus necessarily visible to/used by the UE reader); (2) whether an A-IoT NGAP IE can be directly forwarded to the UE reader and included in related field in R2D msg, or needs to be first transformed to the R2D msg field/info and then signaled to the UE reader in DL.

For R2D, DL direction, below Table 1 exhausts the Rel-19 A-IoT NGAP IEs, with the evaluation in terms of their needs for A-IoT radio interface and whether each can be directly forwarded in Uu DL RRC signalling to the UE reader.

Table 1 Analyses on the Uu impact due to A-IoT NGAP information for R2D, DL direction

| No. | A-IoT NGAP IE items | Child IEs/Info | Relevance to A-IoT radio interface operation | Directly forwarded in DL RRC or not |
| --- | --- | --- | --- | --- |
| Information in “Inventory Request Transfer” |  |  |  |  |
| 1 | A-IoT Correlation Identifier | A-IoT Correlation Identifier (OCTET STRING) | Used to derive “Transaction ID” field in A-IoT Paging message | Related to ID management, See 2.2 |
| 2 | A-IoT Device Identification Requested | Paging ID for single device | Used to fill in the “Paging ID” field in A-IoT paging message | Directly forwarded |
|  |  | Paging ID for group devices |  |  |
|  |  | No paging ID for all devices |  |  |
| 3 | Requested Service Area Information | Global gNB ID | N/A | FFS (Pending RAN3 and SA2 conclusion progress) |
|  |  | Reader Index | FFS NOTE: pending RAN3 and SA2 progress on UE reader selection |  |
|  |  | A-IoT Area ID (list) |  |  |
| 4 | Inventory Assistance Information | Expected D2R Message Size | Used for UE reader’s scheduling in A-IoT radio interface | Directly forwarded. |
|  |  | Approximate Number of Target A-IoT Devices |  |  |
|  |  | Time Interval |  |  |
| 5 | Follow on Command Indication | ENUMERATED (true, ...) | Used for UE reader to decide when to release the A-IoT AS contexts for the on-going service. | Directly forwarded. |
| Information in “Command Request Transfer” |  |  |  |  |
| 6 | A-IoT Correlation Identifier | A-IoT Correlation Identifier (OCTET STRING) | Used to associate the command request message from A-IoT CN with the corresponding inventory message in the same service | Related to ID management, See 2.2 |
| 7 | RAN A-IoT Device NGAP ID | RAN A-IoT Device NGAP ID INTEGER (0…232 -1) | Used to associate the command request message with the associated AS ID. | Related to ID management, See 2.2 |
| 8 | A-IoT NAS PDU | OCTET STRING | Used to fill in “Data SDU” field in R2D Upper Layer data transfer msg. (invisible to UE reader or gNB) | Directly forwarded |
| 9 | Criticality Diagnostics | Procedure Code, Triggering Message, etc. | N/A NOTE: processed by gNB or AMF | N/A |
| Information in “Inventory Response Transfer” |  |  |  |  |
| 10 | A-IoT Correlation Identifier | A-IoT Correlation Identifier (OCTET STRING) | N/A NOTE: The whole IE “Inventory Response Transfer” is used for negotiation between NW nodes, thus not necessarily visible to the UE reader. | N/A |
| 11 | Criticality Diagnostics | Procedure Code, Triggering Message, etc. |  |  |
| Information in “Inventory Failure Transfer” |  |  |  |  |
| 12 | A-IoT Correlation Identifier | A-IoT Correlation Identifier (OCTET STRING) | N/A NOTE: The whole IE “Inventory Failure Transfer” is used for negotiation between NW nodes, thus not necessarily visible to the UE reader. | N/A |
| 13 | Cause | Radio Network Layer Cause, Transport Layer Cause, NAS Cause, etc. |  |  |
| 14 | Criticality Diagnostics | Procedure Code, Triggering Message, etc. |  |  |

Something to be noted for Table 1:

- For the ID management related IEs (i.e. No. 1, 5, 6, 7), how to handle them depends on the specific mechanism adopted for the ID mapping, i.e. whether it is the gNB or UE reader that maps the NGAP related IDs and the A-IoT radio interface related IDs. This is to be discussed below in Section 2.2.

- For the Service Area related IEs (i.e. No. 3), how to handle them fundamentally depends on whether the service area in Topology 2 is still needed and if yes how it is specified. This is tightly related to UE reader selection which should be first justified by RAN3 and SA2.

Based on the above Table 1, it is proposed that at least for the information in A-IoT NGAP IE A-IoT Device Identification Requested, Inventory Assistance Information and A-IoT NAS PDU can be directly forwarded to the UE reader in DL RRC signaling, with the “Service Area” related IEs FFS pending RAN3 and SA2 discussion.

Proposal 1: For R2D, DL direction in Topology 2, the information in the following A-IoT NGAP IE needs to be signaled to the UE reader, and is directly forwarded by the gNB in DL RRC signalling:

- A-IoT Device Identification Requested;

- Follow on Command Indication;

- Inventory Assistance Information;

- A-IoT NAS PDU.

Proposal 1a: For R2D, DL direction, whether information related to A-IoT NGAP IE “Requested Service Area Information” needs to be signaled to the UE reader is pending RAN3 and SA2 progress on UE reader selection.

In fact, there is necessarily a similar evaluation in the D2R, UL direction as the counterpart. By contrast to R2D, DL direction, the situation for D2R, UL direction is much simpler, which only involves (a) whether/how the “Data SDU” in the “R2D Upper Layer Data transfer” message, corresponding to “Inventory Response” and “Command Response” in A-IoT NGAP respectively, needs to be signaled to the gNB and (b) whether the “AS ID” and “Transaction ID” maintained by the UE reader need to be made visible to the gNB.

For the D2R, UL direction, therefore, below Table 2 evaluates the relevance of the A-IoT radio interface information to A-IoT NGAP operation and whether each information is directly forwarded in Uu UL RRC signalling to the gNB.

Table 2 Analyses on the Uu impact due to A-IoT NGAP information for D2R, UL direction

| No. | D2R field/Info in A-IoT radio interface | Relevance to A-IoT NG AP IE |  | Directly forwarded in UL RRC or not |
| --- | --- | --- | --- | --- |
|  |  | Inventory Report | Command Response |  |
| 1 | Transaction ID | Used to derive the “A-IoT Correlation Identifier” in “Inventory Report Transfer” IE | Used to derive “A-IoT Correlation Identifier” field in “Command Response Transfer” IE | Related to ID management, See 2.2 |
| 2 | AS ID | Used to derive the “RAN A-IoT Device NGAP ID” in “Inventory Report Transfer” IE (NOTE: for each item in the Device Report List) | Used to derive the “RAN A-IoT Device NGAP ID” in “Command Response Transfer” IE | Related to ID management, See 2.2 |
| 3 | “Data SDU” field in “D2R Upper Layer Data Transfer” | Used to fill in the “A-IoT NAS PDU” in “Inventory Report Transfer” IE (NOTE: for each item in the Device Report List) | Used to fill in the “A-IoT NAS PDU” in “Command Response Transfer” IE | Directly forwarded |

Based on Table 2, it is proposed that the “D2R SDU” field corresponding to the A-IoT NAS PDU in A-IoT NGAP IEs “Inventory Report Transfer” and “Command Response Transfer” is directly forwarded by the UE reader to the gNB in UL RRC signaling.

Proposal 2: For D2R, UL direction in Topology 2, the “Data SDU” field in “D2R Upper Layer Data Transfer” message is directly forwarded by the UE reader to the gNB in UL RRC signalling (to be included AIoT NGAP IEs of “Inventory Report Transfer” and “Command Response Transfer”).

It is especially noted that for both R2D, DL directions and D2R, UL directions, how the A-IoT specific IDs are signaled, including AS ID vs. RAN A-IoT Device NGAP ID, and A-IoT Correlation Identifier vs. Transaction ID, depends on the ID management mechanism to be adopted in Topology 2, which is to be specifically discussed/concluded below in subclause 2.2.

## Identification management in Topology 2

First the device identification concerning “AS ID” and “RAN A-IoT Device NGAP ID” in Topology 2 is discussed. As per the “Command procedure” in [3] (See Annex 1), for the “Inventory + command” service, the gNB reader in Topology 1, upon reception of an inventory response msg, assigns an “RAN A-IoT Device NGAP ID” to identify the device from which the inventory response msg is received, and sends it along with the inventory response msg to the A-IoT CN. Along with the AS ID of each device assigned by the gNB reader itself, the gNB reader later:

- receives the “Command Request” msg from the A-IoT CN identified by the “RAN A-IoT Device NGAP ID”, and then:

  - For CBRA, maps it to the correct “AS ID”, and sends the “Command Request” msg to the correct device identified by this AS ID in A-IoT radio interface;

  - For CFA, assigns an AS ID and sends the “Command Request” msg to the device along with this assigned AS ID in A-IoT A-IoT radio interface.

- receives the “Command Response” msg from the device on the D2R resources assigned with the “AS ID”, maps it to the correct “RAN A-IoT Device NGAP ID” and sends the “Command Response” to the A-IoT CN along with this “RAN A-IoT Device NGAP ID” identifying the correct device.

For CFA in Rel-19, the AS ID is mainly used to schedule the D2R resources for the (re)transmission of “Command Response” or its segments; since a CFA procedure does not support multiple device access, it is not necessary to associate AS ID with the “RAN A-IoT Device NGAP ID” for a CFA procedure if parallel sessions are not supported in Rel-20. Therefore, such association between “RAN A-IoT Device NGAP ID” and “AS ID” mainly involves CBRA case.

Observation 1: For CBRA In Topology 1, gNB reader associates the “RAN A-IoT Device NGAP ID” and “AS ID” assigned by itself, and transforms the two ID with each other to identify the correct device to which the Command request message is signaled and from which the Command response message is received.

When it comes to Topology 2, the problem is that the “AS ID” and “RAN A-IoT Device NGAP ID” may be assigned by UE reader and gNB respectively, so how they can be associated with each other in CBRA case needs to be addressed. In particularly, the “AS ID” is the ID used by the A-IoT radio interface, it should be assigned by the A-IoT MAC protocol which is only located at the UE reader in Topology 2, but not at gNB anymore. This should be first confirmed by RAN2.

Proposal 3: In Topology 2, the AS ID is assigned by UE reader (by its A-IoT MAC layer).

Then for CBRA case there could be following two options that can enable the mapping between “RAN A-IoT Device NGAP ID” and “AS ID” in Topology 2 for correct device identification.

- [Option 1]: Mapping between “RAN A-IoT Device NGAP ID” and “AS ID” is maintained at the gNB, with “AS ID” signaled along with upper layer data for Command request/response in Uu RRC signalling.

A msg1/2 exchange is extra needed before step 1 for CBRA and then step 1 to step 10a are performed as follows:

- Step 1: UE reader receives the “D2R Upper Layer data transfer” msg for inventory Response from a device as in legacy Rel-19;

- Step 2: UE reader signals the assigned AS ID along with the device’s Inventory Response Data SDU to the gNB in UL RRC signalling;

- Step 3/4: gNB assigns the RAN A-IoT Device NGAP ID and sends it along with the AIoT NAS PDU in Inventory response NGAP msg to the A-IoT CN as in Topology 1, and also establishes the Mapping of “AS ID” and “RAN A-IoT Device NGAP ID”;

- Step 5/6/6a: When the gNB receives the Command request NGAP msg from the A-IoT CN, it maps the RAN A-IoT Device NGAP ID to the AS ID based on the mapping maintained in Step 4, and signals the AS ID along with the Command Request NAS PDU to the UE reader in DL RRC signalling;

- Step 7/8: UE reader, as in legacy Rel-19, performs the R2D and D2R Upper layer data transfer with the received AS ID for Command request and response;

- Step 9: UE reader signals the AS ID for the device’s Command Response Data SDU to the gNB;

- Step 10/10a: gNB maps the AS ID to the RAN A-IoT Device NGAP ID based on the mapping maintained in Step 4, and signals RAN A-IoT Device NGAP ID along with the Command Request NAS PDU to the A-IoT CN.

In this option, the AS ID is directly forwarded in the DL and UL RRC signalling and it is mapped by the gNB from/to the RAN A-IoT Device NGAP ID for Command request and Command response, respectively. RAN A-IoT Device NGAP ID is not visible in Uu in this option.

- [Option 2]: Mapping between “RAN A-IoT Device NGAP ID” and “AS ID” is maintained at the UE reader, with “RAN A-IoT Device NGAP ID” signaled along with Command request/response in Uu RRC signalling.

Compared with option 1, the differences in option 2 are summarized as follows:

- Step 4/4a: gNB, after assigning the RAN A-IoT Device NGAP ID and mapping it to the received AS ID, it signals the mapping relationship to the UE reader, instead of maintaining it by itself. It is the UE reader that maintains the mapping relationship;

- Step 5/6: When the gNB receives the Command request NGAP msg from the A-IoT CN, it signals the RAN A-IoT Device NGAP ID itself, along with the Command Request NAS PDU to the UE reader in DL RRC signalling;

- Step 7: UE reader maps the received RAN A-IoT Device NGAP ID to the AS ID as per the mapping relationship stored in Step 4a, and thus performs the R2D upper layer data transfer with the AS ID for command request as in legacy;

- Step 9/9a: UE reader, upon receiving the D2R upper layer data transfer for command response, maps the AS ID to RAN A-IoT Device NGAP ID as per the mapping relationship stored in Step 4a, and signals the Command Response Data SDU along with RAN A-IoT Device NGAP ID in UL RRC signalling.

In this option, it is the RAN A-IoT Device NGAP ID that is directly forwarded in the DL and UL RRC signalling for Command request and Command response, respectively, and the option relies on the UE reader to map it to/from the AS ID, based on the mapping relationship configured by the gNB.

Comparing the two options, we prefer option 1. The reason is that in option 2, the RAN A-IoT Device NGAP ID is made visible in the Uu RRC signalling, and this is not aligned with design logic for NGAP which should only be used over the network interface. In addition, option 2 requires additionally DL RRC signalling to indicate the UE reader with the mapping relationship which should have been maintained by the gNB itself as in option 1. Also, option 1 can avoid additional impacts/complexity to on UE reader behaviour to map AS ID to/from RAN A-IoT Device NGAP ID.

Therefore, we propose that RAN2 agrees with option 1 on the mapping between AS ID and RAN A-IoT Device NGAP ID in CBRA case, which also means that RAN A-IoT Device NGAP ID is not directly forwarded by the gNB, but will be mapped to AS ID which is then signaled to the UE reader via DL RRC siganlling.

Proposal 4: For CBRA in Topology 2, the gNB maps the AS ID to/from the RAN A-IoT Device NGAP ID as follows:

- For R2D, DL direction, upon reception Command Response over NGAP, the gNB maps RAN A-IoT Device NGAP ID to AS ID which is then signaled to the UE reader in DL RRC signalling;

- For D2R, UL direction, upon reception of “D2R Upper Layer Data Transfer” for Inventory response/command response, the UE reader signals the AS ID in UL RRC signalling to the gNB which is then mapped to the RAN A-IoT Device NGAP ID signaled to A-IoT CN.

For CFA, as discussed above, there is no need to maintain the mapping between AS ID and RAN A-IoT Device NGAP ID at either gNB or UE reader side, so for CFA in topology 2, there is no need to discuss whether to exchange AS ID or RAN A-IoT Device NGAP ID in Uu RRC signalling.

Observation 2: Whether/how to signal “RAN A-IoT Device NGAP ID or AS ID” in Uu RRC signaling and how to maintain their mapping are irrelevant to CFA in Topology 2.

Relating to Table 1 and 2, proposal 4 typically means that “AS ID” is directly forwarded in UL RRC message (w.r.t. Table 2, No.2), whereas “RAN A-IoT Device NGAP ID” is NOT directly forwarded but transformed to AS ID then signaled in DL RRC message (w.r.t. Table 1, No. 7).

Now we focus on the service identification which is related to the “Transaction ID” in A-IoT paging message vs. “A-IoT Correlation Identifier” in A-IoT NGAP message. If we follow Rel-19 Topology 1 assumption that only one service is provided by one reader at a given time without considering the support of multi-reader/multi-service, it looks straightforward that the Transaction ID can be directly assigned by the gNB based on the “A-IoT Correlation Identifier” received from the Inventory Request msg, and signaled to the intended UE reader. The Transaction ID need to be exchanged in Uu afterwards.

Also note that the support of multi-reader/multi-service scenario has not been agreed for Topology 1 in Rel-19, and thus should not be directly considered for Topology 2. Therefore, potential impacts to service identification in Topology 2 due to multi-reader/multi-service scenario (e.g. further exchange of Transaction ID or Correlation Identifier over Uu) is not considered for now.

To this end, it is proposed that RAN2 assumes that the gNB assigns the “Transaction ID” for a service based on “A-IoT Correlation Identifier”, and signals it in DL RRC message to the UE reader (which directly includes it in the A-IoT Paging message). Afterwards, neither “transaction ID” nor “A-IoT Correlation Identifier” needs to be exchanged over Uu (unless RAN2 concludes the support of multi-reader/multi-service scenario in Rel-20).

Proposal 5: RAN2 assumes that the gNB assigns the “Transaction ID” for a service based on “A-IoT Correlation Identifier”, and indicates it in DL RRC signalling to the UE reader (which then includes it in A-IoT Paging message). Neither “Transaction ID” nor “A-IoT Correlation Identifier” needs to be exchanged over Uu afterward.

## SRB transfer specific to A-IoT information

Support of data aggregation for the D2R upper layer SDU

Specifically for the UL A-IoT data forwarding, another issue that is worth looking into is whether:

- The UE reader can aggregate A-IoT data received from multiple devices (e.g. inventory response and/or command response) into the same UL RRC message sent to the gNB, or;

- The UE reader has to forward the A-IoT data from multiple devices one by one.

Basically, forwarding the data from multiple devices one by one may result in signaling storm (due to e.g., signaling to request Uu scheduling) between the reader and the gNB, whereas aggregation of multiple device’s data into the same UL RRC message would have significant saving on signaling overhead at the expense of some response latency.

Actually, in Topology 1, the Inventory Report message over NGAP has already supported the aggregation of multiple devices’ inventory response. This means that the latency caused by the aggregation would not be a critical issue at least for inventory procedure. On the other hand, compared to the A-IoT forwarding over NGAP, we care more about the radio resource consumption and efficiency in the Uu interface. From the UE reader perspective in Topology 2, it also seems unusual for a UE to generate quite a number of UL RRC messages within a short period of time (e.g. due to inventory of huge number of devices), with each containing only one piece for A-IoT data from one device.

Additionally, SA2 agreed that the AIoTF may provide a time interval (contained in Inventory Assistance Information) to the A-IoT RAN, which can be used to assist in aggregating A-IoT device responses for TP1. For TP2, D2R data aggregation on the UE side should be under network control and performed by the UE based on assistance information provided by the network. With regard to the assistance information, similar to TP1, the NG-RAN should at least provide the time interval to the UE reader. Furthermore, to optimize data transmission efficiency and prevent report failures caused by UE state changes (e.g., mobility), the network can consider providing additional criteria information to the UE reader, such as radio condition thresholds, data volume thresholds, and mobility parameters.

Observation 3: In Topology 2, UE reader forwarding the D2R data transmissions one by one may result in signaling storm in the Uu interface.

Observation 4: In Topology 1, it is already supported to aggregate inventory responses from multiple devices into the same Inventory Report message over NGAP.

Based on the above observations and analyses, we propose to support the D2R A-IoT data aggregation in the same UL RRC message. This is at least for the inventory response transfer, similar as the NGAP design in Topology 1.

Proposal 6: In Topology 2, UE reader can aggregate the received D2R data from multiple UEs in the same UL RRC message and forward them to the gNB together. This is at least supported for the D2R upper layer data transfer of inventory response. FFS on the command response.

RRC message to use for A-IoT data transmission

On top of the content of A-IoT related content in 2.1.1 and above data aggregation, another issue is whether the RRC message(s) for A-IoT data transmission is transmitted via existing SBR or a new SRB. The existing SRB for RRC message(s) is defined as highlight below:

“Signalling Radio Bearers" (SRBs) are defined as Radio Bearers (RBs) that are used only for the transmission of RRC and NAS messages. More specifically, the following SRBs are defined:

-	SRB0 is for RRC messages using the CCCH logical channel (except SRB0 of L2 U2N Remote UE);

-	SRB1 is for RRC messages (which may include a piggybacked NAS message) as well as for NAS messages prior to the establishment of SRB2, all using DCCH logical channel (except SRB1 of L2 U2N Remote UE);

-	SRB2 is for NAS messages and for RRC messages which include logged measurement information, all using DCCH logical channel (except SRB2 of L2 U2N Remote UE). SRB2 has a lower priority than SRB1 and may be configured by the network after AS security activation;

-	SRB3 is for specific RRC messages when UE is in (NG)EN-DC or NR-DC, all using DCCH logical channel;

-	SRB4 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB4 has a lower priority than SRB1 and can only be configured by the network after AS security activation.

-	SRB5 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB5 has a lower priority than SRB1 and SRB3 and can only be configured by the SN serving the SCG when the UE is in NR-DC, after AS security activation.

For A-IoT data, the transmission requirements may differ from those of existing control signaling transmitted via SRB1. For instance, A-IoT data typically has lower priority and is latency-tolerant, whereas control signaling is high-priority and latency-sensitive. The existing SRB2 and SRB4 could be considered as potential candidates, but it is used for the specific purpose. Therefore, it is better to decouple theses with the A-IoT transmission and introduce a new SBR to carry RRC messages related to A-IoT data, which can also clearly indicate network support through SRB configuration.

Observation 4: The characters of the RRC message(s) transmission via the existing SRB may differ from that for A-IoT data.

Proposal 7: RAN2 discusses whether to define a new SRB to transmit the RRC message(s) specifically for A-IoT data transmission.

# Resource allocation for A-IoT air interface in Topology 2

## UE reader assistance info for resource allocation

What assistance information can be provided by the UE reader to facilitate gNB’s A-IoT resource allocation is left as FFS from last meeting.

Agreements

2.	gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.   The resource allocation can be triggered by CN request.   UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.  It is up to gNB how the information is used.  It is understood that assistance information is not mandated for the gNB to determine resource allocation.

From our perspective, what UE assistance information is needed may have dependency on how the A-IoT resource allocation looks like, which was another FFS point left from last meeting:

Discussion - Ericsson, Apple, Nokia, CATT and lenovo thinks that a timer is not needed, the gNB is always in control. Lenovo and CATT are not sure the gNB can set the time. - Nokia asks if the reader can tell the reader that it has completed. - Qualcomm thinks that we are going too much into detail, we need to decided if it is periodic resources or continuous or one shot. Mediatek thinks that we could have a one shot resource allocation, or we can have cases where readers can communicate with multiple devices. Huawei thinks that periodic is not likely and one shot doesn’t make sense. The assumption is that the network gives you recourses and the procedure should be completed. - Vivo agrees that we can have one shot and periodic. - Xioami thinks that anyways the gNB can release the connection whenever it wants. One shot is not really validity but rather transmission opportunity - Interdigital explains that allocating resources continuously doesn’t make sense as there may be period of no transmission so the gNB can provide non-contigous finite resources. - Continue discussion next meeting on how resource allocation looks like.

During the meeting, there are below views on this issue:

- One-shot resource allocation

In our understanding, the one-shot resource allocation looks like the dynamic scheduling, i.e. gNB directly allocates the specific A-IoT resource used for each R2D or D2R transmission. We think such a dynamic scheduling like resource allocation may be too much for A-IoT resource allocation, as it results in significant signaling overhead on the Uu interface and can also introduce additional latency to request the per-transmission scheduling. One-shot resource allocation should not be supported for Topology 2.

- Periodic resource allocation

The periodic resource allocation can also lead to extra additional latency for the device/UE reader to wait for the next (set of) A-IoT resource(s) to come to schedule the intended D2R/R2D resources, even if it already received the D2R response from the device and/or the R2D data to be transmitted from the gNB.  Also, due to the timing inaccuracy of the passive device, it is difficult to guarantee that the D2R transmission scheduled by the UE reader will not happen within the interval (i.e. invalid time) of the periodic resource allocation.

- A period of time-domain contiguous resource allocation

This seems to be the simplest and most reasonable way for the A-IoT radio resource allocation. Specifically, the A-IoT resource should be a continuous period of frequency resources in the UL band for Topology 2 when the service is ongoing: when the service starts, gNB allocates the A-IoT resources contiguous in time domain; when the service ends, it releases the resources. We think such a contiguous A-IoT resource location in time domain should also be the assumption for Rel-19 Topology 1.

To this end, we propose that RAN2 assumes the contiguous resource allocation in A-IoT radio interface for Topology 2, and carry out further resource allocation discussions based on that. If companies truly cannot reach a consensus, perhaps a better way is to request feedback from RAN1.

Proposal 8: RAN2 assumes that upon being triggered by the A-IoT CN to initiate a service, gNB allocates A-IoT resources which are continuous in time domain until the service is terminated. One-shot and periodic A-IoT resource allocation are excluded in Rel-20.

Proposal 9: Ask RAN1 for feedback on how A-IoT radio resource configuration should look like, if RAN2 cannot make a conclusion.

Based on above proposal 8, what UE assistance info is needed for A-IoT resource allocation is discussed accordingly. One motivation during the last meeting was to enable the UE reader to signal a resource release indication to facilitate the gNB releasing the A-IoT resources allocated in a timely way. In Topology 1, the gNB reader can decide whether a service is terminated in A-IoT radio interface and release the A-IoT radio resource by its implementation, based on e.g., “Approximate Number of Target A-IoT Devices” NGAP info for inventory service and reception of command response for the command service. By contrast, the difference in Topology 2 is that there can be a case where the UE has already determined that the service data transfer in A-IoT interface is already completed, with only the Uu forwarding of the received A-IoT data necessarily continued. Since gNB now has no direct A-IoT radio interface and the Uu A-IoT data forwarding is still ongoing, the gNB is not aware of the completion of A-IoT operation by this UE reader, and thus may still keep A-IoT resource allocation to this specific UE reader, leading unnecessarily resource wastage.

For such a case, one can consider to enable the UE reader to signal a resource release indication to the gNB in order to avoid potential A-IoT resource wastage. On the other hand, one may also argue that the gNB anyway can decide whether A-IoT service is terminated based on the A-IoT data forwarded from the UE reader in UL, by which it further decides when to release A-IoT resources, and thus the further optimization of UE release indication is not fully necessary.

Therefore, it is proposed that RAN2 discusses whether a A-IoT resource release indication from the UE reader is essential or not.

Proposal 10: RAN2 discusses whether an A-IoT resource release indication from the UE reader is needed (with the purpose to enable gNB’s timely release of A-IoT resources and avoid resource wastage).

Another motivation that companies proposed this UE assistance information is to enable the gNB to add/modify allocated A-IoT resources based on the actual situation on A-IoT radio interface. Based on the discussion above, it is assumed that the gNB allocates the A-IoT resources that are continuous in time domain and last till the end of the service. However, due to the factors like e.g., radio link quality, collisions, etc. in the A-IoT radio interface, the number of resources that are needed by the UE reader may be varies, so it is sometime necessary for the gNB to adjust the number of paging round/access rounds for the device’s access and/or schedule (re)transmissions for D2R transmissions with more resources.

Since the gNB cannot track what is happening on the A-IoT radio interface, it does not know whether the resource it (plans to) allocates is enough or not. We propose to support UE assistance information that requests gNB to allocate more A-IoT resources, and further discuss the specific information that is signaled, e.g. number of resources requested, collision rate, number of retransmission needed, number of access occasions needed, etc.

Proposal 11: UE assistance information can be signaled to the gNB to request more A-IoT resources allocated to the UE reader. FFS the specific information that is signaled (e.g. number of resources requested, collision rate, number of retransmission needed, number of access occasion needed, etc.).

## UE reader behaviour on A-IoT resource scheduling

In the last meeting, it was agreed that UE reader schedules the A-IoT interface transmissions within the resources allocated by the gNB.

Agreements

3. 	the reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

The intended UE reader behaviour for A-IoT scheduling can be further clarified with the following proposal:

Proposal 12: In Topology 2, based on the configured A-IoT radio resources:

- UE reader autonomously selects the resource for each specific R2D/D2R transmission (other than per Msg1 transmission) in the configured set of resources, and signals the selected D2R resource to the device via R2D message.

- Device selects the resources for a Msg1 transmission following legacy Rel-19 CBRA procedure, based on the AO related information controlled/signaled by the UE reader (e.g. in A-IoT paging/Access Trigger message), whereas for other D2R transmissions it uses the D2R resources scheduled by the UE reader.

Also, the UE reader needs to schedule the resources for both PDRCH and PRDCH, and further the resources for different types of messages given a certain type of channel. RAN2 may further clarify whether the A-IoT resource allocation by the gNB is specific for different channels and/or for different msg types, in order to decide whether any restriction to the UE reader scheduling is needed. If RAN2 cannot make a decision in this regard, RAN1 needs to be involved.

Proposal 13: FFS whether A-IoT resources allocated by gNB are specific to different channels/msg types.

# Support of mobility scenarios (i.e. RLF and handover) for Topology 2

As per the objective of Topology 2 work in [1], the enhancements to the UE readers during RLF and handover are also within the scope.

RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling.

During the Rel-19 study item phase, there were related discussion for UE reader mobility in Topology 2 for which some TR conclusions on the validity of A-IoT resource configuration were written down [4].

The radio resources used by A-IoT radio interface between the A-IoT device(s) and UE reader are controlled by the network. The radio resources, which are dedicated for a UE reader, are only configured to the UE reader via dedicated signalling. The mechanisms for shared resource pool amongst UE readers are not considered in this release. The UE reader in coverage of BS scenario is supported. The UE reader may perform the A-IoT procedure on A-IoT radio interface between the reader and the device(s), only if the radio resource configuration is valid in the cell, which is under network control. The radio resource validity across multiple cells is not supported in this release. Hence, for all options below, the resources remain valid only in the same cell in which the resources were configured. […] For the case when the radio resources are allocated semi-statically by the network, the radio resource validities are studied in the following scenarios with candidate options (down-selection can be decided in the WI phase): […] - Scenario 2: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases): - Option 1: The UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition. - Option 2: The UE reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations.

Actually, for the UE reader mobility scenarios, the key issue to address is whether/how to ensure the A-IoT service continuity when the current selected UE reader is experiencing RLF and handover, and this involves in the aspects like whether to stop/suspend/resume the A-IoT transmission in A-IoT radio interface, how/whether to continue the ongoing A-IoT transmission in the target cell, etc.

## Handling of UE reader handover

During the handover, how the UE reader deals with the ongoing operations in the A-IoT radio interface needs to be addressed. There was the Rel-19 TR conclusion that resource validity across multiple cells is not supported [4] with the intention to avoid the inter-cell interference for A-IoT radio interface operations. Following this TR conclusion:

- the UE should consider any A-IoT resources configured by the source cell invalid and stop A-IoT transmission on the A-IoT radio interface during handover, as it detaches from the source cell upon initiation of handover;

- the UE can decide whether it can continue performing A-IoT operations after handover as per the A-IoT radio resource configuration by the target cell: if the target cell allocates the A-IoT radio resources to the UE reader, it can use the resources to perform A-IoT operations/transmissions upon handover completion. Specifically, the serving gNB can signal the target cell allocated A-IoT resources along with HO command to the UE reader. In the inter-gNB handover case, based on inter-node message, serving gNB may signal the A-IoT capability of the UE reader to the target gNB which then decides whether to still select this UE as a reader and allocates A-IoT resources; then the target gNB sends the configured A-IoT resource back to the serving cell to provide them to the UE.

The above procedure is proposed as follows:

Proposal 14: During handover, the UE reader releases the A-IoT resources allocated by the serving cell, stops any A-IoT transmission in A-IoT radio interface and follows the target cell’s A-IoT resource configuration provided along with the HO command to determine whether it can still perform A-IoT operation/transmission after handover completion.

One more aspect to consider is whether ongoing A-IoT service of a UE reader can be suspended during handover and then resumed in the target cell upon handover completion. Such suspension of ongoing A-IoT service intends to avoid data loss and support the service continuity during the UE reader handover case. However, from the network side, such ongoing service resumption may involve in the A-IoT contexts transfer (e.g. Transaction IDs, AS IDs, etc.) between the target gNB and the serving gNB in the inter-gNB handover case, and also requires the correlation of the A-IoT NGAP information from different gNBs to the A-IoT CN. These would lead to complication to NW interfaces, and leads non-trivial RAN3 impacts.

Considering both the benefits to support service continuity and the non-trivial RAN3 impacts to NW interfaces, RAN2 is suggested to discuss whether to support service continuity during UE reader handover, i.e. whether the UE reader can suspend the ongoing A-IoT service during handover and resumes it upon handover completion, if target cell configures A-IoT resources.

Proposal 15: RAN2 further discusses whether it is supported that the ongoing A-IoT service is suspended by the UE reader during handover, and resumed after handover completes (if the UE reader is configured with valid A-IoT resources).

Regarding the Uu operations for A-IoT, in the case of handover, if the UE is not permitted to report received D2R data in the target cell (due to, e.g., the target cell not supporting A-IoT functionality or not selecting the same UE as the reader), it may be forced to clear the A-IoT data buffered, potentially leading to data loss. To mitigate this risk, timely reporting of D2R data in the source cell before handover can be considered, and there are two approaches:

- The UE initiates D2R data reporting based on predefined criteria (e.g., RSRP threshold).

- The UE follows network indication: if the source cell detects that the target cell may not support D2R data reporting, it indicates the UE to report the received D2R data before handover to the target cell.

We propose RAN2 agrees the UL A-IoT data forwarding before UE reader handover, with detail to be further discussed.

Proposal 16: Support uplink A-IoT data forwarding triggered to avoid A-IoT data loss, in the case of UE reader handover. RAN2 further discusses the criteria on how to trigger A-IoT data forwarding in uplink in this case, e.g. based on RSRP-level or NW indication.

## Handling of UE reader RLF

As to UE reader RLF case, one difference from the UE reader HO case is that the UE reader may face the temporary out-of-connection situation and then is (highly) likely to recover from the RLF and stay in the same serving cell. In case the UE reader recovers from RLF in the same serving cell, the ongoing A-IoT service can be resumed, as the gNB has not changed. Therefore, upon the RLF triggered, the UE reader stops any A-IoT transmission, but can suspend the A-IoT operation with the A-IoT contexts (e.g. Transaction ID, AS IDs, etc.) kept, in order to see whether the resumption of the ongoing A-IoT service is possible.

Proposal 17: Upon RLF triggered, UE reader stops A-IoT transmission, suspends the ongoing A-IoT service and keeps the A-IoT AS contexts (e.g. Transaction IDs, AS IDs, etc.).

Regarding A-IoT resource allocated, in the case of RLF recover on the same serving cell, it can be considered that the UE can reuse the A-IoT resource configuration provided by the serving cell and resume the A-IoT transmission after RLF recovery. Specifically, such A-IoT resource reuse can be enabled, in the case of, e.g. the UE reader successfully completes re-establishment to the same serving cell, or it receives explicitly indication from the network explicitly which is subject to further RAN2 discussion.

Proposal 18: The UE can reuse the A-IoT resource configuration and resume the A-IoT transmission over A-IoT interface (e.g. Transaction IDs, AS IDs, etc.), if RLF is recovered in the same cell. FFS the condition to enable this A-IoT resource reuse (e.g. when the UE re-establishes to the same service cell, upon reception of an NW indication on the resource reuse, etc.).

If the UE successfully selects and performs re-establishment to a new cell, then similar to the handover case, the UE reader just follows the new cell’s configuration on the A-IoT operations. As for whether the network supports continuity of the ongoing A-IoT service, this can be further discussed. If continuity is not supported, the UE shall release the suspended A-IoT operations upon successful re-establishment to the new cell. Similar as the UE reader HO case, this is a tradeoff between the data loss reduction and the impacts to network interface.

Proposal 19: If UE re-establishes to a new cell after RLF recovery, the UE follows the new cell’s A-IoT resource configuration, if provide. RAN2 discusses whether it is supported that the ongoing A-IoT service is continued if UE reader re-establishes to a different cell.

When it comes to the Uu operations for A-IoT, similar to the handover case, the UE in the case of RLF may re-establish to a new cell that does not support D2R data reporting, and the data loss may occur if the UE cannot report the received D2R data in time. Therefore, how to avoid or reduce data loss also needs to be considered.

What is different from the handover case is that the network cannot predict RLF events in advance, so that the possible way is that UE initiates D2R data reporting based on predefined criteria (e.g., RSRP threshold) to reduce data loss. This is proposed as follows:

Proposal 20: Support uplink A-IoT data forwarding triggered to avoid A-IoT data loss, in the case of UE reader RLF. RAN2 discusses whether the uplink A-IoT data forwarding in this case is triggered based on RSRP-level.

# Impact on A-IoT MAC behaviour at UE reader

In the A-IoT radio interface, the reader in Topology 2 is now the UE reader, so some operations in A-IoT MAC layer which was originally left to gNB reader implementation in Topology 1 now turns to be the UE behaviour and thus may need to be specified alternatively. These operations/behaviours may include, e.g. reassembly for D2R reception, feedback and retransmission scheduling, generation of related R2D message and field setting, etc.

RAN2 may need to identify what reader behaviour which were left to gNB reader implementation for Topology 1 should now be specified in the A-IoT MAC layer at the UE reader in Topology 2.

Proposal 21: RAN2 identifies which gNB reader behaviours in Topology 1 should be specified in the A-IoT MAC layer at the UE reader in Topology 2.

# Conclusion

In this contribution, RAN2 impacts to support of Topology 2 for A-IOT are discussed. Observations and proposals

Impact on RRC signalling content due to A-IoT NGAP information

Proposal 1: For R2D, DL direction in Topology 2, the information in the following A-IoT NGAP IE needs to be signaled to the UE reader, and is directly forwarded by the gNB in DL RRC signalling:

- A-IoT Device Identification Requested;

- Follow on Command Indication;

- Inventory Assistance Information;

- A-IoT NAS PDU.

Proposal 1a: For R2D, DL direction, whether information related to A-IoT NGAP IE “Requested Service Area Information” needs to be signaled to the UE reader is pending RAN3 and SA2 progress on UE reader selection.

Proposal 2: For D2R, UL direction in Topology 2, the “Data SDU” field in “D2R Upper Layer Data Transfer” message is directly forwarded by the UE reader to the gNB in UL RRC signalling (to be included AIoT NGAP IEs of “Inventory Report Transfer” and “Command Response Transfer”).

Identification management in Topology 2

Observation 1: For CBRA In Topology 1, gNB reader associates the “RAN A-IoT Device NGAP ID” and “AS ID” assigned by itself, and transforms the two ID with each other to identify the correct device to which the Command request message is signaled and from which the Command response message is received.

Observation 2: Whether/how to signal “RAN A-IoT Device NGAP ID or AS ID” in Uu RRC signaling and how to maintain their mapping are irrelevant to CFA in Topology 2.

Proposal 3: In Topology 2, the AS ID is assigned by UE reader (by its A-IoT MAC layer).

Proposal 4: For CBRA in Topology 2, the gNB maps the AS ID to/from the RAN A-IoT Device NGAP ID as follows:

- For R2D, DL direction, upon reception Command Response over NGAP, the gNB maps RAN A-IoT Device NGAP ID to AS ID which is then signaled to the UE reader in DL RRC signalling;

- For D2R, UL direction, upon reception of “D2R Upper Layer Data Transfer” for Inventory response/command response, the UE reader signals the AS ID in UL RRC signalling to the gNB which is then mapped to the RAN A-IoT Device NGAP ID signaled to A-IoT CN.

Proposal 5: RAN2 assumes that the gNB assigns the “Transaction ID” for a service based on “A-IoT Correlation Identifier”, and indicates it in DL RRC signalling to the UE reader (which then includes it in A-IoT Paging message). Neither “Transaction ID” nor “A-IoT Correlation Identifier” needs to be exchanged over Uu afterward.

Support of data aggregation for the D2R upper layer SDU

Observation 3: In Topology 2, UE reader forwarding the D2R data transmissions one by one may result in signaling storm in the Uu interface.

Observation 4: In Topology 1, it is already supported to aggregate inventory responses from multiple devices into the same Inventory Report message over NGAP.

Proposal 6: In Topology 2, UE reader can aggregate the received D2R data from multiple UEs in the same UL RRC message and forward them to the gNB together. This is at least supported for the D2R upper layer data transfer of inventory response. FFS on the command response.

RRC message to use for A-IoT data transmission

Observation 4: The characters of the RRC message(s) transmission via the existing SRB may differ from that for A-IoT data.

Proposal 7: RAN2 discusses whether to define a new SRB to transmit the RRC message(s) specifically for A-IoT data transmission.

UE reader assistance info for resource allocation

Proposal 8: RAN2 assumes that upon being triggered by the A-IoT CN to initiate a service, gNB allocates A-IoT resources which are continuous in time domain until the service is terminated. One-shot and periodic A-IoT resource allocation are excluded in Rel-20.

Proposal 9: Ask RAN1 for feedback on how A-IoT radio resource configuration should look like, if RAN2 cannot make a conclusion.

Proposal 10: RAN2 discusses whether an A-IoT resource release indication from the UE reader is needed (with the purpose to enable gNB’s timely release of A-IoT resources and avoid resource wastage).

Proposal 11: UE assistance information can be signaled to the gNB to request more A-IoT resources allocated to the UE reader. FFS the specific information that is signaled (e.g. number of resources requested, collision rate, number of retransmission needed, number of access occasion needed, etc.).

UE reader behaviour on A-IoT resource scheduling

Proposal 12: In Topology 2, based on the configured A-IoT radio resources:

- UE reader autonomously selects the resource for each specific R2D/D2R transmission (other than per Msg1 transmission) in the configured set of resources, and signals the selected D2R resource to the device via R2D message.

- Device selects the resources for a Msg1 transmission following legacy Rel-19 CBRA procedure, based on the AO related information controlled/signaled by the UE reader (e.g. in A-IoT paging/Access Trigger message), whereas for other D2R transmissions it uses the D2R resources scheduled by the UE reader.

Proposal 13: FFS whether A-IoT resources allocated by gNB are specific to different channels/msg types.

Handling of UE reader handover

Proposal 14: During handover, the UE reader releases the A-IoT resources allocated by the serving cell, stops any A-IoT transmission in A-IoT radio interface and follows the target cell’s A-IoT resource configuration provided along with the HO command to determine whether it can still perform A-IoT operation/transmission after handover completion.

Proposal 15: RAN2 further discusses whether it is supported that the ongoing A-IoT service is suspended by the UE reader during handover, and resumed after handover completes (if the UE reader is configured with valid A-IoT resources).

Proposal 16: Support uplink A-IoT data forwarding triggered to avoid A-IoT data loss, in the case of UE reader handover. RAN2 further discusses the criteria on how to trigger A-IoT data forwarding in uplink in this case, e.g. based on RSRP-level or NW indication.

Handling of UE reader RLF

Proposal 17: Upon RLF triggered, UE reader stops A-IoT transmission, suspends the ongoing A-IoT service and keeps the A-IoT AS contexts (e.g. Transaction IDs, AS IDs, etc.).

Proposal 18: The UE can reuse the A-IoT resource configuration and resume the A-IoT transmission over A-IoT interface (e.g. Transaction IDs, AS IDs, etc.), if RLF is recovered in the same cell. FFS the condition to enable this A-IoT resource reuse (e.g. when the UE re-establishes to the same service cell, upon reception of an NW indication on the resource reuse, etc.).

Proposal 19: If UE re-establishes to a new cell after RLF recovery, the UE follows the new cell’s A-IoT resource configuration, if provide. RAN2 discusses whether it is supported that the ongoing A-IoT service is continued if UE reader re-establishes to a different cell.

Proposal 20: Support uplink A-IoT data forwarding triggered to avoid A-IoT data loss, in the case of UE reader RLF. RAN2 discusses whether the uplink A-IoT data forwarding in this case is triggered based on RSRP-level.

Impact on A-IoT MAC behaviour at UE reader

Proposal 21: RAN2 identifies which gNB reader behaviours in Topology 1 should be specified in the A-IoT MAC layer at the UE reader in Topology 2.

# Reference

[1]	RP-252894	Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2

[2]	RAN2#131bis Meeting minutes

[3]	3GPP TS 23.369	Architecture support for Ambient power-enabled Internet of Things; Stage 2 (Release 19) V19.1.0

[4]	3GPP TR 38.769	Study on solutions for Ambient IoT (Internet of Things) in NR

# Annex 1: Command Procedure in [TS 23.369]

### 6.2.3 Command Procedure Figure 6.2.3-1 depicts the command procedure. The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4. Figure 6.2.3-1: Command Procedure 1. The AF sends the Nnef_AIoT_Command Request (AF ID, Command Type, information about the target AIoT Device(s), [External Target Area information], [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters], [location information requested]) message to NEF. Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s). The approximate number of AIoT Devices (see clause 5.4), if provided, is used to indicate the number of AIoT Devices expected to respond to this AIoT service operation request, which is sent by AIOTF to the NG-RAN in the assistance information as specified in clause 5.4. Command Type provides the operation to be performed and the Command type specific parameters provides the required parameters for the operation. The service operations are described in clause 5.2.2. 2. Step 2 of the Inventory Procedure specified in clause 6.2.2 is performed for External Target Area information processing and AIOTF selection with the following clarifications: - If AIOTF selection fails, the NEF rejects the Nnef_AIoT_Command request and step 6 of this procedure is performed instead. 3. The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, information about the target AIoT Device(s), [Target area information], [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters], [location information requested]) message to the selected AIOTF(s). 4. The AIOTF receives the Naiotf_AIoT_Command Request and checks the parameters included in the request. The AIOTF performs Reader Selection as specified in clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code. The AIOTF generates a Correlation ID corresponding to this AIoT service operation request and the Correlation ID is used for the AIOTF to correlate the service operation responses received from NG-RAN to the request. The AIOTF creates the AIoT Session for the AF service operation request, which is identified by the Correlation ID. The AIOTF performs Reader Selection, see clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT service operation request with an appropriate cause code. The AIOTF determines assistance information as described in clause 5.4, taking into account the parameters provided in the AIoT service operation request. The AIOTF performs AF authorization for AIoT service operation request as described in clause 5.6. The AIOTF may perform AMF selection as described in clause 5.3.4. 5. AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, [cause code]) to the NEF. 6. NEF sends the Nnef_AIoT_Command Response message (accept or reject, [cause code]) to the AF. If the response was a reject the procedure stops here. 7. Step 7 to step12 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications: - In step 7, the AIOTF also includes follow on command indication in the Inventory Request message to inform the NG-RAN command delivery occurs after the inventory. - In step 10, the NG-RAN also includes the RAN AIoT Device NGAP ID for each AIoT Device in the Inventory Report as specified in TS 38.413 [10]. - In step 11, the AIOTF validates the results as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information. The AIOTF updates the corresponding AIoT device context in the AIOTF to include the RAN AIoT Device NGAP ID. If none of successful Inventory response is received, Step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in step 12. 8. For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID], NAS Command Request, [Approximate D2R message size], RAN AIoT Device NGAP ID for each AIoT Device) to the NG-RAN directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data. The Correlation ID is as the same as the Correlation ID generated in step 4. The RAN AIoT Device NGAP ID for each AIoT Device is used by the NG-RAN to determine the AIoT device context in NG-RAN as specified in TS 38.413 [10]. The AIOTF uses the Command Type and Command type specific parameters received in Step 3 to determine the NAS Command Request to send to the AIoT Device, as described in clause 5.2.2. NOTE: Command Request(s) can be sent to NG-RAN when inventory procedure is ongoing. Editor's note: Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3. 9. The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device as defined in TS 38.391 [11]. 10. The AIoT Device sends the AS D2R message (NAS Command Response) to the NG-RAN as defined in TS 38.391 [11]. The NAS Command Response message may include the AIoT data. Editor's note: Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3. 11. The NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response, RAN AIoT Device NGAP ID) to the AIOTF directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4. The AIOTF determines the AIoT device context by the RAN AIoT Device NGAP ID received. 12. The AIOTF reports the result of the Naiotf_AIoT_Command request to the NEF by sending the Naiotf_AIoT_Command Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), AIoT data and optionally location of each AIoT Device), AF ID, [Last Report Indication]). If multiple AIOTFs are involved in the procedure, the NEF may receive Naiotf_AIoT_Command Notify messages from multiple AIOTFs. Based on operator policy, if the location information is requested by the AF and if the location of the reader is configured, the AIOTF uses the Reader ID reported from NG-RAN during inventory in step 7 to determine the AIoT Device Location. When the last report is sent, the AIOTF ends the AIoT Session. 13. The NEF informs the AF of the result of the Nnef_AIoT_Command request by sending the Nnef_AIoT_Command Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), AIoT data and optionally location of each AIoT Device), AF ID, [Last Report Indication]).