3GPP TSG-RAN WG2 Meeting #132   									R2-2508159

Dallas, US, Nov. 17th – Nov. 21th, 2025

Agenda Item:	8.9.2

Source: 	CATT

Title:  	Discussion on paging relaxation for S&F operation

Document for:	Discussion and Decision

Introduction

In the RAN2#131bis meeting [1], the issue of paging relaxation for S&F operation was discussed but reached none consensus.

Come back in the next meeting to check whether we can have an indication (likely in SIB31) without mandating any UE behaviour

Send a LS to SA2 (Cc: RAN3) saying that RAN2 is discussing to have an indication in SIB to inform UEs than no paging is expected (so that the UE may skip paging) and asking them if any paging is expected from MME to RAN when the feeder link is not available (can further work on the LS text in the Post meeting email discussion)

In this contribution, we discuss whether and how to introduce an indication in system information to indicate UE relaxation monitoring paging.

Discussion

‌After RAN2#131bis meeting, RAN2 sent an LS to SA2 and asked whether any paging is expected from MME to RAN when the feeder link is unavailable. If SA2 confirm that no paging is expected from MME to RAN when the feeder link is unavailable, RAN2 may introduce an indication in SIB to inform UEs relaxation monitoring paging.

During the post-email discussion for the LS [2], companies comment that once this bit is set, there is no way to change UE behavior to monitor paging and SIB change, which is very dangerous in fact. We have sympathy on this problem. It is unclear until when the UE stops relaxation monitoring paging if the RAN indicates in SIB that no paging is expected and the UE relaxation monitoring paging. Consider the following scenario, the network broadcasts the paging relaxation indication, and indicates that the cell will switch from S&F mode to normal mode at t-ModeSwitching. If the UE keeps relaxation monitoring paging even if the cell switches to normal mode, it may miss necessary system information update and CN initiation paging from a normal mode satellite.

Observation 1: After the network broadcasts the paging relaxation indication, it is unclear when the UE stops relaxing monitoring paging. This may cause problems in some cases, e.g., if the cell switches from S&F mode to normal mode while the UE is still relaxing monitoring paging, the UE may miss necessary SI update and CN initiation paging from a normal mode satellite.

Therefore, if the indication for relaxing monitoring paging is introduced, it is necessary to clarify when UE stops relaxation monitoring paging. In the current spec, the t-modeSwitching field is introduced to indicate when an NTN cell switches from the S&F operation mode to the normal mode. The t-modeSwitching can be used to address the issues that arise in the case mentioned in Observation 1. However, it is unclear whether using the t-modeSwitching to indicate stopping the relaxation monitoring paging is applicable all cases, as mode switching and paging relaxation are two independent mechanisms. A simpler and clearer way is to introduce a new time point to indicate when UE stops relaxation monitoring paging. If the indication for relaxing monitoring paging is present, the UE not perform monitoring paging only before the indicated time point.

Proposal 1: If the indication for relaxing monitoring paging is introduced, it is also necessary to indicate a time point for UE stopping relaxing monitoring paging, FFS whether to reuse the t-ModeSwitching or introduce a new IE.

Proposal 1a: If Proposal 1 is acceptable, when the indication for relaxing monitoring paging is present, the UE may not perform paging monitoring before the indicated time point.

Conclusions

In this contribution, we have discussed paging relaxation for S&F operation and provide the following observation and proposals:

Observation 1: After the network broadcasts the paging relaxation indication, it is unclear when the UE stops relaxing monitoring paging. This may cause problems in some cases, e.g., if the cell switches from S&F mode to normal mode while the UE is still relaxing monitoring paging, the UE may miss necessary SI update and CN initiation paging from a normal mode satellite.

Proposal 1: If the indication for relaxing monitoring paging is introduced, it is also necessary to indicate a time point for UE stopping relaxing monitoring paging, FFS whether to reuse the t-ModeSwitching or introduce a new IE.

Proposal 1a: If Proposal 1 is acceptable, when the indication for relaxing monitoring paging is present, the UE may not perform paging monitoring before the indicated time point.

Reference

- RAN2#131bis ChairNotes.

- [Post131bis][307][IoT NTN] LS to SA2 (Huawei).