3GPP TSG-RAN WG2 Meeting #132										R2-2508221

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	10.4

Source: 	Sharp

Title:  	Discussion on 6GR Mobility

Document for: 	Discussion

Introduction

In RAN2#131bis meeting, RAN2 made the following agreements on 6GR mobility:

Agreements 1 Study mobility with the following requirements in 6G mobility design: - Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions. - Robustness of mobility procedures - Energy efficiency for both UE and NW 2 Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

This document discusses 6GR mobility aspects that should be considered at the early stage of the study.

Discussion

Mobility Mechanisms

NR introduced various mobility mechanisms across multiple releases on top of the traditional L3 handover. Basically, the L3 handover relies on RRC signalling. This classic mechanism is useful in traditional macro-cell deployment with moderate UE mobility and allowed interruption time. CHO supports quick link switching based on preconfigured condition, and it is useful in high mobility scenario. LTM reduces multi-layer processing and allows prompt mobility-related actions. C-LTM further enhances the robustness for prompt mobility scenarios. Also, RACH-less operation reduces the handover interruption time significantly. We think all those options are useful and the network should have the flexibility to select the implemented mobility mechanisms.

The network should therefore have the flexibility to configure and utilize the most suitable mobility mechanisms depending on deployment scenario, UE capability, and service requirements. Intra-cell mobility, e.g. beam-level mobility, is important for higher-frequency (e.g., mmWave/THz) scenario with beamforming. It relies on L1/L2 procedures for fast beam tracking. If feasible, it is preferred to consider intra-cell and inter-cell mobility mechanisms together in a unified framework of 6G mobility. “Unified” refers to defining a single, consistent mobility framework that aligns procedures and reporting formats across multiple mechanisms (e.g., L3 handover, CHO, LTM, C-LTM, RACH-less, beam-level mobility) under common principles. This ensures coherent configuration, measurement reporting, and UE behaviour without fragmented, mechanism-specific signalling.

Proposal 1: 6GR support unified mobility framework including L3 Handover, CHO, LTM, C-LTM, RACH-less operation and beam-level mobility.

NR supports L3 measurement (for RRM) and L1 measurement (for beam management, LTM), which may be configured through separate signalling paths, even when they use the same reference signals. For example, duplicated SSB configurations (e.g., time/frequency resources, subcarrier spacing, and SSB positions) exist in both the MeasObjectNR for L3 measurements and the LTM-SSB-Config for L1 measurements, even when they refer to the same target cell. Similarly, the same CSI-RS resources can be configured separately in CSI-MeasConfig (for beam management) and LTM-CSI-ResourceConfig (for LTM). This results in duplicated configurations which may increase UE processing and power consumption.

In addition, NR supports multiple reporting mechanisms (RRC-based, MAC-CE-based, and UCI-based) between L1 and L3 measurements. Generally, L3 measurement result is included in RRC message and reported to the gNB to decide to trigger L3 Handover, configure CHO, configure LTM and C-LTM. L1 measurement result is included in UCI or MAC CE and reported to the gNB to decide to trigger LTM. Such coexistence of multiple reporting mechanisms further increases specification and implementation complexity.

In 6GR, to simplify the measurement framework, address the fragmentation observed in NR, and support unified mobility as mentioned above, we think the measurement framework should be unified with at least the following aspects.

- A single reference signal configuration applicable across L1 measurements and L3 measurements.

- A unified reporting mechanism that replaces the multiple existing reporting mechanisms (RRC, MAC CE, and UCI).

Proposal 2: 6GR support unified measurement framework with at least the following aspects.

- A single reference signal configuration applicable across L1 measurements and L3 measurements.

- A unified reporting mechanism that replaces the multiple existing reporting mechanisms (RRC, MAC CE, and UCI).

Intra-RAT vs Inter-RAT

Lossless handover is a key aspect of intra-RAT mobility. Lossless handover avoids performance degradation due to packet loss during MAC reset procedure. This concept should be kept in 6G.

Inter-RAT mobility typically results in lossy handover where ongoing data may be lost during the handover. Considering an initial deployment of 6G, full coverage may not be always available, so frequent 5G-6G handovers are likely to be triggered often. In this scenario, lossy inter-RAT mobility may degrade the user service quality. Thus, in 6G, lossless inter-RAT mobility needs to be discussed and studied.

In MR-DC, a UE may have multiple physical links with both LTE and NR simultaneously. As captured in the SID, DC is not prioritized at this moment. Instead of DC, UE can be connected to only one RAT at a time point. The switching can be done by 6G inter-RAT mobility mechanism.

Proposal 3: For both Intra-RAT and Inter-RAT mobility from/to 6GR (including 5G-6G mobility), lossless handover is a baseline.

IDLE/INACTIVE mode mobility

NR cell (re)selection can serve a baseline for 6GR IDLE/INACTIVE mode mobility. In NR, both the S criteria and R criteria, which consider RSRP and RSRQ, are basic mechanisms for cell (re)selection. These criteria should be supported in the 6GR IDLE/INACTIVE state mobility. In addition, frequency prioritisation, that determines on which cell the UE should attempt to camp, is an essential procedure in NR cell reselection and is also applicable to 6GR. Other NR reselection mechanism, such as slice-aware or mobility-state-based cell reselection may be supported in 6GR.

Proposal 4: RAN2 supports basic mechanisms for NR cell selection/reselection, such as S criteria, R criteria and frequency priority in 6GR IDLE/INACTIVE mode mobility.

The cell reselection procedure in 5G have been expanded in a patchy manner, whenever a new feature was introduced. Specifically, in reselection priorities handling, we have introduced special handling rules for HSDN, V2X, mobile IAB, MBS, NCR, CAG and network slices. The consequence of this patchy approach was a need to specify complicated rules on priority handlings based on the device capabilities, which consumed a lot of time in online/offline discussion time to reach consensus. Thus, in 6GR we should study a simple and future-proof design for cell reselection priority handling.

Proposal 5: Special handling rules should be minimized to achieve a simple and future-proof design for managing 6GR cell reselection priorities.

As the 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR [1], 6GR IDLE/INACTIVE mobility should further support the inter-RAT cell reselection (i.e., 6GR and NR). Whether inter-RAT cell reselection between 6GR and E-UTRA is supported can depend on RAN progress.

In addition, considering the advance of native AI as a fundamental characteristic in 6G, supporting AI/ML in IDLE/INACTIVE mode mobility such as cell (re)selection is worthy of investigation. For instance, in cell reselection, AI/ML can facilitate measurement result prediction, the selection of the best or suitable cell, frequency/cell priority handling etc. Note that we do not propose any particular AI/ML use cases. However, it is important to consider that UEs in idle/inactive mode should be able to know whether the network supports AI/ML features. We see that it will enhance the 6GR network efficiency.

Proposal 6: RAN2 to study 6GR IDLE/INACTIVE mode mobility considers inter-RAT cell reselection between 6GR and NR, and cell reselection mechanism considering AI/ML support of the network/cell.

RLM/RLF

In both 4G and 5G, RLM/RLF is one of the key procedures. According to RLM procedure, UE can know if the RRC connection is still available and if it is still within the serving cell’s coverage due to its moving. Then, based on the RLM/RLF declaration, the UE can make the decision to recover the connection via serving cell or some potential target cell. By this way, UE encountering RLF will not only autonomously maintain the connection via network during the mobility but also avoid causing interference to other normal UEs. For the same reasons, in 6GR, RLM/RLF is needed. We think RLM/RLF in 5G is in good shape and it can be a baseline for further 6GR RLM/RLF discussion.

Proposal 7: RLM/RLF framework in 5G can be a baseline for 6GR RLM/RLF discussion.

Conclusion

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

Proposal 1: 6GR support unified mobility framework including L3 Handover, CHO, LTM, C-LTM, RACH-less operation and beam-level mobility.

Proposal 2: 6GR support unified measurement framework with at least the following aspects.

- A single reference signal configuration applicable across L1 measurements and L3 measurements.

- A unified reporting mechanism that replaces the multiple existing reporting mechanisms (RRC, MAC CE, and UCI).

Proposal 3: For both Intra-RAT and Inter-RAT mobility from/to 6GR (including 5G-6G mobility), lossless handover is a baseline.

Proposal 4: RAN2 supports basic mechanisms for NR cell selection/reselection, such as S criteria, R criteria and frequency priority in 6GR IDLE/INACTIVE mode mobility.

Proposal 5: Special handling rules should be minimized to achieve a simple and future-proof design for managing 6GR cell reselection priorities.

Proposal 6: RAN2 to study 6GR IDLE/INACTIVE mode mobility considers inter-RAT cell reselection between 6GR and NR, and cell reselection mechanism considering AI/ML support of the network/cell.

Proposal 7: RLM/RLF framework in 5G can be a baseline for 6GR RLM/RLF discussion.

References

[1] 3GPP TR 38.914 V0.2.0 (2025-09), “Study on 6G Scenarios and Requirements (Release 20)”