3GPP TSG-RAN WG2 Meeting #132                            R2-2508223

Dallas, USA, Nov.17th–21th, 2025

Agenda Item:	9.7.2

Source: 	Transsion Holdings

Title:  	Discussion on support of IMS voice call over GSO

Document for:	Discussion and Decision

Introduction

In the last meeting, the the support of IMS voice call over NB-IoT NTN was discussed, and the following agreements were made[1]:

Agreements regarding the reply LS to SA2:

1.	Regarding SA2 Q1, RAN2 is not able to make any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC, and thinks the implementation of ROHC functions (e.g., compression and decompression) falls outside of RAN2 scope

2.	Regarding SA2 Q3, RAN2 responds to SA2 that scheduling methods may handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL), but RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues.

3.	Regarding SA2 Q6, RAN2 responds to SA2 that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

4.	Regarding SA2 Q9 and Q10, RAN2 responds to SA2 that it is technically feasible to introduce SRB with RLC UM for voice packet

5.	We add a sentence in the same reply LS saying that majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary.

Agreements regarding the reply LS to SA4:

1.	RAN2 assumes the MAC header size may be 1 to 3 bytes. However, in most common scenario, the total MAC header size is likely to be 3 bytes.

2.	For transmitting voice packets via the UP solution RAN2 expects 1 byte for PDCP + 1 byte for RLC UM + the MAC header size

3.	For transmitting voice packets via the CP solution RAN2 expects 2 bytes for RRC + 2 bytes for RLC AM (the only mode currently specified for CP) or 1 byte for RLC UM (if RLC UM will be introduced for SRB in case a CP based solution will be selected) + the MAC header size

4.	RAN2 will provide further feedback on the expected average RoHC header size

5.	We indicate to SA4 that RAN2 has not started work on SPS for voice but thinks that SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 would require additional specification work in RAN2 to resolve the issues

In this contribution, we provide our views on some of these issues to proceed the work item.

Discussion

CP and UP solutions selection for voice support over NB-IoT-NTN

In the TR 23.700-19 many solutions are proposed to support IMS voice call over NB-IoT NTN, however many of the solutions are from the procedure’s point of view, they lack of the performance analysis and the impact on RAN. When down-select between CP and UP solutions, the selected solution should meet the performance requirements of IMS voice and reduce the impact on RAN impact.

The performance requirements of IMS voice using GEO satellite access defined in SA1 TS22.261 as follows:

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity(DL) | Area traffic capacity(UL) | Overall user density | Activity factor | UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IMS voice call using GEO (note 7) | [1-3] kbit/s | [1-3] kbit/s | N/A | N/A | N/A | N/A | TBD | Handheld | Call set up time (note 8) ≤30 s (note 9) |
| Note 7: All the values defined for IMS voice call using GEO satellite access are generic for both regular and emergency calls. Note 8: call set up time refers to [56]; Note 9: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [57]; |  |  |  |  |  |  |  |  |  |

In legacy, IMS voice is using user plane, i.e., one DRB for IMS signaling and one DRB for voice packet, both the DRBs use GBR bear. For NB-IoT CP solution only SRB1bis is used for NAS message transmission and DRB is not used. If CP solution is used for IMS voice, we may need to decide whether to introduce new SRB for IMS voice, the legacy NAS message data、IMS signaling data and the voice data maybe need to distinguish, the QoS for different data maybe need to be discussed, while for UP solution the existing mechanism maybe can largely reused, so we suggest the UP solution is used for voice support over NB-IoT-NTN.

- Proposal 1: 	UP solution is recommended for voice over NB-IoT-NTN from RAN’s points of view.

Support of Dynamically changed voice data

- In last meeting, the support of dynamically changed voice data triggered by the question from SA2 was discussed and only conclude that scheduling methods may handle the voice packets of different sizes, the details are for further study. Many candidate solutions are raised by companies(e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL). In legacy the SPS can be used for voice data to overcome the scheduling delay, if the dynamic scheduling is used to support the dynamic changed voice data, as its large RTT delay compared with the voice frame period, it may cause the voice data scheduled by dynamic scheduling become obsolete unless the dynamic schedule can be advanced, so we think the dynamic scheduling may not suitable in this case. For two SPS configuration with activate and deactivate, we think it may easy to handle in DL while not in UL as the network may not be able to promptly detect or know the changes in the uplink voice packets. So we think the one SPS configuration with over-provisioning of resources maybe a compromise choice.

- Proposal 2: One SPS configuration with over-provisioning of resources can be used to support dynamically changed voice data.

Call RRC connection setup procedure

For UEs in IDLE mode, when the UE initiates MO call or receives a MT call, the UE may currently camps on a satellite which maybe not suitable for IMS voice. There are two scenarios which the satellite maybe not suitable for IMS voice, one is the satellite may not support NB-IoT-NTN IMS voice, the other one is the satellite may soon stop providing service for the UE. For the first scenario, if the UE still try to initiate RRC connection setup with the satellite, the network may have to release the UE and redirect the UE to a satellite supporting the IMS voice as the NB-IoT UE don’t support handover, which will cause long call setup delay. So the UE is suggest to re-select to a satellite supporting IMS voice when the UE intends to initiate a MO call or receiving a MT call. To achieve this, the satellite IMS voice capability information of both current serving satellite and neighbor satellites should be provided to UE, and the UE trigger a cell re-selection if the UE initiates a voice call if the current camped satellite doesn’t support IMS voice. For the other scenario, if the UE still initiates the voice call on these satellite, it may cause voice interruption and the repeated call setup procedure as the UE don’t support handover. In R18 and R19 the time-based and location-based mobility has been introduced for IoT-NTN to decide to trigger mobility. The similar principle can be used for the UE to decide whether it can use the camped satellite to initiate voice call, thus if the remaining serving time is not enough for the UE to finish a IMS call, the UE re-select to another satellite which can satisfy the time demand of the voice call.

- Proposal 3: The satellite IMS voice capability information of both current serving satellite and neighbor satellites should be provided to UE.

- Proposal 4: UE triggers a cell re-selection if the UE intends to initiate a MO call if the current camped satellite doesn’t support IMS voice.

- Proposal 5: If the remaining serving time is not enough for the UE to finish a IMS call, the UE re-select to another satellite which can satisfy the time demand of the voice call to avoid voice interruption.

- Another issue is the the establishment cause for voice and emergency call is not supported for NB-IoT and need to be introduced when support IMS voice over NB-IoT-NTN.

| Establishment Cause for NB-IoT (Not including voice nor emergency) | Establishment Cause for LTE (Including voice and emergency) |
| --- | --- |
| -- ASN1START EstablishmentCause-NB-r13 ::= ENUMERATED { mt-Access, mo-Signalling, mo-Data, mo-ExceptionData, delayTolerantAccess-v1330, mt-EDT-v1610, spare2, spare1} -- ASN1STOP | EstablishmentCause ::= ENUMERATED { emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare1} |

- Proposal 6:	Introduce establishment cause for Voice call in EstablishmentCause-NB-r13.

Conclusion

In this contribution we discussed issues related to uplink capacity enhancement in IoT-NTN and made the following proposals:

- Proposal 1:	 UP solution is recommended for voice over NB-IoT-NTN.

- Proposal 2: One SPS configuration with over-provisioning of resources can be used to support dynamically changed voice data.

- Proposal 3: The satellite IMS voice capability information of both current serving satellite and neighbor satellites should be provided to UE.

- Proposal 4: UE triggers a cell re-selection if the UE intends to initiate a MO call if the current camped satellite doesn’t support IMS voice.

- Proposal 5: If the remaining serving time is not enough for the UE to finish a IMS call, the UE re-select to another satellite which can satisfy the time demand of the voice call to avoid voice interruption.

- Proposal 6: Introduce establishment cause for Voice call in EstablishmentCause-NB-r13.

Reference

- “Draft_RAN2_131bis_MeetingReport_v1.docx”