3GPP TSG-RAN WG2 Meeting #132                            R2-2508225

Dallas, USA, Nov.17th–21th, 2025

Agenda Item:	10.3.1.3

Source: 	Transsion Holdings

Title:  	Discussion on L2 re-transmission and scheduling in 6G

Document for:	Discussion and Decision

Introduction

In the last meeting, the UP for 6G were discussed, and the following agreements were made[1]:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

In this contribution, we provide our views on these aspects to proceed the study item.

Discussion

Re-transmission functions in Layer 2 for 6G

The L2 re-transmission function includes HARQ in MAC and ARQ in RLC. The HARQ in MAC can provide fast error detection feedback and fast retransmission, but the reliability can’t be 100% guaranteed due to the possible NACK-to-ACK detection in L1, the ARQ in RLC is provided as a supplement to solve the HARQ error and also the ARQ can solve other errors caused by the non-channel conditions. So we think the HRAQ in MAC and ARQ in RLC are still needed in 6G.

- Proposal 1:	 Both the HARQ in MAC and ARQ in RLC are still needed if the MAC Layer and RLC layer exists in 6G.

For ARQ the re-transmission decision is based on RLC STATUS report from the receiving side which is mainly based on timer and the polling from the sending side, so the re-transmission decision is much later compared with HARQ which may not suitable for some services. So the RLC ARQ can be enhanced to ensure reliability while reducing the delay in retransmission decision-making. Now the ARQ decision is based on the STATUS report from the peer entity, but before the status report indicating a missing RLC SDU, the HARQ feedback of NACK in MAC related to the missing RLC SDU may has already provided multiple times, but do the the lack of the real-time HARQ information of MAC, the RLC re-transmission decision is quit delayed. So to reducing the ARQ re-transmission decision delay, the HARQ feedback information in MAC related to a RLC SDU can be provided to RLC.

- Proposal 2:	 The HARQ feedback information in MAC related to a RLC SDU can be provided to RLC to enabled a fast RLC re-transmission decision.

Scheduling enhancement for 6G

  - In last meeting it has been agreed to support at least dynamic grant and configured grant and further study configured grant. One of the configured grant enhancement discussed in the meeting is the contention based configured grant, thus the configured grant are shared by UEs. In R19 IoT-NTN, the CB-Msg3-EDT procedure was introduced in which the PUSCH resources are shared by UEs to send Msg3, with these UE can initiate Msg3 transmission without msg1/RAR, it is considered to be capable of improving the uplink capacity, we think the similar mechanism can be introduce in 6G, thus the shared PUSCH resources can be shared by UEs to initiate Msg3 transmission without msg1/RAR if UEs have a valid TA value

- Proposal 3:	 Introducing shared configured grant for UEs to initiate access in 6G, the CB-Msg3-EDT procedure in R19 IoT-NTN can be a baseline.

Conclusion

In this contribution we discussed the Re-transmission functions in Layer 2 for 6G and made the following proposals:

- Proposal 1:	 Both the HARQ in MAC and ARQ in RLC are still needed if the MAC Layer and RLC layer exists in 6G.

- Proposal 2:	 The HARQ feedback information in MAC related to a RLC SDU can be provided to RLC to enabled a fast RLC re-transmission decision.

- Proposal 3:	 Introducing shared configured grant for UEs to initiate access in 6G, the CB-Msg3-EDT procedure in R19 IoT-NTN can be a baseline.

Reference

- “Draft_RAN2_131bis_MeetingReport_v1.docx”