3GPP TSG-RAN WG2 Meeting #132	           R2-2508498

Dallas, USA, November 17th – 21st, 2025

Agenda item:	10.3.3.1

Source:	China Telecom

Title:	Initial consideration on AI/ML for 6GR

Document for:	Discussion

- Introduction

During the first RAN2 meeting for 6GR study, RAN2 has some initial agreements on both data collection, model transfer/delivery and LCM for AI/ML. In this paper, we would like to provide some initial consideration on AI/ML for 6GR.

- Discussion

RAN2#131bis meeting agreed LCM framework functions are as the following:

Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

In NR, the LCM procedures are designed for AI/ML functionality using offline trained model. Compared to offline trained model, online learning has the following benefits:

- Learn instantly from new data and trends.

- Continuously update, avoiding model staleness.

- No need for costly, periodic full retraining.

During 6GR study phase, we think at least RAN2 can study whether to consider online trained model in LCM framework and procedure design, rather than take the 5G-A LCM design as baseline, before going into details.

Proposal 1: Before detailed design of LCM, RAN2 study whether to consider online trained model in LCM design.

In NR, RAN2 try to reuse resource and reporting configuration framework in legacy non-AI/ML function as much as possible, and spend much time to discuss signaling and procedure for inference configuration and reporting case by case. For example, AI/ML PHY reuses CSI framework for resource for inference and inference reporting configuration for AI/ML based beam management, which is not applicable for AI/ML based positioning and AI/ML mobility. Different frameworks are used for different AI/ML cases. In 6GR, we kindly suggest to consider common signaling/procedure design for different AI/ML cases, at least including applicable functionality reporting, inference configuration and reporting, in order to improve specification impact discussion.

Proposal 2: Try to consider common signaling design for LCM across different AI/ML cases.

In NR, the following figure is captured in TR 38.843 to describe functional framework for AI/ML PHY:

Fig. functional framework for AI/ML for NR Air Interface

Only data collection for network-sided model, functionality management and inference are specified. Regarding performance monitoring, simplified solutions including performance metric, monitoring configuration and monitoring result reporting are discussed and specified. However, some issues are not discussed, e.g. the performance feedback and the validity of performance monitoring.

In our understanding, performance monitoring is important for deciding whether the fallback to non-AI/ML function and retraining model are necessary or not and avoiding network performance decline due to AI/ML functionality. In NR, the UE can report monitoring result to the network and the network calculate the performance metric. Another way is that the UE calculate the performance metric based on the monitoring result and report to the network. However, the UE is unaware of whether the performance metric is good enough in both monitoring methods, since there is no performance feedback. Thus the UE can’t decide whether to request the model training server to retrain the model or switch to another model.

Proposal 3: Study performance feedback and the validity of performance monitoring in 6G AI/ML LCM framework.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: Before detailed design of LCM, RAN2 study whether to consider online trained model in LCM design.

Proposal 2: Try to consider common signaling design for LCM across different AI/ML cases.

Proposal 3: Study performance feedback and the validity of performance monitoring in 6G AI/ML LCM framework.