3GPP TSG-RAN WG2 Meeting #132	R2-2508559

Dallas, US, 17th –21st Nov., 2025

Agenda Item:	8.2.2

Source:	Huawei, HiSilicon

Title:	A-IoT remaining issues

Document for: Discussion and Decision

# 1	Introduction

- Excerpts from some of RAN2#131bis agreements on Rel-19 A-IoT Wait for SA2 conclusion before finalizing length ID field size RAN2 confirms, in addition to delayed response, it is valid that in some cases A-IoT NAS doesn’t provide a response at all. For cases other than integrity failure, AS will indicate no NAS response expected to reader. FFS how (e.g. using 0 SDU & MDI, or new indication). For integrity failure, for now RAN2 assumes that there is no AS response to the reader. Ask SA3 ccCT1 whether a similar mechanism (e.g. AS response to the reader) can be used to indicate to reader no NAS response due to integrity failure. After December, NBC changes should be avoided as much as possible similar to NR MAC process.

Based on the current RAN2 progress and incoming LS, and focusing on the issues without conclusions, this contribution discusses the following issues:

- Security parameter in paging message (Issue 1-7)

- SA2 LS on device permanent ID length (Issue 1-2)

- Data not available case (Issue 3-7)

- Paging ID type (Issue 6-3)

- Reply to CT1 LS on handling of inventory and command collision Cl-256624 (Issue 6-1)

- CT1 LS on no Command Response in NAS

# 2	Discussion

## 2.1	128-bit security parameter in paging message

In the RAN2#131bis meeting, RAN2 concluded to add a optionality bit for 128bits security field in paging message. Some questions were sent to SA3 for further check:

RAN2#131bis agreement Send an LS to SA3, cc:CT1 - Question on whether the security parameter has to be included in every paging message. - Clarify that we have different types of messages, paging message and subsequent paging. - While it is feasible from signaling perspective, RAN2 has concerns - RAN2 has discussed the following concerns and downsides with making the 128bits mandatory for every paging message: - deployments where it may not be always needed - overhead is high and complexity for devices - power consumption overhead with authentication (time consuming, etc) - coverage In RAN2 agree to: - Add a 1 bit optionality bit for 128bits security field in paging message. For now, we state in our specification that this bit is set to present in this release according to SA3 TS. If SA3 confirms that it can be optional after LS reply it shall be updated.

For now, RAN2 wait for SA3 reply for MAC spec further updates.

## 2.2	SA2 LS on device permanent ID length (Issue 1-3)

In the last meeting, for Paging ID extension:

Wait for SA2 conclusion before finalizing length ID field size

According to SA2 LS:

S2-2507793: LS on AIoT Device Permanent ID Length SA2 has discussed extending the length options of the AIoT Device permanent ID (i.e., the Identification Information part) to support 256-bit and 496-bit. The rationale for this extension has received general support. SA2 would like to clarify that the other parts of the AIoT Device permanent ID (i.e. ID type, domain information) is not impacted. SA2 roughly estimated that the filtering information will be around 600-bit in case the Identification Information is 496-bit, and around 360-bit in case the Identification Information is 256-bit. The maximum length of filtering information will be decided by CT4 WG. In addition to filtering information, SA2 understands the overall length of the paging message may need to consider other information like security parameters, etc. SA2 would like to check with RAN2 on whether extending the AIoT Device permanent ID to 256-bit and 496-bit is feasible from RAN2 point of view.

As analysed by SA2, the maximum of filtering information will be around 600 bits. This means current 8-bit “Paging ID Length” is not sufficient. To indicate up-to 600-bit length filtering information, at least 10-bit “Paging ID Length” is needed. RAN2 assume to extend “Paging ID Length” field to 10 bits, once SA2 conclude to extend AIoT Device permanent ID to 496 bits.

Proposal 1:	(Issue 1-3) RAN2 extends the paging ID length field as 10 bits, once SA2 conclude to extend AIoT Device permanent ID to 496 bits.

## 2.3	Data not available case (Issue 3-7)

For the issue of non-available NAS, the discussion progress in the RAN2#131bis meeting is:

RAN2#131bis agreements: - For cases other than integrity failure, AS will indicate no NAS response expected to reader. FFS how (e.g. using 0 SDU & MDI, or new indication). - For integrity failure, for now RAN2 assumes that there is no AS response to the reader. Ask SA3 ccCT1 whether a similar mechanism (e.g. AS response to the reader) can be used to indicate to reader no NAS response due to integrity failure.

For integrity failure, RAN2 wait for SA3/CT1 reply with assumption that no AS response to reader.

For cases other than integrity failure, to indicate no NAS response expected to reader, reusing fields of SDU and MDI is simpler without introducing new impact on MAC format. Since the “SDU=0” already implies the “no immediate D2R response”, we can use the MDI field to indicate the different cases. In our understanding, “more data” (i.e., MDI=1) can imply the device may have more data later, while “no more data” (i.e., MDI=0) is suitable to imply there is no response at all.

Proposal 2:	(Issue 3-7) Use “0 SDU & MDI =0” to indicate “no upper layer data available at all” and “0 SDU & MDI=1” to indicate “no upper layer data available due to delay NAS”.

## 2.4	Delayed NAS in case of CFA inventory (Issue 6-2)

Another new open issue is:

[POST131bis][017][AIoT] MAC CR New issue 6-2: Confirm to apply the no data available case to more general delayed NAS cases, e.g., inventory response message.

In our view, in the current RAN2 agreement, the case “upper layer data is not available to be transmitted” applies not only when command response is delayed, and this includes more general delayed NAS cases. For example, for CFA, some device implementation may not deliver the the inventory response on time, e.g., due to the processing delay caused by security-related operations. This may also cause delayed NAS data. In this case, a delayed inventory response can be scheduled by follow-on R2D Upper Layer Data Transfer messages with “Received Data Size=0”.

Proposal 3:	(Issue 6-2) RAN2 agrees to apply the “no data available” handling to more general delayed NAS cases, e.g., CFRA inventory response is also allowed to be re-scheduled by R2D Upper Layer Data Transfer message with offset=0.

## 2.5	CT1 LS on no Command Response in NAS

CT1 sent LS on no Command Response in NAS to RAN2, i.e., C1-256623:

CT1 LS C1-256623: CT1 has agreed a pCR that the AIoT device shall provide an indication to the lower layer when a received AIoT NAS message was ignored. CT1 kindly asks RAN2 to take the above information into account in A-IoT Data Transmission.

In the agreed CR C1-256640:

CT1 CR (C1-256640): ### 6.2.1 Message too short When a message is received that is too short to contain a complete message type information element, that message shall be ignored, c.f. 3GPP TS 24.007 [3], and the AIoT device shall provide an indication to the lower layers that the message was ignored.

RAN2 should notice that the upper layer provides a NAS layer indication in case of no NAS response.

Proposal 4:	RAN2 can capture the NAS layer indication in MAC spec after RAN2 concludes all the “no upper layer data available” cases.

## 2.6	Paging ID type (Issue 6-3)

In LS from SA2 to CT4:

S2-2509655: SA2 thanks CT4 on the LS on Structure updates of AIoT Identifiers (S2-2508179/C4-253575), and has agreed the attached CR0072 to TS 23.369 to align with CT4 specification. SA2 has also agreed the attached CR0106 to TS 23.369 on the definition of AIoT Identification Information, and kindly asks CT4 to complete the corresponding stage 3 work.

The agreed attached CR0106 includes:

9. Upon reception of the Inventory Request message from the AIOTF, the RAN Reader(s) will execute the inventory operation as specified in TS 38.300 [5] and TS 38.391 [11]. The RAN Reader(s) broadcast the paging message that includes the AIoT Device Identification Information and the security parameters. If the received AIoT Identification Information contains: - Filtering Information, the AIoT Device determines whether it matches the AIoT Identification Information, as described in clause 5.8. - AIoT Device Temporary Identifier, the AIoT Device determines whether it matches the AIoT Identification Information, as described TS 33.369 [9]. - AIoT Device Permanent Identifier, the AIoT Device determines whether it matches the AIoT Identification Information by comparing it with the stored AIoT Device Permanent Identifier. If an AIoT device matches the AIoT Identification Information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its device specific authentication information as specified in TS 33.369 [9] and optionally the AIoT Device Permanent Identifier if privacy protection is not used.

According to the CR, there is a case that the AIoT Identification Information contains Temporary ID (T-ID), and different ID types impact the device behaviour. As described in clause 5.4.3 in TS 33.369:

### 5.4.3 Procedure for AIoT Device identifier protection with Temp ID update during Individual inventory For the protection of AIoT device permanent identifier during the inventory procedure with AIoT device identifier described in clause 5.2.2, the following changes shall apply: - In step 1, AIOTF shall retrieve a T-ID in addition to the RANDAIOT_n from ADM. The ADM shall, based on T-ID type, either fetch the stored T-ID in the AIoT device profile or generate the T-ID as specified in Annex B.1. - In step 2, 3 and 4, the T-ID shall be used as a device identification information. - In step 2 and 3 the AIOTF includes indication of type of T-ID handling. T-ID can be either concealed type or stored type. The concealed type can be based on either the stored T-ID or the permanent identifier. If needed the handling also indicates whether the stored T-ID type shall be updated with or without a command. NG-RAN includes the T-ID handling in the paging message. - In step 4, the AIoT device, based on the T-ID handling indication in the paging message, generates the T-ID in the same way as the ADM did in step 1. The AIoT device determines it needs to reply to the NG-RAN if the generated T-ID matches with the received T-ID. In case the stored T-ID update shall be done without a command, the AIoT Device generates a new Temp_ID_n+1 as specified in Annex B.1 and stores the new Temp ID_n+1. - In step 5 and 6, a device identification information is not included in the D2R message and Inventory Report message. - In step 7, the AIoT device permanent identifier is used as a device identification information. AIOTF requests the ADM to derive a new T-ID as specified in Annex B.1 and to store it in the AIoT Device profile. NOTE 1: The AIOTF identifies the AIoT device by checking the received RESAIoT parameter. Therefore, the device identification information is not needed in the D2R message and Inventory Report message. NOTE 2: In case of concealed T-ID type, every AIoT devices that receive an Inventory Request with T-ID need to perform a T-ID matching by generating a T-ID based on the KAIoT_root and check if the generated T-ID is matched with the received T-ID. It is assumed that the AIoT device that receive the Inventory Request has enough energy to perform this T-ID matching in addition to the Inventory procedure specified in clause 5.2.2. NOTE 3: In case of stored T-ID type, the stored T-IDs on the device side and network side can get out-of synch. The handling of such situation is described in clause

As analysed above, how the device behaves needs some indications:

- whether Filtering Information, AIoT Device Permanent Identifier, concealed, or the stored T-ID is contained in Paging ID,

- if stored T-ID is contained, whether the stored T-ID type shall be updated with or without a command.

Based on the LS, CT4 can update the ID structure with consideration of the needed indication. So, it is suitable to define the Paging ID type indication in the ID structure design by CT4. If so, no additional RAN2 work is needed.

Proposal 5:	(Issue 6-3) RAN2 assumes CT4 will address the Paging ID type indication, since SA2 already informed CT4 to continue the work.

## 2.7	Reply to CT1 LS on handling of inventory and command collision (Issue 6-1)

CT1 sent LS to RAN2 in C1-256624:

Based on above RAN2 agreement, CT1 has discussed the following two scenarios: a) Collision between inventory procedure and inventory procedure: The AIoT NAS layer receives a new inventory from lower layer when the AIoT NAS layer is performing the inventory procedure b) Collision between inventory procedure and command procedure: The AIoT NAS layer receives a new inventory from lower layer when the AIoT NAS layer is performing the command procedure CT1 concluded that there can be an attack scenario of false AIoT pagings that would abort any ongoing valid paging or command, and thereby temporarily or continuously block valid inventory procedure and command procedure execution by the AIoT devices. Hence, CT1 kindly ask RAN2 to provide their view on whether it is possible for the AIoT device to respond to the new AIoT paging after completion of an ongoing inventory or command when collision happens.

CT1 mentioned two issues: 1) collision between two inventory procedures or inventory and command procedures, and 2) false AIoT paging from an attacker would abort the ongoing valid procedure.

For the first issue, in Rel-19, RAN2 agreed that parallel service is not supported, which is also captured in TS 38.300:

TS 38.300 v19.0.0 (2025-09): #### 16.23.5.2 A-IoT Paging A-IoT paging allows the A-IoT reader to trigger one, multiple or all A-IoT device(s) to perform A-IoT CBRA or A-IoT CFA. The A-IoT paging message is transmitted on PRDCH. The A-IoT paging message may include zero or one paging identifier, i.e., AIoT Device Permanent Identifier or Filtering Information as specified in TS 23.369 [68]. If a paging identifier is included, the A-IoT paging message may be addressed to a single A-IoT device or a group of A-IoT devices. If no paging identifier is included, the A-IoT paging message is addressed to all A-IoT devices. The A-IoT paging message also provides configuration for A-IoT access procedure. NOTE: In this release, the A-IoT device is not expected to process parallel service requests indicated by A-IoT paging messages, and relies on network implementation to address the issue of parallel service requests caused by A-IoT reader overlapping scenario.

So the network implementation should avoid collisions between different procedures.

For the second issue, this issue cannot be addressed by RAN2 solution. Based on RAN2 agreement, in a single-reader scenario, a reader may send a new paging message to start a new service procedure even while there is an ongoing service. Thus, a device shall respond to any new paging message to avoid missing the new service. Since the device cannot distinguish between legal and fake paging messages, it will also respond to a fake new paging message from an attacker.

Observation 1:	Based on RAN2 agreement, in a single-reader scenario, the device cannot detect whether the paging is from a legal reader or fake reader, because a legal reader is also allowed to send new paging request even if there is on-going service.

Proposal 6:	(Issue 6-1) RAN2 replies to CT1 on handling of inventory and command collision:

For the collisions between two inventory procedures or inventory and command procedures, it can be assumed that network implementation can handle this correctly;

For the issues of receiving false AIoT paging which abort the ongoing valid procedure, this cannot be addressed by RAN2 solution. RAN2 decides to follow the current RAN2 agreement.

# 3	Conclusion

This contribution makes the following proposals:

SA2 LS on device permanent ID length (Issue 1-3)

Proposal 1:	(Issue 1-3) RAN2 extends the paging ID length field as 10 bits, once SA2 conclude to extend AIoT Device permanent ID to 496 bits.

Data not available case (Issue 3-7)

Proposal 2:	(Issue 3-7) Use “0 SDU & MDI =0” to indicate “no upper layer data available at all” and “0 SDU & MDI=1” to indicate “no upper layer data available due to delay NAS”.

Delayed NAS in case of CFA inventory (Issue 6-2)

Proposal 3:	(Issue 6-2) RAN2 agrees to apply the “no data available” handling to more general delayed NAS cases, e.g., CFRA inventory response is also allowed to be re-scheduled by R2D Upper Layer Data Transfer message with offset=0.

CT1 LS on no Command Response in NAS

Proposal 4:	RAN2 can capture the NAS layer indication in MAC spec after RAN2 concludes all the “no upper layer data available” cases.

Paging ID type (Issue 6-3)

Proposal 5:	(Issue 6-3) RAN2 assumes CT4 will address the Paging ID type indication, since SA2 already inform CT4 to continue the work.

Reply to CT1 LS on handling of inventory and command collision (Issue 6-1)

Observation 1:	Based on RAN2 agreement, in a single-reader scenario, the device cannot detect whether the paging is from a legal reader or fake reader, because a legal reader is also allowed to send new paging request even if there is on-going service.

Proposal 6:	(Issue 6-1) RAN2 replies to CT1 on handling of inventory and command collision:

For the collisions between two inventory procedures or inventory and command procedures, it can be assumed that network implementation can handle this correctly;

For the issues of receiving false AIoT paging which abort the ongoing valid procedure, this cannot be addressed by RAN2 solution. RAN2 decides to follow the current RAN2 agreement.