# 3GPP TSG RAN WG2 #132									 R2-2508583

# Dallas, USA, Nov. 17th – 21st, 2025

# Source:	Hanbat National University

# Title:		Discussion on Data transfer, model transfer, and AIML

# Agenda item:	10.3.3.1

# Document for:	Discussion/Decision

Introduction

RAN2 made the following agreements on data collection framework with some requirements as a baseline of the 6GR[1] in 3GPP RAN2#131-bis meeting [2]:

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

For the LCM, RAN2 also made the following agreements in 3GPP RAN2#131-bis meeting [2]:

Agreements on LCM Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

Meanwhile, Model Transfer requirement is also observed in 3GPP RAN2#131-bis meeting [2]. Due to the lime limitation, however, there was no discussion. In this contribution, hence, we discuss the model transfer regarding requirement and the handling the operation for model transfer related to AI/ML for 6GR.

Discussions

RAN2 made some progress/direction how to study the data collection framework with requirements and considering aspects for this meeting.

We have observed the following:

Observation1: RAN2 considers AI/ML related data as a type of data which may be same or different type to/with general data (i.e., non-AI/ML related data).

We think that AI/ML related data transmission leads to some discussion points with technical impact on the 6GR, regardless of whether AI/ML data is considered as a type of data. Such as:

- How to minimize the impact to the UE battery, UE processing and memory utilization (as agreed in this issue).

- How to activate/deactivate AI/ML related data transmission.

- How to meet the QoS requirements, e.g., latency, priority, packet error rate.

Hence, we propose the following for the discussion in details:

Proposal1: Discuss and study potential impacts by the introduction of AI/ML related data as a type of data, at least how to activate/deactivate the AI/ML related data transmission and to allocate/transmit the AI/ML related data with meeting the requirements of the RAN2 perspective.

Conclusion

In this contribution, we discussed AI/ML related data. As a general design by considering the AI/ML related data as a type of data type, we observe and propose, respectively:

Observation1: RAN2 considers AI/ML related data as a type of data which may be same or different type to/with general data (i.e., non-AI/ML related data).

Proposal1: Discuss and study potential impacts by the introduction of AI/ML related data as a type of data, at least how to activate/deactivate the AI/ML related data transmission and to allocate/transmit the AI/ML related data with meeting the requirements of the RAN2 perspective.

- References

- RP-251881, “Study on 6G Radio,” Jun. 2025.

- 3GPP RAN2 Chair’s notes RAN1#131bis, Oct. 2025.