3GPP TSG-RAN WG2 #132	R2-2508638

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	8.7.2

Source:	InterDigital, Inc.

Title:	Remaining RLC issues for XR

WID/SID:	NR_XR_Ph3-Core – Release 19

Document for:	Discussion

# Introduction

In the previous meeting, two issues on RLC were discussed but they were left for meeting:

- (RLC-N02) Companies will bring contribution in next meeting to explain the use cases and the TP. - (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it

This TDoc discusses those issues.

# Discussion

## RLC-N02

A NOTE was introduced in Clause 5.3.3.3 to capture the agreement from RAN2#130:

NOTE: When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

The intent was to avoid unnecessary polling after expiry of the t-PollRetransmit timer when the data to be polled may have already been discarded. However, the ACK/NACK case is already handled by the specification and the “may” part talks against this normative principle:

#### 5.3.3.3 Reception of a STATUS report Upon reception of a STATUS report from the receiving RLC AM entity the transmitting side of an AM RLC entity shall: - if the STATUS report comprises a positive or negative acknowledgement for the RLC SDU with sequence number equal to POLL_SN: - if t-PollRetransmit is running: - stop and reset t-PollRetransmit.

Hence, the NOTE should cover only the discarding part and we can remove the “positively or negatively acknowledged”.

Proposal 1: Modify the NOTE captured in Clause 5.3.3.3 as follows:

NOTE:	If stopRetxDiscardedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN are indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

## RLC-H02

As noted in section 1, the following was agreed to be discussed further:

- (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it

The issue with indicating ACK_SN as the RX_Highest_Status in this case is that the transmitter is left unaware that the RLC SDUs with SN falling between RX_Highest_Status <= SN < RX_Next are already considered successfully received by the Rx RLC entity and the Tx window could be moved forward.

Observation 2: By the current specification, the transmitter does not get the information of all successfully received RLC SDUs when the RX_Next is larger than RX_Highest_Status upon t-RxDiscard expiry.

It seems unusual such information would not be conveyed to the receiver when available and it seems as a low hanging fruit to be added at this phase. Therefore, we think it should be specified that whenever t-RxDiscard timer expires and STATUS PDU is triggered, the RX_Next is used as the ACK_SN in the resulting STATUS PDU.

Proposal 2: When STATUS PDU is triggered by the t-RxDiscard timer expiry, the ACK_SN is set to RX_Next..

# Conclusion

This document has made the following observations and proposals:

Proposal 1: Modify the NOTE captured in Clause 5.3.3.3 as follows:

NOTE:	If stopRetxDiscardedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN are indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

Observation 2: By the current specification, the transmitter does not get the information of all successfully received RLC SDUs when the RX_Next is larger than RX_Highest_Status upon t-RxDiscard expiry.

Proposal 2: When STATUS PDU is triggered by the t-RxDiscard timer expiry, the ACK_SN is set to RX_Next..