3GPP TSG-RAN WG2 #132	R2-2508645

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	10.3.3.2

Source:	InterDigital, Inc.

Title:	Energy efficiency for 6GR

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion

# Introduction

RAN approved a new SID on 6G Radio at RAN#108 [1] to study the next generation 3GPP radio. The study on 6G contains the following energy efficiency related objectives [1]:

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Target scalable and forward compatible design for diverse device types.

In RAN2#131bis, common UP/CP energy efficiency was discussed briefly. Relevant R1 and R2 agreements are captured in Appendix A. 

This contribution outlines initial views on energy efficiency framework for 6G, common for UP and CP.

# Discussion

Energy efficiency aims to reduce energy consumption, emissions, and operational costs associated with cellular systems, by enabling UE and network energy savings. Energy efficiency can be achieved by adaptation of network and UE transmission and reception resources, while limiting the impact on user performance and experience.

## UE and network energy efficiency

As the SI objective states “Energy efficiency and energy saving: both for network and device”, this should  aim to design energy savings from both the network and UE perspective while overcoming the backward compatibility limitations of 5GA. RAN2 discussed in 131bis how to achieve this, including proposals on balancing UE and NW power/energy savings, or configuration of one or the other.

As striking such balance can be difficult to measure, it can be best to start by focusing on NW energy saving first and considering UE power saving impacts. RAN2 should thus review existing energy and power saving techniques and identify those with the most tangible benefits. For each NW energy saving feature (e.g. cell DTX, on demand signals/channels), RAN2 should study how UE power saving features can align (e.g. PDCCH monitoring). For each UE power saving feature, R2 need to understand and evaluate the impact on NW energy efficiency.

Any UE requirement needed to support baseline 6G network energy savings will most likely become part of the minimum mandatory function set for all 6G UEs. Building on this, RAN2 can then introduce additional UE-based energy-saving techniques.

Observation 1: The design of network-side energy reduction techniques will largely determine the minimal set of mandatory functionalities that must be supported by all 6G devices and device types.

Proposal 1: Energy efficiency should be first addressed from the radio access network operation perspective. For each NW energy saving feature, RAN2 to study how to align UE power saving features and evaluate their impact on network energy saving gains.

## Design targets and observations from NR

6GR design should enable reduction of the operator’s operational costs in terms of spectral usage and energy consumption. While NR provided some support for UE and NW energy saving, the following drawbacks and observations from NR should be considered for 6G:

- fragmentation of supported power saving techniques: NR introduced UE power saving enhancements in R16 and network energy saving (NES) features later in R18, never from day-1 in R15. UE power saving features include LP-WUS/WUR, DCP, PDCCH skipping, bandwidth adaptation, and SCell activation/deactivation. Introduced features vary from one UE to another, as many UEs are not capable of energy and power saving techniques, which makes it difficult for the network to assume a common baseline that all UEs will assume for energy savings. This resulted in relying on cell barring for some UEs in order to support cell-wide energy savings techniques, which in turn makes some features less attractive to use if it only serves a subset of UEs. Some cells thus need to operate without those technique if they serve UEs that don’t support them, or bar such UEs. 6G design in the first release needs to identify which techniques to be assumed for all UEs.

- Backwards compatibility: Even though NR reduced always-on signals compared to LTE, some broadcast signals are still periodically broadcast at fixed periodicities, e.g. for PCells, to support legacy UEs. Among the network energy saving methods studied in the NR NES study item [2], few were considered for NR R18 and R19 specification, as others had considerable impact on legacy UEs. Many evaluated techniques with higher energy saving gains had impact on legacy UEs, e.g. for PCell deployments and due to legacy assumptions for cell common signals and channels. Most considered NES techniques were therefore limited to SCells or for UE dedicated signals and channels, without impacting common signals and channels or camped UEs in non-connected states. The energy saving gain from R18 and R19 NES techniques were therefore limited.

- Disjoint UE and network energy efficiency: NR introduced UE power saving and network energy saving features independently, and often UE-NW features were not designed jointly with interoperability in mind. This resulted in a suboptimal design for the integration of UE and network power saving, and in some cases duplicated or separate signalling were introduced to support such features independently, largely due to the fragmentation of supported techniques.

- Deployment issues: Some NW energy saving features in NR have seen limited field adoption due to backwards compatibility impacts, limited NES gains, or need for baring legacy and non-capable UEs in the cell deploying the NES feature. This can be alleviated if there are clear assumptions on which UE and NW power saving features are mandatory, and if the 6GR design target is made forward compatible to ensure minimal impact in future releases.

- Complexity: Some introduced NES features studied or specified were too complex to implement. In addition, other features with high complexity resulted in limiting the amount of UE power saving.

- Scheduling and QoS impact: NR scheduling assumptions, including PDCCH monitoring, C-DRX, SR/BSR triggering, are largely based on the LTE protocol stack. These assumptions do not consider the delay budget tolerance that data has, in light of energy saving. NR introduced cell DTX/DRX based on a cell-wide periodic pattern for all UEs, but it does not consider the QoS requirements and attributes associated with the data or different UEs. It is therefore important to consider the impact of energy saving on QoS performance.

Observation 2: Fragmentation of supported NR UE and NW techniques across different UE capabilities and releases, as UE and network energy efficiency were developed independently, resulted in limited adoption of NES techniques due to backwards compatibility concerns for legacy and non-capable UEs.

Observation 3: NR energy efficiency had limited consideration for impact of energy efficiency on scheduling and QoS.

To address these drawbacks, the following design targets for 6G should be considered:

- Predictability of UE supported techniques: 6G design in the first release needs to identify which techniques to be assumed for all UEs. Considerable energy saving gains is not realizable without common support from all UEs. There needs to be an understanding of which day-1 power and energy saving features are considered mandatory (e.g. C-DRX, LP-WUS/WUR, PDCCH skipping, cell DTX/DRX, on demand common signals and channels).

- Scalability and forward compatibility: Energy efficiency features need to be possible to introduce and deploy without having to bar UEs of the first release of 6G. Therefore, the design of 6G should be forward compatible to allow initial release UEs to adapt to features deployed by the network later. A new 6G design thus provides prospects to support UE and network power savings natively without backwards compatibility constraints.

- Consideration of network energy and UE power saving: 6G should consider power and energy saving features jointly, to ensure alignment between the UE and NW and achieve most gains for both the UE and the NW. Therefore, 6G should ensure that UE and NW power saving features are designed with interoperability, and while consolidating features and associated signalling.

- Minimization of always on or periodic common signals: 6GR needs to be efficient from the perspective of minimizing transmissions of always-on signals, while not setting hard assumptions on periodicities of broadcast signals and channels.

- Modularity of protocol processing: The impact of scaling UP and CP protocol processing on UE and NW energy saving should be considered, regardless of which features are in use or supported by the UE and considering all UE types and capabilities.

- QoS-awareness: energy efficiency features in use should in general not result in a considerable impact on UE throughput, power consumption, or QoE. All supported verticals should be supported with limited impact while energy efficiency is applied, or the mobility framework should be adaptable to ensure QoS continuity once a serving cell cannot guarantee the UE’s QoS requirements.

- Energy consumption awareness in CP and UP protocols: as a general design target, the design of the UP and CP protocol stack should consider network and UE power consumption associated with protocol processing scalability.

Observation 4: 6GR energy efficiency framework should target commonality of supported energy efficiency techniques by all UEs in a cell.

Observation 5: 6GR energy efficiency framework should target minimization of always on or periodic common signals and QoS-awareness (incl. impact and consideration of delay budgets and throughput).

## Energy efficiency in RAN

The initial focus should be on building the foundation around network-based energy savings and efficiency, prioritizing time-domain techniques to establish the operational baseline. Spatial- and frequency-domain techniques can then be studied in later phases, once the time-domain baseline is in place. From an L2/3 protocol perspective, energy efficiency techniques can generally be classified as follows:

- Control channel monitoring

Forward compatibility should be built in to allow adaptation of monitoring periodicities without having to resort to cell barring. This includes UE and network power saving features for LP-WUS/WUR (e.g. OFDM-based), C-DRX, PDCCH skipping, cell DTX. These mechanisms should be mandatory for all UEs at day-1 to ensure predictability and allow network-wide assumptions. These features should not be specified and configured separately and rather need to be integrated.

Proposal 2: For energy efficiency in control channel monitoring, the following features are supported from day-1: LP-WUS/WUR, C-DRX, PDCCH skipping, and cell DTX/DRX.

Given energy saving techniques at the UE and NW are well correlated, having to handle the features independent of each other (e.g. via independent configurations/signaling mechanisms) can only lead to more inefficiency in terms of latency and achievable energy saving gains.

Therefore, it is beneficial to configure NW-side and UE-side energy saving techniques together, where they activated and deactivated in a coordinated manner. This may not imply a single configuration entity for UE and NW, but rather a UE configuration that inherently allows NW to align multiples UEs and its own operation consistently. For example, a C-DRX cycle configurations can be adaptively applied in accordance with the corresponding cell DTX/DRX configuration. A NW-side or UE-side energy saving configuration can include radio access parameters based on different levels of processing capability, energy saving and capacity/coverage performance. So that different configurations can be flexibly adapted with minimum latency (e.g. from active mode to non-active mode and vice versa) based on NW energy saving operation as well as available network resources to realize a good balance in terms of energy savings and performance.

Proposal 3: Support adaptation of UE power saving configuration based on NW energy saving operation.

Proposal 4: UE adapts C-DRX cycle to align with the active cell DTX/DRX configuration.

- Turn on/off and adaption of common and dedicated signals and channels:

These techniques address the need to minimize always-on periodic signals, and to ensure forward compatibility for deployment without negatively impacting first-release UEs. This includes adaption of common signals for one or more carriers (SSB-less cells, SSB period adaptation), turn on/off or broadcast channels (e.g. SIB-less, SIB adaptation), reception and requests for on-demand common signals and channels (e.g. on-demand SSB/RS/SIBs), time-domain adaptation of data and resources (PUSCH, PDSCH, PUCCH). 

Most energy saving potential comes from increasing the time during which the network turns off, including transmissions, reception, and baseband processing. The network still consumes energy when not transmitting for other activities such as baseband digital beamforming processing. Such idle power consumption can be considerable in dense networks, e.g., during low cell load conditions. Network energy consumption can therefore be reduced when the network does not exchange transmissions with the UE.

Depending on cell load and impact on UE QoS performance requirements, it can be more beneficial to operate a cell in a cell DTX/DRX discontinuous periodic pattern, whereby the cell can be available only for a portion of slots in a radio frame. During the non-active period, it also desirable to adapt the availability of common and dedicated downlink signals and channels for both IDLE and connected mode UEs to realize most NW and UE energy savings, including SSBs, PDCCH, PDSCH, paging, and SI. Similarly for the uplink, common and dedicated uplink signals and channels (e.g. PRACH, PUSCH occasions, PRACH, SRS, UCI) can also adapt to the cell DTX/DRX pattern, while the network is not blind detecting on those UL resources. The UE’s C-DRX cycle configured in connected mode can also switched to align the network’s cycle.

Proposal 5: 6GR supports common cell DTX/DRX cycle for connected and Idle modes, including a single time-domain pattern for common and dedicated signals and channels (SSB, RACH, PUSCH, PUCCH, PDSCH, PDCCH).

- Protocol processing:

These techniques reflect the modularity of protocol processing, where energy consumption awareness is incorporated into both UP and CP stacks, while also maintaining QoS-awareness. This includes impacts on user plane protocols from time domain adaption of user data and control resources, impacts on control plane protocols from time domain adaptation of cell common signals (SI acquisition, cell selection/reselection, paging), as well as energy consumption associated with scaled integrity protection and QoS.

For example, HARQ processing (timeline and processes) can adapt to energy and power saving schemes in operation. In RRC, the protocol can be designed in a modular manner such that the UE can turn on and off specific processing components that are not necessary to save power.  By doing this, the UE can be (re)-configured with different RRC messages, where parameters not needed are not provided in the signalling.

Proposal 6: Energy-efficient protocol processing is considered in 6G by aligning UP/CP procedures with time-domain adaptations of signals and resources, and scaling of power consumption with bandwidth, data rates, and energy efficiency schemes in use.

# Conclusion

This document has made the following observations and proposals:

Observation 1: The design of network-side energy reduction techniques will largely determine the minimal set of mandatory functionalities that must be supported by all 6G devices and device types.

Proposal 1: Energy efficiency should be first addressed from the radio access network operation perspective. For each NW energy saving feature, RAN2 to study how to align UE power saving features and evaluate their impact on network energy saving gains.

Observation 2: Fragmentation of supported NR UE and NW techniques across different UE capabilities and releases, as UE and network energy efficiency were developed independently, resulted in limited adoption of NES techniques due to backwards compatibility concerns for legacy and non-capable UEs.

Observation 3: NR energy efficiency had limited consideration for impact of energy efficiency on scheduling and QoS.

Observation 4: 6GR energy efficiency framework should target commonality of supported energy efficiency techniques by all UEs in a cell.

Observation 5: 6GR energy efficiency framework should target minimization of always on or periodic common signals and QoS-awareness (incl. impact and consideration of delay budgets and throughput).

Proposal 2: For energy efficiency in control channel monitoring, the following features are supported from day-1: LP-WUS/WUR, C-DRX, PDCCH skipping, and cell DTX/DRX.

Proposal 3: Support adaptation of UE power saving configuration based on NW energy saving operation.

Proposal 4: UE adapts C-DRX cycle to align with the active cell DTX/DRX configuration.

Proposal 5: 6GR supports common cell DTX/DRX cycle for connected and Idle modes, including a single time-domain pattern for common and dedicated signals and channels (SSB, RACH, PUSCH, PUCCH, PDSCH, PDCCH).

Proposal 6: Energy-efficient protocol processing is considered in 6G by aligning UP/CP procedures with time-domain adaptations of signals and resources, and scaling of power consumption with bandwidth, data rates, and energy efficiency schemes in use.

# References

- RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator), Prague, CR, June 9-13, 2025.

- TR 38.864, “Study on network energy savings for NR”, v 18.1.0

# Appendix A: Relevant Agreements

RAN2#131bis

For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose.   Focus on solutions that have RAN2 impacts.

RAN1

Agreement

Study and evaluate NW energy savings and the impact on UE performance and user experience with respect to 20ms and longer periodicities of sync signal(s) at least for initial access with the following consideration, but not limited to:

BS assumptions:

- Cell-common signaling (e.g., sync signal(s), broadcast PDCCH, SIB-1, SIB, paging, PRACH), e.g.,

  - Clustered provisioning of different cell-common signaling,

  - On-demand provisioning of different cell-common signaling,

- UE-specific signaling (for low, light, medium loads), e.g.,

  - Clustered provisioning with cell-common signaling,

  - Unclustered provisioning with cell-common signaling,

UE impact:

- Cell search complexity and latency, including frequency search latency,

- UE Power consumption,

- Sync signal detection, coverage and tracking performance,

- RRM, mobility,

- Beam management,

- Other properties are not precluded,

- Improvements to address identified impact, e.g.,

  - Additional sync signal needs,

  - Adaptation of sync signal transmission periodicity,

  - Sparser synch raster.