3GPP TSG-RAN WG2 Meeting #132	R2-2508656

Dallas, USA, Nov. 17th – 21st 2025

Agenda item:		10.3.2.1

Source:		Lenovo

Title:		RRC states and connection management for 6G

Document for:		Discussion and Decision

- Introduction

3GPP has started working on 6G specification. The study in RAN2 has commenced according to the SID approved in [1]. This document takes a look at the important control plane aspects, starting with an analysis of 5G CP as background and focusing on RRC functionality, procedures and modelling including states, connection management, etc.

- Main Control Plane Aspects

The following main aspects should be considered from the beginning for 6G control plane:

2.1 RRC Specification Modelling

RRC specification is arguably one of the most lengthy and complex specification instead of efforts pulled over the years from some of the greatest rapporteurs. This is also the fundamental specification when it comes to Access Stratum Control Plane implementation. Efforts are underway to modernize this (and others) specification. Nevertheless, RAN2 should engage in some fundamental changes, going forward to ease a reader’s life. Here are some salient points.

- Very lengthy specification compromises readability esp. due to nested features: A different specification modelling can be considered: Base-specification + individual functional specifications with their own self compile-able ASN.1 (even with importing ASN.1 from the base specification)

- Long list of specified features, some with almost no deployment (potential): Realistic (e.g., commitment based) deployment evaluation for approving features. This should be discussed directly in RAN Plenary based on company contributions and we do not think any RAN2 input at his point is necessary.

Proposal 1: To tackle long, nested RRC specifications (mainly xx.331 and xx.304) RAN2 should study if a different specification modelling (Base-specification + individual functional specifications with their own self compile-able ASN.1 (even with importing ASN.1 from the base specification)) can be pursued. This should happen in the beginning and parallel to the “Modernization” discussions.

2.2 RRC States

5G NR RRC state modeling defines UE behavior across Idle, Inactive, and Connected states. Idle state conserves power with cell reselection; Inactive state retains UE context for faster resume with reduced signaling; Connected state supports active data transmission/reception and mobility. The model balances energy efficiency, signaling load, and user experience. All 3 states specified in 5G NR have their advantage. RRC Connected and RRC Idle states are indispensable. The latter is the default state after the UE powers on and the UE must continue in this default state until at least the registration procedure with core network is completed.

RRC Inactive state can combine the best of both other states (data transfer possibility of RRC Connected + Power optimized state like RRC Idle) but the deployment of RRC Inactive state is slow. But NA carriers are apparently starting to deploy RRC inactive state.

Observation 1: RRC inactive state has the potential for harnessing power savings, signalling reduction, better latency for small traffic / IoT / RedCap. Initial deployment has started.

Proposal 2: RRC states from 5G i.e., RRC_IDLE, RRC_INACTIVE, RRC_CONNECTED could be taken as baseline.

Before UE camps on a suitable cell, UE should perform cell selection based on S criteria. After UE camps on a suitable cell, UE will perform cell reselection based on ranking mechanism. In 5G, UE in both RRC_IDLE and RRC_INACTIVE follow the same criteria for cell selection reselection. In 6G, it is better to follow the same principle if both RRC_IDLE and RRC_INACTIVE can be supported.

Proposal 3: Same criteria for cell selection and reselection are applicable for both RRC_IDLE and RRC_INACTIVE.

2.3 RRC Connection Management

In 5G NR, RRC connection management governs the transition of a UE between idle, inactive, and connected states, balancing signaling overhead, latency, and energy efficiency. When a UE requires data or control signaling, it establishes an RRC connection through an RRC Setup/resume procedure with the gNB. In connected mode, the network configures radio bearers, security, and mobility parameters. Release 15 introduced the RRC Inactive state to reduce signaling and improve resume latency, while preserving context at both UE and gNB. Enhancements in later releases optimize paging, mobility handling, and state transitions, enabling efficient resource use while maintaining user experience. From the connection management perspective, seems 5G NR can be used as a baseline and some enhancements can be studied e.g., fast transition to connected state.

Proposal 4: 5G RRC Connection Management (at least Transition to and from RRC Connected) can be taken as baseline.

Last meeting, we discussed whether a part of good configuration can be applied if some configurations do not comply. In 5G specifications, RRC reconfiguration message is used to update an RRC connection, e.g. to reconfigure bearers, RLC channels, handover configuration, measurement, CA, DC and NAS dedicated information.

In general, network is aware of the UE capability. The network can in most cases ensure the reconfiguration message is suitable. However, the UE capability may be changed due to e.g., overheat, battery level, UE in MUSIM mode with reduced capability etc. For example, source BS will transmit UE capability to target BS. Then, target BS prepares the candidate configuration based on the received UE capability. After UE receives the candidate configuration prepared by target cell, UE may report the UE capability change to source cell. In this case, source cell will request target cell to prepare the new candidate configuration based on the updated UE capability. It is possible that after UE reports the UE capability change to the network, UE performs CHO or LTM before UE receives the reconfiguration for releasing/updating the previous candidate configuration based on the old UE capability.

Observation 2: After reporting UE capability change, UE may apply the CHO or LTM configuration due to condition fulfilled before receiving the reconfiguration message.

Proposal 5: UE is allowed to apply a part of compliant RRC configuration and avoid re-establishment procedure.

2.4 Dual registration:

Radio aggregation is important for an operator to enable load balancing and likely also to have a better resource utilization if different frequency bands (FR1 vs FR2/ 3) may be better suited for different services. The potential aggregation mechanisms include dual connectivity, dual steering and dual registration. MUSIM can also be looked at as an aggregation solution. In 5G, MUSIM is supported because multi-USIM devices are popular in different countries. For example, a user may have both a personal and a business subscription in one device or has two personal subscriptions in one device for different services. In 6G, we will have similar requirement from the user in some countries. Dual registration allows for data aggregation in the CN. It is applicable regardless of if 6GC is an evolution of 5GC or if both 5GC and 6GC are distinct networks. Connection to both 5G and 6G can be maintained via dual registration. UE can quickly fallback from 6G to 5G. Otherwise, the mobility time from 6G to 5G will be much longer compared to other solutions e.g., Inter-RAT handover. Some may argue that dual RX/TX are required for dual registration. We think that MUSIM-like mechanism can be reused even for a UE with single RX/TX chain. At this early stage it is better to consider dual registration.

Proposal 6: RAN2 should consider if dual registration can be supported from the early stage of 6G.

- Conclusion

In this contribution, we discussed fundamental control plane specific aspects. Based on the discussion, we have the following observation and proposals:

RRC Specification Modelling

Proposal 1: To tackle long, nested RRC specifications (mainly xx.331 and xx.304) RAN2 should study if a different specification modelling (Base-specification + individual functional specifications with their own self compile-able ASN.1 (even with importing ASN.1 from the base specification)) can be pursued. This should happen in the beginning and parallel to the “Modernization” discussions.

RRC States

Observation 1: RRC inactive state has the potential for harnessing power savings, signalling reduction, better latency for small traffic / IoT / RedCap. Initial deployment has started.

Proposal 2: RRC states from 5G i.e., RRC_IDLE, RRC_INACTIVE, RRC_CONNECTED could be taken as baseline.

Proposal 3: Same criteria for cell selection and reselection are applicable for both RRC_IDLE and RRC_INACTIVE.

RRC Connection Management

Proposal 4: 5G RRC Connection Management (at least Transition to and from RRC Connected) can be taken as baseline.

Observation 2: After reporting UE capability change, UE may apply the CHO or LTM configuration due to condition fulfilled before receiving the reconfiguration message.

Proposal 5: UE is allowed to apply a part of compliant RRC configuration and avoid re-establishment procedure.

Dual registration:

Proposal 6: RAN2 should consider if dual registration can be supported from the early stage of 6G.

- References

RP-252912_6G_WG_SID_cl