3GPP TSG-RAN WG2 #132                                          	    R2-2508704

Dallas, USA, November 17-21, 2025

Agenda item:	   8.6.2

Source: 		Qualcomm Incorporated

Title: 	LTM MAC remaining issues

Document for:	Discussion

# 1	Introduction

This paper discusses remaining open issues in MAC for Rel-19 LTM.

# 2	Open issue on Event LTM2 in event-triggered measurement reporting

During the MAC email discussions at and after RAN2#131b [3][4], the updates to measurement report triggering procedure were made to address various issues including Event LTM2 aspect, but there are still additional modification points to be discussed.

### [MAC running CR snippet from post-131bis email discussion [4]] ### 5.35.3 Measurement report triggering #### 5.35.3.1 General […] - BEAM_ENTERING_LIST: list of RS resource index(es) of applicable RS(s) for each ltm-CSI-ReportConfigId, for which the L1 measurement report entering condition has been met for TTT; - BEAM_LEAVING_LIST: list of RS resource index(es) of applicable RS(s) for each ltm-CSI-ReportConfigId, which have been reported in the (Truncated) L1 measurement report MAC CE, and for which the L1 measurement report leaving condition has been met for TTT; - BEAM_REPORTED_LIST: list of RS resource index(es) of applicable RS(s) for each ltm-CSI-ReportConfigId, which have been reported in the (Truncated) L1 measurement report MAC CE, and the L1 measurement report leaving condition has not been met for TTT. Unless explicitly specified otherwise, it is up to UE implementation how to store these variables. […] 4> include the SSBRI or CRI of the concerned RS in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId; […] 4> if the SSBRI or CRI of the concerned RS has been removed from BEAM_REPORTED_LIST: 5> include the SSBRI or CRI of the concerned RS in the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId. […] 4> remove the concerned RS in the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId; 4> include the SSBRI or CRI of the concerned RS in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId.

According to Clause 5.35.3.1, UE maintains three different beam lists (BEAM_ENTERING_LIST, BEAM_LEAVING_LIST, BEAM_REPORTED_LIST) to maintain the event triggered beam information, and they are used to trigger the event-triggered measurement report and the subsequent periodic reports as well.

From Event LTM2 point of view, the RS resource index of applicable RS (highlighted in the snippet above) refers to the serving beam RS, and as agreed before, the serving beam changes doesn’t reset TTT. This means that the serving beam RS can constantly change based on the serving cell beam management procedure, therefore the RS resource index of the serving beam cannot be a fixed value. However, we think it can be left up to UE implementation on how the RS resource index is maintained, and the description may not need to be updated.

Observation 1a: For Event LTM2, the RS resource index of the serving beam cannot be assumed to be a fixed value as TTT is not reset upon the serving beam change, but this RS resource index management can be left to UE implementation w/o further description update.

In the measurement report triggering procedure, however, the description specifies the RS resource index to be either SSBRI or CRI for the concerned RS (highlighted in the snippet above). This is not generally applicable to all event types, but rather applicable to Event LTM3/4/5 that use the candidate beam RS as applicable RS. For candidate beam RSs, UE refers to the associated LTM CSI resource configuration, which includes either LTM SSB resource set or LTM NZP-CSI-RS resource set, SSBRI or CRI would refer to the index within the corresponding resource list. For the serving beam, there is no reference resource list to be mapped to the indicated TCI state, so this part needs to be modified.

Observation 1b: SSBRI or CRI of the concerned RS is only applicable to Event LTM3/4/5 that use the candidate beam RS as applicable RS from LTM SSB resource set or LTM NZP-CSI-RS resource set in LTM CSI resource configuration, respectively. On the other hand, the serving beam RS in Event LTM2 has no such reference resource set to be used for the indicated TCI state.

To resolve the issue, it seems logical to use the same terminology of ‘RS resource index’ instead of explicit ‘SSBRI or CRI’, and leave it to UE implementation on how to define the resource index for the serving beam.

Proposal 1: RAN2 to agree to replace ‘SSBRI or CRI’ with ‘RS resource index’ so that managing RS resource index for the serving beam can be left up to UE implementation, and to the corresponding TP for clause 5.35.3.1 in Annex A.

# 3	Open issues on SP CSI-RS/CSI-IM handling

There were some SP CSI-RS related open issues discussed in post131 email discussion which are captured in [2], but they were not discussed in RAN2#131b.

## 3.1	[MAC-Q04, MAC-N02] Activated SP CSI-RS/CSI-IM resources for target cell for L1-RSRP measurement upon LTM cell switch

In previous RAN1 & RAN2 meetings, there were some agreements on sending the early CSI report to target cell and handling activated SP CSI-RS resources during LTM cell switch, and the latest MAC running CR has captured the corresponding statement as below.

[MAC running CR snippet] ### 5.18.x Activation/Deactivation of Semi-Persistent CSI-RS/CSI-IM resource set for candidate cell […] After reconfiguration with sync that is triggered by LTM, the configured Semi-Persistent CSI-RS/CSI-IM resource sets for all candidate cell(s), except the target cell, are deactivated. After CSI reporting at the target cell after or during cell switch triggered by LTM as specified in clause 5.2.4a in TS 38.214, the configured Semi-Persistent CSI-RS/CSI-IM resource sets for the target cell are deactivated. […]

Focusing on the target cell case, it can be noted that the target cell’s SP CSI-RS/CSI-IM resource deactivation is conditioned on early CSI reporting to the target cell. Therefore, if UE is only configured with SP CSI-RS for L1-RSRP w/o early CSI reporting for a candidate cell, then UE behaviour of SP CSI-RS deactivation is left undefined, which requires a correction.

Observation 2a: The current text in MAC running CR is working only if UE is configured with early CSI acquisition for target cell. In case UE has only activated SP CSI-RS resources for L1-RSRP w/o any early CSI reporting configured, then SP CSI-RS deactivation timing is undefined.

Considering L1-RSRP measurement is meant for LTM cell switch decision to be made by serving cell, it would be logical to deactivate any activated SP CSI-RS from target cell after reconfiguration with sync, like what is done for other candidate cells.

Observation 2c: L1-RSRP measurement report is only for the serving cell to determine the LTM cell switch decision, so even for the target cell, any activated SP CSI-RS resources for L1-RSRP from target cell should be deactivated after reconfiguration with sync.

Proposal 2: RAN2 to agree the deactivation of SP CSI-RS for L1-RSRP from the target cell after reconfiguration with sync, and to adopt the corresponding TP for clause 5.18.x in Annex B.1.

## 3.2	[MAC-Q05] Presence indication of CSI Resource Configuration ID2 in SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE

In RAN2#131, it was pointed out that a single LTM CSI resource configuration can have either ltm-NZP-CSI-RS-ResourceSet or ltm-CSI-IM-ResourceSet-r19, not both. As a consequence, it was agreed that SP CSI-RS/CSI-IM for candidate cell MAC CE could have up to two LTM CSI resource configuration IDs – one for CSI-RS and the other for CSI-IM.

RAN2#131 chair’s note - IM resource and NZP CSI resource cannot be configured for a CSI-RS resource configuration id. - SP-CSI MAC CE includes up to two CSI-RS resource configuration ids. - Detailed change is left to MAC CR rapporteur. MAC running CR snippet ### 6.1.3.12a SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE […] - A/D: This field indicates whether to activate or deactivate the indicated SP CSI-RS resource set for the candidate cell(s) associated with the CSI Resource Configuration ID1 in the same octet, or SP CSI-RS and CSI-IM resource set for the candidate cell(s) associated with the CSI Resource Configuration ID2 in the same octet, respectively. The field is set to 1 to indicate activation, otherwise it indicates deactivation; - CSI Resource Configuration ID1: This field indicates the index of the LTM CSI resource configuration corresponding to LTM-CSI-ResourceConfigId as specified in TS 38.331 [5]. This LTM CSI resource configuration includes an SP CSI-RS resource set for the candidate cell(s) for measurement. The length of the field is 7 bits; - CSI Resource Configuration ID2: This field indicates the index of the LTM CSI resource configuration corresponding to LTM-CSI-ResourceConfigId as specified in TS 38.331 [5]. This LTM CSI resource configuration includes an SP CSI-IM resource set for the candidate cell(s). If the SP CSI-IM resource set for the candidate cell(s) is not configured in TS 38.331 [5], this field and the reserved bit in the same octet are absent. The length of the field is 7 bits; […] Figure 6.1.3.12a-1: SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE Rapporteur response in email discussions [Rapp]: This issue was indeed discussed in RAN2#130 meeting. Everyone agreed that UE could easily identify whether this CSI Resource Configuration ID is CSI Resource Configuration ID1 or CSI Resource Configuration ID2 based on RRC configuration due to the the following agreement： - IM resource and NZP CSI resource cannot be configured for a CSI-RS resource configuration id.

It is worth noting the issue raised here is not about whether UE is able to identify a particular CSI Resource Configuration ID is for CSI-RS or CSI -IM. But it is rather about the ambiguity & redundancy of the presence of CSI Resource Configuration ID2 as per the current running MAC CE.

Regarding the presence of CSI Resource Configuration ID2, UE is supposed to determine the presence implicitly based on RRC configuration of SP CSI-IM resource set for candidate cell(s). There are some issues that are envisioned with the current description as follows:

- SP CSI-RS for L1-RSRP measurements don’t need the SP CSI-IM at all. But under the current description, UE would have to assume CSI Resource Configuration ID2 is present as long as UE is configured with an SP CSI-IM resource set for candidate cell(s).

- Assume UE is configured with the following LTM CSI resource configurations:

  - LTM CSI Resource Configuration ID #0: SP CSI-RS for candidate cell #A

  - LTM CSI Resource Configuration ID #1: SP CSI-IM for candidate cell #A

  - LTM CSI Resource Configuration ID #2: SP CSI-RS for candidate cell #B, #C, #D

What would be the expected UE behaviour to determine the presence of CSI Resource Configuration ID2 when CSI Resource Configuration ID1 is set to #2 (i.e., SP CSI-RS for candidate cell #B, #C, #D)? UE is configured with SP CSI-IM, but it has nothing to do with the signalled CSI Resource Configuration ID1 value.

Both issues are not properly answered by the current MAC running CR. Moreover, if the intention was to let UE validate the corresponding LTM CSI resource configuration for CSI Resource Configuration ID1 to determine the presence of CSI Resource Configuration ID2, then it will cause unnecessary UE side processing latency for the MAC CE processing.

Observation 3a: The current MAC running CR is interpreted as if UE needs to assume CSI Resource Configuration ID2 to be present as long as UE is configured with SP CSI-IM resource set for candidate cells in RRC.

Observation 3b: Such implicit determination rule would cause either unnecessary presence of CSI Resource Configuration ID2 (e.g., SP CSI-RS for L1-RSRP which doesn’t require CSI-IM at all) or unnecessary extra processing of validating the corresponding LTM CSI resource configuration to CSI Resource Configuration ID1.

Proposal 3:  RAN2 to agree replacing “A/D” to “IM” in Octet 2 for the explicit indication of the presence of CSI Resource Configuration ID2 and adopt the TP for clause 6.1.3.12a in Annex B.2.

# Conclusion

This contribution discussed the remaining issues in MAC for LTM. The following observations and proposals have been made:

Observation 1a: For Event LTM2, the RS resource index of the serving beam cannot be assumed to be a fixed value as TTT is not reset upon the serving beam change, but this RS resource index management can be left to UE implementation w/o further description update.

Observation 1b: SSBRI or CRI of the concerned RS is only applicable to Event LTM3/4/5 that use the candidate beam RS as applicable RS from LTM SSB resource set or LTM NZP-CSI-RS resource set in LTM CSI resource configuration, respectively. On the other hand, the serving beam RS in Event LTM2 has no such reference resource set to be used for the indicated TCI state.

Proposal 1: RAN2 to agree to replace ‘SSBRI or CRI’ with ‘RS resource index’ so that managing RS resource index for the serving beam can be left up to UE implementation, and to the corresponding TP for clause 5.35.3.1 in Annex A.

Observation 2a: The current text in MAC running CR is working only if UE is configured with early CSI acquisition for target cell. In case UE has only activated SP CSI-RS resources for L1-RSRP w/o any early CSI reporting configured, then SP CSI-RS deactivation timing is undefined.

Observation 2c: L1-RSRP measurement report is only for the serving cell to determine the LTM cell switch decision, so even for the target cell, any activated SP CSI-RS resources for L1-RSRP from target cell should be deactivated after reconfiguration with sync.

Proposal 2: RAN2 to agree the deactivation of SP CSI-RS for L1-RSRP from the target cell after reconfiguration with sync, and to adopt the corresponding TP for clause 5.18.x in Annex B.1.

Observation 3a: The current MAC running CR is interpreted as if UE needs to assume CSI Resource Configuration ID2 to be present as long as UE is configured with SP CSI-IM resource set for candidate cells in RRC.

Observation 3b: Such implicit determination rule would cause either unnecessary presence of CSI Resource Configuration ID2 (e.g., SP CSI-RS for L1-RSRP which doesn’t require CSI-IM at all) or unnecessary extra processing of validating the corresponding LTM CSI resource configuration to CSI Resource Configuration ID1.

Proposal 3:  RAN2 to agree replacing “A/D” to “IM” in Octet 2 for the explicit indication of the presence of CSI Resource Configuration ID2 and adopt the TP for clause 6.1.3.12a in Annex B.2.

# References

- Chairman Notes; TSG RAN WG2 Meeting #131b

- R2-2507013, List of MAC open issues for R19 mobility, Rapporteur (vivo)

- R2-2507726, Summary of MAC corrections, Rapporteur (vivo)

- R2-2507728, Miscellaneous corrections on MAC for Mob Ph4, Rapporteur (vivo)

# Annex A	TP for Proposal 1

### 5.35.3	Measurement report triggering

#### 5.35.3.1	General

[…]

The MAC entity shall:

1>	for each ltm-CSI-ReportConfigId included in the SpCell for L1 measurement and event triggered measurement report:

2>	if the corresponding ltm-ReportConfigType is set to eventTriggered and there is ltm-EventTriggeredReportContent configuration:

3>	if the eventLTM2 is configured in the corresponding ltm-CSI-ReportConfigId:

4>	if the associated LTM-CSI-ResourceConfig includes ltm-NZP-CSI-RS-ResourceSet:

5>	consider the current beam of serving cell, i.e. the RS configured in the indicated TCI State of the SpCell, as defined in clause 5.1.5 in TS 38.214 [7], to be applicable.

4>	else:

5>	consider the current beam of serving cell, i.e. the SSB QCLed with the RS configured in the indicated TCI State of the SpCell, as defined in clause 5.1.5 in TS 38.214 [7], to be applicable.

3>	if the eventLTM3, eventLTM4, or eventLTM5 is configured in the corresponding ltm-CSI-ReportConfigId:

4>	if ltm-CandidateReportConfigList is configured:

5>	consider any RS configured in the associated LTM-CSI-ResourceConfig of an LTM candidate cell other than the SpCell listed in ltm-CandidateReportConfigList, to be applicable.

4>	else:

5>	consider any RS configured in the associated LTM-CSI-ResourceConfig of an LTM candidate cell other than the SpCell listed, to be applicable.

2>	if the entry condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for the measurement from lower layer during TTT for one or more applicable RSs, which is not in the BEAM_ENTERING_LIST and not in the BEAM_REPORTED_LIST:

3>	if the MR_LIST does not include a measurement reporting entry for the ltm-CSI-ReportConfigId (a first RS triggers the event):

4>	include a measurement reporting entry in the MR_LIST for this ltm-CSI-ReportConfigId;

3> for each applicable RS for which the entry condition is fulfilled during TTT and that is not in BEAM_LEAVING_LIST:

4>	include the RS resource index of the concerned RS in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId;

3>	for each applicable RS for which the entry condition is fulfilled during TTT and that is in BEAM_LEAVING_LIST:

4>	remove the concerned RS in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId;

4> if the RS resource index of the concerned RS has been removed from BEAM_REPORTED_LIST:

5>	include the RS resource index of the concerned RS in the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId.

3>	consider L1 measurement reporting to be triggered for the ltm-CSI-ReportConfigId, as specified in 5.35.4.

2>	else if the leaving condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for the measurement from lower layer during TTT for one or more applicable RSs included in the BEAM_ENTERING_LIST or BEAM_REPORTED_LIST:

3>	for each applicable RS for which the leaving condition is fulfilled during TTT and that is in BEAM_ENTERING_LIST:

4>	remove the concerned RS in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId.

3>	for each applicable RS for which the leaving condition is fulfilled during TTT and that is in BEAM_REPORTED_LIST:

4>	remove the concerned RS in the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId;

4>	include the RS resource index of the concerned RS in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId.

3>	if reportOnLeave is set to true for this ltm-CSI-ReportConfigId:

4>	consider L1 measurement reporting to be triggered for the ltm-CSI-ReportConfigId, as specified in 5.35.4.

2>	upon expiry of the periodical reporting timer for this ltm-CSI-ReportConfigId:

3>	consider L1 measurement reporting to be triggered for the ltm-CSI-ReportConfigId, as specified in 5.35.4.

NOTE 1:	TTT is not restarted if the current beam of serving cell changes and the entry condition is still met with the new current beam.

NOTE 2:	To evaluate the L1 measurement reporting triggering event, the UE uses the latest L1-RSRP measurement from lower layer.

# Annex B.1		TP for Proposal 2

## 5.18.x	Activation/Deactivation of Semi-Persistent CSI-RS/CSI-IM resource set for candidate cell

[…]

After reconfiguration with sync that is triggered by LTM, the configured Semi-Persistent CSI-RS/CSI-IM resource sets for all candidate cell(s) including the target cell, except the ones for CSI reporting at target cell, are deactivated. After CSI reporting at the target cell after or during cell switch triggered by LTM as specified in clause 5.2.4a in TS 38.214, the configured Semi-Persistent CSI-RS/CSI-IM resource sets for the target cell are deactivated.

[…]

# Annex B.2 	TP for Proposal 3

## 6.1.3.12a	SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE

The SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size and consists of the following fields (Figure 6.1.3.12a-1):

  - A/D: This field indicates whether to activate or deactivate the indicated SP CSI-RS resource set for the candidate cell(s) associated with the CSI Resource Configuration ID1 in the same octet. The field is set to 1 to indicate activation, otherwise it indicates deactivation;

-	CSI Resource Configuration ID1: This field indicates the index of the LTM CSI resource configuration corresponding to LTM-CSI-ResourceConfigId as specified in TS 38.331 [5]. This LTM CSI resource configuration includes an SP CSI-RS resource set for the candidate cell(s) for measurement. The length of the field is 7 bits;

  - IM: This field indicates the presence of the CSI Resource Configuration ID2. If the IM field is set to 1, the CSI Resource Configuration ID2 is present in the same octet. If IM field is set to 0 and A/D field is set to 1, then TCI State ID0 is present in the same octet;

-	CSI Resource Configuration ID2: This field indicates the index of the LTM CSI resource configuration corresponding to LTM-CSI-ResourceConfigId as specified in TS 38.331 [5]. This LTM CSI resource configuration includes an SP CSI-IM resource set for the candidate cell. If the IM field is set to 1, then this field is present. The length of the field is 7 bits;

-	TCI State IDi: This field contains TCI-StateId, as specified in TS 38.331 [5], of a TCI State, which is used as QCL source for the resource within the Semi Persistent NZP CSI-RS resource set corresponding to LTM-CSI-ResourceConfigId as specified in TS 38.331 [5]. TCI State ID0 indicates TCI State for the first resource within the Semi Persistent NZP CSI-RS resource set, TCI State ID1 for the second one and so on. If the A/D field is set to 0, the octets containing TCI State ID field(s) are not present. The length of the field is 7 bits;

-	R: Reserved bit, set to 0.

Figure 6.1.3.12a-1: SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE