3GPP TSG-RAN WG2 Meeting #132					       R2-2508876

Dallas, USA, November 17th – 21st , 2025

Agenda item:	10.2.1

Source:	Samsung

Title:	UE capability framework for 6G

Document for:	Discussion and Decision

## Introduction

In the last meeting, the following was agreed as outcome of first 6G discussion on UE capability.

Agreements on UE capabilities for next meeting

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

In this document, we would like to discuss further following up with the above agreements.

## 5G UE capability framework

Basically, most critical issue in UE capability is the excessive size of UE capability signaling. It is observed that RRC processing time is increased and out of memory issue can happen due to large capability size. The following items are considered a root cause for the excessive size of UE capability signaling and we should study to resolve.

#1: Too many bands and band combinations

The number of component carriers in NR band combinations has increased. While NR CA supports up to 16 CCs, RAN4 specifies up to 7 CC CA cases with 6 bands and 40000 band combinations.  Although network requested filter can reduce the reported bands/BCs efficiently, it still requires to report many bands/BCs.

However, we understand that RAN4 is studying band group concept for band combination simplification in 6G study item. In RAN1, a single cell multiple carrier feature has been proposed in 6G in order to utilize fragmented spectrum in 6G. Depending on the outcome of these discussion, this may require change of UE capability signaling structure i.e. it can simplify the band combination structure or make it even worse. In addition, these new concept/feature may affect existing signaling reduction features including fallback BC and reduce intra-band non-contiguous CA. It would be important for RAN2 to consider understand these concept/feature if supported in 6G before RAN2 discuss specific enhancements on bands and BCs related structure.

Observation 1: band and BCs are key factors in the increased UE capability signaling size, however the definition of band and BCs may be changed by new concept/feature in 6G (i.e., band group and single cell with multiple carrier) depending on discussion in RAN1/RAN4.

Proposal 1: RAN2 check with RAN1/RAN4 on general timeline for CA/band/BC structures in order for RAN2 to plan for the enhancement of UE capability signaling structure.

#2: Too many Band/BC dependent capabilities

Numerous lower-level UE capabilities, such as supported bandwidth per band per CC and MIMO parameters per band per CC, are considered. It is because those UE capabilities are affected with both RF and baseband processing capabilities. Since this is one of key factor to increase signaling overhead, in 5G,  FS, FSPC and FSC were introduced to solve the issue.

However, there are some additional areas to reduce redundancy. For example, RAN1 introduced some baseband capabilities that are applicable across bands in the band combination although it is also defined per band (i.e. per band and per band capabilities). Given that this type is frequently used in RAN1 features, it would be necessary to study the proper structure for this.

In addition, RAN4 introduced BCS5 in Rel-17 in order to reduce the burden to signal each BCSs. With BCS5, UE doesn’t need to indicate all possible bandwidth combinations which leads to reduced signaling overhead.

Observation 2: FS, FSPC and FSC are used to efficiently indicate band/BC dependent capabilities but couldn’t cover all baseband dependent capabilities in 5G.

Proposal 2: RAN2 discuss a general structure for band + BC dependent capabilities (e.g. FS, FSPC, FSC, etc.) for 6G and share with RAN1/RAN4 to get their feedback during study item.

#3: Too many optional capabilities

Additionally, the complexity of signalling increases due to the optional nature of most features and feature sets. In every release, a few hundreds of feature groups are defined (RAN1 Rel-17: > 270 and Rel-18 : >250 feature groups). Especially, it was observed that optional capabilities especially in BC in each release/version increases UE capability signaling overhead.

Basically, it could be solved if we support only essential features for 6G. However, given that 6G will incorporate many advanced features inherently from 5G and also 6G will support diverse device types, it will be still challenge to avoid optional capabilities.

Therefore, it would be also required to study how to minimize optional capabilities. In 5G, it is observed that some UE capabilities are introduced to differentiate in each component level rather than to differentiate from overall feature level. In addition, for a feature affecting all WGs, if we consider dependency between UE capabilities among WGs, we can minimize redundant optional capabilities.

Observation 3: optional capabilities especially in BC in each release/version increases UE capability signaling overhead. There might be room to reduce optional capabilities if we can consider differentiation per feature level or dependency between feature groups from WGs.

Proposal 3: RAN2 discuss a general guidance in defining multiple UE capabilities for a feature in order to avoid too many optional UE capabilities.

While the excessive signaling size can be solved by improving capability structure, RAN2 can also study signaling based capability signaling reduction.  Since Release 16 in 5G NR, enhanced mechanisms have been introduced to reduce UE capability signalling and manage excessive UE capability information, including:

- RACS (Radio UE Capability Signaling Optimization): To reduce signalling overhead, an ID (e.g., PLMN-assigned ID) representing UE radio capabilities can be provided in NAS signalling.

- UL RRC segmentation: A UE Capability Information message can be segmented if the size of the encoded RRC message PDU exceeds the maximum PDCP SDU size.

In the last meeting, there is quite good support of reusing RACS given that this can solve signaling overhead issue significantly.  So far, the main reason of this feature not commercialized is because it was introduced in the later release and there is no motivation that NW implements this feature.

Proposal 4: RAN2 study the support of RACS in 6G as a basic tool for UE capability framework.

## Dynamic UE capability change

The following use cases that motivate the support of UE capability change.

- IDC/MUSIM

- Overheating

- Measurement gap

One might argue that the above use cases are the same as 5G NR and UAI approach can be re-used in 6G. However, there are the following drawbacks on the UAI-based approach:

- Less flexibility/Less commonality: UE can request the restriction with specific parameters only and use an option for a specific purpose only (e.g., assistance info of 20 types or more, according to purposes). It makes UAI get heavier. The current MUSIM indication includes BW and MIMO layer as restricted capabilities in addition CC/bands information. However, other capabilities e.g. SRS can be also limited in dual SIM dual active case and without capability change, UE cannot main the connection.

- Not suitable for long-term case: After Cell Reselection, UE should request UAI again if it has still experienced any problem, as in Fig. 6.

- Absolutely under of network control: If network has not reconfigured in the response of the UAI, UE cannot resolve the concerned problem in standardized fashion, and it should finally initiate its implemented option.

Therefore, it is still necessary to study dynamic capability changes to support existing use cases as a baseline. In addition, RAN2 could also look a new use case that requires dynamic UE capability change.

Observation 4: UAI-based dynamic temporary UE capability restriction is not sufficient to handle IDC/MUSIM, overheating, measurement gap use-cases because 1) less flexibility/less commonality, 2) no handling of long-term case and lack of ensuring network response.

Proposal 5: RAN2 study dynamic UE capability change for existing use cases (IDC/MUSIM, overheating and measurement gap use cases) as a baseline.

Figure 1. A drawback with UAI-based Temporary UE Capability Restriction

## RACS-based UE capability change

For UE RAdio Capability Signalling optimization (RACS), an identifier, UE Radio Capability ID was introduced in NR. By assigning the UE Radio Capability ID, to represent the UE radio capabilities, the signalling overhead for UE radio capability information can be dramatically reduced while reporting the ID instead of the full UE radio capabilities.

The ID-based RACS approach can be extended for dynamic temporary UE capability restriction purpose. UE can have multiple of the preferred UE radio capability set corresponding to each UE situation such as power saving, overheating, IDC, MUSIM and so on. If UE can report an ID linked to each capability set whenever it has needed, its capabilities can be dynamically and temporarily changed with less signalling overhead. Because the core network is aware of the the UE capability sets and corresponding IDs with the RACS extension, high signalling efficiency can be achieved in the mobility environment.

As we see UAI parameters on overheating mitigation purpose, there are reduced max. CCs, reduced max, BW and reduced max MIMO-layers. For signalling optimization, we may consider the full UE capabilities and delta UE capabilities. For instance, we can report a full UE capability set, and a delta UE capability set friendly for overheating mitigation (e.g., the parameters above). The delta signalling is mapped to a ID. If the ID is reported as in Fig. 7, the network considers the corresponding parameter set, instead of the values of the full set.

Figure 2. Dynamic Temporary UE Capability Restriction with RACS Enhancements

Proposal 6: RAN2 to study RACS extension for dynamic temporary UE capability restriction purpose, in cooperation with SA2.

## Chipset ID

In the last meeting, chipset ID was proposed to solve IODT issue i.e. in order to identify problematic UEs. In our understanding, chipset ID is a proprietary solution and it would not be necessary if RACS is supported.  In RACS, either manufacturer or PLMN can allocate RACS ID and RACS ID can be provided to sNB to distinguish certain UEs. If there is any error in the field, network could request to fix the issue and to update RACS ID. The main difference between RACS ID and chipset ID is how ID is associated. In RACS, ID is associated to UE capability sets as specified. In case of chipset ID, it is not clear how chipset ID is defined and updated. Without clear specification, it would cause confusion in implementation e.g. when chipset ID is updated.

Observation 5: RACS ID can be used to identify problematic UEs in the field.

Proposal 7: RAN2 should focus on RACS in 6G instead of chipset ID solution.

## Conclusion

In this contribution, we discussed further details on UE capability according to the previous RAN2 agreement. Based on the discussion, we propose the following points.

Observation 1: band and BCs are key factors in the increased UE capability signaling size, however the definition of band and BCs may be changed by new concept/feature in 6G (i.e., band group and single cell with multiple carrier) depending on discussion in RAN1/RAN4.

Proposal 1: RAN2 check with RAN1/RAN4 on general timeline for CA/band/BC structures in order for RAN2 to plan for the enhancement of UE capability signaling structure.

Observation 2: FS, FSPC and FSC are used to efficiently indicate band/BC dependent capabilities but couldn’t cover all baseband dependent capabilities in 5G.

Proposal 2: RAN2 discuss a general structure for band + BC dependent capabilities (e.g. FS, FSPC, FSC, etc.) for 6G and share with RAN1/RAN4 to get their feedback during study item.

Proposal 3: RAN2 discuss a general guidance in defining multiple UE capabilities for a feature in order to avoid too many optional UE capabilities.

Observation 3: optional capabilities especially in BC in each release/version increases UE capability signaling overhead. There might be room to reduce optional capabilities if we can consider differentiation per feature level or dependency between feature groups from WGs.

Proposal 4: RAN2 study the support of RACS in 6G as a basic tool for UE capability framework.

Observation 4: UAI-based dynamic temporary UE capability restriction is not sufficient to handle IDC/MUSIM, overheating, measurement gap use-cases because 1) less flexibility/less commonality, 2) no handling of long-term case and lack of ensuring network response.

Proposal 5: RAN2 study dynamic UE capability change for existing use cases (IDC/MUSIM, overheating and measurement gap use cases) as a baseline.

Proposal 6: RAN2 to study RACS extension for dynamic temporary UE capability restriction purpose, in cooperation with SA2.

Observation 5: RACS ID can be used to identify problematic UEs in the field.

Proposal 7: RAN2 should focus on RACS in 6G instead of chipset ID solution.