3GPP TSG RAN WG2 Meeting #132	             						R2-2508880

Dallas, USA, Nov. 17th –21st, 2025

Agenda Item:	10.3.3.1

Source:	AT&T

Title:	Considerations for 6G Data Framework

Document for:	Discussion/Decision

# Introduction

The study on 6G Radio was agreed in RAN#108 in RP-252912 [1]. The following objectives are of relevance to the AI/ML framework and corresponding data collection in 6G [1]:

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

(6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

Furthermore, RAN2#131bis agreed to study standardized data collection framework in 6G considering a given set of requirements:

Agreement

1.  	Study the standardized data collection framework with these requirements as a baseline at least for AI/ML

- The data collected is secured and data integrity and confidentiality for that data is ensured.

- User data privacy, anonymity and user consent is respected.

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose

- This includes initiating, terminating, and fully managing data transfer.

- MNO has full visibility for standardized data.

- The design is future-proof and extendable.

- The UE data collection should minimize impact to the UE battery, UE processing and memory utilization.

- UE data collection should minimize impact to user traffic transmission and power saving features

These requirements can apply to both UE and NW sided data collection.  FFS if some don’t apply to both.

2.	Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases.

3.	Study termination points (i.e. understand who are producers, consumers of the data, data collection points.  Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP).

4.	Study model transfer/delivery requirements/functions

In this contribution, we discuss a new data framework that satisfies the agreed requirements at the NW and the UE, and the main design principles for enabling such a framework in 6G

# Discussion

As agreed in RAN2#131bis, the standardized data collection framework needs to meet certain requirements, chief among them is MNO visibility and control, extensibility, scalability, and interoperability.

For 5G-A, UE-sided and NW-sided data collection were considered separately, and different solutions were studied and/or specified for each on a per-use case basis. These data collection solutions were retrofitted on the existing 5G architecture, which resulted in data collection solutions that are designed per use case, and some aspects were limited by what the 5G control plane can offer when it carries, e.g. data collection for monitoring.

In the 5G architecture, the control plane (CP) is responsible for signaling and control functions to establish, maintain, and terminate communication sessions between the user and the network, while the user plane (UP) is responsible for all the user data traffic. As the CP contains only the control related information to maintain and manage the communication link between the user and the network, CP by design has a limited amount of bandwidth, low latency requirements and any data transfer on CP is not charged to the users. The UP, however, contains all the data traffic requested by the user and user applications, therefore it has high bandwidth, variable latency/QoS requirements (based on user application) and the users are charged for the data usage on the UP.

UE-sided AI/ML models are typically trained by collecting a large real-world dataset and are then transferred and deployed to the users. The size of training dataset can be in range of 100k to 100M data sample, depending on various aspects such as use case, type of model, generalization aspects etc. Similarly, the model size can vary from 10k to 100M parameter model depending on the use case, model type and other aspects. Due to the large amount of data involved, it is important that the network has full visibility and control when the data is shared, to avoid any network impact. Additionally, the users should not be charged for the AI/ML model training data or model transfer.

Having a CP based solution does not work due to the large amount of data being involved and an UP based solution will not give any visibility to the network and can impact its performance.

There are 3 potential approaches that can be used to design data collection and data management for the native AI/ML in 6G

- Continue with the 5G framework and have a hybrid of CP and UP

- CP overhaul and redesign

- Introduce a new data plane, in parallel with the current CP and UP

The first option is not well suited for a native AI/ML approach and inherits the limitations of the 5G-A AI/ML framework retrofitted on the 5G architecture. A hybrid approach using CP and UP requires significant specification impact and coordination across multiple work groups across RAN and CN. It also increases the signaling load on the CP due to reporting of when and what data is being shared. The increase in signaling load also makes such an approach not scalable with the number of use cases. In addition, for such an approach it is very difficult to optimize data collection for multiple use cases in case some of the input data is common among the use cases.

The second option requires a significant specification impact and results in the 6G framework to be entirely non compatible with the legacy 5G framework. Such a design can allow for several of the core issues with the AI/ML framework to be properly addressed. However, there will still be an issue of increased load on the CP with the number of use cases. In addition, unless we allow the new CP to have higher or adaptable bandwidth there will still be an issue with configuring resource utilization over the new CP, and with unnecessarily using the CP for latency tolerant data transfer/delivery.

The third option is to have an entirely new plane which we name the Data Plane (DP), for all AI/ML related data collection aspects as well as model transfer/delivery. We can also utilize DP for transfer of AI/ML model monitoring metrics between the UE side and the network side. Furthermore, the DP can be designed to perform lifecycle management (LCM) of AI/ML models and functions, including activation, deactivation, switching, and fallback. This will allow all AI/ML related aspects to be managed from within the DP, which enables us to design new use cases quickly and efficiently for native AI/ML. The new plane allows visibility and control for the MNOs over the data flowing through the network.

Proposal 1: Study the introduction of a new data plane to enable the standardized data collection framework, and model transfer/delivery, using AI/ML data as a baseline

As new technologies and services emerge, the demand for network data collection continues to grow. A unified, data collection framework for AI/ML and new services such as sensing, should be  studied. The framework is then extensible  to support a broad range of services and use cases. A core design principle is to preserve a clean, consistent end-to-end path for collected data, spanning collection, transport, and delivery – irrespective of the service involved.

Proposal 2: For 6GR, a unified framework for standardized data collection is supported, including AI/ML, sensing, SON/MDT, QoE, etc.

In 5G-A, configuration for data collection was considered separately for NW-sided data collection and UE-sided data collection, whereas in most of these use cases, the same configuration could have been used for both. Similarly, CSI configuration report for example is needed to configure CSI report for non AI/ML based CSI prediction and AI/ML based CSI prediction, and a common configuration can be used for both.

For 6GR, we should thrive to use common configurations, when possible, for use cases with AI/ML and non AI/ML configurations, as well as common configurations for UE-sided and NW sided data collection.

Proposal 3: For 6GR, a common framework for configuration of data collection for AI/ML and non-AI/ML, as well as for UE-sided and NW-sided model should be considered.

The data plane, by design, combines benefits of both CP and UP solutions. Traffic that flows through the DP is intended to be both originated and terminated either at the UE device or at different entities in the network, similar to CP traffic. Additionally, traffic at DP will not be charged to the user. At the same time, the traffic characteristics of the DP with regards to model transfer/delivery and data collection requirements are like those of the UP traffic.

In addition, the DP is intended to be used on demand. The MNO should consider both device state and network conditions before activating the DP and sending any data through it. For example, a UE should not be paged with a data-collection request when it is in IDLE and conserving energy. Similarly, the MNO, having full visibility of network conditions, can prevent DP activation for UEs in unfavorable radio or network conditions, thereby improving energy efficiency, reducing unnecessary signaling, and protecting user experience.

Proposal 4: A standardized data collection framework through the data plane minimizes impact on the UE battery, UE processing, and memory utilization; it minimizes impact on user traffic transmission and power saving features.

The data plane offers better data management as the network can collect data for multiple use cases simultaneously, which results in improved efficiency (as there may be some common inputs for different AI/ML models operating at different layers or even at different domains/network functions). Furthermore, the network can add relevant additional information and labels to the data. The network can also ensure that the privacy is preserved, and proprietary and security aspects of the data are properly addressed before the data is shared with the network or UE vendors to develop different models. Additionally, it enables the network to control the timing of data collection and model transfer while minimizing the impact on system performance. Finally, the DP architecture provides us with a framework that can be used to quickly develop and deploy AI/ML use cases based on the network requirements, reducing the standardization effort to onboard new use cases based on the established framework.

For both UE-side and network-side data collection, the data plane ensures that AI/ML-related data is logically separated from other traffic on both the UP and the CP. This separation enables clean, scalable evolution of 6G networks with minimal impact on existing services and allows prioritization of AI/ML traffic when needed. In general, AI/ML related data (or new services data such as sensing) are considered lower priority than the user data which flows through the UP and the signaling data which flows through the CP. However, different types of data collected may be prioritized differently. For example, while training data can be transferred using a low priority flow with a very relaxed latency requirement, data related to inference and monitoring are likely to have higher priority and relatively tighter latency bound.

Proposal 5: Standardized data collection through the data plane supports traffic prioritization, such that data collection does not adversely impact existing UP or CP flows.

Proposal 6: For standardized data collection, QoS parameter requirements (e.g., latency, throughput, reliability) for different data types (e.g., training, monitoring, model transfer/delivery) are considered.

The data plane supports multiple termination points inside the MNO, with MNO visibility over the standardized data collection. This reduces the duplication in data collection, saves energy and radio resources and can yield better quality prediction and network automation as data can be shared when and where it is needed inside the network.

Proposal 7: For standardized data collection, a data plane supports multiple termination points inside the MNO, in the RAN, CN and OAM

# Conclusion

In this contribution, we proposed a framework for data collection and model transfer in 6G. We made the following proposals.

Proposal 1: Study the introduction of a new data plane to enable the standardized data collection framework, and model transfer/delivery, using AI/ML data as a baseline

Proposal 2: For 6GR, a unified framework for standardized data collection is supported, including AI/ML, sensing, SON/MDT, QoE, etc.

Proposal 3: For 6GR, a common framework for configuration of data collection for AI/ML and non AI/ML, as well as for UE-sided and NW-sided model should be considered.

Proposal 4: A standardized data collection framework through the data plane minimizes impact on the UE battery, UE processing, and memory utilization; it minimizes impact on user traffic transmission and power saving features.

Proposal 5: Standardized data collection through the data plane supports traffic prioritization, such that data collection does not adversely impact existing UP or CP flows.

Proposal 6: For standardized data collection, QoS parameter requirements (e.g., latency, throughput, reliability) for different data types (e.g., training, monitoring, model transfer/delivery) are considered.

Proposal 7: For standardized data collection, a data plane supports multiple termination points inside the MNO, in the RAN, CN and OAM.

# References

[1] RP-252912, New SID: Study on 6G Radio, NTT Docomo, Prague Czech Republic

[2] RAN2 131bis Chair notes, Prague, Czech Republic