3GPP TSG-RAN WG2#132	R2-2508945

Dallas, USA, Nov 17th – 21st 2025

Source: 	Fujitsu

Agenda item:	10.3.2.1

Title:	Discussion on RRC modelling and connection management in 6G

Document for:	Discussion and decision

# Introduction

The SID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

At RAN2#131Bis meeting in Prague, the following agreements were achieved w.r.t RRC states:

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

In this contribution, we would like to provide our views on RRC functions, RRC modelling and connection management.

# Discussion

## 6G RRC Functions

The control plane functionalities and design are directly impacted by the RAN architecture choice, the key performance indicators to be met by 6GR (e.g. KPIs for latency, mobility, energy efficiency etc), the separation of CP and UP, the support for virtualization, the support for cloudification of the RAN, the support for RAN sharing, network slicing, support for existing and new services etc. As per TS 38.300 the following functions are provided by the RRC layer in 5G NR.

Table 1 - Summary of Functions provided by NR RRC

| System Information Provisioning, Emergency Warning messages and Positioning Information | Paging notifications, Bearer configuration (SRB/DRB) and handling, QoS management |
| --- | --- |
| Connection control: establishment/resume, re-establishment, release, configuration/reconfiguration | Lower protocol layer configuration and reconfiguration, radio resource configuration, SCell addition, modification and release etc |
| Security key management, access stratum security configuration | Measurement configuration, Mobility and UE context management |
| PLMN selection, cell selection and reselection, cell access control and barring | UE capability handling |
| RLF handling, SON, ANR, MDT, etc | Services: VoNR, Sidelink, MBMS, etc, |
| Network sharing, network slicing etc | Other functions like allocation of UE Identifiers, handling UE assistance information, transport of NAS messages and generic error handling |

The above listed 5G NR RRC functions have played crucial role in commercialization of 5G NR and some of them extended in 5G Advanced to support new capabilities. Some of the above-mentioned RRC functions may need to be supported in the baseline 6GR and some functions may get specified in future releases. As the technical discussion during study item progresses some of the functions may get reused from 5G NR RRC, some of them get simplified by avoiding duplicate methods or avoiding unnecessary complexity, some of them get re-designed from scratch as per 6G requirements which are changed compared to 5G requirements, some of them get distributed between RRC and MAC depending on the architecture choice and enhancements needed to improve the performance compared to 5G NR.

We propose the following:

Proposal#1:

- NR RRC functions are considered as baseline for 6GR RRC.

- List of baseline 6GR RRC functions to be considered during study item is FFS.

### RRC Procedures

In 5G NR, following baseline RRC procedures are defined:

- RRC Setup  Establishing the RRC Connection

- RRC Reestablishment  Reestablishing the RRC Connection after encountering failure condition

- RRC Resume  Resuming the RRC Connection

- RRC ReConfiguration  Reconfiguring the UE due to L1/L2/L3 configuration update

- SecurityModeCommand  Establishing the Access Stratum Security

- UECapabilityEnquiry  Requesting the UE to report its radio capabilities

- Counter Check à Security related procedure to verify if there is a “Man in the Middle Attack”

There are other procedures like DL/UL Information Transfer and UE assistance information request. Most of the baseline NR RRC procedures shall be adopted for 6G, except the RRCReestablishment and RRCResume can be further studied for unification. Such unification attempt was studied in 5G study items and in our view there was no technical blocking point to unify these two procedures which basically are doing the same thing but trigger condition at UE is different. From procedure point of view functionally these two procedures are same. Obviously, the need of Resume procedure depends on the definition of RRC_INACTIVE state. If such a state is defined, then it is worthwhile studying the unification aspects of the two procedures.

Observation#1: From procedure point of view RRC Reestablishment and RRC Resume are achieving the same functionality, whereas the trigger at UE is different.

Observation#2: Resume procedure has a dependency on the definition of RRC_INACTIVE state.

We therefore propose:

Proposal#2:

- NR RRC procedures are considered as baseline for 6GR except the Reestablishment and Resume procedure.

- If RRC_INACTIVE state is defined, then it is worthwhile studying the unification of Reestablishment and Resume state.

## RRC Modelling

According to [1], a “Single technology framework based on a stand-alone architecture” is envisaged for next generation 6G systems targeting the IMT-2030 requirements. This means the initial deployment of 6G system will be primarily standalone system unlike the initial 5G NR system deployment which were based on the Non-standalone (NSA) architecture relying on tight interworking between LTE RAT as anchor/master and NR as secondary RAT. The 5G NR control plane architecture is flexible and modular supporting inter node RRC messages, encapsulation of RRC messages generated by one RAT to be transported in RRC container messages of another RAT in case of tight inter-RAT interworking and embedding NAS container in RRC messages. To support new services and additional deployment architecture (apart from the standalone) in future the 6GR control plane architecture needs to be flexible, modular and forward compatible right from the beginning so that backward incompatibility can be avoided in future 6GR releases.

Further, existing LTE and 5G NR deployment (both standalone and non-standalone) in many countries leverage on the distributed split RAN architecture with the standardized interfaces between CU-CP and DU (i.e. F1 interface) and CU-CP and CU-UP (i.e. E1 interface). The 6G RAN architecture as per [1], RAN2 and RAN3 will study “RAN internal functional split, interfaces, protocol stacks and procedures”. In our companion contribution we have proposed modular RRC design [2] based on “Split RRC Design” such that the CU-CP and DU shall be able to send RRC Reconfiguration messages towards the UE corresponding to the L3 configuration update and L1/L2 configuration update handled by the CU-CP and DU respectively. Such Split RRC design with Anchor RRC hosted in CU-CP and Auxiliary RRC hosted in DU offers several technical benefits as outlined in [2]

Observation#3: Split RRC design allows CU-CP and DU to manage reconfigurations towards UE independently, therefore reducing control plane latency, minimizing coordination between CU-CP and DU, reducing the processing load at CU-CP and relaxing the requirements on the F1 interface.

### RRC Signalling

At the RAN2#131bis meeting, it was agreed to use the ASN.1 encoding for the RRC signalling and study the improvements for the ASN.1 encoding so that readability is improved [3]. In the company contributions submitted to RAN2#131bis, several companies proposed to simplify the ASN.1 encoding, avoid duplication of information elements, and make delta signalling more useful [4-8]. We concur with the views on the issues related to ASN.1 signalling enlisted by [4-8]. Delta signalling is a useful technique available from the LTE/NR RRC signalling toolkit which should be adopted in the 6G RRC signalling. It helps in reducing the signalling overhead. FullConfig should be used in scenarios where delta configuration cannot be used e.g. handover between gNB involving two different vendors. Therefore, we propose:

Proposal#3: Delta configuration technique shall be adopted in the 6G RRC signalling toolkit and studied further for improvements.

One way of simplifying the RRC signalling is discussing the RRC structure framework. The 5G NR RRC framework is a nested framework involving parent structure comprising the top-level information elements and from which actual configuration in the child structure is referred. In our opinion we need extensive discussion in RAN2 on how the parent structure schema and what information elements should be introduced in the parent structure. Further, if the parent structure needs to be further extended in future, then what type of information elements can be included in the extended parent structure. Similarly, child structure schema and what type of information elements shall be included in the child structure and the extended child structure. Typically, in RRC specifications rules related to RRC information elements schema, extension rules and NEED codes are defined. However, sometimes due to fast pace of finalizing the release or sometimes due to overlooking the ASN.1 review, or large number of features getting introduced in subsequent releases these rules are sometimes not strictly followed. This happens rarely but we need to be cautious.

Figure 1 Example of Nested RRC Structure

A simple approach to define the RRC signalling framework would be to group the essential information elements related to mandatory features in parent structure or extended patent structure. The essential information elements or fields would be mandatory such that the value of such IEs or fields do not change frequently and some IEs or fields of the mandatory features which are further optimized by top-up UE capability these are optional and may change with reconfiguration. This means fullConfig for IEs and fields in the parent structure which are essential that do not change frequently whereas delta configuration for optional IEs and fields which are not signalled initially and configured later or updated later. For optional features which are solely based on UE capability, all the IEs and fields are optional and hence they can be grouped together in child structure or extended child structure. Reconfiguration of such IEs and fields are essentially managed by delta signalling. One example of such simplified parent and child structure is shown in Figure 1.

Proposal#4: RAN2 may consider the Nested RRC structure comprising the Parent and Child Structure as depicted in Figure 1.

### RRC state design

5G NR standards support the RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. At the RAN2#131bis meeting, it was agreed to study all the three states [3]. The RRC_INACTIVE state is not only a power efficient state from the UE perspective but also reduces latency allowing quick data transfer. However, in majority of the network deployments around the globe the RRC_INACTIVE state is not supported widely. From the 5G NR experience it seems the RRC_INACTIVE support from network perspective is more beneficial for specific services catering to those use cases the operator intends to provision (e.g. RedCap, IoT etc.) or features like Small Data Transmission (SDT) needs to be supported. Further, from network operator perspective the implementation of the RRC_INACTIVE on top of the RRC_IDLE introduces complexity to support the RAN paging in addition to the support of CN paging, configuration of the RAN-based Notification area, RRC suspend/resume procedures for connection control, the UE context management at RAN and small data transfer without state transition.

Therefore, 6GR should support the RRC_IDLE and RRC_CONNECTED as baseline. RAN2 needs to further study whether to support RRC_INACTIVE state and the modelling method for RRC states.

If RRC_INACTIVE state/sub-state is supported, RRC states in 6GR have the following possibilities:

- Modelling 1: as NR, three RRC states including RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED

- Modelling 2: two RRC states including RRC_IDLE (with RRC_INACTIVE-like sub-state) and RRC_CONNECTED

- Modelling 3: two RRC states including RRC_IDLE and RRC_CONNECTED (with RRC_INACTIVE-like sub-state)

From our perspective, Modelling 1 and Modelling 2 are preferred because some functions like paging and I-DRX which are common in RRC_IDLE and RRC_INACTIVE can be unified. This simplifies the functionality and also the specification effort. Modelling 3 is not preferred as it makes functionality complex and also increases the specification effort. If Modelling 1/2 is adopted, RAN2 should further study the potential enhancements or simplification to alleviate some of the pain points in NR.

This is independent of the Core network states that SA2 may study for the 6G system. RAN2 and SA2 may need coordination in the beginning of study item in the form of joint discussion and exchange of the usual liaison statements on the RRC and CN states and their relation, possible avoidance of duplicate functions like RAN paging and CN paging, UE state transparent to the CN etc. From our perspective we prefer that the UE state is transparent to the CN, wherein the paging responsibility is solely handled by the RAN. Whenever, data is available for the UE, the CN simply sends the DL data to the last serving node of the UE. However, such a decision cannot be taken at RAN2, but RAN2 can indicate such preference to SA2 for simplifying the paging.

Considering the 5G NR experience, we propose the following:

Proposal#5

- 6GR shall support the RRC_IDLE and RRC_CONNECTED states as baseline.

- During the study consult with SA2 on the CN defined UE states and its relationship with RRC states.

- Indicate RAN2 preference to SA2 to keep UE state transparent to CN, thus avoiding CN paging.

- Support of RRC_INACTIVE state and modelling methods 1/2 shall be further studied during the study item to alleviate some of the pain points inherent in 5G NR.

If RAN2 subsequently agree to define the RRC_INACTIVE state, then the important question remains how to ensure the feature is commercialized in the network to gain the benefits of the feature. In 5G NR the feature is mandatory at the UE side from Day1, however the network side implementation of the feature is not ubiquitous. The network side implementation of the feature depends on how the RRC_IDLE and RRC_INACTIVE functionality is defined. If the RRC_IDLE is defined as a state during UE power ON or UE recovery (e.g. coverage hole), then obviously the network will not release the UE to RRC_IDLE and it has to mandatorily release the UE to RRC_INACTIVE. However, if the functionality is duplicated in RRC_IDLE and RRC_INACTIVE then it is network implementation decision whether to release the UE to RRC_IDLE or RRC_INACTIVE. If such a choice is available at the network side, then there is high probability the RRC_INACTIVE may not get commercialized even in 6G.

Observation#4: If some of the functionality of RRC_IDLE and RRC_INACTIVE is duplicated then there is high probability the network will always release the UE to RRC_IDLE.

Proposal#6: If RAN2 decides to introduce the RRC_INACTIVE state in 6GR, then RAN2 should strive to ensure the feature is commercialized in real deployments from Day1.

### Connection management

As mentioned in above section regardless of the RRC states to be considered and studied in 6GR, from the UE perspective the UE at a given point of time is expected to be in a single RRC state.

Regardless of the RAN architecture, evolution of the RAN architecture, support of multiple radio spectrum sharing (MRSS), support of dual registration from the UE perspective etc., the UE supports one RRC instance i.e. RRC connection management of the UE is under the control of single RAT during the lifetime of the RRC connection.

We propose the following:

Proposal#7

- From UE perspective for connection management, the UE is expected to be in a single RRC state at a given point of time.

- From the UE perspective the UE supports one RRC instance i.e. RRC connection management of the UE is under the control of single RAT during the lifetime of the RRC connection.

# Conclusion

In this paper we presented some views on RRC functions, RRC procedures. RRC modelling, RRC state design and connection management and have come up with the following observations and proposals to be discussed and agreed:

Proposal#1: On RRC Functions

- NR RRC functions are considered as baseline for 6GR RRC.

- List of baseline 6GR RRC functions to be considered during study item is FFS.

Observation#1: From procedure point of view RRC Reestablishment and RRC Resume are achieving the same functionality, whereas the trigger at UE is different.

Observation#2: Resume procedure has a dependency on the definition of RRC_INACTIVE state.

Proposal#2: On RRC Procedures

- NR RRC procedures are considered as baseline for 6GR except the Reestablishment and Resume procedure.

- If RRC_INACTIVE state is defined, then it is worthwhile study the unification of Reestablishment and Resume state.

Observation#3: Split RRC design allows CU-CP and DU to manage reconfigurations towards UE independently, therefore reducing control plane latency, minimizing coordination between CU-CP and DU, reducing the processing load at CU-CP and relaxing the requirements on the F1 interface.

Proposal#3: Delta configuration technique shall be adopted in the 6G RRC signalling toolkit and studied further for improvements.

Proposal#4: RAN2 may consider the Nested RRC structure comprising the Parent and Child Structure as depicted in Figure 1.

Proposal#5: On RRC_INACTIVE

- 6GR shall support the RRC_IDLE and RRC_CONNECTED states as baseline.

- During the study consult with SA2 on the CN defined UE states and its relationship with RRC states.

- Indicate RAN2 preference to SA2 to keep UE state transparent to CN, thus avoiding CN paging.

- Support of RRC_INACTIVE state and modelling methods 1/2 shall be further studied during the study item to alleviate some of the pain points inherent in 5G NR.

Observation#4: If some of the functionality of RRC_IDLE and RRC_INACTIVE is duplicated then there is high probability the network will always release the UE to RRC_IDLE.

Proposal#6: If RAN2 decides to introduce the RRC_INACTIVE state in 6GR, then RAN2 should strive to ensure the feature is commercialized in real deployments from Day1.

Proposal#7: On UE Connection Management

- From UE perspective for connection management, the UE is expected to be in a single RRC state at a given point of time.

- From the UE perspective the UE supports one RRC instance i.e. RRC connection management of the UE is under the control of single RAT during the lifetime of the RRC connection.

# References

- RP-252912, Revised SID: Study on 6G Radio.

- R2-2508946x, Discussion on RRC Structure and (re)configuration in 6G, Fujitsu.

- R2-25xxxx, Chairman Notes, RAN2#131Bis meeting, 13-17 Oct, 2025, Prague.

- R2-2507146,	On 6G RRC design, Nokia.

- R2-2506957,	Control plane architecture and 6G RRC protocol design, MediaTek Inc.

- R2-2507073,	RRC, ASN.1 and other signalling aspects for 6G, Ericsson.

- R2-2507172,	Views on Control Plane for 6G, Qualcomm Incorporated.

- R2-2507433,	Control Plane for 6GR,	InterDigital, Inc.